3GPP TSG RAN #109			RP-251924

Beijing, China, September 15 – 18, 2025

Agenda item: 9.1.4.4

Source:	Apple

Title:   	Moderator's summary for RAN4 REL-20 5G-Adv: RRM topics

Document for: Information

# Introduction

In RP-251879 (WF for RAN4 led REL-20 topics) led by RAN4 chair, the following agreements were reached:

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

In RP-251928 (Release 20 5G-Adv RAN4 Package) prepared by RAN/RAN1/RAN2/RAN4 Chairs, the following proposed scope is provided for discussion in RAN#109.

This document summarizes the relevant contributions on this topic and provides moderator recommendations for further discussions with an aim to reach an agreement on R20 WI scope.

# List of relevant contributions and their views

| RP‑252036 | Views on Rel-20 RRM topics for 5G-A | ZTE Corporation, Sanechips | Proposal 1: propose to keep the SCell activation delay reduction without CSI reporting with the detailed objective as following - SCell activation delay reduction by scheduling SCell prior to valid CSI report for intra-band co-located CA, including SSB-less cases - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. Proposal 2: propose to rephrase the objective in the following way: - Core part: (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 less than 5MHz WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - SCell activation delay reduction by scheduling Scell prior to valid CSI report for intra-band co-located CA, including SSB-less cases - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - Perf part: - (e)RedCap UE enhancement - To specify the test case for RLM/BFD relaxation for (e)Redcap UE - To specify the test case for RLM/BFD relaxation for Redcap UE with less than 5MHz; - To specify the demod requirement for Redcap UE with less than 5MHz; - Fast SCell activation - To specify the test case for Fast Scell activation |
| --- | --- | --- | --- |
| RP-252069 | Views on 5G-A Rel-20 RRM topics | Samsung | (e) RedCap UE enhancements - Core part objective o(e) RedCap UE enhancements Specify RRM requirements for relaxing UE measurements of RLM / BFD for (e) RedCap UE including both 1Rx and 2Rx Follow the defined UE measurement requirement in Rel 17 UE power saving WI as much as possible. Specify UE RF and RRM, requirements for less than 5MHz support for (e) RedCap UE including both 1Rx and 2Rx Follow the outcomes of Rel 18 less then 5MHz WI as much as possible.  - Performance part objective o( RedCap UE enhancements Specify necessary RRM test cases for the RLM/BFD relaxation enhancements Specify necessary RRM and UE demodulation performance requirements for less than 5MHz for (e) RedCap UE |
| RP-252158 |  | Views on Rel-20 RAN4 RRM 3 MHz (e)Redcap |  | Anterix, Rakuten |  | - Specify core RRM requirements while minimizing specification impact to support operation in 3 MHz (15 PRB) channel bandwidth led by RAN4. - Support 15 PRB implementations of 3 MHz channel bandwidths, FR1, 15 kHz SCS - Utilize existing (e)Redcap 2 Rx capabilities defined in Rel-17, Rel-18 and Rel-19 unless otherwise specified. - 1 Rx and/or 1 Tx antenna support, No CA/DC, Reduced modulation: Max 64-QAM DL, 16-QAM UL, Reduced MIMO, Power-saving: eDRX, small data transmissions, RRM measurement relaxations, Standalone (SA) only, HD/FD-FDD operation, Early UE identification support, Small data transmission optimizations, NTN support - Add Radio Link Monitoring (RLM) and Beam Failure Detection (BFD) for connected mode (e)Redcap UE power saving. - Mostly RAN 4 led items such as - Specify necessary RRM performance requirements (RAN4) - Specify necessary UE RF requirements, demodulation performance and CSI reporting requirements (RAN4) - Specify necessary BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4) |  |
| RP-252191 | Views on Rel-20 RRM topics in 5G-A | vivo | Proposal 1: For SCellactivation, it is preferred to considered co-located intra-band CA only. Proposal 2: Objectives for RLM/BFD measurement relaxation for RedCap are •Specify RLM/BFD measurement relaxation requirements for RedCap [ •Both 1Rx and 2Rx UEs are included. •Reuse conclusions of Rel 17 power saving WI as baseline. Proposal 3: Objective for fast SCell activation for intra band co located CA is •Specify enhanced SCell activation requirements for intra band co located CA [RAN4] |
| RP‑252368 | RAN4 R20 RRM | OPPO | Proposal 1: Specify RLM/BFD relaxation requirements for Redcap UE, including both 1Rx and 2Rx. And R17 UE power saving WI conclusions can be reused as much as possible. Proposal 2: Consider SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting in Rel-20. Proposal 3: Prefer to have no further enhancement for A-TRS/TRS based FR2 unknown SCell activation. |
| RP-252373 | Discussion on RRM proposals for 5G-Advanced Release 20 | Ericsson AB. | - Add RLM/BFD relaxation for Redcap devices in Rel-20 RRM. If less than 5MHz for eRedCap is introduced, RAN4 to leverage on conclusions from release 18 less than 5MHz work and only work on the missing requirements for 1Rx and HD-FDD cases. - Observation 1: Extending the Rel-19 FR2 CA measurement delay reduction work to FR1 may not require significant amount of work. Observation 2: RAN4 RRM workload is an issue in general, and not approving any further work on fast scell activation requirements is yet another option. Further enhancing Scell activation delay requirements is in Rel-20 is not necessary. If there is any introduction of fast SCell activation objective, RAN4 to consider Scell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting. As an another alternative apply Rel-19 FR2 delay reduction requirements to FR1. |
| RP-252486 | Qualcomm views on RRM topics for 5GA Rel-20 | Qualcomm Incorporated | •(e)RedCap UE enhancement ◦Introduce RLM/BFD relaxation based similarly to R17 UE power saving WI •Adopt the same solution to minimize the work and enable simple implementation ◦<5MHz for (e)RedCap for 1Rx and 2Rx •UE RF requirements for 1Rx •RRM and demod requirements for both 1Rx and 2Rx •Reuse requirements already defined for 3MHz eMBB as much as possible - Fast SCell activation - Multiple rounds of SCell activation enhancements have been defined in previous releases, however these enhancements are not deployed - There is no need for another round of enhancements given the previous enhancements are not yet deployedRAN4 discussions for such enhancements consume a lot of time and could have RAN1/2 impact also - If something is to be approved, scope should be limited to a minimum - Keep only MAC CE based SCell activation for intra-band continuous CA with CSI report removal. No CSI report enhancement and/or handling of SSB-less cases - A-TRS/TRS based SCell activation is already defined but not deployed |
| RP-252496 | Apple’s view on Rel-20 RAN4 package for RRM | Apple | RAN4 to specify RLM/BFD relaxation requirement for (e)RedCap UE - Use R17 RLM/BFD relaxation for legacy UE as baseline e.g., the relaxation criteria and the scaling factor for relaxation requirement - Consider both 1Rx and 2Rx for (e)RedCap UE RLM/BFD Discuss and specify offset due to 1Rx (applies to the criteria for relaxation) Use the relaxation between 1Rx and 2Rx in legacy (e)RedCap requirement as baseline. e.g., the RLM OOS evaluation period for 1Rx (e)RedCap is relaxed from 2Rx (e)RedCap case (doubled OOS evaluation period) RAN4 to discuss and specify fast SCell activation and fast PSCell addition/activation - TRS/A-TRS based SCell activation Including the scenarios which are not covered by R17 and R18 SCell activation enhancement, e.g., inter-band FR1 SCell activation or intra-band non-contiguous FR1 SCell activation. - TRS/A-TRS based PSCell addition and SCG activation Use the side conditions from R17/R18 TRS/A-TRS based SCell activation as baseline, in terms of RTD, EPRE difference and collocation state. |
| RP-252513 | Rel-20 RAN4 RRM | MediaTek Inc. | - MediaTek views to (e)RedCapUE enhancementThe scope is stable with strong support and high market demand. - For <5MHz, example bands, RF and Demodworks are also needed. - •MediaTek views to Fast SCellactivation Fine if nothing is agreed under this objective. SCellactivation is already very complicated in NR and new enhancements has been introduced for every earlier NR releases. - If this objective is to be introduced, the 1stcandidate (CSI enhancement) is more preferred than the 2ndcandidate (A-TRS), due to smaller workload. To minimize the workload, we suggest focusing on considering scheduling on SCellbefore UE’s valid CSI report.A late start of this objective can be considered. |
| RP-252523 | Proposal on RRM project scope | Xiaomi Technology | •Core part: •Specify RF requirements for optional3 MHz channel bandwidth •Candidate example bands: n100, n106, n26, n28 and n85 (pending on operators’ request) •Specify necessary RRM requirements with 3MHz CHBW for (e)Redcap UE •Specify relaxed RLM/BFD core requirements for (e)RedCapUE Note 1: consider both 1Rx and 2Rx (e)Redcap UEs Note 2:leverage exiting 3MHz requirements for normal UE as much as possible •Perf part: •Specify necessary RRM test cases for RLM/BFD relaxation •Specify necessary RRM test cases for 3MHz •Specify demodulation requirements for 3MHz Core part: •Specify A-TRS based SCellactivation enhancement requirements for unknown SCellactivation [RAN4] •Specify necessary capability signaling [RAN2] Perf part: •Specify corresponding RRM test cases |
| RP-252525 | Views on 5G-A Rel-20 RRM topics | LG Electronics Deutschland | - (e)RedCapUE enhancement - Motivation - Until current Release, RLM/BFD relaxation for power saving has been specified for non-RedCapUE in FR1 and FR2. - From power saving perspective, RLM/BFD relaxation for (e)RedCapUE is beneficial. - Objective - Specify RLM/BFD relaxation requirements for (e)RedCapUEConsider 1Rx and 2Rx UE - Consider the conclusions of R17 UE power saving as baseline - Fast SCell Activation - To consider overall TU in Rel-20 NR and 6G SI, limited scope for fast SCell activation could be considered - Objective - Specify the requirements for SCell activation delay reduction for intra-band co-located CA including SSB-less cases and enhanced CSI reporting |
| RP-252535 | Views on RRM topics for RAN4 Rel-20 | China Telecom | Fast SCell activation - »Justification: The total SCell activation delay includes THARQ, Tactivation_time and TCSI_reporting, where TCSI_reporting is CSI reporting delay. In intra-band co-located contiguous CA scenario, the shortest delay requirement for Tactivation_time is 3ms, but the test case configuration for TCSI_reporting is 15ms, which is larger than Tactivation_time. If CSI is available in one active serving cell contiguous to the SCell, the CSI reporting delay may not be necessary for the SCell being activated. - Periodical RS (e.g. SSB) is mainly used for SCell activation procedures in current specification, A-TRS/TRS based fast SCell activation is introduced in Rel-17, which is applicable for known cases. - »Objective: Enhance CSI reporting delay in SCell activation delay requirements for intra-band co-located contiguous CA. - Specify A-TRS/TRS based SCell activation enhancement requirements for unknown SCell activation including both inter-band CA and intra-band non-contiguous CA. |
| RP-252555 | Views on Rel-20 RRM topics for 5G-A | China Unicom | - SCell activation enhancement for Intra-band continuous CA - ﻿﻿Motivation: Intra-band CA (2cc/3cc) has been widely deployed in commercial 5G network in NR RAN sharing network (i.e. band n78 ). - ﻿﻿Potential objective: ﻿﻿SCell activation delay reduction by completing SCell activation without valid CSI reporting for intra-band co-located CA including SSB-less cases. |
| RP-252583 | (e)Redcap enhancement in Rel-20 | Sony, Nordic Semiconductor ASA, Anterix | Based on the endorsed WF, the following objectives for Rel-20 (e)Redcap enhancement is proposed: - Specify 3Mhz CBW for TN and NTN (e)RedCap, including both 1Rx and 2Rx - For RF complexity reduction, study and specify 3MHz only (e)RedCap for HD-FDD mode in Rel-20. - Study feasibility to support 3MHz only (e)Redcap for both UL and DL directions or UL direction only. - Work from less than 5MHz WIs in previous releases shall be reused as much as possible - Taking band n106 as example band to start the work, target to support all frequency bands in FR1 in Rel-20 - Specify relaxed RLM/BFD evaluation period requirements for (e)Redcap devices, including both 1Rx and 2Rx - Relaxation criteria (i.e. good serving cell- and low mobility criteria) from Rel-17 relaxed RLM/BFD work for non-RedCap UEs shall be reused. |
| RP-252601 | Views on Rel-20 RRM Topics | CMCC | Proposal：Specify RRM requirements to support RLM/BFD relaxation for both 1Rx RedCap UEs and 2Rx RedCap UEs, and the Rel-17 relaxed RLM/BFD requirements are used as baseline. Proposal：Specify RRM requirements for A-TRS/TRS based unknown SCell activation for FR1, including both inter-band CA and intra-band non-contiguous CA. |
| RP-252662 | Considerations on RAN4-led Rel-20 5G-Adv RRM topics | Nokia | (e)RedCap UE enhancement: - For RLM/BFD relaxation: - include both 1Rx and 2Rx receive branches; - include HD-FDD operation; - define requirements for both FR1 and FR2. For Less than 5MHz for (e)RedCap: - include both 1Rx and 2Rx receive branches; - include HD-FDD operation; - define requirements for FR1 only. Fast SCell activation: Exclude proposals which have impact on other WGs. Agree only one objective for Fast SCell activation. - Due to lack of gain, do not include into the Fast SCell activation objective: The SCell to be activated is an unknown FR2 SCell in a band in which there is not already an active cell. - Due to lack of gain, do not include into the Fast SCell activation objective: SCell to be activated is an unknown FR1 SCell which is not contiguous to an active cell in the same band. - Due to lack of gain, do not include into the Fast SCell activation objective: The SCell to be activated is an unknown FR1 SCell which is not in the same band as an active cell. RAN4 does not work on any objective related to A-TRS/TRS aspects under the Fast SCell activation objective. Fast SCell activation objective: enable early SCell scheduling during SCell activation prior to sending valid CSI report. Fast SCell activation work shall not change the existing SCell activation delay start and end points. |
| RP-252713 | Views on RRM works for Rel-20 5G-A | CATT | Proposal 1: RAN4 to define relaxed RLM/BFD requirements for 1Rx/2Rx RedCap UEs. Proposal 2: RAN4 to define less than 5MHz requirements for 1Rx/2Rx RedCap UEs. Proposal 3: RAN4 to define requirements for fast SCell activation delay reduction in intra-band co-located CA by leveraging SSB-less operation and enhanced CSI reporting. The objectives of the work item are the following: - Specify RRM core and performance requirements for (e)RedCap UEs enhancement [RAN4] - Specify RRM requirements for (e)RedCap UEs to support RLM/BFD relaxation - Note 1: Rel-17 requirements of UE power saving defined for normal UEs can be reused as a baseline. - Specify RRM requirements for (e)RedCap UEs to support 3 MHz channel bandwidth. - Note 2: Rel-18 requirements of less than 5MHz defined for normal UEs can be reused as a baseline. - Note 3: Both 1Rx and 2Rx (e)RedCap UEs are consider. - Specify RRM core and performance requirements for fast SCell activation [RAN4] - Specify RRM requirements to enhance SCell activation delay reduction leveraging the SSB-less operation and enhanced CSI reporting Only intra-band co-located CA scenario is considered. - Note 4: Whether to consider inter-band CA scenario can be checked in June 2026. - Note 5: No RAN1/RAN2 impact is expected. |
| RP-252741 | Rel-20 RRM enhancement with fast SCell activation | Huawei, HiSilicon, China Telecom, China Unicom, NTT DOCOMO, Rakuten Mobile, Spark NZ, Telia Company, KDDI, Bell Mobility, TELUS | Proposal 1 >For fast SCell activation, include the following objective as part of Rel 20 5 G Advanced WI on RRM enhancement >SCell activation delay reduction by completing SCell activation without valid CSI reporting for intra band co located CA including SSB less cases |
| RP-252750 | Views on RAN4 Rel-20 5G-A - RRM topics | Intel Corporation | Proposal #1: RRM objective “(e)RedCap UE enhancement” - The work objectives shall include definition of requirements for - (e)RedCap RRM requirements for RLM/BFD relaxation based on solutions defined in Rel-17 NR_UE_pow_sav_enh WI - (e)RedCap UE RF / RRM / Demod requirements for 3 MHz operation with 15 PRB transmission bandwidth configuration and 12 PRB SSB - Handle the objective as a separate work item with ~12 months duration Proposal #2: RRM objective “Fast SCell activation” - The work objectives shall include definition of - RRM requirements for SCell activation delay reduction for intra-band co-located CA including the following solutions - Scheduling of SCell before without a valid CSI report incl. scenarios with SCell with and without SSB transmissions - Scheduling SCell prior without the need for TRS reception for SSB-less SCell scenarios |

# Moderator recommendations for discussions

## (e)RedCap UE enhancement

It is recommended to consider the following WI scope in the WID:

- Specify the following requirements for (e)RedCap UEs

  - Relaxed RLM/BFD RRM requirements

    - Use R17 RLM/BFD relaxation for legacy UE as baseline

  - Support of less than 5 MHz channel bandwidth (15PRB transmission bandwidth configuration and 12PRB SSB)

    - RF requirements, considering example bands: n100, n106, n26, n28 and n85

      - [UE RF scope including potential complexity reduction]

      - [The requirements shall apply to all (e)Redcap UEs.

      - In addition, support (e)Redcap UEs that are restricted to not being scheduled more than 3MHz within the channel bandwidth at any instant in time in uplink.

        - Note: Different uplink 3MHz portions of the channel bandwidth can be scheduled at different times. For the DL, all channel bandwidths are supported and no scheduling restriction or DL interruption is needed.]

    - RRM requirements including Relaxed RLM/BFD RRM requirements

    - Note 1: Use R18 3MHz requirements for non-RedCap UE as a baseline

  - Note 2: Consider both 1Rx and 2Rx (e)Redcap UEs and HD-FDD

Breakout room discussion:

E///: “considering RF complexity with 3MH only” contradicts Note 1. Recommend removing that bullet. Sony: the key is to enable 3MHz only for UL TX. Xiaomi: from UE capability signaling perspective, the UE can signal what channel bandwidth can be supported. Moderator: Defer comments on the bullet “[UE RF scope including potential complexity reduction]” to email discussion.

Offline Discussion

| Company | Comments |
| --- | --- |
| Anterix | From a historical perspective the intent of the original statement "[UE RF scope including potential complexity reduction]" was based on the fact that Rel-18 introduced 3 MHz CBW and devices. (e)Redcap was explicitly not supported in Rel-18 3 MHz devices. The original intent was to utilize existing (e)Redcap 2 Rx capabilities defined in Rel-17/18. These features by definition reduce the complexity of 3 MHz devices e.g. HD/FD-FDD operation, 1 Rx and/or 1 Tx antenna support, RRM measurement relaxations...etc. There was no intent to create additional simplifications or complexity reductions but simply enable redcap for Less than 5 MHz devices. |
| Nokia | - Relaxed RLM/BFD requirements have already been done for non-RedCap UEs, but not applicable to (e)RedCap UEs. So the objective should be (a) to make the existing R17 relaxed RLM/BFD requirements applicable to 2Rx (e)RedCap UEs, and (b) to define relaxed RLM/BFD requirements for 1Rx (e)RedCap UEs using the Rel-17 RLM/BFD relaxation for legacy UEs as baseline. We therefore propose the following wording: Relaxed RLM/BFD RRM requirements: - Rel-17 RLM/BFD RRM relaxation to be applicable also for (e)RedCap 2Rx UEs - Introduce RLM/BFD RRM relaxation for (e)RedCap 1Rx UEs, using Rel-17 RLM/BFD relaxation for legacy UE as baseline. Consider e.g., the relaxation criteria and the scaling factor for relaxation requirement - Note 1 can then be deleted. - For support of <5MHz channel bandwidth bullet, it is unclear what “UE RF scope including potential complexity reduction” means. We should delete this and add “The minimum supported RF bandwidth of RedCap devices shall not be changed.” - “RRM requirements including Relaxed RLM/BFD RRM requirements” is unclear. Replace with “RRM requirements, including reusing the above relaxed RLM/BFD RRM requirements for the <5MHz case.” |
| Sony | To clarify the RF complexity reduction aspect: - With the introducing of 3MHz to (e)Redcap device, we see it is feasible to provide additional complexity reduction comparing to legacy (e)Redcap devices. Our proposal is to allow HD-FDD eRedcap device in Rel-20 to NOT mandatorily support bandwidth (or equivalent RB length) larger than 3MHz in a given band. - We see significant complexity reduction when the HD-FDD eRedcap device operate with 3MHz, which can meet all existing RAN4 requirements without need of any SAW filter. This would unleash the full advantage of HD-FDD eRedcap, and the detail can be found in RP-252583. - 3MHz Redcap was proposed in the previous RAN meetings under Rel-20 RAN4 led Wis discussion, e.g., RP-251766, and it is inline with the agreed WF To avoid any impact to the network design, we consider to only not mandate the UE CBW > 3MHz in UL direction, or equivalent RB length larger than 20 PRB in UL direction. The RF bandwidth or DL bandwidth can remain the same as legacy devices. In addition, it is sufficient to just define a new UE capability to indicate that the UE would not support UE CBW > 3MHz in a given band, we don’t see the need to change the definition of existing (e)Redcap or define a new device type. With the above clarification, and the comments provided from previous companies, we propose the following revision to clarify the scope of RF complexity reduction: For RF complexity reduction, allowing UE to optionally support UE CBW (or equivalent RB length) > 3MHz in UL direction for HD-FDD (e)Redcap devices. The DL CBW support is not changed comparing legacy (e)Redcap devices. |
| Ericsson | The bullet “UE RF scope including potential complexity reduction” is too generic and a large scope; potentially creation of a new device type. In our view, we should consider the following: - UEs that support the maximum Redcap channel bandwidth in both DL and UL should be included and are the baseline. - UEs that support the maximum Redcap channel bandwidth in DL and 3MHz only in UL, and are able to retune the 3MHz channel bandwidth within the Redcap channel bandwidth with sufficiently low retuning delay. We should also be clear that if the second bulled has any impact on RAN1/2 scope then we only do the first bullet; support all Redcap bandwidths in both DL and UL (as well as 3MHz) |
| Sony | Reply to Ericsson: Based on our offline discussion, we clarify the intention as below: Redcap UE is restricted to not being scheduled more than 3MHz within the Redcap bandwidth at any instant in time in uplink. The scheduler is free to schedule different uplink 3MHz portions of the Redcap bandwidth at different times. For the DL, all redcap bandwidths are supported and no scheduling restriction |
| Anterix | A majority of the devices on our network are IoT that require the necessity for a low complexity/cost device ecosystem, therefore the necessity for (e)Redcap devices to replace NB-IoT and CATM. We are supportive of the latest Sony text above. Looking at Ericsson and Nokia comments we agree and feel that NOTE 2 should be modified and included in the first bullet as < 5MHz 2 Rx will copy over existing specs for 2 Rx and 1 Rx is part of the further simplification. |

## Fast SCell activation

It is recommended to consider the following WI scope in the WID:

- For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026)

- Note 1: No new PDSCH demodulation requirement shall be defined.

Offline Discussion

| Company | Comments |
| --- | --- |
| Qualcomm | It should be clarified what exactly this delay reduction is. Right now the base station can already schedule UEs before a CQI report is sent, the timeline when the UE should have DL on and ready to receive is quite clear. Also, this should not introduce new demod performance requirements. |
| Nokia | The network is already “allowed to schedule”. We propose: “…specify RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report” |
| Ericsson | The RAN chair suggestion is to start this from Q4 2026; this should be captured. We should also capture that a potential outcome may be that there is not a need or possibility to specify anything. If the valid CSI report is not defined as the end of the delay, it is not clear how the 3GPP spec then defines what the delay is. So we should allow for the possibility that no requirement is defined. We assume this should be simple; if it is not then we should not include it in the package. Proposal to update the wording: - For intra-band co-located contiguous CA including SSB-less cases, consider and if needed specify RRM requirements for SCell activation delay reduction assuming that the network may schedule the SCell before a valid CSI report is received from UE |
| CMCC | We support to allow schedule the SCell before a valid CSI report. But the current wording is not clear. One interpretention is that the SCell activation delay reduction is based on SSB-less + scheduling before valid CSI report. From our perspective, we do not see the necessity that scheduling before valid CSI report need to be coupled with SSB-less. Based on above consideration, it is suggested to change the wording as following: “For intra-band co-located contiguous CA including SSB-less cases, specify RRM requirements for SCell activation delay reduction by allowing network to schedule the SCell before a valid CSI report is received from UE” |
| Huawei | We support to include the objective in the package, and we also support the updated wording from the AH session at Tuesday morning. To QC, the delay to be reduced here is from the time point when SCell activation command is received by the UE to the time point when UE transmit and receive data on the SCell. As mentioned by QC and Nokia, NW is already “allowed” to schedule UE before a valid CQI report is sent, but UE is only required to response the NW scheduling after the valid CQI is sent, which means data scheduling on SCell prior to valid CSI is not supported in current spec. We agree with E/// that this objective should be simple, but we do see there is a need to specify the UE requirement clearly in the spec. We can either change the ending point of SCell activation, or we can explicitly specify the UE requirement, e.g. “UE shall be capable to receive PDCCH/PDSCH not later than …”. The exact spec impact can be discussed in RAN4 in WI phase. |

# Conclusion

Based on the email discussion and breakout room discussion, the following WI scope is agreeable:

## (e)RedCap UE enhancement

- Specify the following requirements for (e)RedCap UEs

  - Relaxed RLM/BFD RRM requirements

    - Use R17 RLM/BFD relaxation for legacy UE as baseline

  - Support of less than 5 MHz channel bandwidth (15PRB transmission bandwidth configuration and 12PRB SSB)

    - RF requirements, considering example bands: n100, n106, n26, n28 and n85

      - [The requirements shall apply to all (e)Redcap UEs.

      - In addition, for RF complexity reduction, RAN4 is tasked to study the following aspects for one quarter:

        - support (e)Redcap UEs that are restricted to not being scheduled more than 3MHz within the channel bandwidth at any instant in time in uplink.

          - Note: Different uplink 3MHz portions of the channel bandwidth can be scheduled at different times. For the DL, all channel bandwidths are supported and no scheduling restriction or DL interruption is needed.]

    - RRM requirements including Relaxed RLM/BFD RRM requirements

    - Note 1: Use R18 3MHz requirements for non-RedCap UE as a baseline

  - Note 2: Consider both 1Rx and 2Rx (e)Redcap UEs and HD-FDD

## Fast SCell activation

- For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026)

- Note 1: No new PDSCH demodulation requirement shall be defined.