3GPP TSG RAN Meeting #109										RP-251965

Beijing, China, Sep. 15-18, 2025

Status Report to TSG

Agenda item:			9.5.2

| WI / SI Name | Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_AIML_Mob |  |  |  |  |
| Unique ID | 1020084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242393 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 09/2025 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 100% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Zhongda Du |
|  | Company | OPPO |
|  | Email | duzhongda@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

Agreements in RAN2#131 meeting:

Agreements on L3 beam-level prediction

For RRM measurement prediction, L3 beam-level prediction is feasible for network sided model.  CB after spec impact/overhead etc on the different cases.

For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model.  However, there are concerns on complexity and other WGs.

Agreements 1. Confirm that as a baseline following agreements on applicability reporting in AI-based beam management are applicable for AI mobility: - UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration. - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. - When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). - The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported predicted values after UE indicates that an inference configuration is not applicable. - The UE shall report when an inference configuration becomes non-applicable. - How to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration. - Whether Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. - RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). NOTE: these agreements will be aligned with AI PHY agreements at the end of this week and will not be re-discussed

Agreements

- For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

A monitoring window, a window over which the performance monitoring metric can be calculated, can be configured for performance monitoring.  Up to WI on which cases (if any) this monitoring window is needed.

Agreements

Aim for a single framework for event prediction and performance monitoring.  From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring.  Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction.

What the UE requires to the determine applicability is similar to RRM prediction.

Agreements

1	For RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW for the associated AIML model

- Event-related information (event type and event-related parameters)

2	For RRM measurement event prediction, the event prediction report can include:

-	RRM measurements (For normative phase to define what types of measurements)

-	Time-related information about the predicted event (e.g., the time/window of occurrence)

3	For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure.   This can be achieved without spec impact for measurement reporting.  FFS for normative phase if additional information needs to be added.

- Agreements 1 For request/configuration for UE side data collection, the following in AI/ML PHY agreements are also applied as a baseline for RRM measurement prediction and measurement event prediction for connected mode. - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection - Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request. - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). NOTE this can be aligned with AI/ML PHY agreements at the end of this - For UE-side data collection, UE can perform measurement by re-using MOs configured for legacy RRM measurement. - The full list of candidate measurement configuration is not needed for AI mobility. Capture following options in the TR. Up to normative phase to determine solution. - Option 1 - Network can configure a set of candidate frequencies the UE can request. - The UE can indicate a preference for data collection within the set of candidate frequencies. - Option 2 - The UE can indicate preferred frequencies for data collection (under network control). FFS what mechanism to use. UP to normative phase if other information is required.

UE can perform data collection in IDLE/INACTIVE mode without any specification impacts.

For network sided models for inference:

- For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report.

- For other cases there is no specification impact.   Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam)

3	Study item can conclude that all scenarios and sub-cases are feasible.

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

Agreements in RAN4#116 meeting:

RAN4 requirements for RRM Measurement prediction:

Issue 2-1: Impacts of measurement error on prediction accuracy

Agreement:

RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy:

- As a baseline:

  - For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

  - For RF error model, use a truncated Gaussian distribution.

    - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

Issue 2-2: Relative RSRP accuracy definition

Agreement:

Relative RSRP accuracy definition

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

- Note: It is not precluded to update the definition based on further RAN2 progress in WI phase.

RAN4 requirements for measurement event prediction:

Issue 3-1: Performance metric for indirect measurement event prediction

Agreement:

- On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase.

- Capture the candidate proposals in the WF for information purpose.

Testability and interoperability:

Issue 1-1-1: Testing of Generalization aspects

Agreement:

- Start discussion on the requirement and testing aspects related to the generalization in WI phase.

Issue 1-1-2: Interoperability

Agreement:

- RAN4 does not identify any interoperability issue which impacts the feasibility.

Issue 1-1-6: LCM related testability aspects

Agreement:

  - Leave the LCM related testing discussion to WI stage after the requirements are clear.

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2 contributions in RAN2#131

R2-2505114	Discussions on functionality management for RRM measuerment prediction	DOCOMO Beijing Labs

R2-2505115	Discussions on data collection for UE sided model	DOCOMO Beijing Labs

R2-2505116	Discussions on NW-sided model	DOCOMO Beijing Labs

R2-2505128	Discussion on functionality management for RRM measurement prediction	vivo

R2-2505129	Recommendation on specification for L3 beam-level prediction	vivo, Qualcomm Incorporated, Huawei, HiSilicon, CATT, ZTE, Ericsson, CMCC, NTT DOCOMO, Interdigital

R2-2505130	Discussion on functionality management for RRM measurement event prediction	vivo

R2-2505131	Discussion on UE side data collection	vivo

R2-2505132	Discussion on NW-sided mode lin AI enhanced mobility	vivo

R2-2505151	Discussion on functionality management for RRM prediction	Xiaomi

R2-2505152	Discussion on functionality management for event prediction	Xiaomi

R2-2505153	Discussion on data collection	Xiaomi

R2-2505154	Discussion on NW sided model	Xiaomi

R2-2505161	Text proposal of TR 38.744 to capture spec impact_v15	OPPO

R2-2505184	Text proposal of TR 38.744 on model complexity	OPPO

R2-2505185	Text porposal of TR 38.744 on conclusion of AI mobility SID	OPPO

R2-2505186	Discussion on functionality management for RRM measurement prediction v3	OPPO

R2-2505187	Discussion on functionality management for measurement event prediction V3	OPPO

R2-2505188	Discussion on data collection for UE sided model-V2	OPPO

R2-2505189	Discussion on LCM procedures of network sided model	OPPO

R2-2505190	Discussion on AI/ML performance monitoring for network-sided model	KT Corp.

R2-2505216	Functionality management for RRM measurement prediction	CATT, Turkcell

R2-2505217	Functionality management for RRM measurement event prediction	CATT, Turkcell

R2-2505218	Discussion on UE-sided data collection	CATT, Turkcell

R2-2505219	Discussion on Network sided model	CATT, Turkcell

R2-2505337	Discussion on UE-side AIML model for RRM measurement prediction 	NEC

R2-2505347	Discussion on data collection for AI/ML mobility	Samsung

R2-2505357	Discussion on functionality management for RRM measurement prediction	Samsung

R2-2505358	Discussion on functionality management for RRM measurement event prediction	Samsung

R2-2505359	Discussion on Network-sided model	Samsung

R2-2505382	Network sided model	Ericsson

R2-2505383	Data collection for UE sided models	Ericsson

R2-2505440	UE-sided measurement prediction	Apple

R2-2505441	Event prediction	Apple

R2-2505442	UE-sided data collection	Apple

R2-2505443	On network-sided models	Apple

R2-2505460	Functionality management for RRM measurement prediction	Lenovo

R2-2505461	Functionality management for RRM measurement event prediction	Lenovo

R2-2505473	Discussion on Functionality Management for RRM Measurement Prediction	MediaTek Inc.

R2-2505474	Discussion on Functionality Management for RRM measurement event prediction	MediaTek Inc.

R2-2505475	Discussion on Data Collection for UE-sided Model	MediaTek Inc.

R2-2505512	Discussion on functionality management for measurement event prediction	CMCC

R2-2505513	Discussion on data collection for UE-sided model	CMCC

R2-2505569	Discussion on Functionality management for RRM measurement prediction for UE sided model	KT Corp.

R2-2505637	Discussion on NW-side model input	Sharp

R2-2505654	UE side Data collection for measurement/ event prediction	Sony

R2-2505671	Functionality management for RRM measurement prediction	Huawei, HiSilicon

R2-2505672	Functionality management for measurement event prediction	Huawei, HiSilicon

R2-2505673	Discussion on UE-side data collection	Huawei, HiSilicon

R2-2505674	Discussion on NW-sided model support	Huawei, HiSilicon

R2-2505695	Discussion on network sided model	Lenovo

R2-2505710	Discussion on functionality management for RRM measurement prediction	Spreadtrum, UNISOC

R2-2505711	Discussion on functionality management for RRM measurement event prediction	Spreadtrum, UNISOC

R2-2505836	UE-side Model – Functionality Management for RRM Measurement Prediction	Ericsson

R2-2505837	UE Sided model – Functionality Management for RRM Event Prediction	Ericsson

R2-2505874	Functionality management for UE sided model for RRM measurement prediction	Interdigital Inc.

R2-2505875	Functionality management for UE sided model for RRM measurement event prediction	Interdigital Inc.

R2-2505876	Data collection for UE sided model	Interdigital Inc.

R2-2505877	Network sided model for AI/ML mobility	Interdigital Inc.

R2-2505887	Functionality management for RRM measurement prediction for UE-side model	Qualcomm Incorporated

R2-2505888	Functionality management for Measurement Event prediction for UE-side model	Qualcomm Incorporated

R2-2505889	Data collection for UE-side model	Qualcomm Incorporated

R2-2505890	Network-side model	Qualcomm Incorporated

R2-2505915	UE-side data collection for AI Mobility	Lenovo

R2-2505964	Discussion on RRM measurement prediction with UE sided model	CMCC

R2-2505965	Discussion on RRM measurement prediction with NW sided model	CMCC

R2-2505984	Functionality Management for AIML Mobility	Continental Automotive

R2-2506006	Discussion on Functionality management for RRM measurement event prediction	NEC

R2-2506007	Discussion on AIML mobility for Network Sided Model Performance	NEC

R2-2506030	Discussion on functionality management for RRM measurement prediction	ASUSTeK

R2-2506031	Discussion on functionality management for measurement event prediction	ASUSTeK

R2-2506126	Discussion on functionality management for RRM measurement prediction	ZTE Corporation

R2-2506127	Discussion on functionality management for measurement event prediction	ZTE Corporation

R2-2506128	Discussion on data collection for UE side model	ZTE Corporation

R2-2506129	Discussion on AI mobility for network side model	ZTE Corporation

R2-2506133	Considerations on network-sided model	Nokia, Nokia Shanghai Bell

R2-2506134	Functionality management for measurement event prediction	Nokia

R2-2506135	Functionality management for RRM measurement prediction	Nokia

R2-2506136	Data collection for UE-sided model	Nokia

R2-2506164	Discussion on NW-side model input	Sharp

R2-2506177	Discussion on UE-sided Model Functionality Management	Sharp

R2-2506180	Discussion on performance monitoring	Sharp

RAN4 Contributions in RAN4#116

R4-2509057	Topic summary for [116][214] FS_NR_AIML_Mob_Part1	Moderator (Nokia)

R4-2509058	Topic summary for [116][215] FS_NR_AIML_Mob_Part2	Moderator (OPPO)

R4-2509302	Discussion on impacts of AIML RRM measurement prediction on RRM requirements	CATT

R4-2509303	Discussion on impacts of AIML measurement event prediction on RRM requirements	CATT

R4-2509304	Discussion on testability and interoperability issues for AIML mobility	CATT

R4-2509431	General aspects on AI/ML for mobility in NR	Apple

R4-2509432	On RAN4 Impacts for RRM measurement prediction in AIML Mobility	Apple

R4-2509433	On RAN4 Impacts for Measurement Event Prediction in AIML Mobility	Apple

R4-2509434	Testability and Interoperability Issues for AIML Mobility	Apple

R4-2509645	Discussion on testability and interoperability for AI/ML mobility	Tejas Network Limited

R4-2509646	Study of RAN4 impacts for RRM measurement prediction for AIML mobility in NR	Tejas Network Limited

R4-2509672	TP on RAN4 aspects for TR 38.744	OPPO, Nokia

R4-2509673	Study of impacts on RAN4 requirements for RRM measurement prediction	OPPO

R4-2509674	Study of impacts on RAN4 requirements for measurement event prediction	OPPO

R4-2509675	Study of testability and interoperability for AI mobility	OPPO

R4-2509750	Discussion of RAN4 impacts for RRM measurement prediction	LG Electronics Inc.

R4-2509763	Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility	Xiaomi

R4-2509764	Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility	Xiaomi

R4-2509773	Discussion on study of testability and interoperability for AIML mobility	Xiaomi

R4-2509922	On RAN4 Impacts for RRM measurement prediction in AIML Mobility	Nokia

R4-2509923	On RAN4 Impacts for Measurement Event Prediction in AIML Mobility	Nokia

R4-2509924	On General aspects in AIML Mobility	Nokia

R4-2510106	Discussion on general aspects for AI mobility	MediaTek Inc.

R4-2510107	Discussion on RAN4 impacts for measurement event prediction	MediaTek Inc.

R4-2510108	Discussion on testability and interoperability of AI mobility	MediaTek Inc.

R4-2510167	Discussion on general part for AI/ML for mobility	CMCC

R4-2510168	Discussion on RAN4 impacts for RRM measurement prediction	CMCC

R4-2510169	Discussion on RAN4 impacts for measurement event prediction	CMCC

R4-2510170	Discussion on testability and interoperability for AI/ML for mobility	CMCC

R4-2510443	Discussion on the interoperability and testability aspects	ZTE Corporation, Sanechips

R4-2510444	Study of RAN4 impacts for RRM measurement prediction	ZTE Corporation, Sanechips

R4-2510445	Discussion on General Aspects of AIML Mobility	ZTE Corporation, Sanechips

R4-2510446	Discussion on impacts for measurement event prediction	ZTE Corporation, Sanechips

R4-2510486	Discussion on general aspects for AI mobility	vivo

R4-2510487	Discussion on RAN4 impacts for RRM measurement prediction	vivo

R4-2510488	Discussion on RAN4 impacts for measurement event prediction	vivo

R4-2510489	Discussion on testability and interoperability for AI mobility	vivo

R4-2510577	Discussion on genereal aspects in AIML mobility	Huawei, HiSilicon

R4-2510578	Discussion on impacts for RRM measurement prediction	Huawei, HiSilicon

R4-2510579	Discussion on impacts for measurement event prediction	Huawei, HiSilicon

R4-2510580	Discussion on testability and interoperability issues in AIML mobility	Huawei, HiSilicon

R4-2510706	Discussions on requirements for RRM measurement prediction on AIML mobility	NTT DOCOMO, INC.

R4-2510707	Discussions on requirements for measurement event prediction on AIML mobility	NTT DOCOMO, INC.

R4-2510761	Corrections to TP on RAN4 aspects in TR 38.744	Ericsson

R4-2510762	General discussion on AI/ML for mobility	Ericsson

R4-2510763	On requirements for RRM measurement prediction for AI/ML based mobility	Ericsson

R4-2510764	On requirements for even prediction for AI/ML based mobility	Ericsson

R4-2510765	On testing of AI/ML based mobility	Ericsson

R4-2511092	Discussion on AI mobility regarding testability and interoperability	Samsung

R4-2511223	Testability and Interoperability Issues for AIML Mobility	Nokia

R4-2511226	Discussion on AI mobility regarding RAN4 requirements impact	Samsung

R4-2511516	Impact of AI based mobility on RRM measurement prediction	Qualcomm Incorporated

R4-2511517	Impact of AI based mobility on RRM event prediction	Qualcomm Incorporated

R4-2511598	Discussion on testability aspects for AI/ML based mobility	Rohde & Schwarz

R4-2512126	Ad-hoc minutes for FS_NR_AIML_Mob	Nokia

R4-2512127	WF on FS_NR_AIML_Mob_Part1 Nokia

R4-2512128	WF on FS_NR_AIML_Mob_Part2	OPPO

R4-2512160	TP on RAN4 aspects for TR 38.744	OPPO, Nokia