3GPP TSG RAN Meeting #109												RP-252176

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4  Requirements of new and existing services

Source:	Robert Bosch GmbH, Toyota ITC, BMW AG, Volkswagen AG, Continental Automotive, Fraunhofer IIS

Title:	Text proposal for Requirements of New and Existing Services for Vehicular Use Cases

Document for:	Discussion/Decision

# Introduction

This contribution presents a Text Proposal for TR 38.914, Section 5.4, Requirements of New and Existing Services. The proposal first establishes the need for service continuity from Day-1, ensuring that 6G supports existing 5G services, particularly voice, data/eMBB, and URLLC, from Day-1, to guarantee early and successful adoption of the technology. Secondly, it demonstrates the needs for seamless mobility between the harmonized TN and NTN as a fundamental requirement in 6G.

Finally, to address the automotive evolution towards autonomous driving and advanced cockpits, the contribution introduces requirements for new capabilities mapped to IMT-2030 usage scenarios. This includes proposing HRLLC, AI services, immersive communication, and ubiquitous connectivity as new automotive service requirements.

# Text Proposal

======================Start of the changes========================

## 5.4	Requirements of New and Existing Services

Editorial note: To be captured in the main “Requirements of New and Existing Services” sections:

As a baseline, 6G RAN should support all requirements specified by services defined for the 5G.

5.4.X Requirements of New and Existing Services for Connected Vehicles

Editorial note: To be captured as separate section or to be captured in the different respective service’s sections:

Connected vehicles require service continuity (see Note 1) of voice, data/eMBB, and URLLC services from Day-1 of 6G deployment. Wherein the service should be maintained during the long-life cycle (15+ years, see Note 2) of the vehicles.

The 6G should ensure service continuity of connected vehicles moving between 5G and 6G coverage areas with limited interruption time.

TN/NTN mobility should be included from 6G Day-1 guaranteeing also limited interruption time of service.

The 6G should also support advanced connected mobility services, e.g., connected autonomous driving, immersive infotainment, and advanced cockpits, utilizing a harmonized design with other UE types, including the following services with their respective TPR investigated in this Technical Report:

- Immersive Communication

- HRLLC

- Ubiquitous Connectivity with TN and NTN

- AI Services

Note 1: for specific regions, some connectivity services could be mandated based on the regulatory requirements, e.g., voice and date.

Note 2: based on regulatory requirements in some regions and the typical development cycle.

# ========== End of Changes ==========

# Proposal 1: Capture the Text Proposal in Section 2 of this TDoc to 3GPP TSG RAN level SI Technical Report TR 38.914, Section 5.4 “Requirements of New and Existing Services”.

# Conclusion

The following proposal is presented:

Proposal 1: Capture the Text Proposal in Section 2 of this TDoc to 3GPP TSG RAN level SI Technical Report TR 38.914, Section 5.4 “Requirements of New and Existing Services”.