3GPP TSG RAN Meeting #109					RP-252226

Beijing, China, September 15-18, 2025

Title:	Operational requirements related to resilience of the 6G Radio

Source:	ST Engineering iDirect

Type: 	pCR

Document for:	Discussion

Agenda Item:	8.2.1.3 Requirements for architecture and migration

# Introduction

Following the ITU Recommendation M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond” [1], the goals for 6G include the availability of increased security, integrity, and privacy from day one [2].

Observation 1: The IMT-2030 design principles include increased security and resiliency.

Proposal 1: The overall goals for 6G should include further security enhancements and reliability improvements.

This document aims at providing requirements related to resilience of the 6G Radio in clause 5.3 “Operational requirements” of the TR 38.914.

# Discussion

Over the past years, several security weaknesses have been reported related to 5G New Radio (NR).  Security threats of 5G NR physical layer are presented in, inter alia, the following publications.

•	A. K. Yerrapragada, et al., "Physical Layer Security for Beyond 5G: Ultra Secure Low Latency Communications," in IEEE Open Journal of the Communications Society, vol. 2, pp. 2232-2242, 2021, doi: 10.1109/OJCOMS.2021.3105185

•	Y. Arjoune and S. Faruque, "Smart Jamming Attacks in 5G New Radio: A Review," 2020 10th Annual Computing and Communication Workshop and Conference (CCWC), Las Vegas, NV, USA, 2020, pp. 1010-1015, doi: 10.1109/CCWC47524.2020.9031175

•	Y. Wu, et al., "A Survey of Physical Layer Security Techniques for 5G Wireless Networks and Challenges Ahead," in IEEE Journal on Selected Areas in Communications, vol. 36, no. 4, pp. 679-695, April 2018, doi: 10.1109/JSAC.2018.2825560

•	Harvanek M, et al., “Survey on 5G Physical Layer Security Threats and Countermeasures” in Sensors, vol. 17, 2024, doi: 10.3390/s24175523

•	S. Sullivan, et al., "5G Security Challenges and Solutions: A Review by OSI Layers," in IEEE Access, vol. 9, pp. 116294-116314, 2021, doi: 10.1109/ACCESS.2021.3105396

•	M. Mitev, et al., "What Physical Layer Security Can Do for 6G Security," in IEEE Open Journal of Vehicular Technology, vol. 4, pp. 375-388, 2023, doi: 10.1109/OJVT.2023.3245071

Observation 2: Many academic publications highlight PHY layer security issues of 5G NR.

As an example, the 5G broadcast and control channels can be easily jammed or eavesdropped because of their narrowband nature. Also, traffic maps with network activity of the different UEs in the network can be created as the downlink control information (DCI) with downlink and uplink scheduling information is broadcasted by the gNB without encryption.

In previous plenary meetings, numerous contributions have highlighted the need to improve security of the 6G Radio, and several contributions discussed the need to introduce physical layer security [3].

Observation 3: Lower layer security is considered an essential topic for 6G by many companies.

It is proposed to add a requirement to provide channels and signals with increased physical layer security, e.g.,

- Make the 6G broadcast and control channels and signals more resilient against jamming, spoofing, and eavesdropping

- Enable an option to hide the network activity, and if possible, the network traffic, e.g., for governmental or private 6G use cases

As an example, the existing Synchronization Signal Block (SSB) and Physical Downlink Control Channel (PDCCH) can be easily jammed and could be complemented by adding a secure SSB and a secure PDCCH.

Proposal 2: Add a requirement to have secure channels available in 6G Radio, in addition to existing channels.

# Text proposal for TR 38.914

Based on the above discussion we propose the following text proposal.

====================Start of the changes==================

## 5.3 Operational requirements

#### 5.3.5 Network Resilience

##### 5.3.5.x Resilience requirements for the 6G Radio

The 6G Radio shall support additional secure channels and signals that are more resilient against threats such as jamming, spoofing, and eavesdropping, and that can co-exist with the existing channels and signals.

==================End of the changes========================

Proposal 3: Add the above text proposal in clause 5.3 of the TR 38.914.

# Conclusion

In this contribution, the following observations and proposals are made.

Observation 1: The IMT-2030 design principles include increased security and resiliency.

Observation 2: Many academic publications highlight PHY layer security issues of 5G NR.

Observation 3: Lower layer security is considered an essential topic for 6G by many companies.

Proposal 1: The overall goals for 6G should include further security enhancements and reliability improvements.

Proposal 2: Add a requirement to have secure channels available in 6G Radio, in addition to existing channels.

Proposal 3: Add the text proposal from Sect. 3 in clause 5.3 of the TR 38.914.

# References

[1] Recommendation ITU-R M.2160-0, “Framework and overall objectives of the future development of IMT for 2030 and beyond”, November, 2023

[2] 6GWS-250243, “Chair’s summary of the workshop on 6G”, Incheon, Korea, March 10 – 11, 2025

[3] R1-2506156, “Physical-layer security considerations for 6G Radio (6GR)”, RAN1#122, Bengaluru, India, August 25 – 29, 2025