3GPP TSG-RAN WG5 Meeting #108	R5-255335

Bangaluru, India, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-3 | CR | 0177 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test applicabilities for NR RSTD and UE Rx-Tx time difference test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added test applicability for NR RSTD reporting period test cases - Added test applicability for UE Rx-Tx time difference reporting period and accuracy test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4-11, Table 4-12, Table A.4.3-6B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-1 CR 0639, 0640, 0641, 0642, 0643, 0644, 0645, 0646, 0647, 0648, 0649, 0650, 0655, 0656 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

Table 4-11: Applicability of tests and additional information for testing for test cases in TS 37.571-1 [5] for NR

| Clause | TC Title | Release of LPP/SLPP | Applicability |  | Additional Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions |  | Release RAT |
| 14 | NR RSTD measurement requirements |  |  |  |  |  |  |  |  |
| 14.2.1 | NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.2 | NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.3 | NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.4 | NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.5 | NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Rel-17 | C85nr | All FR1 NR UEs. The UEs shall support DL-TDOA and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.2.6_1 | NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA: Subtest 1 | Rel-17 | C86nr | All FR1 NR UEs. The UEs shall support DL-TDOA and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types |  |  |  |  | Rel-17 |
| 14.2.6_2 | NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA: Subtest 2 | Rel-17 | C87nr | All FR1 NR UEs. The UEs shall support DL-TDOA and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.2.7 | NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR1 SA | Rel-17 | C88nr | All FR1 NR UEs. The UEs shall support UE assisted DL-TDOA and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 14.2.8 | NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C54nr | All FR2 NR UEs. The UEs shall support DL-TDOA and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.2.9_1s | NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 1 | Rel-17 | C55nr | All FR2 NR UEs. The UEs shall support DL-TDOA and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types |  |  |  |  | Rel-17 |
| 14.2.9_2s | NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 2 | Rel-17 | C56nr | All FR2 NR UEs. The UEs shall support DL-TDOA and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.2.10 | NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Rel-17 | C57nr | All FR2 NR UEs. The UEs shall support UE assisted DL-TDOA and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 14.2.11 | NR RSTD measurement reporting delay test case for PRS aggregation in FR1 | Rel-18 | C105nr | All FR1 NR UEs. The UEs shall support DL-TDOA and DL-PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 14.2.12 | NR RSTD measurement reporting delay test case for PRS aggregation in FR2 | Rel-18 | C106nr | All FR2 NR UEs. The UEs shall support DL-TDOA and DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 14.3.1 | NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.2 | NR RSTD measurement accuracy test case for dual positioning frequency layer in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.3 | NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.4 | NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.5 | NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA | Rel-17 | C85nr | All FR1 NR UEs. The UEs shall support DL-TDOA and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.3.6 | NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR1 SA | Rel-17 | C88nr | All FR1 NR UEs. The UEs shall support UE assisted DL-TDOA and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 14.3.7 | NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C54nr | All FR2 NR UEs. The UEs shall support DL-TDOA and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.3.8 | NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Rel-17 | C57nr | All FR2 NR UEs. The UEs shall support UE assisted DL-TDOA and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 14.3.9 | NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA | Rel-18 | C105nr | All FR1 NR UEs. The UEs shall support DL-TDOA and DL-PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 14.3.10 | NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA | Rel-18 | C106nr | All FR2 NR UEs. The UEs shall support DL-TDOA and DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 15 | UE Rx-Tx time difference measurement requirements |  |  |  |  |  |  |  |  |
| 15.2.1 | UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Rel-16 | C46nr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.2 | UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR1 SA | Rel-16 | C46nr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.3 | UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Rel-16 | C47nr | All FR2 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.4 | UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA | Rel-16 | C47nr | All FR2 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.5 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Rel-17 | C89nr | All FR1 NR UEs. The UEs shall support UE assisted Multi-RTT and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.2.6_1 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA: Subtest 1 | Rel-17 | C90nr | All FR1 NR UEs. The UEs shall support Multi-RTT and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types |  |  |  |  | Rel-17 |
| 15.2.6_2 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA: Subtest 2 | Rel-17 | C91nr | All FR1 NR UEs. The UEs shall support Multi-RTT and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type s and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.2.7 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with multiple RxTx TEGs in FR1 SA | Rel-17 | C92nr | All FR1 NR UEs. The UEs shall support Multi-RTT and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 15.2.8 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C58nr | All FR2 NR UEs. The UEs shall support UE assisted Multi-RTT and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.2.9_1s | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 1 | Rel-17 | C59nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types |  |  |  |  | Rel-17 |
| 15.2.9_2s | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 2 | Rel-17 | C60nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type s and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.2.10 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA | Rel-17 | C61nr | All FR2 NR UEs. The UEs shall support Multi-RTT and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 15.2.11 | UE Rx-Tx time difference measurement reporting delay test case for PRS aggregation in FR1 SA | Rel-18 | C100nr | All FR1 NR UEs. The UEs shall support Multi-RTT and DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 15.2.12 | UE Rx-Tx time difference measurement reporting delay test case for PRS aggregation in FR2 SA | Rel-18 | C101nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 15.2.13 | UE Rx-Tx time difference measurement reporting delay test case in FR1 for Redcap UE without Rx FH | Rel-18 | C102nr | All FR1 NR redcap UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-18 |
| 15.2.14 | UE Rx-Tx time difference measurement reporting delay test case in FR2 for Redcap UE without Rx FH | Rel-18 | C103nr | All FR2 NR redcap UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-18 |
| 15.2.15 | UE Rx-Tx time difference measurement reporting delay test case in FR2 for Redcap UE without Rx FH | Rel-18 | C104nr | All NR redcap UEs. The UEs shall support Multi-RTT and maximumPRS-BandwidthAcrossAllHopsFR1 for and PRS frequency hopping measurement and report and UE supporting per hop bandwidth ≥ 10 MHz for SCS 15 kHz and UE supporting per hop bandwidth = 20 MHz for SCS 30 kHz |  |  |  |  | Rel-18 |
| 15.2.16 | UE Rx-Tx time difference measurement reporting delay test case in FR2 for Redcap UE with Rx FH | Rel-18 | C105nr | All NR redcap UEs. The UEs shall support Multi-RTT and maximumPRS-BandwidthAcrossAllHopsFR1 for and PRS frequency hopping measurement and report and the UE supporting per hop bandwidth = 100 MHz for SCS 120 kHz |  |  |  |  | Rel-18 |
| 15.3.1 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Rel-16 | C46nr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.3.2 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layers in FR2 SA | Rel-16 | C47nr | All FR2 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.3.3 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR1 SA | Rel-17 | C89nr | All FR1 NR UEs. The UEs shall support UE assisted Multi-RTT and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.3.4 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR1 SA | Rel-17 | C92nr | All FR1 NR UEs. The UEs shall support Multi-RTT and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 15.3.5 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C58nr | All FR2 NR UEs. The UEs shall support UE assisted Multi-RTT and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.3.6 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Rel-17 | C61nr | All FR2 NR UEs. The UEs shall support Multi-RTT and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 15.3.7 | UE Rx-Tx time difference measurement accuracy test case for PRS aggregation in FR1 SA | Rel-18 | C100nr | All FR1 NR UEs. The UEs shall support Multi-RTT and DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 15.3.8 | UE Rx-Tx time difference measurement accuracy test case for PRS aggregation in FR2 SA | Rel-18 | C101nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT positioning method and aggregated DL PRS measurement and report |  |  |  |  | Rel-18 |
| 15.3.9 | UE Rx-Tx time difference measurement accuracy test case in FR1 for Redcap UE without Rx FH | Rel-18 | C102nr | All FR1 NR redcap UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-18 |
| 15.3.10 | UE Rx-Tx time difference measurement accuracy test case in FR2 for Redcap UE without Rx FH | Rel-18 | C103nr | All FR2 NR redcap UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-18 |
| 15.3.11 | UE Rx-Tx time difference measurement accuracy test case in FR1 for Redcap UE with Rx FH | Rel-18 | C104nr | All NR redcap UEs. The UEs shall support Multi-RTT and maximumPRS-BandwidthAcrossAllHopsFR1 for and PRS frequency hopping measurement and report and UE supporting per hop bandwidth ≥ 10 MHz for SCS 15 kHz and UE supporting per hop bandwidth = 20 MHz for SCS 30 kHz |  |  |  |  | Rel-18 |
| 15.3.12 | UE Rx-Tx time difference measurement accuracy test case in FR2 for Redcap UE with Rx FH | Rel-18 | C105nr | All NR redcap UEs. The UEs shall support Multi-RTT and maximumPRS-BandwidthAcrossAllHopsFR1 for and PRS frequency hopping measurement and report and the UE supporting per hop bandwidth = 100 MHz for SCS 120 kHz |  |  |  |  | Rel-18 |
| 15.4.1 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR1 SA | Rel-17 | C77nr | All FR1 NR UEs. The UEs shall support Multi-RTT and DL-PRS measurement in RRC_INACTIVE state | pc_ra_SDT_r17 |  |  |  | Rel-17 |
| 15.4.2 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Rel-17 | C78nr | All FR1 NR UEs. The UEs shall support Multi-RTT and DL-PRS measurement in RRC_INACTIVE state and reduced number of samples for PRS measurement in RRC_INACTIVE state | pc_ra_SDT_r17 |  |  |  | Rel-17 |
| 15.4.3 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA | Rel-17 | C79nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS measurement in RRC_INACTIVE state | pc_ra_SDT_r17 |  |  |  | Rel-17 |
| 15.4.4 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C80nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS measurement in RRC_INACTIVE state and reduced number of samples for PRS measurement in RRC_INACTIVE state | pc_ra_SDT_r17 |  |  |  | Rel-17 |
| 15.5.3 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Rel-17 | C79nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS measurement in RRC_INACTIVE state | pc_ra_SDT_r17 |  |  |  | Rel-17 |
| 15.5.4 | UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA | Rel-17 | C80nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS measurement and reduced number of samples for PRS measurements in RRC_INACTIVE state | pc_ra_SDT_r17 |  |  |  | Rel-17 |
| Note 1: If the signal type for BDS supported by the UE includes B1C then Rel-16 of LPP is required. If the signal type for BDS supported by the UE includes B2a and/or B3I then Rel-17 of LPP is required. |  |  |  |  |  |  |  |  |  |

Table 4-12: Applicability of tests Conditions for RAT-independent test cases in TS 37.571-1 [5] for NR

C01nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 THEN R ELSE N/A

C02nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 THEN R ELSE N/A

C03nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 THEN R ELSE N/A

C04nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 THEN R ELSE N/A

C05nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/36 THEN R ELSE N/A

C06nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND A.4.3-2/3 THEN R ELSE N/A

C07nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND A.4.3-2/3 THEN R ELSE N/A

C08nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND A.4.3-2/3 THEN R ELSE N/A

C09nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND A.4.3-2/3 THEN R ELSE N/A

C10nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/36 AND A.4.3-2/3 THEN R ELSE N/A

C11nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 THEN R ELSE N/A

C12nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/38 THEN R ELSE N/A

C13nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND A.4.3-2/3 THEN R ELSE N/A

C14nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/38 AND A.4.3-2/3 THEN R ELSE N/A

C15nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/37 THEN R ELSE N/A

C16nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/37 AND A.4.3-2/3 THEN R ELSE N/A

C17nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/39 THEN R ELSE N/A

C18nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/39 AND A.4.3-2/3 THEN R ELSE N/A

C19nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/6 THEN R ELSE N/A

C20nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/7 THEN R ELSE N/A

C21nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/9 THEN R ELSE N/A

C22nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/8 THEN R ELSE N/A

C23nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/36 THEN R ELSE N/A

C24nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/37 THEN R ELSE N/A

C25nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/18 THEN R ELSE N/A

C26nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/38 THEN R ELSE N/A

C27nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/40 THEN R ELSE N/A

C28nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/41 THEN R ELSE N/A

C29nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/40 AND A.4.3-2/3 THEN R ELSE N/A

C30nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/41 AND A.4.3-2/3 THEN R ELSE N/A

C31nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/40 THEN R ELSE N/A

C32nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/41 THEN R ELSE N/A

C33nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/39 THEN R ELSE N/A

C34nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND A.4.3-2/3 THEN R ELSE N/A

C35nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND A.4.3-2/3 THEN R ELSE N/A

C36nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND A.4.3-2/3 THEN R ELSE N/A

C37nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND A.4.3-2/3 THEN R ELSE N/A

C38nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/36 AND A.4.3-2/3 THEN R ELSE N/A

C39nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/37 AND A.4.3-2/3 THEN R ELSE N/A

C40nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND A.4.3-2/3 THEN R ELSE N/A

C41nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/38 AND A.4.3-2/3 THEN R ELSE N/A

C42nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/39 AND A.4.3-2/3 THEN R ELSE N/A

C43nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/40 AND A.4.3-2/3 THEN R ELSE N/A

C44nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/41 AND A.4.3-2/3 THEN R ELSE N/A

C45nr IF A.4.1-1/6 AND A.4.3-2/20 THEN R ELSE N/A

C46nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 THEN R ELSE N/A

C47nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 THEN R ELSE N/A

C48nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A

C49nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A

C50nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) THEN R ELSE N/A

C51nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A

C52nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A

C53nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) THEN R ELSE N/A

C54nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/23 THEN R ELSE N/A

C55nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C56nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C57nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/32 AND A.4.3-6G/1 THEN R ELSE N/A

C58nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6B/23 THEN R ELSE N/A

C59nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C60nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C61nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6G/1 THEN R ELSE N/A

C62nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/23 THEN R ELSE N/A

C63nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C64nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C65nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 THEN R ELSE N/A

C66nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 THEN R ELSE N/A

C67nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C68nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C69nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 THEN R ELSE N/A

C70nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 THEN R ELSE N/A

C71nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C72nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C73nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6F/11 THEN R ELSE N/A

C74nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/36 AND A.4.3-6F/11 THEN R ELSE N/A

C75nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/11 THEN R ELSE N/A

C76nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/36 AND A.4.3-6E/11 THEN R ELSE N/A

C77nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 AND A.4.3-6C/11 THEN R ELSE N/A

C78nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 AND A.4.3-6B/36 AND A.4.3-6C/11 THEN R ELSE N/A

C79nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6C/11 THEN R ELSE N/A

C80nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6B/36 AND A.4.3-6C/11 THEN R ELSE N/A

C81nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/11THEN R ELSE N/A

C82nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 AND A.4.3-6B/36 THEN R ELSE N/A

C83nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/11THEN R ELSE N/A

C84nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 AND A.4.3-6B/36 THEN R ELSE N/A

C85nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/23 THEN R ELSE N/A

C86nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C87nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C88nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/32 AND A.4.3-6G/1 THEN R ELSE N/A

C89nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 AND A.4.3-6B/23 THEN R ELSE N/A

C90nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C91nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C92nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 AND A.4.3-6G/1 THEN R ELSE N/A

C93nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/23 THEN R ELSE N/A

C94nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C95nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A

C96nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6F/11 THEN R ELSE N/A

C97nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/36 AND A.4.3-6F/11 THEN R ELSE N/A

C98nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/11 THEN R ELSE N/A

C99nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/36 AND A.4.3-6E/11 THEN R ELSE N/A

C100nr IF A.4.1-4/1 AND A.4.1-5/1 AND A.4.3-2/29 AND A.4.3-2/54 AND A.4.3-6C/13 THEN R ELSE N/A

C101nr IF A.4.1-4/1 AND A.4.1-5/2 AND A.4.3-2/29 AND A.4.3-2/54 AND A.4.3-6C/13 THEN R ELSE N/A

C102nr IF A.4.1-4/1 AND [16] A.4.3.12-1/2 AND A.4.1-5/1 AND A.4.3-2/29

C103nr IF A.4.1-4/1 AND [16] A.4.3.12-1/2 AND A.4.1-5/2 AND A.4.3-2/29

C104nr IF A.4.1-4/1 AND [16] A.4.3.12-1/2 AND A.4.3-2/29 AND (A.4.3-6B/18 AND (A.4.3-6B/40 OR A.4.3-6B/41 OR A.4.3-6B/42 OR A.4.3-6B/43 OR A.4.3-6B/44 OR A.4.3-6B/45)) AND (A.4.3-6B/19 AND A.4.3-6B/41)

C104nr IF A.4.1-4/1 AND [16] A.4.3.12-1/2 AND A.4.3-2/29 AND (A.4.3-6B/21 AND A.4.3-6B/46)

C105nr IF A.4.1-4/1 AND A.4.1-5/1 AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-2/54 AND A.4.3-6F/13 THEN R ELSE N/A

C106nr IF A.4.1-4/1 AND A.4.1-5/2 AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-2/54 AND A.4.3-6F/13 THEN R ELSE N/A

< Unchanged sections omitted >

Table A.4.3-6B: NR DL-PRS Capability

| Item | NR DL-PRS Capability | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourceSetPerTrpPerFrequencyLayer |  |
| 2 | maxNrOfTRP-AcrossFreqs | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfTRP_AcrossFreqs |  |
| 3 | maxNrOfPosLayer | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfPosLayer |  |
| 4 | maxNrOfDL-PRS-ResourcesPerResourceSet | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourcesPerResourceSet |  |
| 5 | maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourcesPerPositioningFrequencylayer |  |
| 6 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-fr1-Only | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourcesAcrossAllFL_TRP_ResourceSet_fr1_Only |  |
| 7 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-fr2-Only | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourcesAcrossAllFL_TRP_ResourceSet_fr2_Only |  |
| 8 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet - fr1-FR2Mix-fr1 | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourcesAcrossAllFL_TRP_ResourceSet_fr1_FR2Mix_fr1 |  |
| 9 | Support of ssb-FromNeighCellAsQCL for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_ssb_FromNeighCellAsQCL |  |
| 10 | Support of prs-FromServNeighCellAsQCL for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_prs_FromServNeighCellAsQCL |  |
| 11 | maxSupportedFreqLayers | 37.355, 6.4.3 | Rel-16 | pc_maxSupportedFreqLayers |  |
| 12 | Support of simulLTE-NR-PRS | 37.355, 6.4.3 | Rel-16 | pc_simulLTE_NR_PRS |  |
| 13 | supportedBandwidthPRS in FR1 for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1 |  |
| 14 | supportedBandwidthPRS in FR2 for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR2 |  |
| 15 | dl-PRS-BufferType for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_dl_PRS_BufferType |  |
| 16 | durationOfPRS-ProcessingSymbols for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_durationOfPRS_ProcessingSymbols |  |
| 17 | PRS-ProcessingSymbolsInEveryTms for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_PRS_ProcessingSymbolsInEveryTms |  |
| 18 | maxNumOfDL-PRS-ResProcessedPerSlot in 15kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15 |  |
| 19 | maxNumOfDL-PRS-ResProcessedPerSlot in 30kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30 |  |
| 20 | maxNumOfDL-PRS-ResProcessedPerSlot in 60kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60 |  |
| 21 | maxNumOfDL-PRS-ResProcessedPerSlot in 120kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120 |  |
| 22 | maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-fr1-FR2Mix-fr2 | 37.355, 6.4.3 | Rel-16 | pc_maxNrOfDL_PRS_ResourcesAcrossAllFL_TRP_ResourceSet_fr1_FR2Mix_fr2 |  |
| 23 | supportedDL-PRS-ProcessingSamples-RRC-CONNECTED for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_supportedDL_PRS_ProcessingSamples_RRC_CONNECTED |  |
| 24 | prsProcessingType for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_prsProcessingType |  |
| 25 | ppw-dl-PRS-BufferType for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_dl_PRS_BufferType |  |
| 26 | ppw-durationOfPRS-ProcessingSymbolsN for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_durationOfPRS_ProcessingSymbolsN |  |
| 27 | ppw-durationOfPRS-ProcessingSymbolsT for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_durationOfPRS_ProcessingSymbolsT |  |
| 28 | ppw-durationOfPRS-ProcessingSymbolsN2 for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_durationOfPRS_ProcessingSymbolsN2 |  |
| 29 | ppw-durationOfPRS-ProcessingSymbolsT2 for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_durationOfPRS_ProcessingSymbolsT2 |  |
| 30 | ppw-maxNumOfDL-PRS-ResProcessedPerSlot in 15kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15 |  |
| 31 | ppw-maxNumOfDL-PRS-ResProcessedPerSlot in 30kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30 |  |
| 32 | ppw-maxNumOfDL-PRS-ResProcessedPerSlot in 60kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60 |  |
| 33 | ppw-maxNumOfDL-PRS-ResProcessedPerSlot in 120kHz SCS for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120 |  |
| 34 | ppw-maxNumOfDL-Bandwidth in FR1 for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_maxNumOfDL_Bandwidth_FR1 |  |
| 35 | ppw-maxNumOfDL-Bandwidth in FR2 for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_ppw_maxNumOfDL_Bandwidth_FR2 |  |
| 36 | supportedDL-PRS-ProcessingSamples-RRC-Inactive for at least one NR band | 37.355, 6.4.3 | Rel-17 | pc_supportedDL_PRS_ProcessingSamples_RRC_Inactive |  |
| 37 | Supporting one of maximum DL-PRS bandwidth mhz40, mhz50, mhz80 and mhz100 across all hops for FR1 for at least one NR band | 37.355, 6.4.3 | Rel-18 | pc_maximumPRS_BandwidthAcrossAllHopsFR1 |  |
| 38 | Supporting one of maximum DL-PRS bandwidth mhz100, mhz200 and mhz400 across all hops for FR2 for at least one NR band | 37.355, 6.4.3 | Rel-18 | pc_maximumPRS_BandwidthAcrossAllHopsFR2 |  |
| 39 | supportedBandwidthPRS in FR1 equal to 5MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_5MHz |  |
| 40 | supportedBandwidthPRS in FR1 equal to 10MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_10MHz |  |
| 41 | supportedBandwidthPRS in FR1 equal to 20MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_20MHz |  |
| 42 | supportedBandwidthPRS in FR1 equal to 40MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_40MHz |  |
| 43 | supportedBandwidthPRS in FR1 equal to 50MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_50MHz |  |
| 44 | supportedBandwidthPRS in FR1 equal to 80MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_80MHz |  |
| 45 | supportedBandwidthPRS in FR1 equal to 100MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR1_100MHz |  |
| 46 | supportedBandwidthPRS in FR2 equal to 100MHz for at least one NR band | 37.355, 6.4.3 | Rel-16 | pc_supportedBandwidthPRS_FR2_100MHz |  |