3GPP TSG-RAN5 Meeting #108	R5-254202

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3540 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Implementation of RRM NR Intra-band Contiguous CA procedure in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For RRM testing TS 38,133 defines SSB and CORESET reference channel at the carrier lower edge to avoid overlap of SSB and CORESET#0 with the Reference Measurement Channel (RMC). The current test frequencies in TS 38.508-1 clause 4.3.1 are based on expanding the full range of the tested bands. This means that Low and High test frequencies are calculated such that carrier is at the band edges. Due to the sparse synchronisation raster this will result in that CORESET#0 and the SSB will be offset from the carrier lower edge. As discussed in R5-206789 at RAN5#89-e will FR2 test frequencies calculated as in clause 4.3.1.2 not meet the RRM requirements as defined in TS 38.533 for SSB and CORESET#0 reference channels for the case of SSB SCS=240kHz. As discussed in R5-203321 at RAN5#88-e the conclusion to achieve necessary dB range to enable RRM testing for FR2 was to keep all the allocated RBs within the CORESET (24RBs). This will not be posisble using current test frequencies specified for SSB SCS=120 kHz in clause 4.3.1.2 due to SSB and CORESET#0 offset from carrier lower edge. To enable FR2 RRM testing for SSB SCS=120 kHz there is a need to specify specific test frequncies for FR2 RRM testing and SSB SCS=120 kHz. New frequencies were already introduced in chapter 7.2.3 for Single Carrier Bands based on the principles as introduced to Annex C in TS 38.508-1 in CR 1580. A new section to calculate the frequencies for CA RRM in Annex C is needed Additionaly: There are missing references in the procedure and the formula refered to calculate in section 2d the Fcarrier is not accurate |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section to determine SSB, CORESET#0 and signalling parameters for RRM CA were added to meet with RRM testing requirements References missing were included and specific formula for Fcarrier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There won’t be any procudere specifically to determine SSB, CORESET#0 and signalling parameters for RRM CA and It will lead to errors when doing the calculation of the Fcarrier |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related CRs: R5-254205, R5-254210 and R5-254302 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

# C.4	Determination of SSB and CORESET#0 for RRM testing with SSB SCS 120 kHz and 240 kHz

## C.4.1	General

The requirements to be met and the principles used for determining the SSB and CORESET#0 for a PCell used in RRM test cases are:

1.	The complete SSB and CORESET#0 shall be within the carrier’s channel bandwidth.

2.	The SSB centre frequency (SSref) shall be on the synchronisation raster.

3.	The SSB shall be kept as close as possible to the carrier’s lower edge centre frequency.

4.	The CORESET#0 configuration is selected using 24 RBs and OffsetRBs = 0 according to Table 13-8 and Index 0 for SCSSSB =120 KHz in TS 38.213 [22] and Table 13-10 and Index 0 for SCSSSB =240 kHz in TS 38.213 [22].

5.	The first SSB subcarrier shall be aligned with the defined resource grid given by SCS indicated by subCarrierSpacingCommon in the MIB.

## C.4.2	Determination of SSB, CORESET#0 and signalling parameters

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs = 0 and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	The target test frequencies for Low, Mid and High ranges are calculated as described in clause C.2.1.1.

For each of Low, Mid and High ranges do:

2.	Determine SSB and CORESET#0:

2a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

2b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-2 [7], clause 5.4.3.1 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-2 [8].

2c.	Calculate the FSSref for the selected GSCN value in step 2b in accordance to TS 38.101-2 [7], clause 5.4.3.1 for FR2.

2d.	Calculate the frequency FSSBlow and shift the carrier frequency to achieve FcarrierLow equal or as close as possible FSSBlow on the carrier’s frequency raster.

FSSBlow = FSSref - BWSSB / 2

Fcarrier = calculated using the formula in clause C.2.1.1 with FDL_Low = FSSBlow

FcarrierLow = Ceil((FSSBlow + 12 * SCSCarrier * (NRB / 2)) / ΔFRaster) * ΔFRaster

2e.	Calculate kSSB

kSSB = (FSSBlow - FcarrierLow) / subCarrierSpacingCommon (MIB, FR2) (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / subCarrierSpacingCommon (MIB, FR2). kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 3 else modify the carrier frequency to get valid value of kSSB and kSSB MOD N = 0.

3.	Calculate Point A frequency, ΔFoffsetToCarrier and ΔFOffsetToPointA:
The CORESET#0 configuration is selected using 24 RBs and OffsetRBs = 0 according to Table 13-8 and Index 0 for SCSSSB =120 KHz in TS 38.213 [22] and Table 13-10 and Index 0 for SCSSSB =240 kHz in TS 38.213 [22] (see C.4.1). This means that FOffsetToPointA = FcarrierLow. By selecting Point A equal to FcarrierLow this gives:

FPointA = FcarrierLow

ΔFoffsetToCarrier = 0

ΔFOffsetToPointA = 0

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | 0 (Index=0 in table 13-8 for SCSSSB =120 KHz in TS 38.213 [22] and table 13-10 for SCSSSB =240 KHz in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | 0 The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | 0 The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

## C.4.3	Determination of SSB, CORESET#0 and signalling parameters for NR Intra-band Contiguous CA for PCell

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs = 0 and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	The target test frequencies for Low, Mid and High ranges are calculated as described in clause C.2.4.2.

For each of Low, Mid and High ranges do:

2.	Determine SSB and CORESET#0:

2a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

2b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-2 [7], clause 5.4.3.1 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-2 [8].

2c.	Calculate the FSSref for the selected GSCN value in step 2b in accordance to TS 38.101-2 [7], clause 5.4.3.1 for FR2.

2d.	Calculate the frequency FSSBlow and shift the carrier frequency to achieve FcarrierLow equal or as close as possible FSSBlow on the carrier’s frequency raster.

FSSBlow = FSSref - BWSSB / 2

Fcarrier = Ceil((FSSBlow + 12 * SCSCarrier * (NRB / 2)) / ΔFRaster) * ΔFRaster

FcarrierLow = see formula for FcarrierLow in Table C.1-1 with new value of Fcarrier

2e.	Calculate kSSB

kSSB = (FSSBlow - FcarrierLow) / subCarrierSpacingCommon (MIB, FR2) (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / subCarrierSpacingCommon (MIB, FR2). kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 3 else modify the carrier frequency to get valid value of kSSB and kSSB MOD N = 0.

3.	Calculate Point A frequency, ΔFoffsetToCarrier and ΔFOffsetToPointA:
The CORESET#0 configuration is selected using 24 RBs and OffsetRBs = 0 according to Table 13-8 and Index 0 for SCSSSB =120 KHz in TS 38.213 [22] and Table 13-10 and Index 0 for SCSSSB =240 kHz in TS 38.213 [22] (see C.4.1). This means that FOffsetToPointA = FcarrierLow. By selecting Point A equal to FcarrierLow this gives:

FPointA = FcarrierLow

ΔFoffsetToCarrier = 0

ΔFOffsetToPointA = 0

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | 0 (Index=0 in table 13-8 for SCSSSB =120 KHz in TS 38.213 [22] and table 13-10 for SCSSSB =240 KHz in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | 0 The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | 0 The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

## C.4.3	Determination of SSB, CORESET#0 and signalling parameters for NR Intra-band Contiguous CA for SCell

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs = 0 and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	The target test frequencies for Low, Mid and High ranges are calculated as described in clause C.2.4.2.2

2.	Determine SSB and CORESET#0:

2a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Max

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

2b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-1 [7] and TS 38.101-2 [8].

2c.	Calculate the FSSref for the selected GSCN value in step 1b in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2.

2d.	Calculate the frequency FoffsetToPointA, which is the lowest subcarrier of the lowest resource block with the subcarrier spacing being a multiple of resource blocks expressed in terms of common PRB size and overlaps with the SS/PBCH block subcarrier 0 of the first resource block of the SS/PBCH block, FSSBlow (TS 38.211 [3], clause 7.4.3.1):

FSSBlow = FSSref - BWSSB / 2

FoffsetToPointA = CRBsize * Floor(Round((FSSBlow - FcarrierLow) / CRBsize),5) + FcarrierLow Note 1

Note 1:	The “Round” operation is needed to avoid that platforms using “float” representation will give erroneous results. Hence, the need for the “Round” operation is platform dependant. The “Round” operation is to be done to 5 decimals. It is needed to avoid that a calculated value of for example 5.9999999999 is Floored to the value of 5.

2e.	Calculate the maximum OffsetRBs value with FCORESET0Low >= FcarrierLow:

Max_OffsetRBs = Round((FoffsetToPointA - FcarrierLow) / CRBsize) Note 2

Note 2:	The “Round” operation is needed to avoid that platforms using “float” representation will give erroneous results. Hence, the need for the “Round” operation is platform dependant. It is needed to make sure that the Max_OffsetRBs is an integer. For example a calculated value of Max_OffsetRBs = 5.9999999999 shall result in a Max_OffsetRBs = 6.

2f.	Select the largest valid OffsetRBs value equal or smaller than the calculated max value, Max_OffsetRBs in step 1e within the applicable values for the carrier in TS 38.213 [4], table 13-1 to 13-10 limited to the table indexes with number of RBs and number of symbols  equal to the minimum value of in the table and minimum value of  for the selected .If a valid OffsetRBs value is found, then continue from step 2g.

If no valid OffsetRBs value is found, then select the next valid GSCN with FSSref <= FSSref_Max within the valid GSCN range for the carrier and repeat steps 2b to 2f.

If no valid OffsetRBs value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.3 and the procedure is completed.

2g.	Calculate kSSB

kSSB = (FSSBlow - FoffsetToPointA) / {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / {15 kHz for FR1; subCarrierSpacingCommon (MIB) for FR2}. kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 2.

If kSSB is not an integer value or kSSB MOD N <> 0, then select the next valid GSCN with FSSref < FSSref_Max within the valid GSCN range for the carrier and repeat steps 2b to 2g. 

If N > 1 and no valid kSSB value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g for a maximum shift of 3 * ΔFRaster (see clause C.3.1, Note 1). Cases requiring more than 3*ΔFRaster shifts shall be analyzed individually and, if eventually agreed, marked in section 4.3 to have been obtained applying certain exceptions over this section C.3.2. In case of an exception, for Mid, Mid-Low and Mid-High, shifting frequency up can also be considered to achieve the frequency closer to the ideal case.

If no valid kSSB value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

3.	Calculate Point A frequency, ΔFoffsetToCarrier and ΔFOffsetToPointA:
The CORESET#0 configuration is selected using 24 RBs and OffsetRBs = 0 according to Table 13-8 and Index 0 for SCSSSB =120 KHz in TS 38.213 [22] and Table 13-10 and Index 0 for SCSSSB =240 kHz in TS 38.213 [22] (see C.4.1). This means that FOffsetToPointA = FcarrierLow. By selecting Point A equal to FcarrierLow this gives:

FPointA = FcarrierLow

ΔFoffsetToCarrier = 0

ΔFOffsetToPointA = 0

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | 0 (Index=0 in table 13-8 for SCSSSB =120 KHz in TS 38.213 [22] and table 13-10 for SCSSSB =240 KHz in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | 0 The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | 0 The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>