3GPP TSG-RAN5 Meeting #108	R5-254205

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3541 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RRM NR Intra-band Contiguous CA configurations in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For RRM testing TS 38,133 defines SSB and CORESET reference channel at the carrier lower edge to avoid overlap of SSB and CORESET#0 with the Reference Measurement Channel (RMC). The current test frequencies in TS 38.508-1 clause 4.3.1 are based on expanding the full range of the tested bands. This means that Low and High test frequencies are calculated such that carrier is at the band edges. Due to the sparse synchronisation raster this will result in that CORESET#0 and the SSB will be offset from the carrier lower edge. As discussed in R5-206789 at RAN5#89-e will FR2 test frequencies calculated as in clause 4.3.1.2 not meet the RRM requirements as defined in TS 38.533 for SSB and CORESET#0 reference channels for the case of SSB SCS=240kHz. As discussed in R5-203321 at RAN5#88-e the conclusion to achieve necessary dB range to enable RRM testing for FR2 was to keep all the allocated RBs within the CORESET (24RBs). This will not be posisble using current test frequencies specified for SSB SCS=120 kHz in clause 4.3.1.2 due to SSB and CORESET#0 offset from carrier lower edge. To enable FR2 RRM testing for SSB SCS=120 kHz there is a need to specify specific test frequncies for FR2 RRM testing and SSB SCS=120 kHz. New frequencies were already introduced in chapter 7.2.3 for Single Carrier Bands based on the principles as introduced to Annex C in TS 38.508-1 in CR 1580. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New tables for intra-band contiguous CA configurations in FR2 were added to meet with RRM testing requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA Bands won’t meet RRM testing requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related CRs: R5-254202, R5-254210 and R5-254302 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 7.2.3.3	NR inter-band CA configurations in FR2

Editor’s note: further testability analysis for RRM inter-band CA configurations might be required.

For FR2 NR operating bands the test frequencies for RRM testing are specified in clause 4.3.1.2.2.

#### 7.2.3.4	NR intra-band contiguous CA configurations in FR2

##### 7.2.3.4.1	NR Intra-band contiguous CA configurations for CA_n257

7.2.3.4.1.1	CA_n257B

Editor’s note:	CA_n257B Test frequencies are FFS.

7.2.3.4.1.2	CA_n257C

Editor’s note:	CA_n257C Test frequencies are FFS.

7.2.3.4.1.3	CA_n257D

Editor’s note:	CA_n257D Test frequencies are FFS.

7.2.3.4.1.4	CA_n257E

Editor’s note:	CA_n257E Test frequencies are FFS.

7.2.3.4.1.5	CA_n257F

Editor’s note:	CA_n257F Test frequencies are FFS.

7.2.3.4.1.6	CA_n257G

Table 7.2.3.4.1.6-1: NR Intra-Band contiguous CA configuration CA_n257G, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

FFS

Table 7.2.3.4.1.6-2: NR Intra-Band contiguous CA configuration CA_n257G, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 |  | CC1 |  | 100 |  | 120 |  | 66 |  | Downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | & | Mid | 27963.84 | 2078563 | 27916.32 | 2077771 | 0 | 120 |  | 22469 | 2078011 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  |  |  |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 |  | 100 |  | 120 |  | 66 |  | Downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | & | Mid | 28063.80 | 2080229 | 28016.28 | 2079437 | 0 | 120 |  | 22475 | 2079739 |  | 7 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  |  |  |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.2.3.4.1.7	CA_n257H

Editor’s note:	CA_n257H Test frequencies are FFS.

7.2.3.4.1.8	CA_n257I

Editor’s note:	CA_n257I Test frequencies are FFS.

7.2.3.4.1.9	CA_n257J

Editor’s note:	CA_n257J Test frequencies are FFS.

7.2.3.4.1.10	CA_n257K

Editor’s note:	CA_n257K Test frequencies are FFS.

7.2.3.4.1.11	CA_n257L

Editor’s note:	CA_n257L Test frequencies are FFS.

7.2.3.4.1.12	CA_n257M

Editor’s note:	CA_n257M Test frequencies are FFS.

##### 7.2.3.4.2	NR Intra-band contiguous CA configurations for CA_n258

7.2.3.4.2.1	CA_n258B

Editor’s note:	CA_n258B Test frequencies are FFS.

7.2.3.4.2.2	CA_n258C

Editor’s note:	CA_n258C Test frequencies are FFS.

7.2.3.4.2.3	CA_n258D

Editor’s note:	CA_n258D Test frequencies are FFS.

7.2.3.4.2.4	CA_n258E

Editor’s note:	CA_n258E Test frequencies are FFS.

7.2.3.4.2.5	CA_n258F

Editor’s note:	CA_n258F Test frequencies are FFS.

7.2.3.4.2.6	CA_n258G

Table 7.2.3.4.2.6-1: NR Intra-Band contiguous CA configuration CA_n258G, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

FFS

Table 7.2.3.4.2.6-2: NR Intra-Band contiguous CA configuration CA_n258G (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | & | Mid | 25838.40 | 2042915 | 25790.88 | 2042347 | 0 |  | 22346 | 2042587 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | & | Mid | 25938.36 | 2044805 | 25890.84 | 2044013 | 0 |  | 22352 | 2044315 | 7 | 0 | 0 (0) | 0 |
|  |  |  |  |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.2.3.4.2.7	CA_n258H

Editor’s note:	CA_n258H Test frequencies are FFS.

7.2.3.4.2.8	CA_n258I

Editor’s note:	CA_n258I Test frequencies are FFS.

7.2.3.4.2.9	CA_n258J

Editor’s note:	CA_n258J Test frequencies are FFS.

7.2.3.4.2.10	CA_n258K

Editor’s note:	CA_n258K Test frequencies are FFS.

7.2.3.4.2.11	CA_n258L

Editor’s note:	CA_n258L Test frequencies are FFS.

7.2.3.4.2.12	CA_n258M

Editor’s note:	CA_n258M Test frequencies are FFS.

##### 7.2.3.4.3	NR Intra-band contiguous CA configurations for CA_n259

##### FFS

##### 7.2.3.4.4	NR Intra-band contiguous CA configurations for CA_n260

FFS

##### 7.2.3.4.5	NR Intra-band contiguous CA configurations for CA_n261

7.2.3.4.5.1	CA_n261B

Editor’s note:	CA_n261B Test frequencies are FFS.

7.2.3.4.5.2	CA_n261C

Editor’s note:	CA_n261C Test frequencies are FFS.

7.2.3.4.5.3	CA_n261D

Editor’s note:	CA_n261D Test frequencies are FFS.

7.2.3.4.5.4	CA_n261E

Editor’s note:	CA_n261E Test frequencies are FFS.

7.2.3.4.5.5	CA_n261F

Editor’s note:	CA_n261F Test frequencies are FFS.

7.2.3.4.5.6	CA_n261G

Table 7.2.3.4.5.6-1: NR Intra-Band contiguous CA configuration CA_n261G, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

FFS

Table 7.2.3.4.5.6-2: NR Intra-Band contiguous CA configuration CA_n261G (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | & | Mid | 27877.44 | 2077123 | 27829.92 | 2076331 | 0 |  |  | 22464 | 2076571 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | & | Mid | 27977.40 | 2078789 | 27929.88 | 2077997 | 0 |  |  | 22470 | 2078299 |  | 7 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.2.3.4.5.7	CA_n261H

Editor’s note:	CA_n261H Test frequencies are FFS.

7.2.3.4.5.8	CA_n261I

Editor’s note:	CA_n258I Test frequencies are FFS.

7.2.3.4.5.9	CA_n261J

Editor’s note:	CA_n261J Test frequencies are FFS.

7.2.3.4.5.10	CA_n261K

Editor’s note:	CA_n261K Test frequencies are FFS.

7.2.3.4.5.11	CA_n261L

Editor’s note:	CA_n261L Test frequencies are FFS.

7.2.3.4.5.12	CA_n261M

Editor’s note:	CA_n261M Test frequencies are FFS.

7.2.3.4.5.12	CA_n261O

Editor’s note:	CA_n261O Test frequencies are FFS.

7.2.3.4.5.12	CA_n261P

Editor’s note:	CA_n261P Test frequencies are FFS.

7.2.3.4.5.12	CA_n261Q

Editor’s note:	CA_n261 Test frequencies are FFS.

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>