3GPP TSG-RAN5 Meeting #108	R5-254047

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1179 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Antenna Assumptions in Annex M |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 07-07-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some of the antenna assumptions were incorrect or needed to be refined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Antenna assumptions were corrected and properly defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Antenna assumptions are not defined properly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | M.2.1.3, M.2.1.6, M.3.1.1.1, M.3.1.1.5, M.3.2.1.1, M.3.2.1.3, M.3.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

Annex M:(normative)
Measurement grids

This appendix describes the assumptions and definition of the minimum number of measurement grid points for various grid types. Further details can be found in [5].

A total of three measurement grids are considered:

-	Beam Peak Search Grid: using this grid, the TX and RX beam peak direction will be determined. 3D EIRP scans are used to determine the TX beam peak direction and 3D Throughput/RSRP/EIS scans for RX beam peak directions.

-	Spherical Coverage Grid: using this grid, the CDF of the EIRP/EIS distribution in 3D is calculated to determine the spherical coverage performance.

-	TRP Measurement Grid: using this grid, the total power radiated by the DUT in the TX beam peak direction is determined by integrating the EIRP measurements taken on the sampling grid.

<<< START OF CHANGES >>>

# M.2	Beam Peak Search Grid

Editor’s note: Other implementations are not precluded as far as the respective analysis are presented and included in this TS

## M.2.1	UE Power classes

### M.2.1.1	Power class 1 devices

The antenna array assumptions for the MU simulations are outlined in Table M.2.1.1-0 and M.2.1.1-0a for PC1.

Table M.2.1.1-0: Single Antenna Element Radiation Pattern for PC1 and PC5

| Antenna element horizontal radiation pattern | AE,Hϕ=-min12ϕϕ3dB2,AmdB Am =25 dB |
| --- | --- |
| Horizontal half-power beamwidth of single element | ϕ3dB=90° |
| Antenna element vertical radiation pattern | AE,Vθ=-min12θ-90θ3dB2,SLAv SLAv =25 dB |
| Vertical half-power beamwidth of single array element | θ3dB=90° |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 5 dBi |

Table M.2.1.1-0a: Composite Antenna Array Radiation Pattern for PC1 and PC5

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | 12 x 12 (default), 6 x 6 (optional for PC5) |
| Horizontal radiating element spacing | dh/λ = 0.5 |
| Vertical radiating element spacing | dv/λ = 0.5 |

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use for beam peak search the following measurement grids leading to a systematic error of “Beam Peak Search” of 0.7 dB:

-	Constant density grid (using the charged particle implementation) with at least 3000 grid points.

-	Constant step size grid with at least 4902 grid points, corresponding to an angular step size of 3.6º.

For better measurement uncertainties, finer measurement grids as shown in Table M.2.1.1-1 may be used. Choice of grids among these 2 types of grids is up to test system implementation.

Table M.2.1.1-1: Minimum number of unique grid points for sample systematic errors

| Systematic Error of ‘Beam Peak Search’: Offset from Beam Peak at which CDF is 5% | Minimum Number of Unique Grid Points for Constant Step Size Grid | Minimum Number of Unique Grid Points for Constant Density Grid |
| --- | --- | --- |
| 0.3dB | 10226 (2.5o step size) | 7000 |
| 0.4dB | N/A | 5000 |
| 0.5dB | 7082 (3ostep size) | 4500 |
| 0.6dB | N/A | 3500 |
| 0.7dB | 4902 (3.6o step size) | 3000 |

### M.2.1.2	Power class 2 devices

TBD

### M.2.1.3	Power class 3 devices

The antenna array assumptions for the MU simulations are outlined in Table M.2.1.3-0 and M.2.1.3-0a for PC3.

Table M.2.1.3-0: Single Antenna Element Radiation Pattern for PC3

| Antenna element horizontal radiation pattern | , Am =30 dB |
| --- | --- |
| Horizontal half-power beamwidth of single element | 260° for 8 x 2 antenna array configuration, 90° for other optional configurations |
| Antenna element vertical radiation pattern | , SLAv =30 dB |
| Vertical half-power beamwidth of single array element | 130º for 8 x 2 antenna array configuration, 90° for other optional configurations |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 1.5 dBi |

Table M.2.1.3-0a: Composite Antenna Array Radiation Pattern for PC3

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | 8 x 2 (default), 4 x 2 (optional), 6 x 2 (optional) |
| Horizontal radiating element spacing dh/λ | 0.5 |
| Vertical radiating element spacing dv/λ | 0.5 |

In order to make a reasonable trade-off between measurement uncertainties, at least 800 (constant density grid with charged particle implementation) or 1106 (constant step size grid) measurement grid points shall be used for beam peak search procedures. For better measurement uncertainties, finer measurement grids as shown below may be used. Choice of grids among these 2 types of grids is up to test system implementation.

Table M.2.1.3-1: Minimum number of unique grid points for sample systematic errors (non-sparse antenna arrays)

| Systematic Error of ‘Beam Peak Search’: Offset from Beam Peak at which CDF is 5% | Minimum Number of Unique Grid Points for Constant Step Size Grid | Minimum Number of Unique Grid Points for Constant Density Grid (charged particle implementation) |
| --- | --- | --- |
| 0.2dB | 2522 (5o step size) | 2000 |
| 0.3dB | 1742 (6o step size) | 1500 |
| 0.4dB | N/A | 1000 |
| 0.5dB | 1106 (7.5ostep size) | 800 |

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10 of [11], devices with an M x N (M ≥ N) configuration with M ≤ 4 and N ≤ 2 can utilize either of the following minimum number of grid points with the same systematic error of ‘Beam Peak Search’ of 0.5dB for beam peak search procedures:

- 	310 (constant density grid with charged particle implementation) measurement grid points.

-	422 (constant step size grid with Dq=Df=12.0o) measurement grid points.

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10 of [11], devices with an M x N (M ≥ N) configuration with 4 < M ≤ 6 and N ≤ 2 can utilize either of the following minimum number of grid points with the same systematic error of ‘Beam Peak Search’ of 0.5dB for beam peak search procedures:

- 	575 (constant density grid with charged particle implementation) measurement grid points.

-	762 (constant step size grid with Dq=Df=9.0o) measurement grid points.

### M.2.1.4	Power class 4 devices

TBD

### M.2.1.5	Power class 5 devices

The same antenna array assumptions and measurement grids as in Clause M.2.1.1 apply.

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10a of [11], devices with an M x N (M ≥ N) configuration with M ≤ 6 and N ≤ 6 can utilize either of the following minimum number of grid points with the same systematic error of ‘Beam Peak Search’ of 0.7dB for beam peak search procedures:

-	Constant density grid (using the charged particle implementation) with at least 750 grid points.

-	Constant step size grid with at least 1106 grid points, corresponding to an angular step size of 7.5º.

### M.2.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.2.1.1 with a default antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use for beam peak search the following measurement grids leading to a systematic error of “Beam Peak Search” of 0.7 dB:

-	Constant density grid (using the charged particle implementation) with at least 750 grid points.

-	Constant step size grid with at least 1106 grid points, corresponding to an angular step size of 7.5º.

## M.2.2	Coarse and fine measurement grids

The baseline beam peak search is based on a single and fine beam peak search grid to determine the TX/RX beam peak of the DUT in any given direction. This means that even in sectors where poor EIRP/EIS performance is observed, a very fine grid is used to search for the TX/RX beam peak.

An optimized approach, based on an initial coarse search followed by a subsequent fine search could reduce the number of beam peak search grid points significantly. The basis for this approach is to use a coarse grid with fewer number of points than the ones described in section M.2.1 in the first stage to identify candidate regions that contain the global beam peak and search for the global beam peak with the fine grid in the second stage with a minimum number of points described in section M.2.1.

As an example, Figure M.2.2-1 illustrates the coarse and fine measurement grid approach applied to TX beam search; while this illustration is for EIRP, it can easily be extended to RX beam peak search using EIS.For simplification purposes, 2D coarse and fine searches are illustrated but the concept can be extended to 3D easily. The UE is assumed to form a total of six beams in the 2D plane as illustrated on the left of Figure M.2.2-1. In the centre of Figure M.2.2-1, the 36 coarse beam peak search grid points in the 2D plane are illustrated. On the right, the grey dots on the respective antenna patterns illustrate the measured EIRP values towards each coarse grid point direction based on the respective beam steering directions. This illustration shows that the EIRP beam peak of the coarse search, EIRPCSBP, is found to be the peak of the orange beam while the global TX beam peak (red beam) was not identified due to the coarse sampling of the grid points.

Figure M.2.2-1: Illustration of the Coarse Search Approach for TX Beam Peak Search. Left: Antenna Pattern assumptions in 2D, Centre: Coarse beam peak search grid points/discrete antenna measurement positions, Right: TX beam EIRP measurements per grid point

The proposed fine search approach is illustrated further in Figure M.2.2-2. A fine search region starting from the beam peak identified in the coarse search, EIRPCSBP, over a range of FS is used to identify the regions that need to be investigated more closely with the fine search algorithm. The fine search range FS is a function of the angular spacing of the coarse beam peak search grid as well as the beam width of the reference antenna pattern considered for smartphone UEs.

Figure M.2.2-2: Illustration of the fine beam peak search grid. Left: identify the measurement grid points that yielded EIRP values within the fine search region, right: placement of fine beam peak search grid points

In order to maintain the same MU as the fine beam peak search measurement grids in Clause M.2.1.3, i.e., 0.5 dB for PC3 UEs, the minimum DFS from Table M.2.2-1 for constant step-size grids and from Table M.2.2-2 for constant density grids shall be applied to the coarse&fine search for PC3 UEs. The results presented in these tables utilize coarse measurement grids that match the spherical coverage grids from Clause M.3.

Table M.2.2-1: Measurement grid parameters for the constant step-size coarse&fine beam peak search measurement grids for PC3 UEs with a coarse grid of Dq=Df=15° (spherical coverage grid).

| Antenna Configuration Grid Parameters | 8x2 | 6x2 | 4x2 |
| --- | --- | --- | --- |
| DFS [dB] with fine grid Dq=Df=7.5° (Note 1) | 2.5 | 1.5 | 0.5 |
| DFS [dB] with fine grid Dq=Df=9° (Note 2) |  | 1.5 |  |
| DFS [dB] with fine grid Dq=Df=12° (Note 2) |  |  | 0.5 |
| Coarse grid with Dq=Df [°] | 15 | 15 | 15 |
| Note 1: Local searches in the “fine search region” are performed on the 8 fine grid points surrounding each coarse grid point within the DFS region (Figure M.2.2-3). Note 2: Local searches in the “fine search region” are performed on the fine grid points surrounding each coarse grid point within the DFS region that are within a conical region (half angle) of 1.5*step size of the fine grid, as illustrated in Figure M.2.2-4. |  |  |  |

Table M.2.2-2: Measurement grid parameters for the constant-density coarse&fine beam peak search measurement grids with a coarse grid using 200 unique grid points (spherical coverage grid)

| Antenna Configuration Grid Parameters | 8x2 | 6x2 | 4x2 |
| --- | --- | --- | --- |
| DFS [dB] | 3 | 1.5 | 0.5 |
| Number of Unique Grid Points (fine grid) | 800 | 575 | 310 |
| Number of Unique Grid Points (coarse grid) | 200 | 200 | 200 |
| Min. Conical Region (Half Angle) Surrounding Coarse Gird Point to Identify Fine Grid Points [°] (Note 1) | 11.25 | 13.7 | 19.3 |
| Note 1: Local searches in the “fine search region” are performed on the fine grid points surrounding each coarse grid point within the DFS region that are within tabulated conical region (half angle), as illustrated in Figure M.2.2-5. |  |  |  |

When the coarse&fine searches with constant-step size grids are utilizing step sizes of Dcoarse=Dqcoarse=Dfcoarse with Dcoarse=2Dfine, 8 fine grid points are selected for each coarse grid point within DFS as outlined highlighted in Figure M.2.2-3.

For the 6x2 and 4x2 configurations utilizing the spherical coverage grid with constant step size grids, outlined in Clause M.3, as coarse measurement grid, the coarse and fine grid step sizes are no longer an integer multiple of each other. The above approach to select the 8 closest neighbours of the coarse grid point, Figure M.2.2-3, is no longer applicable. Instead, a different approach shall be applied to those constant-step size grids as well as all constant-density grids. Here, the fine grid points surrounding a coarse grid point identified to be within DFS shall be contained within a conical region around that coarse grid point. This approach is further visualized in Figure M.2.2-4 for the constant step size grids and in Figure M.2.2-5 for the constant density grids. The half-angle of the cone shall match the values in Table M.2.2-1 and Table M.2.2-2, respectively, which correspond to 1.5Dfine for the constant step size grids and 1.5*maximum separation between a fine grid point and its 6 closest neighbours.

In these figures, red grid points correspond to fine grid points and the blue points correspond to the coarse grid points. The conical region around a sample coarse grid point is visualized in yellow, while the fine grid points within those regions are highlighted in green.

Figure M.2.2-3: Illustration: Coarse & Fine Constant Step Size Grids with Dcoarse=2Dfine (with D=Dq=Df)

Figure M.2.2-4: Coarse & Fine Constant Step Size Grids for sample PC3 grids, left: 8x2, centre: 6x2, right: 4x2.

Figure M.2.2-5: Coarse & Fine Constant Density Grids for sample PC3 grids, left: 8x2, centre: 6x2, right: 4x2.

The metric using a coarse & fine grid approach for the TX beam peak search is EIRP for both grids. For RX beam peak search, the metric is EIS for coarse grids and for fine grids.

# M.3	Spherical Coverage Grid

Editor’s note: Other implementations are not precluded as far as the respective analysis are presented and included in this TS

## M.3.1	EIRP spherical coverage

### M.3.1.1	UE Power classes

#### M.3.1.1.1	Power class 1 devices

The same antenna array assumptions as in Clause M.2.1.1 apply. Additionally, the following assumptions apply:

- a single antenna array is integrated inside DUT

- beam steering assumptions as follows:

-	In the xz plane, 4° beam steering granularity (from 30° to 150°)

-	In the xy plane, 4° beam steering granularity (from -60° to 60°)

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB and 0.04dB Mean Error

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB and 0.06dB Mean Error

For better measurement uncertainties, finer measurement grids as shown in Tables M.3.1.1.1-1 and M.3.1.1.1-2 may be used. Choice of grids among these 2 types of grids is up to test system implementation.

There is no need to have the Tx beam peak placed on a measurement grid point.

For constant step size measurement grids, the CDF analyses require the PDFs to be scaled by sin(theta) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1.

Table M.3.1.1.1-1: Statistical results of EIRP85%CDF for the 12x12 antenna array for constant step size measurement grids and the beam peak oriented in completely random orientations.

| Step Size [o] | Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- | --- |
| 12 | 422 | 0.10 | 0.03 |
| 15 | 266 | 0.12 | 0.06 |
| 20 | 146 | 0.23 | 0.05 |

Table M.3.1.1.1-2: Statistical results of EIRP85%CDF for the 12x12 antenna array for constant density measurement grids and the beam peak oriented in completely random orientations.

| Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- |
| 150 | 0.15 | 0.06 |
| 175 | 0.13 | 0.04 |
| 200 | 0.13 | 0.04 |

#### M.3.1.1.2	Power class 2 devices

TBD

#### M.3.1.1.3	Power class 3 devices

The same antenna array assumptions as in Clause M.2.1.3 apply. Additionally, the following assumptions apply:

- two antenna arrays integrated inside DUT, one in the front and one in the back

- the implementation loss for the antenna near the front is 5dB less than that for the antenna near the back

- beam steering assumptions as follows:

-	In the xz plane, 45° beam steering granularity (from 45° to 135°)

-	In the xy plane, 25° beam steering granularity (from -90° to 90°)

In order to make a reasonable trade-off between measurement uncertainties, at least 200 (constant density grid with charged particle implementation) or 266 (constant step size grid) measurement grid points shall be used for EIRP spherical coverage procedure. For better measurement uncertainties, finer measurement grids as shown below may be used. Choice of grids among these 2 types of grids is up to test system implementation.

There is no need to have the Tx beam peak placed on a measurement grid point.

For constant step size measurement grids, the CDF analyses require the PDFs to be scaled by sin(theta) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1.

Table M.3.1.1.3-1: Statistical results of EIRP50%CDF for the 8x2 antenna array for constant density measurement grids (with charged particle implementation) and the beam peak oriented in completely random orientations errors (non-sparse antenna arrays)

| Number of unique grid points | STD [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- |
| 200 | 0.11 | 0.02 |
| 300 | 0.08 | 0.01 |
| 400 | 0.07 | 0.01 |
| 500 | 0.06 | 0.01 |

Table M.3.1.1.3-2: Statistical results of EIRP50%CDF for the 8x2 antenna array for constant step size measurement grids and the beam peak oriented in completely random orientations errors (non-sparse antenna arrays)

| Step Size [o] | Number of unique grid points | STD [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- | --- |
| 9 | 762 | 0.05 | 0.00 |
| 10 | 614 | 0.06 | 0.00 |
| 12 | 422 | 0.07 | 0.01 |
| 15 | 266 | 0.12 | 0.01 |

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10 of [11], devices with an M x N (M ≥ N) configuration with M ≤ 4 and N ≤ 2 can utilize either of the following minimum number of grid points for spherical coverage procedures:

-	180 (constant density grid with charged particle implementation) measurement grid points with std. deviation of 0.12dB.

-	266 (constant step size grid with Dq=Df=15.0o) measurement grid points with std. deviation of 0.11dB.

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10 of [11], devices with an M x N (M ≥ N) configuration with 4 < M ≤ 6 and N ≤ 2 can utilize either of the following minimum number of grid points for spherical coverage procedures:

-	200 (constant density grid with charged particle implementation) measurement grid points with std. deviation of 0.14dB.

-	266 (constant step size grid with Dq=Df=15.0o) measurement grid points with std. deviation of 0.15dB.

#### M.3.1.1.4	Power class 4 devices

TBD

#### M.3.1.1.5	Power class 5 devices

The same antenna array, integration, and beam steering assumptions and measurement grids as in Clause M.3.1.1.1 apply.

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10a of [11], devices with an M x N (M ≥ N) configuration with M ≤ 6 and N ≤ 6 can utilize either of the following minimum number of grid points for spherical coverage procedures:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB.

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB.

#### M.3.1.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.3.1.1.1 with an antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB.

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB.

## M.3.2	EIS spherical coverage

### M.3.2.1	UE Power classes

#### M.3.2.1.1	Power class 1 devices

The same antenna array and integration assumptions as in Clause M.2.1.1 apply. Additionally, the same beam steering assumptions as in Clause M.3.1.1.1 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB and 0.04dB Mean Error

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB and 0.06dB Mean Error

-	the MU element ‘Systematic error related to EIS spherical coverage’ is the DL step size, i.e., 0.2dB.

Choice of grids among these 2 types of grids is up to test system implementation.

There is no need to have the Rx beam peak placed on a measurement grid point.

For constant step size measurement grids, the CCDF analyses require the PDFs to be scaled by sin(theta) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1.

#### M.3.2.1.2	Power class 2 devices

TBD

#### M.3.2.1.3	Power class 3 devices

The same antenna array and integration assumptions as in Clause M.2.1.3 apply. Additionally, the same beam steering assumptions as in Clause M.3.1.1.3 apply.

In order to make a reasonable trade-off between measurement uncertainties, at least 200 (constant density grid with charged particle implementation) or 266 (constant step size grid) measurement grid points shall be used for EIS spherical coverage procedure. For better measurement uncertainties, finer measurement grids as shown below may be used. Choice of grid(s) among these 2 types of grids is up to test system implementation.

There is no need to have the Rx beam peak placed on a measurement grid point.

For constant step size measurement grids, the CCDF analyses require the PDFs to be scaled by sin(theta) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1.

Table M.3.2.1.3-1: Statistical results of EIS50%CDF for the 8x2 antenna array for constant step size measurement grids and the beam peak oriented in completely random orientations errors (non-sparse antenna arrays)

|  |  | DL Power Step Size: infinitesimal |  | DL Power Step Size: 0.1dB |  | DL Power Step Size: 0.5dB |  | DL Power Step Size: 1dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step Size [o] | Number of unique grid points | STD [dB] | ¦Mean Error¦ [dB] | STD [dB] | ¦Mean Error¦ [dB] | STD [dB] | ¦Mean Error¦ [dB] | STD [dB] | ¦Mean Error¦ [dB] |
| 6.0 | 1742 | 0.03 | 0.00 | 0.03 | 0.10 | 0.03 | 0.50 | 0.02 | 1.02 |
| 9.0 | 762 | 0.05 | 0.00 | 0.05 | 0.10 | 0.05 | 0.50 | 0.04 | 1.02 |
| 10.0 | 614 | 0.06 | 0.00 | 0.06 | 0.10 | 0.06 | 0.50 | 0.05 | 1.02 |
| 12.0 | 422 | 0.08 | 0.01 | 0.07 | 0.10 | 0.07 | 0.50 | 0.07 | 1.02 |
| 15.0 | 266 | 0.12 | 0.02 | 0.12 | 0.10 | 0.11 | 0.50 | 0.10 | 1.02 |

Table M.3.2.1.3-2: Statistical results of EIS50%CDF for the 8x2 antenna array for constant density measurement grids (with charged particle implementation) and the beam peak oriented in completely random orientations errors (non-sparse antenna arrays)

|  | DL Power Step Size: infinitesimal |  | DL Power Step Size: 0.1dB |  | DL Power Step Size: 0.5dB |  | DL Power Step Size: 1dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of unique grid points | STD [dB] | ¦Mean Error¦ [dB] | STD [dB] | ¦Mean Error¦ [dB] | STD [dB] | ¦Mean Error¦ [dB] | STD [dB] | ¦Mean Error¦ [dB] |
| 200 | 0.10 | 0.02 | 0.10 | 0.10 | 0.10 | 0.50 | 0.09 | 1.01 |
| 300 | 0.08 | 0.01 | 0.08 | 0.10 | 0.08 | 0.50 | 0.07 | 1.01 |
| 400 | 0.06 | 0.01 | 0.06 | 0.10 | 0.06 | 0.50 | 0.05 | 1.01 |
| 500 | 0.06 | 0.01 | 0.06 | 0.10 | 0.06 | 0.50 | 0.05 | 1.01 |

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10 of [11], devices with an M x N (M ≥ N) configuration with M ≤ 4 and N ≤ 2 can utilize either of the following minimum number of grid points for spherical coverage procedures:

-	180 (constant density grid with charged particle implementation) measurement grid points with std. deviation of 0.12dB.

-	266 (constant step size grid with Dq=Df=15.0o) measurement grid points with std. deviation of 0.11dB.

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10 of [11], devices with an M x N (M ≥ N) configuration with 4 < M ≤ 6 and N ≤ 2 can utilize either of the following minimum number of grid points for spherical coverage procedures:

-	200 (constant density grid with charged particle implementation) measurement grid points with std. deviation of 0.14dB.

-	266 (constant step size grid with Dq=Df=15.0o) measurement grid points with std. deviation of 0.15dB.

#### M.3.2.1.4	Power class 4 devices

TBD

#### M.3.2.1.5	Power class 5 devices

The same antenna array and integration assumptions and measurement grids as in Clause M.3.2.1.1 apply. Additionally, the same beam steering assumptions as in Clause M.3.1.1.5 apply.

Based on an optional vendor declaration with respect to the antenna array configuration, see Table A.4.3.9-10a of [11], devices with an M x N (M ≥ N) configuration with M ≤ 6 and N ≤ 6 can utilize either of the following minimum number of grid points for spherical coverage procedures:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB.

-	the MU element ‘Systematic error related to EIS spherical coverage’ is the DL step size, i.e., 0.2dB

#### M.3.2.1.6	Power class 6 devices

The same antenna array assumptions and measurement grids as in Clause M.3.2.1.1 with an antenna array configuration of 6 x 6 apply.

In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

-	constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB

-	constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB.

-	the MU element ‘Systematic error related to EIS spherical coverage’ is the DL step size, i.e., 0.2dB

Choice of grids among these 2 types of grids is up to test system implementation.

There is no need to have the Rx beam peak placed on a measurement grid point.

For constant step size measurement grids, the CCDF analyses require the PDFs to be scaled by sin(theta) or the normalized Clenshaw-Curtis weights W()/W(90o), introduced in Section M.4.2.1.

<<< Skip Unchanged Sections >>>

<<< END OF CHANGES >>>