3GPP TSG RAN Meeting #109					        RP-252328

Beijing, China, September 15-18, 2025

Source:	AT&T

Title:	TSG RAN Guidance to RAN1 after RAN1 #122

Agenda Item:	8.2.2

Document for: 	Discussion/Decision

# 1. Introduction

At the TSG RAN #108 meeting, a new study item for the 6G Radio (6GR) design was approved [1]. Subsequently, RAN1 commenced detailed studies during RAN1 #122 [2]. This document identifies areas where RAN guidance is needed for RAN1 to progress the physical layer design of 6GR according to agreements and outcomes from RAN1 #122.

# 2. Aspects Related to Diverse Device Types

At the RAN1 #122 meeting, several agreements were reached pertaining to the support of diverse device types in 6G. Some of these agreements readily identified additional actions for RAN #109, e.g., in the form of guidance or even confirmation.

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

At least from a RAN1 perspective, the discussions should focus on the properties that define mandatory baseline functionalities supported by all 6G UEs. In our view, these are tied to a lowest-tier 6G UE for “ease of exposition”. For example, LTE Release 8 specified initial access within 1.4 MHz bandwidths (PSS/SSS/PBCH) for all 4G UEs while also mandating bandwidths of up to 20 MHz for all 4G UEs in the initial release. This, later, required the definition of bandwidth reduced (BR) system information (SIB1-BR) to allow initial access with 1.4 MHz within a >1.4 MHz carrier. Among the many detriments of this design is duplication of common control signaling (SIB1 and SIB1-BR). Hence, there is broad support to define one common initial access procedure for all 6G UEs. The discussion, however, is rather unrelated to specific device types and when they are introduced. For example, LTE Rel. 8 could have limited the initial access to 1.4 MHz even for MBB devices in a 20 MHz carrier while still not introducing Cat-M type devices until later. The key difference is that in this hypothetical scenario a SIB1-BR type of design would not have been needed.

- Proposal 1: RAN1 discussions should focus on the properties that define mandatory baseline functionalities supported by all 6G UEs. These discussions, however, are rather unrelated to specific device types and when they are introduced.

As for the need of a plurality of device types, that are normatively specified, this discussion seems more relevant to the work item phase of 6GR, and even then, it may not be affirmed. As a baseline, 3GPP specifications should provide a platform or toolbox from which features can be configured. The less features are constrained to certain use cases, the better. This prevents siloed specifications with “verticals specific solutions” including NTN, unlicensed spectrum, IoT and so forth. For example, coverage enhancements can equally benefit NTN, IoT, and eMBB. Consequently, at least at this early stage, RAN1 studies should focus on functionalities supported by 6GR with utmost care to a scalable design that defines a single RAT which can be scaled from a minimum set of KPIs upwards to the most demanding ones. This bottom-up scalability exactly establishes the aforementioned platform and toolbox approach with one set of specifications that support all use cases with minimum customization and increased economy of scale.

- Proposal 2: Normative specification of a plurality of device types can be discussed in the work item phase of 6GR, and even then may not be affirmed

For the aspects of “minimum spectrum allocation” and “smallest maximum supported RF and BB UE BW without spectrum aggregation” we believe 6GR should support 3 MHz bands while the detailed design can be left to the working groups. Specifically, the design doesn’t need to and maybe shouldn’t be optimized for 3 MHz. Additionally, as the working groups embark on discussing specific design proposals, smallest maximum supported RF bandwidths that differ in the DL an dUL should be discussed.

- Proposal 3: 6GR should support 3 MHz bands. The detailed design can be left to the working groups. Smallest maximum supported RF bandwidths that differ in the DL and UL should be discussed.

# 3. Aspects Related to 5G-6G Migration

RAN1 #122 witnessed extensive discussions on 6GR framework including waveform, channel coding, etc. Prior to that, RAN had identified “Migration from 5G NR to 6GR” as one of the 6GR study objectives in the 6G SID agreed in RAN#108 [1].

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

The following is captured in the RAN1 #122 chair’s notes as guidance for future meetings. Note that these were not agreed and are not part of the formal report.

Note: Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.: Proponents to characterize the main motivation for modification/additional waveform proposals: - Targeted link direction, i.e. DL, UL or both - Targeted use case (e.g. NTN, specific frequency range, etc.), if any - Potential motivations metrics used, and quantified gains for a proposal, e.g. - Coverage - Network energy efficiency - UE energy efficiency - Spectral efficiency - High speed tolerance - Scheduling flexibility - Integration with ISAC Proponents provide information on the following aspects, if applicable - MRSS compatibility - Target channels/signals, e.g. all channels, PxSCH only, etc. - MIMO (SU and MU-MIMO) compatibility - Target modulations, and impact to other modulations, if applicable - Multi-user multiplexing/scheduling flexibility - Multiplexing/coexistence with baseline waveforms - Impact on synchronization and initial access - Expected specification impact - Transmitter/receiver complexity and impact to power consumption. Chairman Guidance For 6GR control channel coding, - Evaluations can be provided in form of BLER and FAR results. - Evaluations/analysis can be provided for complexity, decoding latency, - Other metrics are not precluded. - Proponent companies to provide evaluation assumptions and methodologies for respective evaluation. - Proponent companies to provide details of channel coding extension compared with NR channel coding - Proponent companies to provide justification for the channel coding extension, compared with control channel codes as defined in 5G NR. Chairman Guidance For 6GR data channel coding, - Evaluations can be provided in form of BLER results. - Evaluation/analysis on throughput, complexity, and decoding latency can be provided - Other metrics are not precluded. - Proponent companies to provide their target scenarios and requirements, evaluation assumptions and methodologies for respective evaluation/analysis, e.g., decoding algorithm and details, information sizes, code rates, HARQ scheme, channel type, modulation order, target BLER, etc. - Proponent companies to provide details of channel coding extension compared with NR channel coding. - Proponent companies to provide justification for the channel coding extension, and how to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off, compared with data channel codes as defined in 5G NR.

In our view, when discussing novel 6GR design proposals, RAN1 studies should proactively emphasize their impact on migrating from 5G to 6GR deployments. This could, for instance, be accomplished in the form of high-level descriptions of the required modifications to existing 5G NR features, schemes, and/or functionalities. Such design outlines should never be construed to impose a certain implementation solution or approach. Rather, they provide a “means of feasibility” argument by the proponent(s) of a new/modified feature that can help the 3GPP community evaluate the needed technological upgrades required to support that feature.

- Proposal 4: RAN1 studies should proactively emphasize the impact of novel 6GR design proposals on migrating from 5G to 6GR deployments. This could, for instance, be accomplished in the form of high-level descriptions of the required modifications to existing 5G NR features, schemes, and/or functionalities.

To ensure reasonable evaluation of potential 6GR features, RAN1 6G studies should additionally emphasize the impact of novel 6GR design proposals on performance and efficiency with the legacy NR design as a reference.

- Proposal 5: RAN1 6G studies should additionally emphasize the impact of novel 6GR design proposals on performance and efficiency with the legacy NR design as a reference.

Performance corresponds to classical communication metrics used in legacy generations, e.g., throughput, spectral efficiency, latency, reliability, etc. 3GPP should strive to significantly improve on the legacy performance.  Efficiency includes preliminary evaluation of the underlying operational/implementation efficiency needed to support the potential 6GR new feature, including energy efficiency, design complexity at the transmitter/receiver sides, algorithm/scheme complexity, as well as other aspects impacting the cost efficiency of prospective 6G deployment, e.g., dataset/metadata signaling overhead over backhaul, etc. New/updated performance metrics may need to be introduced to quantify efficiency, e.g., energy efficiency, as to be recommended by the 6GR study on evaluation assumptions.

# 4. Conclusion

This contribution provided our views on RAN1 development following the RAN1#122 meeting. We have the following proposals:

- Proposal 1: RAN1 discussions should focus on the properties that define mandatory baseline functionalities supported by all 6G UEs. These discussions, however, are rather unrelated to specific device types and when they are introduced.

- Proposal 2: Normative specification of a plurality of device types can be discussed in the work item phase of 6GR, and even then may not be affirmed

- Proposal 3: 6GR should support 3 MHz bands. The detailed design can be left to the working groups. Smallest maximum supported RF bandwidths that differ in the DL an dUL should be discussed.

- Proposal 4: RAN1 studies should proactively emphasize the impact of novel 6GR design proposals on migrating from 5G to 6GR deployments. This could, for instance, be accomplished in the form of high-level descriptions of the required modifications to existing 5G NR features, schemes, and/or functionalities.

- Proposal 5: RAN1 6G studies should additionally emphasize the impact of novel 6GR design proposals on performance and efficiency with the legacy NR design as a reference.

# References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- Draft Meeting Report, 3GPP TSG RAN WG1#122, Bengaluru, India