3GPP TSG RAN Meeting #109					RP-252341

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Google

Title:	Key Performance Indicators for 6G

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussions on key performance indicators for 6G.

# Discussion

The AI traffic (e.g., Generative AI) is a new type of traffic which is an important service to be provided by 6G. The AI traffic is fundamentally different from the traditional traffics, e.g., web browsing or video streaming traffics. The AI traffic is usually token based, in which a token is a piece of a word that an AI model understands and a single word might be one token or broken into multiple tokens. As shown in Figure 1, one AI model, e.g., large language model (LLM), can generate inference result sequentially which is consist of a lot of tokens according to instructions (prompt) received from the user. The server would start to communicate with users after generating a few tokens in a Packet instead of after generating all the tokens. The process repeats, with the next batch of tokens going into the next packet, creating a steady stream of data packets for the AI traffic.

Figure 1: AI traffic communication

Nowadays there are the following types of AI traffics:

- AI based summary. The AI writes a summary for a paper. The number of output tokens is smaller than the number of input tokens. The LLM may generate one token within 1-5ms.

- AI based text generation. The AI generates a text based on some instructions. Usually, the number of output tokens is larger than the number of input tokens. In one example, to generate a text with 800 words, 1000 output tokens may be required. The LLM may generate one token within 1-5ms.

- AI based image/video generation. The AI generates an image or a video according to some instructions. Compared to a text generation, the video generation would require more output tokens. For example, to generate a 5s video (e.g., 81 frames) with 720p, the number of output tokens could be 40K-80K or more. The LLM may generate one video within 90s.

- Interactive AI. The AI inference can be interrupted and updated according to the updated instructions from the user. This type of AI would have higher delay requirement and certain package loss would lead to incorrect inference results. The LLM may generate one token within 1ms.

For AI traffic, if one packet with output tokens is lost, the whole inference result will be broken. For example, for AI based video generation, if one output token is missing, the user cannot get the video. Therefore, instead of the user throughput based on the success/failure status for each packet, the user throughput based on multi-package should be defined as one KPI.

Proposal 1: Support the burst-packet data rate as a KPI for the AI traffic.

- If one packet is lost, all the packets are considered as lost

# Conclusion

In this contribution, we provided discussion on the KPI for 6G. Based on the discussion, the following proposals are provided.

Proposal 1: Support the burst-packet data rate as a KPI for the AI traffic.

- If one packet is lost, all the packets are considered as lost