3GPP TSG-RAN WG4 Meeting #116	R4-2512348

# Bangalore, India, 25 Aug - 29 Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5761 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for NR Radio Resource Management (RRM) Phase 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-07-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the core requirements for NR Radio Resource Management (RRM) Phase 5, including CSSF optimization, BSF optimization and EMR-based fast SCell activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft bigCR includes the following endorsed draft CRs: Draft CRs endorsed in RAN4#114bis: Rx BSF optimization: T-doc; Company; Title. R4-2503357; OPPO; DraftCR on Multi-Rx based RRC Connection Release with Redirection requirements. R4-2504957; CATT; DraftCR on SSB-based intra-frequency measurement without gap delay reduction by optimizing Rx BSF. R4-2504958; CMCC; DraftCR for optimizing Rx beam sweeping factor on SSB based Inter-frequency measurement with MG. R4-2504959; Nokia; Draft CR HO requirements and FBS requirement applicability. R4-2504960; ZTE Corporation, Sanechips; Draft CR on FBS based RRC Re-establishment for R19 RRM enh. R4-2504970; Apple; Draft CR on SSB based Intra-frequency measurement with MG. R4-2504971; MediaTek Germany GmbH; draft CR for Rx beam sweeping optimization for SSB based Intra-frequency measurement with NCSG. Draft CRs endorsed in RAN4#115: CSSF optimization T-doc; Company; Title. R4-2508319; Apple, CATT, Ericsson; draft CR for Rel-19 CSSF enhancement solution 1 (one CC measurement per band). R4-2508333; Huawei, HiSilicon; CR on L3 measurement delay by utilising 3-searcher solution to optimise CSSF outside gap for Rel-19. Rx BSF optimization: T-doc; Company; Title. R4-2508427; OPPO; DraftCR on Multi-Rx based RRC Connection Release with Redirection requirements. R4-2508428; Nokia; DraftCR for FBS requirement maintenance. R4-2505995; CATT, OPPO; DraftCR on Multi-Rx based inter-frequency measurement with NCSG. R4-2506615; CMCC; DraftCR for optimizing Rx beam sweeping factor on RRC connection release with redirection to NR. R4-2508431; Huawei, HiSilicon; DraftCR on FBS L3 measurement for inter-frequency without gap. EMR-based fast SCell activation: T-doc; Company; Title. R4-2508432; CATT, Apple, Huawei, HiSilicon, Ericsson; CR for fast SCell activation for UE supporting Rel-18 EMR. R4-2507222; Nokia; draftCR to 38.133 on Fast SCell activation with EMR. Draft CRs endorsed in RAN4#116: CSSF optimization T-doc; Company; Title. R4-2512347; OPPO, Huawei, HiSilicon; Draft CR for CSSF optimization of RRM Phase 5. Rx BSF optimization: T-doc; Company; Title. R4-2512289; Apple, CATT, Nokia, CMCC, Huawei, HiSilicon; Draft CR on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. |
| Consequences if not approved: |  | the core requirements for NR Radio Resource Management (RRM) Phase 5, including CSSF optimization, BSF optimization and EMR-based fast SCell activation, are missing in R19 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6.x (new), 6.1.1.4.2, 6.1.1.5.2, 6.2.1.2.1, 6.2.3.2.1, 8.3.2A (new), 8.3.4, 8.3.12, 9.2.5.1, 9.2.3.2, 9.2.6.2, 9.2.7.1, 9.3.4, 9.3.9.1, 9.3.10.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 3.6.20	Applicability of requirements for UE supporting L3 fast beam sweeping operation in FR2-1

The requirements related to the support of Rel-19 L3 fast Rx beam sweeping are applicable when

-	For intra-frequency measurements, the carrier configured for L3 SSB measurement in the FR2-1 band is the serving carrier, and

-	For inter-frequency measurements, in addition to the serving carrier, at most one carrier in the FR2-1 band is configured for L3 SSB measurement, and

-	UE is not configured with CA or DC, and

-	L3 fast Rx beam sweeping operation is activated.

L3 fast Rx beam sweeping operation is activated for intra-frequency measurements and inter-frequency measurements with measurement gap provided that

-	UE is configured with triggering conditions indicated by fbs-ThresholdP-r19 and/or fbs-ThresholdQ-r19, and the configured triggering conditions are satisfied.

-	The time since L3 fast Rx beam sweeping operation is activated is not larger than 90 s.

L3 fast Rx beam sweeping operation is activated when UE is indicated to perform handover or RRC Connection Release with Redirection to NR cell procedure.

<End of Change 1>

<Start of Change 2>

##### 6.1.1.4.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	If the target cell is a known intra-frequency cell, then Tsearch = 0 ms.

-	If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms.

-	If the target cell is a known inter-frequency cell, then

-	For a UE supporting ncd-SSB-BWP-Wor-r18:

-	if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0 ms;

-	if the measured SSB and the target SSB for handover belong to the same NR target cell and are not the same SSB, Tsearch = Trs ms provided one of the following conditions is satisfied:

-	The measured SSB is the CD-SSB in the target cell and the HO target SSB is the NCD-SSB in the first active DL BWP, or

-	The measured SSB is the NCD-SSB in the target cell and the HO target SSB is the CD-SSB in the first active DL BWP, or

-	The measured SSB is the NCD-SSB in the target cell and the HO target SSB is the NCD-SSB and they are within different DL BWPs

-	For a UE not supporting ncd-SSB-BWP-Wor-r18, then Tsearch = 0 ms.

-	If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, Tsearch = N*3* Trs ms.

Where

N = reduced beam sweeping factor when the UE supports Rel-19 L3 fast Rx beam sweeping and the conditions in clause 3.6.20 are fulfilled, otherwise N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2.

Tprocessing is time for UE processing. Tprocessing can be up to 20 ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ =  Trs for both known and unknown target cell.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If such measObjectNRs configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

In FR2, the target cell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the handover command:

-	the UE has sent a valid measurement report for the target cell and

-	One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the target cell also remains detectable during the handover delay according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell.

otherwise, it is unknown.

<End of Change 2>

<Start of Change 3>

##### 6.1.1.5.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When in inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	If the target cell is a known intra-frequency cell, then Tsearch = 0 ms.

-	If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms.

-	If the target cell is a known inter-frequency cell, then

-	For a UE supporting ncd-SSB-BWP-Wor-r18:

-	if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0 ms;

-	if the measured SSB and the target SSB for handover belong to the same NR target cell and are not the same SSB, Tsearch = Trs ms provided one of the following conditions is satisfied:

-	The measured SSB is the CD-SSB in the target cell and the HO target SSB is the NCD-SSB in the first active DL BWP, or

-	The measured SSB is the NCD-SSB in the target cell and the HO target SSB is the CD-SSB in the first active DL BWP, or

-	The measured SSB is the NCD-SSB in the target cell and the HO target SSB is the NCD-SSB and they are within different DL BWPs

-	For a UE not supporting ncd-SSB-BWP-Wor-r18, then Tsearch = 0 ms.

-	If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, Tsearch = N*3* Trs ms.

Where

N = reduced beam sweeping factor when the UE supports Rel-19 L3 fast Rx beam sweeping and the conditions in clause 3.6.20 are fulfilled, otherwise N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2.

Tprocessing is time for UE processing. Tprocessing can be up 40 ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ =  Trs for both known and unknown target cell.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If such measObjectNRs configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In FR2, the target cell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the handover command:

-	the UE has sent a valid measurement report for the target cell and

-	One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3,

-	One of the SSBs measured from the target cell also remains detectable during the handover delay according to the cell identification conditions specified in clause 9.3.

otherwise, it is unknown.

<End of Change 3>

<Start of Change 4>

6.2.1.2.1	UE Re-establishment delay requirement

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

The intra-frequency target NR cell shall be considered detectable if each relevant SSB can satisfy that:

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively, and

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band are fulfilled.

The inter-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively, and

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding NR Band are fulfilled.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the FR of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.2.1.2.1-1 when highSpeedMeasFlagFR2-r17 is not configured or UE is not capable of FR2 power class 6 and table 6.2.1.2.1-3 when highSpeedMeasFlagFR2-r17 is configured and UE is capable of FR2 power class 6.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the FR of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.2.1.2.1-2.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target NR cell.

TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3].

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target intra-frequency NR cell is known, else Nfreq = 2 and Tidentify_intra_NR = 0 if the target inter-frequency NR cell is known.

There is no requirement if the target cell does not contain the UE context.

In the requirement defined in the below tables, the target FR1 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, (M1Note2 x TSMTC + Tproc)) |
| ≥ -8 | FR2-2 | N/A | MAX (1000 ms, 120 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2-1 | N/A | M2Note1,Note3 + Tproc |
| < -8 | FR2-2 | N/A | 5280Note1 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: M1 = 80 and Tproc = 0 ms for the UE not capable of Rel-19 L3 fast Rx beam sweeping, otherwise, M1 = N1*10, N1 = reduced N and Tproc = 2 ms. NOTE 3: M2 = 3520 and Tproc = 0 ms for the UE not capable of Rel-19 L3 fast Rx beam sweeping, otherwise, M2 = N1*440, N1 = reduced N and Tproc = 2 ms. |  |  |  |

Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, (M1Note2 x TSMTC, i + Tproc)) |
| ≥ -8 | FR2-2 | N/A | MAX (1000 ms, 156 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2-1 | N/A | M2Note1,Note3 + Tproc |
| < -8 | FR2-2 | N/A | 6000 Note1 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: M1 = 104 and Tproc = 0 ms for the UE not capable of Rel-19 L3 fast Rx beam sweeping, otherwise, M1 = N1*13, N1 = reduced N and Tproc = 2 ms. NOTE 3: M2 = 4000 and Tproc = 0 ms for the UE not capable of Rel-19 L3 fast Rx beam sweeping, otherwise, M2 = N1*500, N1 = reduced N and Tproc = 2 ms. |  |  |  |

Table 6.2.1.2.1-3: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell when highSpeedMeasFlagFR2-r17 is configured (Frequency range FR2)

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 10 xN2 x TSMTC)) |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: When SMTC <= 40 ms, N2=2 when highSpeedMeasFlagFR2-r17 = set1; N2=6 when highSpeedMeasFlagFR2-r17 = set2. |  |  |  |

<End of Change 4>

<Start of Change 5>

##### 6.2.3.2.1	RRC connection release with redirection to NR

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detectable when for each relevant SSB, the following side conditions are met:

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding NR Band are fulfilled.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

Tidentify-NR: It is the time to identify the target NR cell and depends on the FR of the target NR cell. It is defined in table 6.2.3.2.1-1. Note that Tidentify-NR = TPSS/SSS-sync + Tmeas, in which TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released. TRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise,

-	there is no requirement if the SSB transmission periodicity is larger than 20 ms.

Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 11 x Trs) |
| FR2-1 | MAX (880 ms, Nx11 x Trs + Tproc) |
| FR2-2 | MAX (880 ms, 12x11 x Trs) |
| NOTE: If the UE has been provided with higher layer signalling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. NOTE2: If the UE supports Rel-19 L3 fast Rx beam sweeping, N = reduced beam sweeping factor indicated via UE capability and Tproc = 2 ms, otherwise N = 8 and Tproc = 0. |  |

<End of Change 5>

<Start of Change 6>

### 8.3.2A	SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Tactivation_time is the SCell activation delay in millisecond, which is defined as:

Requirements in this clause apply if the SCell is unknown, provided:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode,

The SCell is unknown if the corresponding known conditions specified in clause 8.3.2 are not met.

If the SCell belongs to FR1, Tactivation_time is TFirstSSB_MAX + Trs + 5ms provided that the side condition Ês/Iot ≥ -2dB is fulfilled, if the following conditions are met:

-	During the period equal to 5s for FR1 before the reception of the SCell activation command:

-	the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3.

-	the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e., a valid CQI reporting)

-	If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

-	If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter.

-	the SSB measured remains detectable according to the IDLE/INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

If the SCell belongs to FR2, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, and if the following conditions are met:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3.

-	the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e. a valid CQI reporting)

-	If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

-	If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter.

-	the SSB measured remains detectable according to the IDLE/ INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

Then Tactivation_time is:

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX  + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

Where:

TFirstSSB_MAX, Trs, TSMTC_MAX,  TFineTiming and TRRC_delay are same as that defined in clause 8.3.2.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to SCell activation command, and the TCI state is selected based on one of the latest reported SSB indexes

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to SCell activation command, and the TCI state is selected based on one of the latest reported SSB indexes;

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to SCell activation command, the TCI state for semi-persistent CSI-RS is selected based on one of the latest reported SSB indexes.

<End of Change 6>

<Start of Change 7>

### 8.3.4	Direct SCell Activation at SCell addition

The requirements in this clause apply for UE being configured in the RRC reconfiguration message, TS 38.331 [2], with one SCell for which the parameter sCellState is set to activated. If the RRC reconfiguration message for direct SCell activation also configures PSCell addition or PSCell change, the direct SCell activation delay may be longer than the requirements defined in this clause.

If the RRC reconfiguration message for direct SCell activation also configures TCI state information, the requirements in clause 8.3.2 based on that TCI state activation command is received at the same time as SCell activation command shall apply.

The UE shall configure the SCell in activated state upon successful completion of the RRC reconfiguration procedure within the specified delay. The UE shall be capable to transmit valid CSI report and apply actions for the directly activated SCell no later than in slot n+NdirectNR slot length ,

where:

-	Slot n is the last slot overlapping with the PDSCH containing the RRC reconfiguration message,

-	Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3 ms for the cases specified in clause 8.3.2 that TCI state is not indicated within Tactivation_time; otherwise, Ndirect = TRRC_Process + T1 + THARQ + Tactivation_time + TCSI_Reporting

-	TRRC_Process: RRC procedure delay as specified in clause 11.2 of TS 36.331 [16] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the RRC procedure delay defined in clause 12 of TS 38.331 [2],

-	T1: Delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message,

NOTE:	T1 is UE implementation dependent.

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3],

If target SCell is unknown, Tactivation_time is specified in 8.3.2A provided the following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode,

Otherwise:

If the SCell is known and belongs to FR1, TCSI_Reporting is specified in clause 8.3.2 and Tactivation_time is defined as:

-	TFirstSSB+ 5 ms, if the measurement period of the SCell being activated is equal to or smaller than 2400 ms.

-	TFirstSSB_MAX + Trs + 5 ms, if measurement period of the SCell being activated is larger than 2400 ms.

where,

the measurement period in table 9.2.5.2-1 applies if the target SCell was in an intra-frequency layer corresponding to an activated SCell;

the measurement period in table 9.2.5.2-3 applies if the target SCell was in an intra-frequency layer corresponding to a deactivated SCell;

the measurement period in table 9.3.5-1 applies if the target SCell was in an inter-frequency layer.

-	Otherwise, Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX shall override the existing ones:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1NR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRCProcess+T1NR slot length

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

-	If a UE supports, reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, the reduced Tactivation_time  specified in clause 8.3.2 when UE supports these capabilities is applicable for Direct SCell activation at SCell addition also.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.321 [7] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The SCell in FR1 is known provided the following conditions are met for the SCell:

-	During the last 5 seconds before the reception of the direct SCell configuration command:

-	the UE has sent a valid measurement report for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell identification conditions specified in clauses 9.2 and 9.3,

-	the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise, the SCell is unknown.

The SCell in FR2 is known provided it meets the corresponding conditions as defined in clause 8.3.2. Otherwise, the SCell is unknown.

The UE may be allowed to cause interruptions to serving cells on other component carriers during an interruption window, as specified in clause 8.2. The starting point of an interruption window on SpCell or any activated SCell shall not occur before slot n+1, and shall not occur after slot n+1+TRRC_Process+T1+TXNR slot length, where NR slot length is with respect to the numerology of the SCell being activated, and TX is:

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes TFineTiming.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

Starting from the slot n+TRRC_Process + T1NR slot length until the UE has completed the direct SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

<Start of Change 7>

<End of Change 8>

### 8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell

The requirements in this clause shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

If the UE has a valid TA for transmitting on an SCell then the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3*Ttarget_PL-RS)+TCSI_Reporting_afterNR slot length,

Where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running.

-	THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tactivation_time is the SCell activation delay in milliseconds.

If target SCell is unknown, Tactivation_time is specified in 8.3.2A provided the following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode,

For the UE capable of l3-MeasUnknownSCellActivation-r18, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in milliseconds as specified in clause 8.3.17 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L3 report for unknown case, if UE reports valid L3-RSRP before receiving TCI activation command;

-	First valid L1-RSRP reporting for unknown case, if UE reports valid L3-RSRP after receiving TCI activation command.

Otherwise, Tactivation_time is the SCell activation delay in milliseconds as specified in clause 8.3.2 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during PUCCH SCell activation.

-	TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource.

-	TCSI_processing is the UE processing time for CSI reporting.

-	TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS)

If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot lengthTHARQ+Tactivation_time+TCSI_ReportingNR slot length, and shall be capable to perform uplink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length n+THARQ+Tdelay_PUCCH_SCell+TCSI_Reporting_PUCCH_SCellNR slot lengthand shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length, where:

Tdelay_PUCCH_SCell = Tactivation_time + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS) + TCSI_reporting_after

Where:

-	Tactivation_time is the SCell activation delay in milliseconds.

If target SCell is unknown, Tactivation_time is specified in 8.3.2A provided the following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode,

For the UE capable of l3-MeasUnknownSCellActivation-r18, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in milliseconds as specified in clause 8.3.17 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L3 report for unknown case, if UE reports valid L3-RSRP before receiving TCI activation command;

-	First valid L1-RSRP reporting for unknown case, if UE reports valid L3-RSRP after receiving TCI activation command;

Otherwise, Tactivation_time is the SCell activation delay in milliseconds as specified in clause 8.3.2 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (when applicable in FR2) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during PUCCH SCell activation.

-	TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource.

-	TCSI_processing is the UE processing time for CSI reporting.

-	TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS)

-	T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the PUCCH SCell after the slot n+THARQ+Tactivation_timeNR slot length.

-	T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in table 8.1-1 of TS 38.213 [3]

-	T2 is the delay from slot n+THARQ+Tactivation_time+T1NR slot lengthuntil UE has obtained a valid TA command for the target PUCCH Scell being activated. Slot n is the slot where the UE receives PUCCH SCell activation command.

-	T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, as specified in clause 4.2 in TS 38.213 [3].

The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal:

-	The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for L3 measurement reporting

-	The target pathloss reference signal determination is based on the latest L3 measurement reporting

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3 dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB ≥-3 dB

If the target SCell is unknown and following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode,

the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for EMR reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The target pathloss reference signal determination is based on the EMR reporting in 4.7.3 and 5.8.3.

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs reported in EMR report with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB reported in EMR report≥-3dB

For the UE capable of l3-MeasUnknownSCellActivation-r18, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 reporting after SCell activation command and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the L3 reporting after SCell activation command

--	The target pathloss reference signal determination is based on either the latest L1-RSRP measurement reporting or the L3 reporting after SCell activation command, if UE reports both before receiving TCI activation command

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3 dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB ≥-3 dB

Otherwise, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for L1-RSRP measurement reporting

-	The target pathloss reference signal determination is based on the latest L1-RSRP measurement reporting

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3 dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB ≥-3 dB

Otherwise, the pathloss reference signal is unknown.

The above delay requirement shall apply provided that:

-	The target pathloss reference signal determined during PUCCH SCell activation is known otherwise longer activation time is expected if the pathloss reference signal is unknown; and

-	The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and

-	No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay can be extended.

The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is the same as the interruption in single SCell activation requirement in clause 8.3.2.

In addition to the interruption due to RF retuning during PUCCH SCell activation, if the UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH, and PRACH on PUCCH SCell and PUCCH/PUSCH/SRS on other active serving cell are fully or partially overlapping in time, the UE shall transmit PRACH on PUCCH SCell and is allowed to drop or cause interruption to SRS or PUCCH or PUSCH transmission on the SpCell or on any activated SCell. Otherwise, UE is not allowed to drop or cause any interruption to SRS or PUCCH or PUSCH transmission on SpCell or on any activated SCell.

For unknown PUCCH SCell activation in FR2, the requirement only applies when UE supports ReportingCrossPUCCH-Grp-r16, and UE is configured with CSI reporting via SpCell. For unknown PUCCH SCell activation in FR1, the requirement only applies when UE supports ReportingCrossPUCCH-Grp-r16, and UE is configured with CSI reporting via SpCell, if ‘ssb-PositionInBurst’ indicates multiple SSBs but TCI state indication is not provided in same MAC PDU with SCell activation.

The requirement for unknown PUCCH SCell applies provided that the PDCCH order (when applicable) and the activation commands for TCI, UL spatial relation and PL-RS (when applicable) are based on the latest valid L1-RSRP reporting or the L3 reporting after SCell activation command via Primary PUCCH group or EMR reporting as defined in 8.3.2A, 4.7.3 and 5.8.3.

<End of Change 8>

<Start of Change 9>

##### 9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.1-1.

For UE support FR1 only EN-DC 3-searcher capability configured with the E-UTRA-NR dual connectivity operation and none of SMTC occasions of outside gap measurement objects in one FR are overlapped with per-FR measurement GAP (if configured) in another FR, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.1-2.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gp |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1+NPSCC_CSIRS+ NPSCC_CCA_RSSI/CO | NSCC_SSB +Y+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1+NPSCC_CSIRS | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS |
| EN-DC with FR2 only inter band CA | N/A | N/A | 1+NPSCC_CSIRS | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1+NPSCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+NSCC_CSIRS_FR2_NCM) Note 3 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | 1+NPSCC_CSIRS | N/A | NSCC_SSB+Y+2x NSCC_CSIRS | NSCC_SSB+Y+2x NSCC_CSIRS |
| NOTE 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating band are included for EN-DC with FR2 only inter-band CA NOTE 6: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. For UE supporting CSSF enhancement for one serving CC measurement per-band for intra-frequency measurements without MG, NSCC_SSB is the number of SCCs to be measured following the principles specified in clause 9.2.3.1 and 9.2.3.2. NOTE 10: NPSCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11 If a measurement object configured by PSCell and an NR inter-RAT measurement object configured by E-UTRAN PCell are on the same serving carrier, they shall be counted as one intra-frequency measurement object, provided that they meet the measurement object merging conditions [in clause 9.1.3.2], otherwise they are counted separately as two measurement objects. |  |  |  |  |  |  |

Table 9.1.5.1.1-2: Enhanced CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1+NPSCC_CSIRS+NPSCC_CCA_RSSI/CO | ⌈0.5×(NSCC_SSB +Y+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) ⌉ | N/A | N/A | N/A | ⌈0.5×(NSCC_SSB +Y+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) ⌉ |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1+NPSCC_CSIRS | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS |
| EN-DC with FR2 only inter band CA | N/A | N/A | 1+NPSCC_CSIRS | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1+NPSCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+NSCC_CSIRS_FR2_NCM) Note 3 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | 1+NPSCC_CSIRS | N/A | NSCC_SSB+Y+2x NSCC_CSIRS | NSCC_SSB+Y+2x NSCC_CSIRS |
| NOTE 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating band are included for EN-DC with FR2 only inter-band CA NOTE 6: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPSCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: If a measurement object configured by PSCell and an NR inter-RAT measurement object configured by E-UTRAN PCell are on the same serving carrier, they shall be counted as one intra-frequency measurement object, provided that they meet the measurement object merging conditions in clause 9.1.3.2, otherwise they are counted separately as two measurement objects. |  |  |  |  |  |  |

##### 9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, E-UTRA inter-RAT measurement object without measurement gap, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

For UE supports FR1 only CA and FR1 only NR-DC 3-searcher capability or FR1+FR2 CA (PCell is FR1 only) 3-searcher capability in SA operation mode and none of SMTC occasions of outside gap measurement objects in one FR are overlapped with per-FR measurement GAP (if configured) in another FR, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, E-UTRA inter-RAT measurement object without measurement gap, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.2-2.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. For UE supporting CSSF enhancement for one serving CC measurement per-band for intra-frequency measurements without MG, NSCC_SSB is the number of SCCs to be measured following the principles specified in clause 9.2.3.1 and 9.2.3.2. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. |  |  |  |  |  |  |  |

Table 9.1.5.1.2-2: Enhanced CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA Note12 | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | 0.5×(NSCC_SSB +2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) +Y+Z | N/A | N/A | N/A | 0.5×(NSCC_SSB +2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) +Y+Z | (NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1, Note 12 | 1+NPCC_CSIRS | NSCC_SSB+2*(Y+Z) +2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM | N/A | 1+ NSCC_CSIRS_FR2_NCM Note 3,5 | NSCC_SSB+Y+Z +2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM | NSCC_SSB+2*(Y+Z)+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM | 2 x ( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 12: For UE supporting FR1 only CA and FR1 only NR-DC 3-searcher capability or FR1+FR2 CA (PCell is FR1 only) 3-searcher capability in SA operation mode, the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG that are being measured outside of MG. |  |  |  |  |  |  |  |

##### 9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurement, inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS based L3 measurement will be as specified in table 9.1.5.1.3-1.

For UE supports FR1 only CA and FR1 only NR-DC 3-searcher capability or  FR1+FR2 NR-DC (PCell is FR1 only) 3-searcher capability configured with NR-DC operation and none of SMTC occasions of outside gap measurement objects in one FR are overlapped with per-FR measurement GAP (if configured) in another FR, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurement, inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS based L3 measurement will be as specified in table 9.1.5.1.3-2.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PSCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | N/A | 2x(1+ NPSCC_CSIRS) Note 2 | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PSCell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |
| NOTE 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. NOTE 2: CSSFoutside_gap,i =1 if no SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on PSCC; CSSFoutside_gap,i =2 if no SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on PSCC. NOTE 3: Y is the number of configured inter-frequency SSB based frequency layers without MG that are being measured outside of MG; otherwise, it is 0. NOTE 4: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 5: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. NOTE 6: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. For UE supporting CSSF enhancement for one CC measurement per-band for intra-frequency measurements without MG, NSCC_SSB is the number of SCCs to be measured following the principles specified in clause 9.2.3.1 and 9.2.3.2. |  |  |  |  |  |  |

Table 9.1.5.1.3-2: Enhanced CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PSCell) Note 1 | 1+NPCC_CSIRS | ( NSCC_SSB +Y+2xNSCC_CSIRS) | N/A | (1+ NPSCC_CSIRS) Note 2 | (NSCC_SSB +Y+2x NSCC_CSIRS ) | (NSCC_SSB +Y+2x NSCC_CSIRS ) |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PSCell) | 1+NPCC_CSIRS | ( NSCC_SSB +Y+2xNSCC_CSIRS) | (1+ NPSCC_CSIRS) Note 2 | N/A | N/A | (NSCC_SSB +Y+2x NSCC_CSIRS ) |
| NOTE 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. NOTE 2: CSSFoutside_gap,i =1 if no SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on PSCC; CSSFoutside_gap,i =2 if no SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on PSCC. NOTE 3: Y is the number of configured inter-frequency SSB based frequency layers without MG that are being measured outside of MG; otherwise, it is 0. NOTE 4: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 5: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. NOTE 6: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. |  |  |  |  |  |  |

##### 9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps

For UE configured with NE-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurement and inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS based L3 measurement will be as specified in table 9.1.5.1.4-1.

Table 9.1.5.1.4-1: CSSFoutside_gap,i scaling factor for NE-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1+NPCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS | N/A | N/A | N/A | NSCC_SSB +Y+2x NSCC_CSIRS |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS |
| NE-DC with FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating band are included for NE-DC with FR2 only inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. For UE supporting CSSF enhancement for one serving CC measurement per-band for intra-frequency measurements without MG, NSCC_SSB is the number of SCCs to be measured following the principles specified in clause 9.2.3.1 and 9.2.3.2. |  |  |  |  |  |  |

<End of Change 9>

<Start of Change 10>

#### 9.2.3.2	Requirements for FR2

For UE supporting CSSF enhancement for one serving CC measurement per-band, if UE receives network indication via enable one serving carrier measurement in a band to enable one serving carrier measurement for an FR2 band,

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

- 	PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in the band; or

-	PSCC, when UE is configured with EN-DC with PSCC in the band; or

-	PSCC, when UE is configured with NR-DC with PSCC in the band; or

-	an SCC, if such SCC is the only one SCC on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band; or

-	an SCC, indicated by network via enable one serving carrier measurement in a band, if such SCC is one of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band;

-	otherwise

-	one SCC determined by UE implementation.

UE is not required to perform SS-RSRP, SS-RSRQ, and SS-SINR measurements on both serving and neighbour cell for each of the other intra-frequency layer(s) in the same band.

For UE not supporting CSSF enhancement for one serving CC measurement per-band, or UE supporting CSSF enhancement for one serving CC measurement per-band but does not receive network indication via enable one serving carrier measurement in a band to enable one serving carrier measurement for each FR2 band,

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

<End of Change 10>

<Start of Change 11>

#### 9.2.5.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise the UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_processing) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra + TSSB_processing) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_intra is given in table 9.2.5.1-11; otherwise, TPSS/SSS_sync_intra is given in table 9.2.5.1-2.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra is given in table 9.2.5.1-1 for FR1 and  table 9.2.5.1-2 for FR2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra is given in table 9.2.5.1-17 for FR1 and table 9.2.5.1-18 for FR2.

-	Otherwise, TPSS/SSS_sync_intra is given in tables 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (CC with deactivated SCell) or 9.2.5.1-5 (CC with deactivated SCell) or 9.2.5.1-9 (CC with deactivated SCell) or 9.2.5.1-11 or 9.2.5.1-12 (CC with deactivated PSCell) or 9.2.5.1-13 (CC with deactivated PSCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured

-	For UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to operate on a target cell with 12 PRB SSB in FR1, TSSB_time_index_intra_less_than_5Mhz is given in table 9.2.5.1-23.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra is given in table 9.2.5.1-3 for FR1 and table 9.2.5.1-15 for FR2-2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra is given in table 9.2.5.1-19 for FR1.

-	For UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to perform measurements on either a deactivated SCell or a deactivated PSCell with 12PRB SSB in FR1, TSSB_time_index_intra_less_than_5MHz is given in table 9.2.5.1-24 (deactivated SCell), table 9.2.5.1-24a (deactivated SCell with highSpeedMeasCA-Scell-r17)  or in table 9.2.5.1-25 (deactivated PSCell).

-	Otherwise, TSSB_time_index_intra is given in tables 9.2.5.1-3, 9.2.5.1-15 (FR2-2), 9.2.5.1-6 (CC with deactivated SCell), 9.2.5.1-10 (CC with deactivated SCell) or 9.2.5.1-14 (CC with deactivated PSCell).

-	TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.5.2-7; otherwise, TSSB_measurement_period_intra is given in table 9.2.5.2-2.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra is given in table 9.2.5.2-1 for FR1 and  table 9.2.5.2-2 for FR2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra is given in table 9.2.5.2-10 for FR1 and  table table 9.2.5.2-11 for FR2.

-	For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2 is configured, the TSSB_measurement_period_intra given in table 9.2.5.2-7 (if SMTC ≤ 40ms) and table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

-	Otherwise, TSSB_measurement_period_intra is given in table 9.2.5.2-1, table 9.2.5.2-2, table 9.2.5.2-3 (CC with deactivated SCell), 9.2.5.2-4 (CC with deactivated SCell), 9.2.5.2-5 or 9.2.5.2-6 (CC with deactivated SCell), 9.2.5.2-8 (CC with deactivated PSCell) or 9.2.5.2-9 (CC with deactivated PSCell).

TSSB_processing: it is the time period used to process multiple beams received in one SMTC. TSSB_processing = 2 ms for UE supporting Rel-19 L3 fast Rx beam sweeping when the conditions in clause 3.6.20 are fulfilled. Otherwise TSSB_processing = 0.

-	CSSFintra: it is a carrier specific scaling factor and is determined

-	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP, or

-	 when intra-frequency SMTC is fully non overlapping with GAP for UE indicating no-gap-with-interruption, or

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating no-gap-no-interruption, or

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting power class 6, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

For a UE that supports Rel-19 L3 fast Rx beam sweeping , when the conditions in clause 3.6.20 are fulfilled and when highSpeedMeasFlagFR2-r17 is not configured, the following values shall apply for Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps:

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =3* Nreduced_Rx_BSF.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =3* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via Rel-19 L3 fast Rx beam sweeping.

When UE supports concurrent GAPs, i.e., supports the following capability or capabilities’ combination:

-	concurrentMeasGap-r17, or

-	concurrentMeasGapsPreMG-r18, or

-	concurrentMeasGapsNCSG-r18,

Or when UE supports musim-GapPreference-r17 or both concurrent measurement GAPs and musim-GapPreference-r17 and UE concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

When UE supports [MUSIM-GapConfig-17] and the SMTC occasion of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

-	Otherwise, when the UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.	If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in table 9.2.5.1-1, table 9.2.5.1-2, table 9.2.5.1-3, table 9.2.5.1-4, table 9.2.5.1-5 and table 9.2.5.1-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in table 9.2.5.1-1, table 9.2.5.1-2, table 9.2.5.1-3, table 9.2.5.1-4, table 9.2.5.1-5, table 9.2.5.1-6, table 9.2.5.1-12, table 9.2.5.1-13 and table 9.2.5.1-14 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

-	When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement shall be conducted without gap and without interruption regardless of the NeedForGaps’ status reporting.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-4: Time period for PSS/SSS detection, CC with deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, CC with deactivated SCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.2.5.1-6: Time period for time index detection, CC with deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.2.5.1-7: Void

Table 9.2.5.1-8: Void

Table 9.2.5.1-9: Time period for PSS/SSS detection, CC with deactivated SCell (FR1), when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-10: Time period for time index detection, CC with deactivated SCell (FR1)，when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp)x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-11: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2) when SMTC period ≤ 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.5.1-12: Time period for PSS/SSS detection, CC with deactivated PSCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-13: Time period for PSS/SSS detection, CC with deactivated PSCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-14: Time period for time index detection, CC with deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-15: Time period for time index detection (Frequency range FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(MSSB_index_intra x Kp x SMTC period) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5 x MSSB_index_intra x Kp) x max(SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(MSSB_index_intra x Kp )x DRX cycle x CSSFintra |

Table 9.2.5.1-16: Void

Table 9.2.5.1-17: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, 5 x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320 ms] | max( 600 ms, ceil(M2 Note 2x 5) x [max(80 ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320 ms] | 5 x [DRX cycle x] CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.2.5.1-18: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320 ms] | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x [max(80 ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320 ms] | [ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x DRX cycle] x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. NOTE 3: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.2.5.1-19: Time period for time index detection for UE indicating no-gap-with-interruption (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, 3 x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320 ms] | max(120 ms, ceil (M2 Note 2 x 3) x [max(80 ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320 ms] | 3 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-20: Void

Table 9.2.5.1-21: Void

Table 9.2.5.1-22: Void

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, ceil(7 x Kp) x SMTC period) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(7 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.2.5.1-24: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5MHz |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-24a: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1), when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TSSB_time_index_intra_less_than_5MHz |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-25: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5MHz |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

<End of Change 11>

<Start of Change 12>

#### 9.2.6.2	Intra-frequency cell identification

When a measurement gap is provided or an activated Pre-MG is provided without any pre-MG status changed during the measurement period, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_processing ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra + TSSB_processing ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in tables 9.2.6.2-1, 9.2.6.2-2 or 9.2.6.2-9.

-	For UEs supporting Rel-19 L3 fast Rx beam sweeping, when the conditions in clause 3.6.20 are fulfilled, TSSB_processing = 2 ms; otherwise, TSSB_processing = 0.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_intra is given in table 9.2.6.2-9; otherwise, TPSS/SSS_sync_intra is given in table 9.2.6.2-2.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.6.2-3 or 9.2.6.2-10 (for FR2-2).

-	For UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to operate on a target cell with 12 PRB SSB in FR1, TSSB_time_index_intra_less_than_5Mhz is given in table 9.2.6.2-12.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.6.3-1 or 9.2.6.3-2.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.6.3-4; otherwise, T SSB_measurement_period_intra is given in table 9.2.6.3-2.

-	For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2-r17 is configured, the TSSB_measurement_period_intra given in table 9.2.6.3-4 (if SMTC ≤ 40ms) and table 9.2.6.3-2 (if SMTC > 40ms) shall apply for SCC.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with or the UE does not support concurrent GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps and per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP and MUSIM gap occasions within the window, and

Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [MUSIM-GapConfig-17] and the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target intra-frequency is expected.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36.

Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-1 power class 6, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36.

-	MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

For a UE that supports Rel-19 L3 fast Rx beam sweeping when the conditions in clause 3.6.20 are fulfilled and when highSpeedMeasFlagFR2-r17 is not configured, the following values shall apply for Mpss/sss_sync_with_gaps and Mmeas_period_with_gaps:

Mpss/sss_sync_with_gaps: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_with_gaps =3* Nreduced_Rx_BSF.

Mmeas_period_with_gaps: For a UE supporting FR2-1 power class 3, Mmeas_period_with_gaps =3* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via Rel-19 L3 fast Rx beam sweeping .

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in table 9.2.6.2-1, table 9.2.6.2-2, and table 9.2.6.2-3 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in table 9.2.6.2-1, table 9.2.6.2-2, and table 9.2.6.2-3 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.2-2: Time period for PSS/SSS detection (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, Mpss/sss_sync_with_gaps x KFR x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5x Mpss/sss_sync_with_gaps x KFR x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(Mpss/sss_sync_with_gaps x KFR x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.6.2-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.2-7: Void

Table 9.2.6.2-8: Void

Table 9.2.6.2-8: Void

Table 9.2.6.2-9: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2) when SMTC period ≤ 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, M1Note 2 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note2 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | max(600 ms, ceil(Mpss/sss_sync_with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil( Mpss/sss_sync_with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.6.2-10: Time period for time index detection (FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(MSSB_index_intra x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5 x MSSB_index_intra x Kgap) x max(MGRP, SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(MSSB_index_intra x Kgap)x DRX cycle x CSSFintra |

Table 9.2.6.2-11: Void

Table 9.2.6.2-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120 ms, 7 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 7 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Void NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

<End of Change 12>

<Start of Change 13>

### 9.2.7	Intra-frequency measurements with NCSG

#### 9.2.7.1	Intra-frequency cell identification

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_processing) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra + TSSB_processing) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.7.1-1, 9.2.7.1-2, 9.2.7.1-4 (deactivated Scell) or 9.2.7.1-5 (deactivated SCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.7.1-3 or 9.2.7.1-6 (deactivated SCell).

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.7.2-1, 9.2.7.2-2, 9.2.7.2-3, 9.2.7.2-4 (deactivated SCell) or 9.2.7.2-5 (deactivated SCell).

TSSB_processing: it is the time period used to process multiple beams received in one SMTC. TSSB_processing = 2 ms for UE supporting Rel-19 L3 fast Rx beam sweeping, when he conditions in clause 3.6.20 are fulfilled. Otherwise, TSSB_processing = 0

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in clause 9.1.5.3 for measurement conducted within NCSG.

KNCSG is the scaling factor for a SSB frequency layer to be measured within an associated NCSG pattern. KNCSG = 1 when the UE is not configured with concurrent GAPs. Otherwise, KNCSG = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAP and per-FR GAP within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated NCSG within the window W, including those overlapped with other GAP occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated NCSG within the window W after accounting for GAP collisions by applying the GAP collision rule in clauses 9.1.8.3, 9.1.12.3, and 9.1.13.3.

-	xRP = MGRP when configured GAP is MG, and xRP = VIRP when configured GAP is NCSG.

When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

Mpss/sss_sync_with_ncsg: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync with_ncsg=40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_ncsg =24. For a UE supporting FR2 power class 3, Mpss/sss_sync with_ncsg =24. For a UE supporting power class 4, Mpss/sss_sync with_ncsg =24

Mmeas_period_ with_ncsg: For a UE supporting power class 1 or 5, Mmeas_period_ with_ncsg =40. For a UE supporting power class 2, Mmeas_period_ with_ncsg =24. For a UE supporting power class 3, Mmeas_period_ with_ncsg =24. For a UE supporting power class 4, Mmeas_period with_ncsg =24.

For a UE that supports Rel-19 L3 fast Rx beam sweeping, when the conditions in clause 3.6.20 are fulfilled and when highSpeedMeasFlagFR2-r17 is not configured, the following values shall apply for Mpss/sss_sync_with_ncsg and Mmeas_period_with_ncsg:

Mpss/sss_sync_with_ncsg: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_with_ncsg =3* Nreduced_Rx_BSF.

Mmeas_period_with_ncsg: For a UE supporting FR2-1 power class 3, Mmeas_period_with_ncsg =3* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via Rel-19 L3 fast Rx beam sweeping.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with NCSG and smtc2 is partially overlapping with NCSG, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 9.2.7.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.7.1-2: Time period for PSS/SSS detection with NCSG (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_ncsg x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_ncsg x KNCSG) x max(VIRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Mpss/sss_sync_with_ncsg x KNCSG x max(VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-3: Time period for time index detection with NCSG (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, 3 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(M2Note 1x 3 x KNCSG) x max(VIRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 3 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.7.1-4: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | 5 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | 5 x KNCSG x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-5: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-6: Time period for time index detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | 3 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | 3 x KNCSG x max(measCycleSCell, VIRP,DRX cycle) x CSSFintra |

<End of Change 13>

<Start of Change 14>

### 9.3.4	Inter-frequency measurement with measurement gaps

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_processing) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ TSSB_processing) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1, table 9.3.4-2, table 9.3.4-5 when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17 and table 9.3.4-9 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.4-7 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-2 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_inter is given in table 9.3.4-9; otherwise, TPSS/SSS_sync_inter is given in table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3, table 9.3.4-6 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and table 9.3.4-10 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.4-8 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-4 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-4 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_inter is given in table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in table 9.3.5-2.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1, table 9.3.5-2, table 9.3.5-3 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and in table 9.3.5-5 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.5-4 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.5-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.5-2 applies for an inter-frequency carrier configured only by MCG.

TSSB_processing: the time period used to process multiple beams received in one SMTC. TSSB_processing = 2 ms for UE supporting Rel-19 L3 fast Rx beam sweeping, when the conditions in clause 3.6.20 are fulfilled. Otherwise, TSSB_processing = 0.

-	For UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to operate on a target cell with 12 PRB SSB in FR1, TSSB_time_index_intra_less_than_5Mhz is given in table 9.3.4-11.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, TSSB_measurement_period_inter is given in table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in table 9.3.5-2.

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60.

MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60.

For a UE that supports Rel-19 L3 fast Rx beam sweeping, when the conditions in clause 3.6.20 are fulfilled and when highSpeedMeasFlagFR2-r17 is not configured, the following values shall apply for Mpss/sss_sync_inter, MSSB_index_inter and Mmeas_period_inter:

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter =5* Nreduced_Rx_BSF.

MSSB_index_inter: For a UE supporting FR2-1 power class 3, MSSB_index_inter = 3* Nreduced_Rx_BSF.

Mmeas_period_inter: For a UE supporting FR2-1 power class 3, Mmeas_period_inter =5* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via Rel-19 L3 fast Rx beam sweeping.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP occasions and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Kgap is only applicable for UE supporting concurrent GAPs or MUSIM gaps. When concurrent GAPs or MUSIM gaps are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-frequency is expected.

Table 9.3.4-1: Time period for PSS/SSS detection (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8 * Kgap) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(8*1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-2: Time period for PSS/SSS detection, (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Kgap Mpss/sss_sync_inter x KFR) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter x KFR) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mpss/sss_sync_inter x KFR) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 4: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.3.4-3: Time period for time index detection (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, Ceil(3 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(3 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-4: Time period for time index detection (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap MSSB_index_inter) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-5: Time period for PSS/SSS detection when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600 ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160 ms | max(600 ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160 ms < DRX cycle ≤ 320 ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320 ms | N4 x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: N4=6 if SMTC periodicity > 40 ms, otherwise N4=5 |  |

Table 9.3.4-6: Time period for time index detection when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 M2 NOTE3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

Table 9.3.4-7: Time period for PSS/SSS detection when the inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Kgap Mpss/sss_sync_inter) Max(MGRP , measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mpss/sss_sync_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-8: Time period for time index detection when inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap MSSB_index_inter) Max(MGRP, measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-9: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2-1) when SMTC period ≤40ms

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600 ms, M1Note 3 x Kgap x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note 3 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | max(600 ms, ceil(Mpss/sss_sync_with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320 ms | Ceil( Mpss/sss_sync_with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.4-10: Time period for time index detection when when highSpeedMeasFlagFR2-r17 is configured (Frequency range FR2-1) when SMTC period <= 40 ms

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap , M1Note 3) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 80 ms | Max(200 ms, Ceil(1.5 * Kgap M1Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.4-11: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter_less_than_5Mhz |
| --- | --- |
| No DRX | Max(120 ms, Ceil(6 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120ms, Ceil(6 M2 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(6* Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. |  |

#### 9.3.4.1	Void

#### 9.3.4.2	Void

<End of Change 14>

<Start of Change 15>

### 9.3.10	Inter-frequency measurement with NCSG

#### 9.3.10.1	Inter-frequency cell identification

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured. Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_processing) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter + TSSB_processing) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.10.1-1 and table 9.3.10.1-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.10.1-3 and table 9.3.10.1-4.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.10.2-1 and table 9.3.10.2-2.

TSSB_processing: it is the time period used to process multiple beams received in one SMTC. TSSB_processing = 2 ms for UE supporting Rel-19 L3 fast Rx beam sweeping when the conditions in clause 3.6.20 are fulfilled. Otherwise, TSSB_processing = 0.

Mpss/sss_sync_inter: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync_inter = 40 samples.

MSSB_index_inter: For a UE supporting FR2 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2 power class 4, MSSB_index_inter = 24 samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1 or 5, Mmeas_period_inter =64 samples. For a UE supporting FR2 power class 2, Mmeas_period_inter=40 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =40 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 40 samples.

For a UE that supports Rel-19 L3 fast Rx beam sweeping, when the conditions in clause 3.6.20 are fulfilled and when highSpeedMeasFlagFR2-r17 is not configured, the following values shall apply for Mpss/sss_sync_inter, MSSB_index_inter and Mmeas_period_inter:

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter =5* Nreduced_Rx_BSF.

MSSB_index_inter: For a UE supporting FR2-1 power class 3, MSSB_index_inter = 3* Nreduced_Rx_BSF.

Mmeas_period_inter: For a UE supporting FR2-1 power class 3, Mmeas_period_inter =5* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced Rx beam sweeping factor reported by UE via Rel-19 L3 fast Rx beam sweeping.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in clause 9.1.5.3 for measurement conducted within NCSG.

-	KNCSG is the scaling factor for a SSB frequency layer to be measured within an associated NCSG pattern. KNCSG = 1 when the UE is not configured with concurrent GAPs. Otherwise, KNCSG = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs and per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated NCSG within the window W, including those overlapped with other GAP occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated NCSG within the window W after accounting for GAP collisions by applying the GAP collision rule in clauses 9.1.8.3, 9.1.12.3, and 9.1.13.3.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

Table 9.3.10.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, 8 KNCSG Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(8*1.5 KNCSG) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | 8 KNCSG DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.10.1-2: Time period for PSS/SSS detection with NCSG (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Mpss/sss_sync_inter KNCSG Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, (1.5 Mpss/sss_sync_inter KNCSG) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Mpss/sss_sync_inter KNCSG DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.10.1-3: Time period for time index detection with NCSG (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, 3 KNCSG Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 1.5 KNCSG) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | 3 KNCSG DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.10.1-4: Time period for time index detection with NCSG (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, MSSB_index_inter KNCSG Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, (1.5 MSSB_index_inter KNCSG) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | MSSB_index_inter KNCSG DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

<End of Change 15>