3GPP TSG RAN #109			RP-252470

Beijing, China, Sept. 15-18, 2025

Agenda Item:	8.2.1.1

Source: 	LG Electronics

Title: 	Discussion on 6G deployment scenarios

Document for:	Discussion and decision

Introduction

We discuss the recommendation on the deployment scenarios on 6GR in this contribution.

Discussion

- 2.1	Urban grid and Highway scenario

Considering the expanded commercial needs and deployments of autonomous driving, the automotive-related applications are one of the most important and promising motivation towards 6G era. From communication perspective, 6GR can not only support the techniques for low latency and high reliability communication, but also support rich services providing infotainment experience to the driver based on e.g. XR-based immersive communication. From sensing perspective, 6GR should be able to provide safe driving through situational awareness to avoid collisions, as well as to make driving more efficient in both energy consumption and traffic management point of view, based on sensing.

With these reasons, Urban grid and Highway scenario that supports automotive-related deployment scenarios should be considered for 6G deployment scenario. We suggest removing the brackets in Section 4.11 and 4.12 in TR 38.914, and study the relevant deployment scenarios.

Proposal 1: For 6GR deployment scenarios, urban grid and highway scenario should be studied. Remove the bracket in [Urban grid] and [Highway Scenario] in Section 4.11 and 4.12 in TR 38.914.

As a starting point of discussion for Urban grid and Highway scenario, we can consider the relevant deployment scenarios and parameters from TR 37.885 [3]. We propose to include the following tables for Urban grid and Highway scenario in Section 4.11 and 4.12 in TR 38.914.

Table 1: Attributes for Urban grid

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro BS: 4GHz Micro BS: 4GHz |
| Aggregated system bandwidth | 200MHz |
| Layout | Baseline: Macro only (with the road configuration in Figure 6.1.9-1 in [1] and BS placement as depicted in Figure A.1.3-1 in [2]) |
| ISD | Inter Macro: 500m |
| BS antenna elements | Macro BS: antenna height 25m Micro BS: antenna height 5m |
| UE antenna elements | - Type 1 vehicle (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters - Type 2 vehicle (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters - Type 3 vehicle (truck/bus): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters |
| User distribution and UE speed | - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. - Vehicle speed for the urban grid scenario in [3] |

Table 2: Attributes for Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro BS: 2GHz, 4GHz Micro BS: 4GHz |
| Aggregated system bandwidth | 200MHz |
| Layout | Baseline: Macro only (straight line BS placement with Road configuration in [2]) |
| ISD | 1732m 500m |
| BS antenna elements | Macro BS: - 35m for ISD 1732m - 25m for ISD 500m Micro BS: 5m |
| UE antenna elements | - Type 1 vehicle (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters - Type 2 vehicle (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters - Type 3 vehicle (truck/bus): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters |
| User distribution and UE speed | - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. - Vehicle speed for the highway scenario in [3] |

Proposal 2: The attributes for Urban grid and Highway scenario in Table 1 and Table 2 are included in Section 4.11 and 4.12 in TR 38.914.

The commercial automotive radar systems operate in 76 GHz to 81 GHz frequency band, which provides higher accuracy with larger bandwidth, enables smaller size device implementation, and allows radar signal transmission of higher power level. We can use available 3GPP licensed spectrum for 3GPP ISAC, but can also consider to use the regulated spectrum for automotive radar application, which may help easier deployment of 3GPP ISAC in the automotive industry.

Proposal 3: Automotive radar spectrum (76-81GHz) as well as 3GPP licensed spectrum can be considered for 3GPP ISAC system in automotive radar application.

- 2.2	Non-terrestrial Network (NTN)

When discussing and defining parameters for 6G NTN deployment scenarios in [4], we think that the following 5G NTN-related content of TR 38.913 [1] in Table 3 can be considered as a starting point. It would also be desirable to consider satellite payload parameters such as the absolute number of simultaneously active beams or the ratio of simultaneously active beams, which were additionally introduced in NR-NTN.

Table 3: 5G NTN Deployment Scenarios in TR 38.913 [1]

### 6.1.12 Satellite extension to Terrestrial This deployment scenario is defined to allow for the provision of services for those areas where the terrestrial service is not available and also for those services that can be more efficiently supported by the satellite systems such as broadcasting service. Satellite acts as a fill-in especially on roadways and rural areas where the terrestrial service isn’t available. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services. Some of its attributes are listed in Table 6.1.12-1. Table 6.1.12-1: Examples for Satellite Deployment Attributes; Deployment-1; Deployment-2; Deployment-3. Carrier Frequency NOTE1; Around 1.5 or 2 GHz for both DL and UL; Around 20 GHz for DL Around 30 GHz for UL; Around 40 or 50 GHz. Duplexing; FDD; FDD; FDD. Satellite architecture; Bent-pipe NOTE2; Bent-pipe, On-Board Processing; Bent-pipe, On-Board Processing. Typical satellite system positioning in the 5G architecture; Access network; Backhaul network; Backhaul network. System Bandwidth (DL + UL); Up to 2*10 MHz; Up to 2*250 MHz; Up to 2 * 1000 MHz. Satellite Orbit; GEO, LEO; LEO, MEO, GEO; LEO, MEO, GEO. UE Distribution; 100% Outdoors; 100% Outdoors; 100% Outdoors. UE Mobility; Fixed, Portable, Mobile NOTE3; Fixed, Portable, Mobile; Fixed, Portable, Mobile. NOTE1: The carrier frequencies noted here are for evaluation purpose only, satellites are deployed in wide range of frequency bands including L band (1-2GHz), S band (2-4GHz), C band (3.4-6.725 GHz), Ku band (10.7-14.8 GHz), Ka band (17.3-21.2 GHz, 27.0-31.0 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz) and more. Here the around xGHz is used to denote the carrier frequency close to xGHz rather than used as band proxy to denote a band range. NOTE2: Bent pipe refers to the architecure where the satellite transponders are transparent—only amplify and change frequency but preserve the waveform. On Board Processing satellite transponders incorperate regeneration — including modulating and coding the waveform. NOTE3: Mobile consitutes of both hand-helds and other moving platform receivers such as automobiles, ships, planes etc. Currently the hand-helds are limited to L and S bands but the research is ongoing to support higher bands.

At the RAN1#122 meeting, the SCS value(s) ​​supported for each frequency band/range were discussed under the AI of 6GR Frame Structure in [5], and the following agreements were reached. Here, as highlighted in yellow below, the frequency bands/ranges supported for 6GR will be subject to the plenary decision.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

From this perspective, our view is that, since 6GR targets the harmonization of TN and NTN, the frequency bands/ranges defined for 5G NTN should at least be included for 6G NTN deployment scenarios. Specifically, for example, the NR-NTN FR1/2 bands defined in TS 38.101-5 [6] are as follows:

Table 4: NR-NTN FR1/2 Bands in TS 38.101-5 [6]

Table 5.2.2-1: NTN satellite bands in FR1-NTN NTN satellite operating band1; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 – 1626.5 MHz; 2483.5 – 2500 MHz; FDD. n252; 2000 – 2020 MHz; 2180 – 2200 MHz; FDD. n248; 14000 MHz - 14500 MHz; 10700 MHz – 127502,3 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 127502,3 MHz; FDD. NOTE 1: NTN satellite bands are numbered in descending order from n256. NOTE 2: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 3: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n5121; 27500 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5112; 28350 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5103; 27500 MHz - 28350 MHz; 17300 MHz - 20200 MHz; FDD. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504,5 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504,5 MHz; FDD. NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT..

VLEO-based NTN is one of the emerging deployment scenarios that should be focused on in 6GR, which can enable support of services with low latency requirements due to reduced RTT values. In this case, it would be necessary to determine parameters such as altitude, maximum RTT, maximum Doppler shift, and maximum Doppler rate, etc. As altitude decreases, overall latency will decrease, but due to the rapid movement of satellites and the rotation of the Earth, a number of satellites are needed to ensure continuity of service. If the altitude is less than 300km, then the satellite may decay quickly due to atmospheric drag. For example, assuming the altitude of VLEO is 300 km, the RTT is 4msec when the satellite with transparent payload is located at the nadir point.

Proposal 4: The 5G NTN deployment scenarios of TR 38.913 [1] in Table 3 can be considered as a starting point for 6G NTN deployment scenarios. Additionally, in 6G NTN deployment scenarios, the frequency ranges/bands defined in 5G NTN (e.g., NR-NTN FR1/2 bands of TS 38.101-5 [6] in Table 4) are included at least, and VLEO is considered as one of the primary satellite types.

Summary

In this contribution, we discussed the deployment scenarios for 6GR, and the following proposals were made:

Proposal 1: For 6GR deployment scenarios, urban grid and highway scenario should be studied. Remove the bracket in [Urban grid] and [Highway Scenario] in Section 4.11 and 4.12 in TR 38.914.

Proposal 2: The attributes for Urban grid and Highway scenario in Table 1 and Table 2 are included in Section 4.11 and 4.12 in TR 38.914.

Proposal 3: Automotive radar spectrum (76-81GHz) as well as 3GPP licensed spectrum can be considered for 3GPP ISAC system in automotive radar application.

Proposal 4: The 5G NTN deployment scenarios of TR 38.913 [1] in Table 3 can be considered as a starting point for 6G NTN deployment scenarios. Additionally, in 6G NTN deployment scenarios, the frequency ranges/bands defined in 5G NTN (e.g., NR-NTN FR1/2 bands of TS 38.101-5 [6] in Table 4) are included at least, and VLEO is considered as one of the primary satellite types.

Reference

[1]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[2]	3GPP TR 36.885: "Study on LTE-based V2X services".

[3]	3GPP TR 37.885: "Study on evaluation methodology of new V2X use cases for LTE and NR".

[4]	RP-251395, “Revised SID: Study on 6G Scenarios and requirements,” CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

[5]	RP-251881, “New SID: Study on 6G Radio,” NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

[6]	3GPP TS 38.101-5: "User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements NR"