3GPP TSG RAN Meeting #109	RP-252747

Beijing, China, September 15th -18th, 2025

Agenda item:	10.7

Source:	Nordic Semiconductor, Thales, QC, Sony, Iridium, Aalyria

Title:	Extended guard band for satellite bands

Document for:	Discussion

# Introduction

At the WG RAN4 #116 an LS from ETSI was received [1] asking RAN4 to provide opinion on benefits of ETSI’s Nominated guard band concept. RAN4 replied in [2] the following:

3GPP RAN4 discussed the ETSI SES(25)000071/R4-2509023 task and would like to report the following feedback to TC SES: - The nominated channel bandwidth concept is considered to be useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting ETSI SEM requirements. The nominated channel bandwidth concept can be adopted to the 3GPP specifications with modifications, such as restriction on the possible extended guard-band size(s) and fixed symmetric extended guard-band values. - The utilization of the above concept would be under network’s control to use it depending on the deployment. - 3GPP RAN WG4 plans to work on the design of this concept aiming at a general framework that would be applicable to other satellite bands.

It is obvious that RAN4 cannot work on the general framework without approval from RAN plenary. In this contribution we clarify, on high level, the amount of work required to introduce general framework of extended guard band for NTN IoT satellite bands and request RAN to approve small work package/task as part of R19 TEI.

# Discussion

## On benefits of extend guard band for satellite bands

The main motivation to introduce extended guard bands are the very strict requirements of ETSI in MSS bands. The requirements for spectral emission mask resulted in very large A-MPR values in 3GPP (see Table 6.2B.3.2-2 for NS_05 in 1610-1626.5 part of L-band, i.e. NS applicable to b249 and b254), making NB-IoT system operation infeasible in practice in those bands. While RAN4 has been intensively working on high power UEs for NTN, and even 5dB of A-MPR for PC3 can result in link budget of UL being insufficient for operation in the band.

Observation-1: Strict ETSI requirements in MSS bands resulted in very high A-MPRs and make 3GPP NB-IoT system deployment infeasible in such bands in practice.

Table 6.2B.3.2-2: A-MPR for "NS_05N" with 15kHz SCS for Power Class 3

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤15 |  | ≤10.5 | ≤8 | ≤2 | ≤1.5 | ≤1.5 | ≤2 | ≤8 | ≤10.5 | ≤15 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |
| A-MPR (dB) | ≤ 11 |  |  |  | ≤ 3.5 |  |  |  | ≤ 11 |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 8.5 |  |  |  |  |  | ≤ 8.5 |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 4.5 |  |  |  |  |  |  |  |  |  |  |  |

While strict requirements exist in ETSI, ETSI also allows a UE to declare a Nominated bandwidth being as much as 5x larger than channel bandwidth defined in 3GPP, this allowing UEs to operate in legacy proprietary NTN systems without A-MPRs. As LS reply from 3GPP to ETSI states, Nominated bandwidth can be adopted with modification as framework of extended guard band in 3GPP. Except in 3GPP:

- extended guard bands would not be declared by UE, but under control of network.

- guard bands would be symmetric.

Observation-2: Nominated bandwidth of ETSI can be adopted by 3GPP as general extended guard band concept and is essential to reduce A-MPRs to 0dB. Nominated bandwidth concept can be adopted in 3GPP with the following modifications:

- in 3GPP extended guard bands would not be declared by a UE, but would be under control of network, and

- guard bands would be symmetric.

## On open issues of extended guard band after RAN4#116

RAN4#116 progressed already significantly with the design of extended guard bands. Under thread [315 – IoT NTN phase 3] see WF in [3], the following was agreed:

#### Issue 2-1: Regarding meeting the ETSI SEM requirement for NB-IoT/NR NTN, whether the “nominated BW (Bn) concept” (e.g., guard-band, extended BW) is useful from 3GPP perspective. - Agreement: - The nominated channel bandwidth concept is considered to be useful to minimize NB-IoT/NR NTN UL power backoff (i.e., A-MPR) for meeting ETSI SEM requirements. The nominated channel bandwidth concept can be adopted to the 3GPP specifications with modifications, such as restriction on the possible extended guard-band size(s) and fixed symmetric extended guard-band values. - The utilization of the above concept would be under network's control to use it depending on the deployment. - 3GPP RAN WG4 aims to work on the design of this concept as a general framework. - Introduction of the concept is considered earliest in Release 19. #### Issue 2-2: If nominated BW concept (e.g., extended guard-band/extended BW) can be applicable to NB-IoT/NR NTN, then how to specify it. - Agreement: - For meeting ETSI SEM requirements, 3GPP can use fixed extended guard-band value such as 400kHz for meeting nominated BW concept for FR1 NB-IoT NTN band, e.g., apply additional 100 kHz guard band on both sides. - FFS on NR-NTN. #### Issue 2-3: Introduction of extended guard bands for NTN IOT. - Agreement: - Introduce the following UE capability to be added to Rel-19 capability feature list based on RAN guidance: Features; Index; Feature group; Components; Prerequisite feature groups; Field name in TS 36.331; Parent IE in TS 36.331; Need of FDD/TDD differentiation; Note; Mandatory /Optional. TBD; Extended guard band for A-MPR reduction; 1. Support of extended guard-band feature; [NTN-extendedGuardBand-A-MPR-r19]; [FeatureSetUplink-v1900]; TBD; The feature can be supported in below scenarios: RAN4 intends for the NTN IoT bands.; Optional with capability signalling.

Further details were discussed under thread [316 – IoT NTN TDD] see WF in [4]

Agreement: Confirm the following statements from [116][315] are applicable to 249: - The nominated BW concept will be applied to frequency bands b249 and b254. - For meeting ETSI SEM requirements, 3GPP can use fixed GB or extended BW such as 400 kHz for meeting nominated BW concept for FR1 NB-IoT NTN band, e.g., apply additional 100 kHz guard band on both sides. Agreement: Confirm the general principles from [116][315] are applicable to 249: - A general framework to enable extended guard-band/channel for NTN IoT - The framework is applicable to any NTN IOT band - Extended guard-band/channel size limited to [one or two] options at most, e.g., 400kHz and/or 300kHz extended channel bandwidth for NTN IOT - Extended guard-band/channel is symmetrical to the transmission channel bandwidth Agreement: Confirm the signalling principles from [116][315] are applicable to 249: - Signalling indicating that SAN uses extended guard band for the NTN IoT band - Option 1: a new parameter indicating value of extended guard band. - Option 2: introduce one new NS value for combination of legacy NS and extended guard band - Option 3: introduce one additional NS to indicate extended guard band, UE receives two NS: one legacy NS and one NS indicating extended guard band - Other - UE capability signalling, where a UE indicates support of extended guard band feature ([per UE/per band]); if capability is not indicated, then the UE does not support the feature (or doesn’t need to use it). Agreement: Confirm that general behaviour captured with the following table is applicable to 249: UE extended guardband capability; Network configuration. No extended guardband; Is extended guardband. UE capabilty reported; UE applies defined AMPR if needed; No AMPR is applied. No UE capability reported; UE cannot understand SAN guard-band configuration, and a UE applies A-MPR, if needed. Agreement: Confirm that the following specific behaviour (using 100 kHz extended guard band as an example) is applicable to 249: - If extended guard band is signalled as 100 kHz and UE indicates capability NTN-extendedGuardBand-A-MPR -r19, then a UE follows “extended” SEM with extended guard band and A-MPR=0 - If extended guard band is not signalled by the network, then a UE follows legacy SEM without extended guard band and uses, if needed, the band specific A-MPRs - If UE does not indicate extendedGuardBand-A-MPR -r19, then a UE follows legacy SEM without extended guard band and uses, if needed, the band specific A-MPRs

Above agreements can be summarized with the following observation:

Observation-3:  Concept of extended bandwidth does not require much specification effort anymore:

- Capability is tentatively agreed and missing only a Work item under which it could be introduced in R19.

- Band 249 and 254 are the bands which would benefit the most from introduction of general extended guard band concept, other NTN IoT and NR bands could benefit as well.

- Network signalling is open in RAN4, however 3 options were identified in RAN4#116 and one option could be down-selected in RAN4#116bis.

- Behaviour is simple and does not require too much specification changes or further work.

Therefore, we have the following proposal to RAN#109 plenary to consider creating a RAN task for the extended guard band feature with the following content:

Proposal: In RAN#109, RAN plenary to create a RAN task and allow WGs to work on the following objectives in Q4 2025:

- Add the already agreed tentative capability of extended guard band in RAN#109 to R19 feature list. [RAN, RAN2]

- Allow RAN4 to conclude discussion on extended guard band signalling in RAN4#116b and allow signalling to be included in RAN#132 to R19 ASN.1. [RAN4, RAN2]

- Allow RAN4 to specify the behaviour in below table to be generally applicable to all NTN IoT bands for UEs in R19 and onwards. [RAN4]

| UE extended guard band capability | Network configuration |  |
| --- | --- | --- |
|  | No extended guard band | Is extended guard band |
| UE capability reported | UE applies defined AMPR if needed | No AMPR is applied |
| No UE capability reported | UE cannot understand SAN guard-band configuration, and a UE applies A-MPR, if needed |  |

- Note: Remaining work may be handled under maintenance of [FFS:  IoT_NTN_TDD or IoT_NTN_Ph3]

# Conclusions

In this contribution, we discussed introduction of extended guard band concept in 3GPP.  The following is observed and proposed:

Observation-1: Strict ETSI requirements in MSS bands resulted in very high A-MPRs and make 3GPP NB-IoT system deployment infeasible in such bands in practice.

Observation-2: Nominated bandwidth of ETSI can be adopted by 3GPP as general extended guard band concept and is essential to reduce A-MPRs to 0dB. Nominated bandwidth concept can be adopted in 3GPP with the following modifications:

- in 3GPP extended guard bands would not be declared by a UE, but would be under control of network, and

- guard bands would be symmetric.

Observation-3:  Concept of extended bandwidth does not require much specification effort anymore:

- Capability is tentatively agreed and missing only a Work item under which it could be introduced in R19.

- Band 249 and 254 are the bands which would benefit the most from introduction of general extended guard band concept, other NTN IoT and NR bands could benefit as well.

- Network signalling is open in RAN4, however 3 options were identified in RAN4#116 and one option could be down-selected in RAN4#116bis.

- Behaviour is simple and does not require too much specification changes or further work.

Proposal: In RAN#109, RAN plenary to create a RAN task and allow WGs to work on the following objectives in Q4 2025:

- Add the already agreed tentative capability of extended guard band in RAN#109 to R19 feature list. [RAN, RAN2]

- Allow RAN4 to conclude discussion on extended guard band signalling in RAN4#116b and allow signalling to be included in RAN#132 to R19 ASN.1. [RAN4, RAN2]

- Allow RAN4 to specify the behaviour in below table to be generally applicable to all NTN IoT bands for UEs in R19 and onwards. [RAN4]

| UE extended guard band capability | Network configuration |  |
| --- | --- | --- |
|  | No extended guard band | Is extended guard band |
| UE capability reported | UE applies defined AMPR if needed | No AMPR is applied |
| No UE capability reported | UE cannot understand SAN guard-band configuration, and a UE applies A-MPR, if needed |  |

- Note: Remaining work may be handled under maintenance of [FFS:  IoT_NTN_TDD or IoT_NTN_Ph3]

# References

- R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI

- R4-2512539, LS reply to ETSI TC SES, RAN4

- R4-2512688, Way Forward for [116][315] IoT_NTN_Ph3, Moderator (MediaTek)

- R4-2512666, Way Forward for [116][316] IoT_NTN_TDD_UE_SAN_RF, Moderator (Thales)