3GPP TSG-RAN Meeting #110									       RP-253079

Baltimore, USA, December 8-11, 2025

Agenda Item:		8.2.1.2

Source:		Deutsche Telekom, Vodafone, Orange, Telecom Italia, Turkcell, Odido Spark NZ, BT, Bouygues Telecom, Telefonica, Telia Company, SK Telecom, KPN, Rakuten Mobile, CK Hutchison, Telenor, Boost Mobile Network, Jio Platforms

Title:		TP on Requirements for architecture and migration in 6G for TR 38.914

Document for:		Discussion and decision

## 1	Introduction

The RAN P SI on “Study on 6G scenarios and requirements” has been approved in December 2024 [1].

At the RAN#108 a multiple operator contribution in [2] defined key requirements for 6G for inclusion into the newly created TR 38.914 [3] but has not been discussed nor any agreements have been taken as basis for inclusion to the TR.

With this contribution the cosigning operators list their key requirements for 6G on the topic of architecture and migration for inclusion into the TR 38.914.

## 2	Text Proposal

*** START TEXT PROPOSAL ***

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

  - An open Lower Layer Split (LLS) between the RU and the DU shall be supported and standardised outside 3GPP

  - An open Xn-like interface between the 6GNBs shall be supported

  - As part of the 3GPP Study Item the need for open F1-like interface between the DU and the CU shall be studied

  - As part of the 3GPP Study Item the need for open E1-like interface between the CU-CP and the CU-UP shall be studied

  - An open N2-like interface between the 6G RAN and the CN for 6G shall be supported

  - An open N3-like interface between the 6G RAN and the CN for 6G shall be supported

  - An open multi-vendor interface from 6G RAN to the OMC (like entity) shall be supported

  - The 6G RAN shall support Open Application Programming Interfaces (“APIs”)

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

- The 6G RAN architecture shall be software based and optimised, wherever possible

- The 6G RAT & RAN architecture shall support a streamlined protocol for each layer and procedure

- The 6G RAN architecture shall allow cloud implementations, allowing independent selection of HW and SW for digital elements

- The 6G RAN architecture part of the 3GPP Study Item shall study protocols optimised for cloud native implementations

- The 6G RAT & RAN architecture shall support a smooth migration path from NR connected to 5GCN

- The 6G RAT & RAN architecture shall work without DC (Dual Connectivity) with NR

- The 6G RAT shall support Multi-RAT Spectrum Sharing (“MRSS”), including Carrier Aggregation of MRSS carriers, on new and existing Radio Access Network HW with minimal radio overhead and without the need of a mandatory HW refresh

- The 6G RAT shall support a non-standalone mode (DC between NR and 6G RAT) with NR as the Primary Cell (“N6DC”)

  - NOTE: The final decision if this mode (single solution) is standardised shall be based on the 6G RAN WG and SA WG SIs results

- The 6G RAN architecture shall generally keep the 5G functional split (e.g. Authentication, Charging, etc in the CN, while Handover decisions, etc remain in the RAN), while optimising functions which have (partly) a duplication in today’s architecture (e.g. paging)

*** END TEXT PROPOSAL ***

## 3	Proposal

It is proposed by the cosigning operators to agree the above TP to be captured in the new TR 38.914 v0.2.0

## References

[1] 	RP-243327 “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#107

[2] 	RP-251682 “Key operator requirements for 6G”, Deutsche Telekom, Vodafone, Orange, Telecom Italia, Telefonica, BT, Bouygues Telecom, Spark NZ, Odido, Turkcell, Telenor, Telus, CKH IOD UK, Jio Platforms, Reliance Jio, KPN, Boost Mobile Network, 3GPP RAN#108

[3]	RP-252882 “3GPP TR 38.914 V0.2.0”, 3GPP RAN#109