3GPP TSG RAN Meeting #110					RP-253096

Baltimore, USA, December 15-18, 2025

Agenda Item:	8.2.1.3.1           Operational requirements: Spectrum

Source:	Thales, Iridium, Viasat, ST Engineering iDirect, Novamint, Fraunhofer IIS, Fraunhofer HHI, Eutelsat, EchoStar, Gatehouse Satcom, Airbus, SES, ESA, Hispasat, Sateliot

Title:	TP on Spectrum for NTN

Document type/for: Discussion/Decision

# 1	Introduction

This document aims at providing requirements for NTN in clause 5.3.1	Spectrum of the TR 38.914.

# 2	Discussion

6GR should be designed to operate in the same frequency bands as in 5G.

In addition, it should be able to operate in new bands such as C band (3.8-4.2 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz).

# 3 Text proposal for TR 38.914

Based on the above discussion we propose the following text proposal.

====================Start of the changes==================

## 5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

### 5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

5.3.1.x NTN

The service link of non-terrestrial networks, refers to the radio link between satellite(s) and User Equipment, and will use the 3GPP defined 6G radio interface.

This service may operate in satellite service allocated frequency bands which can be paired (Downlink and Uplink in separate bands) or unpaired (Downlink and Uplink in the same band). While paired bands can apply to all orbits, unpaired bands are typically restricted to the Low Earth Orbits.

Satellite service links shall be able to operate in the following frequency bands:

- L band (1-2GHz), S band (2-4GHz), C band (3.8-4.2 GHz),

- Ku band (10.7-14.5 GHz), Ka band (17.3-21.2 GHz, 27.0-30.0 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz).

The 6G Radio shall be able to support

- channel bandwidth between [~1] MHz and 800 MHz (including native support of 5, 10, 20, 40, 50, 100, 200 and 400 MHz etc.)

- FDD (Frequency Division Duplexing), and TDD (Time Division Duplexing) as well as Half Duplex Frequency Division Duplexing (HD-FDD) mode at UE level

- Full-duplex and Half-duplex FDD at Network side (Satellite payload)

- TN and NTN spectrum sharing.

==================End of the changes========================

# 4	Conclusion

Proposal: Add the text proposal in clauses 5.3.1 of the TR 38.914.