3GPP TSG RAN Meeting #110	RP-253120

Baltimore, USA, December 8-11, 2025

Source: 	CMCC

Title:	Discussion on 6G RAN Architecture

Agenda item:	8.2.1.2

Document for:	Discussion

- Introduction

After several RAN plenary meetings’ discussion, TR 38.914 captures the following requirements for 6G architecture and migration [1]:

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

Besides, in the latest RAN3 meeting, some agreements were achieved for overall RAN architecture [2]:

## 5.3 Overall RAN architecture Editor’s note: Figure 5.3-1 and title aim at providing a single framework based on a stand-alone architecture for existing and new services, taking into account legacy communication services, and is subject to study progress including new services. The names of the logical nodes and the interfaces are FFS. The 6G RAN consists of 6G RAN logical nodes providing the 6G U-plane and C-plane protocol terminations towards the UE. The 6G RAN logical nodes may be interconnected with each other by means of a 6G RAN node to 6G RAN node interface. The 6G RAN logical nodes are connected to the CN for 6G by means of an interface between 6G RAN node and CN for 6G. The 6G-RAN logical nodes may be connected to one or more CN(s) for 6G. The 6G RAN architecture is illustrated in Figure 5.3-1. Figure 5.3-1 The RAN Architecture

In this contribution we provide our consideration on the requirements that should be supported by 6G RAN.

- Discussion

Based on the 6G SID, 6G will support some new services/scenarios, like AI and sensing, therefore, the related requirements should be included, except for those already captured in [1]. What’s more, we also identify some issues can be further optimized in 6G based on 5G lessons. Therefore, we think at least the following requirements should be addressed in 6GR.

- Data collection function: Based on the progress of data collection/transfer in RAN2 and RAN3, we believe that 6G RAN architecture shall support data collection terminated in RAN side, share the collected data to other entities, as well as request and use the data collected from other entities. In our understanding, data terminated in RAN means that the data collection is triggered by RAN, and RAN decides which data will be collected and send the data collection configuration to the UE. Furthermore, we believe that a new and specific RAN data collection function can avoid duplicated data collection and allow reusability of collected data for different use cases, and it is beneficial for improving efficiency since it allows unified interaction with other entities compared to today independent deployment within the base station. The new and specific RAN data collection function is applicable for the scenarios where the data is used for multiple cells or base stations, such as AI/ML based mobility. On the other hand, the latency for data collection by a new and specific RAN data collection function is lower than data collection by OAM. The details can be found in our contribution [3].

Figure.1 Overall architecture of AI Data Collection Function in 6G RAN

- Sensing: Sensing is identified as an objective of 6GR. Considering the enormous deployment areas, 6G sensing should take data privacy, service latency and deployment flexibility into consideration. To achieve these requirements, Sensing Function Node (SF Node) deployed in RAN (illustrated in Figure.2) should be considered in 6G network architecture. To further improve sensing performance, the impact on RAN architecture of multiple Base Stations cooperation (including integrated AI and sensing) should be considered [4].

Figure.2 Proposed Radio Access Network architecture for sensing in 6G RAN

- Cooperative Communication: To improve the spectral efficiency and achieve consistent user experience, 6G should support multi-node access, cooperative transmission, and flexible handover. To fulfill these requirements, the 6G RAN architecture should incorporate the following aspects:

- High-PHY/Low-PHY functional split, with High-PHY deployed at the DU to perform centralized decision-making, while Low-PHY carrying out distributed execution;

- CU/DU splitting, which should be retained to ensure scalability and flexibility and enable coordination for performance features, load management, and real-time performance optimization in cooperative communication;

- Inter-DU interface, which should be open to support inter-site coordination across DUs and enable real-time information exchange at L1/L2;

- Fusion processing unit, which should be introduced at layers such as High-PHY and MAC to support multi-point data fusion processing.

Figure.3 RAN architecture for cooperative communication in 6G

- Service-based interface (SBI): Replacing the pre-established peer-to-peer (p2p) signalling interface between the RAN and the CN with an SBI will help to improve interaction efficiency. On one hand, the RAN can more directly expose information to CN NFs or third-party applications. On the other hand, new capabilities of the RAN such as computing, AI, and sensing can adopt a unified interaction model with corresponding capabilities in the CN, enabling efficient coordination of these new features. For the RAN service definition, (some) NGAP procedures can be defined as services first before gradually considering the servitization of other interfaces such as XnAP, E1AP and F1AP, details can be found in [5].

Figure.4 Service-based interface

- Network Digital twin (NDT): NDT enables proactive network state prediction and policy pre-verification, thereby helping to achieve a higher level of network autonomy. It needs to be introduced into 6G RAN due to the following reasons:

  - NDT has been introduced in SA5 to support autonomous network management and operation. However, the NDT deployed in MnS(Management Service) cannot meet the near real-time predicting and pre-validation requirements of RAN.

  - Self-organizing network (SON) cannot pre-validate configurations, and therefore cannot guarantee that configuration, optimization, or fault-related parameters will not have negative impacts.

Therefore, NDT in 6G RAN needs to be supported from 6G Day 1, as illustrated in Figure.5. Based on dynamic network mapping and modeling, NDT enables near real-time network prediction and policy pre-validation (e.g., AI-driven policies) in RAN, thereby reducing fault risks, and lowering costs. Its performance elevation has been proven in several RAN senarios such as proactive link adaptation and AI-based resource allocation senarios. To fully unleash the capabilities of NDT, 6G RAN should standardize bi-directional interfaces between NDT and RAN network elements to support near real-time or real-time data collection and controls. It should also standardize the functions and invocation interfaces of various twin models from multi-vendor devices. Its specific analysis provided in our contribution [6].

Figure.5 Proposed Radio Access Network architecture for NDT in 6G RAN

- Differentiated user and service assurance: In current commercial network, due to the real-time service attributes cannot be obtained by RAN, it is hard for the operator to provide a finer service assurance mechanism to the user. Since some high-priority users in the operator’s network are not receiving a service experience commensurate with what they have paid for, results in the operator receiving some complaints from those users. Currently, it is agreed to support service awareness in 6G, which can solve this issues in some sense. What’s more, 6G RAN needs to ensure the user experience according to the priority of users/services to achieve a differentiated user and service assurance mechanism.

- 6GR and 4G spectrum sharing: In current network deployments, a large number of 4G terminals remain active. These 4G terminals include toC personal UEs and toB commercial devices, such as Cat 1, Cat 1bis, and Cat 4 terminals. While toC devices are easy to be replaced in the future, toB terminals will continue to exist for a long time. And it can be foreseen that toB applications will keep growing. Given that these toB applications have a slow phase-out rate, potentially lasting 10 years or more, if the 4G frequency bands cannot be utilized for 6G due to the continued presence of 4G toB applications, the valuable low-frequency spectrum during non-busy hours would remain underutilized, resulting in significant resource wastage. Considering the requirements of LTE toB applications and spectrum utilization, the dynamic spectrum sharing should be supported between 6G and 4G, other than compressing 4G spectrum allocation for dedicated 6G use, details can be seen in [7].

- UE aggregation: In the future, there will be more and more applications with high UL bitrates. Considering the limitation of UL UE transmission power, UE aggregation is a flexible way to achieve the required UL bitrates. What’s more, UE aggregation can help to improve the stability and reduce delay of uplink services as well, that is, if the channel condition of a terminal is deteriorating, another terminal can be used to make up for the traffic performance unsteadiness caused by channel condition variation. Therefore, UE aggregation can be considered from 6G Day1.

- Text Proposal for TR 38.914

Based on the discussion in Section 2, we propose the following text proposal for TR 38.914 for the general requirements for architecture of 6G RAN.

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and 4G RAT.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	The 6G RAN architecture shall support the new and specific data collection function which allows data collection triggered by RAN and terminated in RAN.

-	The RAN architecture shall allow for RAN data plane.

-	The RAN architecture shall allow integrated sensing and communication function(s).

-	The design of RAN-CN interfaces shall allow service-based interface.

-	The RAN architecture shall support UE aggregation.

-	The RAN architecture shall support differentiated user and service assurance.

-	The RAN architecture shall allow for network digital twin (NDT).

The 6G RAN architecture is illustrated in Figure 5.2-X below.

Figure 5.2-X: Overall Architecture

- Conclusion

In this contribution, we propose to add the text proposal in Section 3 to TR 38.914.

- References

- 3GPP TR 38.914 Study on 6G Scenarios and Requirements; (Release 20) V0.2.0 (2025-09);

- R3-258861 (pCR for 38.760-3) RAN Architecture, 3GPP TSG-RAN WG3 Meeting #130, InterDigital, Samsung, Qualcomm;

- RP-253121 Requirements for RAN data collection architecture, 3GPP TSG RAN Meeting #110, CMCC, CHTTL, T-Mobile USA, FiberCop, NTT Docomo, China Unicom；

- RP-253122	Views on architecture of 6G RAN for Sensing, 3GPP TSG RAN Meeting #110, CMCC;

- RP-253130	Consideration on 6G service-based RAN, 3GPP TSG RAN Meeting #110, CMCC；

- RP-253131 Requirements for Network Digital Twin in 6G RAN, 3GPP TSG RAN Meeting #110, CMCC;

- RP-253123	Views on coexistence of 6G and 4G, 3GPP TSG RAN Meeting #110, CMCC.