3GPP TSG-RAN WG Meeting #110       	                            RP-253121

Baltimore, USA, December 8-11 2025

Agenda item:	8.2.1.2

Source:	CMCC, CHTTL, T-Mobile USA, FiberCop, NTT Docomo, China Unicom

Title:	Requirements for RAN data collection architecture

WID/SID:	FS_6G_RAN_Scen_Req

Document for:	Discussion

- Introduction

In the last meeting [1], RAN plenary discussed the requirements for architecture and migration for 6G, and agreed the pCR for 38.914 [2].

This contribution will focus on the requirements for RAN data collection architecture.

- Discussion

## 2.1 The progress of data collection/transfer in RAN2

During RAN2#131bis meeting, RAN2 discussed the requirements on AI/ML data collection/management/transfer, and achieved the following agreements [3]:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

Furthermore, RAN2 discussed the data types and endpoints for various use cases (e.g. AI/ML, sensing, SON/MDT, QoE) and achieved the following agreements in RAN2#132 meeting [4].

Agreements Study the follow aspects as a starting point: Data types (non-real time) - AI/ML (data collection for offline training) - SON/MDT, QoE - (details on what is included in 6G depends on RANP) - Sensing (the detailed characteristics (including what cases are real time or not) will be discussed in later phase, after some RAN1 and RAN2 discussions in the sensing session) QoS (e.g. data volume and latency) - Define some categories we want to base our discussion on. Details of QoS for some cases may depend on other WGs. Termination points (impacted group) - UE to RAN - UE to CN - UE to OAM - UE to UE server Understand the consumer (may be different than the termination) of the data and whether data has to be decodable by RAN.

It can be observed that the termination points for some use cases can be between UE to RAN, such as for NW side data collection for NW-sided model training, SON report and RAN-visible QoE.

Observation 1: RAN2 agreed that the termination points for some use cases can be between UE to RAN, such as for NW side data collection for NW-sided model training, SON report and RAN-visible QoE.

## 2.2 The progress of data collection/transfer in RAN3

During RAN3#130 meeting, RAN3 agreed that 6G RAN architecture supports data collection and the following principles in [5].

The 6G RAN architecture supports data collection according to the following principles: - Reusability of collected data is supported. - Data collected or generated by the 6G RAN can be used by the RAN and it can be made available to other entities, if needed. FFS which entities. - The 6G RAN can request data, and use the data collected from other entities. FFS which entities. NOTE 1: The collection, storage and usage of data follows the security principles which will be defined by relevant WGs.

Furthermore, there is an FFS in RAN3 on triggering of data collection which needs further discussion.

Triggering of data collection needs further discussion

Observation 2: RAN3 agreed that 6G RAN architecture supports data collection and follows the principles including supporting reusability of collected data, sharing data in RAN to other entities and requesting/using the data from other entities.

## 2.3 Requirements on data collection

Based on the progress of data collection/transfer in RAN2 and RAN3, we believe that 6G RAN architecture shall support data collection terminated in RAN side, share the collected data to other entities, as well as request and use the data collected from other entities. In our understanding, data terminated in RAN means that the data collection is triggered by RAN, and RAN decides which data will be collected and send the data collection configuration to the UE.

Furthermore, we believe that a new and specific RAN data collection function can avoid duplicated data collection and allow reusability of collected data for different use cases, and it is beneficial for improving efficiency since it allows unified interaction with other entities compared to today independent deployment within the base station. The new and specific RAN data collection function is applicable for the scenarios where the data is used for multiple cells or base stations, such as AI/ML based mobility. On the other hand, the latency for data collection by a new and specific RAN data collection function is lower than data collection by OAM.

Proposal 1: 6G RAN architecture shall support the new and specific data collection function which allows data collection triggered by RAN and terminated in RAN.

# 3	Conclusion

Here are the observations and proposals for requirements for data collection architecture.

Observation 1: RAN2 agreed that the termination points for some use cases can be between UE to RAN, such as for NW side data collection for NW-sided model training, SON report and RAN-visible QoE.

Observation 2: RAN3 agreed that 6G RAN architecture supports data collection and follows the principles including supporting reusability of collected data, sharing data in RAN to other entities and requesting/using the data from other entities.

Proposal 1: 6G RAN architecture shall support the new and specific data collection function which allows data collection triggered by RAN and terminated in RAN.

# 4	Text Proposal to TR 38.914

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	The 6G RAN architecture shall support the new and specific data collection function which allows data collection triggered by RAN and terminated in RAN.

# 5	Reference

- RP-252971 DRAFT Meeting Report for TSG RAN meeting #109

- RP-252870 pCR for 38.914 on Requirements for architecture and migration for 6G

- R2-2508002 RAN2#131bis Meeting Report

- R2_132_ChairNotes

- R3-258871 Consideration on data collection framework