3GPP TSG RAN #110	RP-253142

Baltimore, USA, 8th – 11th December, 2025

Agenda item:			8.2.1.1

Source:	MediaTek Inc.

Title:		Latency-related requirements

Document for:					Discussion and Decision

# Introduction

This contribution makes proposals for practical requirements for the following latency-related KPIs: User Plane Latency, Composite TPR, and Mobility Interruption Time.

# Discussion

User Plane latency

The agreed target requirement for user plane latency is acceptable for IMT-2030 purposes. However, the target operating point of that KPI is nott so relevant in guiding the practical system design. Therefore, we recommend that more importance is placed on aspects such as:

- Robust performance for latency-bound traffic

- The realistic impacts and constraints in relation to scheduling.

Proposal 1: Add the following text to the User plane latency KPI in section 5.1.7 of TR38.914

The following practical aspects shall be key focus areas for 6G latency design:

-	Ability to guarantee robust latency performance for practical packet sizes including for users at the 5% point of the CDF for latency (to be evaluated via the Composite requirement).

-	The impact of components such as HARQ, scheduling delays and support (e.g. SR/BSR) for uplink scheduling under typical assumptions (duplex mode constraints, dynamic scheduling) on latency shall be taken into account in the design.

Composite TPR

For ITU-R recommended TPR, a satisfied user ratio of 90% was agreed. There is a risk that only modelling a value of 90% does not give a true reflection of the robustness to practical latency performance of the 6GR system. Therefore, for internal 3GPP requirements, we recommend to also consider a value of 95%. It is acknowledged that the target number of connections may need to be reduced accordingly for this figure.

Proposal 2: Add “95%” to the user satisfaction ratio values to be modelled. Target number of connections per cell can be TBD.

Mobility interruption time

At last RAN plenary, there was a reluctance to consider practical values for Mobility interruption for the purposes of IMT-2030 TPR, and a very scenario-specific target of 0ms (valid for multi-TRP operation) was agreed.

It is important to improve “practical” mobility support in 6G, including beam-based mobility in “non-ideal” scenarios, and we believe that:

- Interruption requirement of ≤10 milliseconds would be a realistic target for inter-site switching cases (different DU, same CU).

- Data rates should be able to be optimized immediately after the interruption.

Proposal 3: 6G mobility should target a practical interruption time target of ≤10 ms for mobility between TRPs/cells (different DU, same CU) with single TRP operation by the user, and the design should target optimization of data rates immediately after the mobility interruption.

# 3	Proposals

The following is proposed:

Proposal 1: Add the following text to the User plane latency KPI in section 5.1.7 of TR38.914

The following practical aspects shall be key focus areas for 6G latency design:

-	Ability to guarantee robust latency performance for practical packet sizes including for users at the 5% point of the CDF for latency (to be evaluated via the Composite requirement).

-	The impact of components such as HARQ, scheduling delays and support (e.g. SR/BSR) for uplink scheduling under typical assumptions (duplex mode constraints, dynamic scheduling) on latency shall be taken into account in the design.

Proposal 2: Add “95%” to the user satisfaction ratio values to be modelled for the Composite Requirement for internal 3GPP evaluation. Target number of connections per cell can be TBD.

Proposal 3: 6G mobility should target a practical interruption time target of ≤10 ms for mobility between TRPs/cells (different DU, same CU) with single TRP operation by the user, and the design should target optimization of data rates immediately after the mobility interruption.

# 4	References

[1]	3GPP TR 38.914 v0.1.0 “Study on 6G Scenarios and Requirements”.