3GPP TSG RAN Meeting #110	RP-253161

# Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.3.1

Source:	Samsung

Title:	Discussion on 6G Spectrum

Document for:	Discussion and Decision

1	Introduction

The minimum channel bandwidth, and the minimum RF and DBB channel bandwidths supported by a UE, were extensively discussed in previous RANP and RAN1 meetings. It is generally understood that RAN decides the minimum channel bandwidth, considering available spectrum by operators, and the minimum UE RF and DBB bandwidths, considering the various UE types, and RAN1 can then design common signals/channels for initial access that would be applicable for all UE types (e.g. having bandwidth that is not larger than the minimum UE RF/DBB bandwidth).

2	Discussion

Minimum Channel bandwidth

In Rel-15 NR, most FR1 bands support 5 MHz as the minimum channel bandwidth, except for bands n41, n77, and n78 using 10 MHz, and band n79 using 40 MHz as the minimum channel bandwidth. All FR2 bands support 50 MHz as the minimum channel bandwidth. In Rel-18, 3 MHz minimum channel bandwidth was also introduced for a set of FR1 bands, such as bands n26, n28, n31, n72, n85, n100, and n106.

For 6GR, similar minimum channel bandwidths can be considered at least for the bands re-farmed from NR to 6GR (e.g., FR1 and FR2), since existing channelization and band allocation for NR could be used for 6GR as starting point. For new bands around 7GHz and around 15 GHz, a minimum channel bandwidth similar to the upper range of FR1 or lower range of FR2 can be considered. For example, spectrum around 7 GHz can be regarded as FR1-extension so its min CBW can be aligned with upper FR1 TDD band. For around 15 GHz, a new FR can apply with either separate SCS (60kHz) or the same SCS (120kHz) as FR2-1, and the minimum CBW can be larger, such as 50 MHz. Further, the maximum channel bandwidth is set to 200 MHz as a baseline with an optional extension to 400 MHz for bands where sufficient contiguous spectrum is available.

It is often assumed that the minimum channel bandwidth would impact the design of the physical layer, especially for signals and channels associated with initial access procedures. In particular, the following was agreed in RAN1#123.

Agreement If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, - Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable - Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

Basically, “Opt1” may imply an assumption of a larger than 3 MHz minimum channel bandwidth for the design of common signals/channels and then adjusting for the case of 3 MHz, for example by puncturing as in NR, while “Opt2” considers that the design directly considers 3 MHz as the minimum channel bandwidth. Both can scale with the SCS for the given channel bandwidth. However, not all common signals/channels require the larger capacity offered by a larger channel bandwidth. Some, such as the SSB, can have a 3 MHz channel bandwidth for other reasons, such as for enabling larger SSB periodicity for NES without increasing search time, and that remains applicable regardless of whether the minimum channel bandwidth is larger than 3 MHz.

Proposal 1: For 6GR, consider the following minimum channel bandwidths:

- 5 MHz for FR1 FDD bands;

- 10 MHz for lower FR1 TDD bands;

- 20 MHz for upper FR1 TDD bands and around 7GHz bands;

- 50 MHz for around 15GHz bands and FR2 bands.

For the minimum BW supported by a UE, the following was agreed.

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

The smallest maximum supported RF and BB UE BW needs to be separately considered. The UE supported “smallest maximum” BW is understood to be the minimum BW that all UEs (including the low-tier UE) need to support. Low-tier UEs in 6GR aim to replace NR (e)RedCap UEs, and potentially high end eMTC UEs, with 7Mbps ~ 10Mbps as peak data rate. Therefore, the smallest maximum BB BW supported by 6GR UEs needs to consider the balance between supported smallest peak data rate versus complexity and overhead. For example, with 5 MHz BB bandwidth at least for data channel (around 25 PRBs for 15 kHz SCS), and with 16QAM, the data rate can reach ~16 Mbps for FDD. The data rate for TDD is lower depending on the UL/DL configuration. Therefore, 5 MHz is good enough as the smallest maximum UE supported BB BW, while having the advantages of smaller device cost/complexity and power consumption over 20 MHz. RF bandwidth is another factor for device cost/complexity savings. However, restricting the RF bandwidth to smaller than 20MHz may lose some flexibility while increasing complexity, e.g., additional/frequent RF retuning. Thus, it is reasonable to set the smallest maximum UE-supported RF BW as 20 MHz.

Proposal 2: For 6GR, the smallest maximum UE-supported RF BW is 20MHz and BB BW is 5MHz.

We have noticed that there was some discussion in RAN4#117 on the minimum CBW and accordingly the following agreements were reached from RAN4 perspective. Our interpretation is the agreement is only to provide the input from RAN4 perspective which could be considered during RAN-P decision.

- Agreement: - Define minimum CBW based on SCS (e.g., 5 MHz for 15 kHz, 10 MHz for 30 kHz, 50 MHz for 120 kHz). - Support 3 MHz with 15kHz SCS for particular bands subject to operator requests, commercial need and spectrum regulations, - Work with RAN1 for the chosen minimum CBW in initial access design to accommodate majority spectrum scenarios.

Specifically, “Define minimum CBW based on SCS (e.g., 5 MHz for 15 kHz, 10 MHz for 30 kHz, 50 MHz for 120 kHz)” is just to provide some examples from RAN4 perspective, which can be considered in RAN-P. “Support 3 MHz with 15kHz SCS for particular bands subject to operator requests, commercial need and spectrum regulations,” is similar to 5G to handle specific request from operators, which can be considered in future. However, we still see the potential risk of duplicating works between RAN and WGs. To avoid that, we would like to ask RAN guidance to avoid the similar situation.

Proposal 3: To avoid duplicated work between RAN and WGs, RAN guidance is needed on topics including minimum CBW, device types and other identified ones.

3	Conclusions and Proposals

Proposal 1: For 6GR, consider the following minimum channel bandwidths:

- 5 MHz for FR1 FDD bands;

- 10 MHz for lower FR1 TDD bands;

- 20 MHz for upper FR1 TDD bands and around 7GHz bands;

- 50 MHz for around 15GHz bands and FR2 bands.

Proposal 2: For 6GR, the smallest maximum UE-supported RF BW is 20MHz and BB BW is 5MHz.

Proposal 3: To avoid duplicated work between RAN and WGs, RAN guidance is needed on topics including minimum CBW, device types and other identified ones.