3GPP TSG-RAN Meeting #110	RP-253249

Baltimore, USA, 8-11 December, 2025

Agenda item:		8.2.1.4

Source:	Xiaomi

Title:	Requirements of Integrated Sensing And Communication (ISAC)

Document for:		Discussion and Decision

Introduction

In the RAN#106 meeting, a RAN plenary study item on “Study on 6G scenarios and requirements” [1] was approved. In the RAN#108 meeting, the RAN WG study item on “Study on 6G Radio” [2] was then approved and in RAN#109. It was agreed to focus on use cases for detection and tracking and to study all sensing modes.

In this contribution, our views on 6G Requirements for Integrated Sensing And Communication (ISAC) study are provided.

Discussion

Integrated Sensing and Communication (ISAC) is envisioned to become a key application for 6G. It extends and complements the communication oriented 3GPP system with new sensing capabilities for broader business opportunities with new use cases. Additionally, it can also be used to enhance the achievable air-interface performance for communication.

In SA1 TR 22.837 [3] and TS 22.137 [4] 32 use cases are proposed for sensing, which are categorized into three groups, i.e., object detection and tracking, environment monitoring and motion monitoring (see Annex A). They cover a wide field of new applications and services. Additionally, 20 advanced use cases are further identified in the current SA1 TR 22.870 [5], e.g., environment reconstruction, etc. To keep the evaluation efforts manageable, it was agreed in RAN#109 to study object detection and tracking. We support this decision, even though the early support of advanced use cases may be favourable in one sense. However, it would result into a broad scope of the study. Considering the limited available time for the 6G ISAC study, i.e., about one year, we prefer to focus in Rel-20 on the basic sensing use cases of object detection/tracking.

The list of use cases for object detection/tracking is still very large. In TR 22.837, for example, seventeen different applications are listed. Therefore, we propose to carefully select a few use cases and define the representative deployment scenarios [6] accordingly.

Proposal 1: To facilitate requirement definition for sensing as a new service in 6G, RAN selects a subset of use cases for object detection and tracking and defines the representative deployment scenarios accordingly.

This selection should be based on two criteria, i.e., near future commercial needs and already existing work in RAN that can be re-used. For the latter, the RAN1 study in Rel-19 has developed the ISAC channel model for object detection/tracking [7]. Regarding the commercial needs, we see the following services to be attractive for the near future:

- Low-altitude economy: This is an emerging business area driven by UAVs, and their detection/tracking are an essential part of the service. Different UAV target types should be supported. The sensing transmitter/receiver can be TRPs and/or UEs. We assume that the UAV itself is not a sensing node, and just treated as a target.

- Human intruder detection: This is another promising category, offering safety while keeping privacy (e.g. because no cameras are needed). Also here, the sensing transmitter/receiver can be TRPs and/or UEs. We assume that the human itself is just treated as a target.

- Automotive: For better support of automatic driving, early accident indication and intruder detection on the road, sensing should be supported. The sensing transmitter/receiver can be TRPs and/or UEs. In details, the UE can be vehicle UE, pedestrian UE, and RSU type UE.

- AGV: The application of ISAC in industry use cases is attractive to extend business models of 3GPP. The sensing transmitter/receiver can be TRPs and/or UEs. In details, the UE can be an AGV UE and/or a pedestrian UE.

Proposal 2: To facilitate requirement definition for sensing as a new service in 6G, RAN prioritizes detection and tracking for the following use cases

- UAV related use cases

- Human related use cases

- Automotive related use cases

- AGV related use cases

Proposal 3: Sensing modes with UE involvement should be studied for detection and tracking at least for following target types: UAV, human, vehicle and AGV.

Regarding the requirements for the above-described new sensing services, the following high level design requirements can be given, enabling the efficient support of sensing, while minimizing the negative impact on communication:

- Sensing Performance: The sensing operation should be guaranteed for certain KPIs (e.g., accuracy, coverage, latency), so that the application knows what sensing performance it can expect.

- Impact on communication: The impact of ISAC on communication should be minimized, to not cause an unacceptable degradation of existing communication performance (e.g., with respect to throughput, latency, reliability). It should be possible to schedule sensing operations so that they can be harmonized with communication.

- Efficient use of radio resources: Sensing resources should not be over-provisioned relative to their actual need. Reuse of existing communication signals (SSB, CSI-RS, DMRS) for sensing where possible.

- Scalability/Flexibility: ISAC should scale with target density and sensing demand, without requiring dedicated infrastructure everywhere. Support for distributed transmitters and receivers without overwhelming coordination overhead.

- Interoperability: At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

Proposal 4: For supporting 6G sensing services, the following requirements can be considered as a guidance,

- Sensing Performance (e.g. accuracy, coverage, latency) should be guaranteed

- Impact on communication shall be minimized

  - Meeting given sensing requirements should not cause unacceptable degradation of communication KPIs (e.g. throughput, latency, reliability)

- Radio resources should be used efficiently

  - Sensing resources should not be over-provisioned.

  - Reuse of existing communication signals (SSB, CSI-RS, DMRS) for sensing where possible

- Scalability/Flexibility

  - ISAC should scale with target density and sensing demand, without requiring dedicated infrastructure everywhere

  - Support for distributed transmitters and receivers without overwhelming coordination overhead

- Interoperability

  - At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

Conclusion

In this contribution, we discuss requirements for architecture and migration and make the following observations and proposals:

Proposal 1: To facilitate requirement definition for sensing as a new service in 6G, RAN selects a subset of use cases for object detection and tracking and defines the representative deployment scenarios accordingly.

Proposal 2: To facilitate requirement definition for sensing as a new service in 6G, RAN prioritizes detection and tracking for the following use cases

- UAV related use cases

- Human related use cases

- Automotive related use cases

- AGV related use cases

Proposal 3: Sensing modes with UE involvement should be studied for detection and tracking at least for following target types: UAV, human, vehicle and AGV.

Proposal 4: For supporting 6G sensing services, the following requirements can be considered as a guidance,

- Sensing Performance (e.g. accuracy, coverage, latency) should be guaranteed

- Impact on communication shall be minimized

  - Meeting given sensing requirements should not cause unacceptable degradation of communication KPIs (e.g. throughput, latency, reliability)

- Radio resources should be used efficiently

  - Sensing resources should not be over-provisioned.

  - Reuse of existing communication signals (SSB, CSI-RS, DMRS) for sensing where possible

- Scalability/Flexibility

  - ISAC should scale with target density and sensing demand, without requiring dedicated infrastructure everywhere

  - Support for distributed transmitters and receivers without overwhelming coordination overhead

- Interoperability

  - At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

- References

[1] RP-243327, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, “New SID: Study on 6G Scenarios and requirements”

[2] RP-251881, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[3] 3GPP TSG SA, “Feasibility Study on Integrated Sensing and Communication”, TR 22.837, V19.4.0

[4] 3GPP TSG SA, “Service requirements for Integrated Sensing and Communication; Stage 1”, TS 22.137, V19.4.0

[5] 3GPP TSG SA, “Study on Use Cases and Service Requirements; Stage 1”, TR 22.870, V0.3.1

[6] RP-253247, Xiaomi, “Deployment scenarios for ISAC”

[7] 3GPP TSG RAN, “Study on channel model for frequencies from 0.5 to 100 GHz”, TR 38.901, V19.0.0

- Annex A: Use case grouping defined in TS 22.137

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 5 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
| Environment monitoring | 5 | 95 | 10 | 0.2 (NOTE 4) | N/A | N/A | N/A | N/A | 60000 | 60 to 600 | 0.1 to 5 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring) |
| Motion monitoring | 6 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 5) |
|  | 7 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 to 50 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing (NOTE 6) |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |