3GPP TSG RAN Meeting #110                                                       RP-253253

Baltimore, USA, December 8-11, 2025

Source:	vivo

Title:	Discussion on 6G device types

Agenda Item:	8.2.1.3.2

Document for:	Discussion and Decision

Introduction

Following objective for 6G device types are captured in the SID for RAN 6G study [1].

- Target scalable and forward compatible design for diverse device types

Following agreements reached in RAN1#122 on device types including supported BW for IoT device. The 3rd agreement was further discussed together with minimum spectrum allocation in RAN#109 with the clarification that the 3rd agreement is applicable to FR1, and RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110.

Agreement - Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

It was further discussed in RAN#109 and following agreements reached with the conclusion that “proposal 3 & 4 are endorsed for RAN only (no WG discussion)”.

Agreement Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

Then, in RAN1#122 bis and RAN1 #123, RAN1 continued discussion on what should be commonly applicable to all 6G device types and aspects to determine smallest maximum supported RF and BB UE BW, and the following agreements reached.

Agreement - For scalable 6GR design for diverse device types, RAN1 can at least consider the following, targeting applicable to all 6G device types, - Basic initial access procedures from RAN1 perspective - Other PHY features after initial access procedure, e.g., Other DL/UL control, scheduling/HARQ - Coverage features to meet the identified coverage target - Energy saving both at BS and UE sides - MRSS - Note: whether these features are supported, mandatory or optional is separate discussion Agreement - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least - Overall device complexity - Overall system performance impact - Energy efficiency for both BS and UE - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

In this contribution, we further discuss device types with some detailed views based on RAN and RAN1 agreements above.

Discussion

Possible 6G device types

In the era of 6GR, the mobile system will provide its services to a variety of devices, from low-end IoT devices to high-end advanced eMBB devices. The design of 6GR should learn from the lessons of LTE & NR systems, wherein several new device types were introduced in later releases, causing too many different sub-categories serving similar service, e.g., NB-IoT with category NB1 & NB2, and MTC with category M1 & M2, leading to difficulties in market adoption. Therefore, clear blueprint for 6GR device types should be laid out in the first release to avoid compatibility issues in later releases, and the number of device types shall be limited for economy of scales.

Undoubtedly, eMBB still stands as the mainstream for mobile operators. One device type including Smartphones, immersive eMBB can be the starting point. It can be further studied if FWA/CPE can be a separate device type from smartphone/immersive eMBB due to their significant difference in use cases and form factors.

Meanwhile, to support massive communication, one device type for IoT should be introduced. Based on lessons learned from LTE & NR systems, single device type covering different IoT service is desired to maximize the economy of scale. It is noted that, ambient IoT is a different type of IoT evolved in a separate track, thus is not considered here.

In between eMBB and IoT device types, a device type for reduced capability eMBB can be considered due to inherent form factor limitation, e.g., for wearables.

Consider three device types as starting point for 6G diverse device types,

- Two eMBB device types:

  - Type A: High-tier eMBB device, including Smartphones, immersive eMBB

  - Type B: Reduced capability eMBB device, including wearable and XR

- Single IoT device type:

  - Type C: low-tier device type for IoT

- FFS FWA/CPE as separate device type or included in Type A

How to distinguish/identify 6G device types

To distinguish/identify different device types, a minimum set of mandatory features for baseband and RF can be defined, e.g., Maximum UE bandwidth, Number of Tx/Rx antennas/chains and Maximum MIMO layers, Peak data rate, Max modulation order, Duplex mode.

Other parameters/factors discussed in RAN#109 may not be device type specific features. At least, coverage and energy efficiency should be common feature for all device types, which was already agreed in RAN1 though details, e.g., which energy efficiency techniques are commonly supported by all device types needs further discussion in RAN1.

At this stage, it is sufficient for RAN to focus on the baseband and RF mandatory features for device type.  Table 1 provides our suggested values for these features for device type A, B and C.

Table 1 Suggested three device types with mandatory baseband and RF capability

|  | Device type A | Device type B | Device type C |
| --- | --- | --- | --- |
| Typical Device/service | Smartphones, immersive eMBB FFS CPE and FWA may be in a separate device type | Reduced capability eMBB, e.g., wearable/XR, etc | IoT |
| Downlink peak data rate | At least ~5Gbps | 200Mbps ~ 1Gbps | ~10Mbps |
| Uplink peak data rate | At least ~1Gbps | 50Mbps ~ 200Mbps | ~5Mbps |
| Supported maximum downlink channel bandwidth | 200 MHz [Optional 400MHz] | 100 MHz | 20MHz |
| Supported maximum uplink channel bandwidth | 100/200 MHz [Optional 400MHz] | 100 MHz | 20MHz |
| Supported max modulation order | 256 QAM for DL [Optional 1024QAM] 256QAM for UL | [256 or 64] QAM for DL 64QAM for UL | 64QAM for DL 64QAM for UL |
| Supported maximum Downlink MIMO layer/Rx antenna | At least 4 layers (4Rx) [Optional up to 8 layers (8 Rx)] | 1~2 layers (1~2 Rx) | 1 layer(1Rx) [Optional up to 2 layers (2 Rx)] |
| Supported maximum Uplink MIMO layer/Tx antenna | At least 2 layers (2 Tx) [Optional up to 4/8 layers (4/8 Tx)] | 1 layer (1Tx) | 1 layer(1Tx) |
| Duplex mode | TDD, FDD | TDD, FDD | TDD, FDD, half-duplex FDD |

Maximum UE bandwidth

Four options were listed as candidate value for smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type, which is for type C device (IoT) discussed above. While the down-selection of the value is to be determined by RAN, the physical layer-related factors for selecting an appropriate value agreed by RAN1 offer valuable insights.

  - Overall device complexity

In NR, RAN1 analyzed complexity reduction provided by UE RF and BB bandwidth reduction for Redcap and eRedcap. As captured in TR 38.865, significant cost reduction can be achieved by reducing 100MHz RF&BB to 20MHz RF&BB BW, e.g., reduction is up to 68.59% for TDD 1Rx and 60.93% for HD-FDD 1Rx according to tables 7.2.2-2 and 7.2.2.2-3. Further reducing UE RF&BB BW to 5MHz, or 20MHz RF & 5MHz BW, additional cost reduction is minimal, e.g., only additional 5.49% or 4.66% reduction for HD-FDD 1Rx, and 3.53% or 2.53% reduction for TDD 1Rx. On the other hand, while keeping BW unchanged but reducing peak data rate, e.g., 20MHz BW with peak data rate restriction, the cost can be reduced around 3% compared to without peak data rate limitation, as captured in table 7.3.2-1~6 in TR 38.865.

In LTE system, Cat1bis supporting 20MHz BW are widely deployed. Due to the large economy of scale the commercial Cat1bis device cost is comparable to that of LTE-M and NB-IoT, though its BW is much larger than LTE-M and NB-IoT. This proves the attainable cost reduction from further reducing the UE BW below 20MHz is minimal, compared to that from extension of economy of scale.

  - Overall system performance impact and a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

As captured in SID, common 6G Radio design should meet mobile broadband service requirements as a high priority, which can be extended to meet the vertical needs. Therefore, single structure for SSB/SIB1 and other common signals/channels with sufficient bandwidth shared by different device types without degrading eMBB performance should be the prime design objective. As evaluated in TR 38.865, assuming the system bandwidth is 20MHz while UE BW capability is 5MHz, performance degradation is observed for common channels such as PDCCH CSS and SIB1 PDSCH, when the common channels are transmitted within 5MHz compared to 20 MHz. Consequently, if these common channels have to be confined within BW smaller than 20MHz due to limitation of IoT device, eMBB would suffer coverage loss or negatively impact latency and power consumption, which should be avoided.

  - Energy efficiency for both BS and UE

Though 20MHz BW would result in larger peak power than smaller BW, the increase would be trivial especially for uplink in case of same Tx power, because the peak power is dominated by PA max output power rather than BW. Furthermore, the average power consumption could be lower due to faster transmission enabled by 20MHz BW, which is beneficial for both BS and UE energy efficiency.

Based on analysis above, maximum supported RF and BB UE BW without spectrum aggregation for type C device (IoT) should be 20MHz.

Regarding UE channel bandwidth for other device types, similar bandwidth as NR eMBB should be supported for type B device for high throughput. For type A device, larger DL bandwidth of at least 200MHz can be supported to dramatically increase DL data rate or capacity. Maximum UL bandwidth up to 200MHz is considered to improve UL data rate. However, device type A supporting UL bandwidth up to 100MHz should also be considered at current stage, since it is not fully clear that 200MHz UL transmission can be supported by all type A devices especially from perspective of availability of commercial PA.

Additionally, larger BW can be considered for CPE/FWA considering less form factor restriction, e.g., maximum DL/UL BW up to 400MHz.

Number of Tx/Rx antennas/chains and Maximum MIMO layers

RAN already agreed minimum number of Tx/Rx antenna is 1 for lowest-tier device for IoT. In addition, type C device may optionally support two Rx antennas. Thus, downlink MIMO layer no more than 2 is supported by type C device.

For type A devices, larger number of antennas with higher antenna gain can be supported, e.g., 4 layers for DL and 2 layers for UL as mandatory, and optionally up to 8 layers can be considered.

For type B, the supported MIMO layer could be reduced due to form factor limitation (e.g., for XR and wearable devices). Single Tx antenna, one or two Rx antennas thus single layer for UL and up to 2 layers for DL is considered.

Modulation order

For type A devices, to deliver ultra-high data speeds and immersive multimedia experiences, at least 256 QAM for DL and 256QAM for UL should be mandatory supported. Optionally, 1024QAM for DL can be further considered.

For type B devices, lower modulation order, e.g., 256QAM for DL and 64QAM for UL, or 64QAM for DL/UL as NR Redcap can be sufficient for required services while achievable even with the form factor limitation.

Restricting the maximum modulation order can reduce complexity by reducing the amount of RF and baseband processing, which is beneficial at least for device type C. Similar as NR eRedcap, 64QAM can be considered as mandatory supported.

Peak data rate

To meet various service requirements/scenarios, e.g., from smart cities, smart metering to immersive multimedia, different device types supporting different data rates shall be considered in the first release of 6GR. With limited bandwidth and limited number of Rx/Tx antennas, and also potential peak data rate restriction to reduce the BB cost (e.g., memory), type C device can support up to 10 Mbps, while up to 5 Gbps can be supported by type A device to deliver lightning-fast data speeds and immersive multimedia experiences. In-between type A and type C device, 100Mbps ~ 1Gbps data rate can be considered for type B device.

Duplex mode

Half duplex mode can reduce cost as well as power, thus, half duplex FDD & TDD can be beneficial for device type C. The device C optionally support either half duplex or full duplex FDD. For device A & B, full duplex FDD should be Mandatory.

6GR considers RF mandatory baseband and RF features for three device types shown in the table 1 as the starting point.

Native support of diverse device types by a single 6G-RAT

Support diverse device types by a single 6G-RAT means the system shall be inherently flexible/scalable enough to meet the connectivity requirements of all possible device types, including at least following aspects:

- common basic initial access procedures based on a single common signals/channel for all device types,

- common coverage features for all device types to meet the identified coverage target

- common energy saving features for all device types.

Consider following operational requirements for diverse device types:

- 6G RAT shall define a limited set of diverse device types.

- 6G RAT shall be inherently flexible/scalable enough to meet the connectivity requirements of all possible device types, including at least common basic initial access procedures based on a single common signals/channel for all device types, common coverage features for all device types to meet the identified coverage target and common energy saving features for all device types.

Conclusion

In this contribution, we provide our considerations on 6G device types and the proposals are as following:

- Consider three device types as starting point for 6G diverse device types,

- Two eMBB device types:

  - Type A: High-tier eMBB device, including Smartphones, immersive eMBB

  - Type B: Reduced capability eMBB device, including wearable and XR

- Single IoT device type:

  - Type C: low-tier device type for IoT

- FFS FWA/CPE as separate device type or included in Type A

6GR considers following RF mandatory baseband and RF features for three device types shown in the table as the starting point.

- Table 1 Suggested three device types with mandatory baseband and RF capability

|  | Device type A | Device type B | Device type C |
| --- | --- | --- | --- |
| Typical Device/service | Smartphones, immersive eMBB FFS CPE and FWA may be in a separate device type | Reduced capability eMBB, e.g., wearable/XR, etc | IoT |
| Downlink peak data rate | At least ~5Gbps | 200Mbps ~ 1Gbps | ~10Mbps |
| Uplink peak data rate | At least ~1Gbps | 50Mbps ~ 200Mbps | ~5Mbps |
| Supported maximum downlink channel bandwidth | 200 MHz [Optional 400MHz] | 100 MHz | 20MHz |
| Supported maximum uplink channel bandwidth | 100/200 MHz [Optional 400MHz] | 100 MHz | 20MHz |
| Supported max modulation order | 256 QAM for DL [Optional 1024QAM] 256QAM for UL | [256 or 64] QAM for DL 64QAM for UL | 64QAM for DL 64 QAM for UL |
| Supported maximum Downlink MIMO layer/Rx antenna | At least 4 layers (4Rx) [Optional up to 8 layers (8 Rx)] | 1~2 layers (1~2 Rx) | 1 layer(1Rx) [Optional up to 2 layers (2 Rx)] |
| Supported maximum Uplink MIMO layer/Tx antenna | At least 2 layers (2 Tx) [Optional up to 4/8 layers (4/8 Tx)] | 1 layer (1Tx) | 1 layer(1Tx) |
| Duplex mode | TDD, FDD | TDD, FDD | TDD, FDD, half-duplex FDD |

Consider following operational requirements for diverse device types:

- 6G RAT shall define a limited set of diverse device types.

- 6G RAT shall be inherently flexible/scalable enough to meet the connectivity requirements of all possible device types, including at least common basic initial access procedures based on a single common signals/channel for all device types, common coverage features for all device types to meet the identified coverage target and common energy saving features for all device types.

Reference

[1] RP-251395, “Revised SID: Study on 6G Scenarios and requirements”; CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.