3GPP TSG RAN Meeting #110	RP-253406

Baltimore, 8-11 December 2025

Agenda item:			9.1.5

Source:	SES

Title:	Analysis of the Tx-Rx Separation of Ku Band Phase 2 Band Combination.

Document for:	Information and Discussion

# Introduction

Work on the Priority 1 of the Ku Band has been progressing well to date. The next phase of the normative work will focus on the Standardisation of the Phase 2 Uplink and Downlink band combinations as shown in the following Figure 1.

The proposed Work Item RP-253405 covers Full and Half Duplex solution for Downlink and Uplink Band A.

In this contribution we present analysis of how the Tx-Rx Separation issue could be resolved during the normative phase 2 of the Ku Band combination.

Figure 1 Ku Band Downlink and Uplink Banad A (Phase 2)

# Ku Band Uplink Bands

## Ku Band Uplink Band B

Figure 2 illustrates the Ku Band uplink and downlink spectrum along with the terminal configuration for Uplink Band B. The downlink spectrum spans 10.7 to 12.75 GHz (ITU Regions 1 and 3), while the uplink spectrum covers 13.75 to 14.5 GHz. The guard-band between the Ku Band downlink and uplink spectra ranges from 1 to 1.3 GHz.

For the terminal, the power amplifier’s output can vary between 1W and 100W, with a nominal power of 20W (33 dBm) assumed. The received Ku Band signal is typically extremely weak, with a nominal receiver sensitivity of -120 dBm. Additionally, the Ku Band LNB may saturate if the input power exceeds its 1 dB compression point, typically around -55 dBm. Given the significant power disparity between the transmit and receive signals, the diplexer is a critical component, filtering the transmit power to an acceptable level and minimizing TX leakage into the RX port. The minimum diplexer rejection is 60 dB, though it can exceed 100 dB.

Figure 2: Uplink Band B Configuration.

Observation 1: For Downlink and Uplink Band B band combination, the 1 GHz separation between the Downlink and Uplink Band B is a natural guard band.

## Ku Band Uplink Band A

Figure 3 illustrates the Ku Band uplink and downlink spectrum along with the terminal configuration for Uplink Band A. The downlink spectrum extends from 10.7 to 12.75 GHz (ITU Regions 1 and 3), while the uplink spectrum spans 12.75 to 13.25 GHz. Notably, the guard-band separating the Ku Band downlink and uplink spectra for Uplink Band A is minimal.

Figure 3: Uplink Band A Configuration

A typical diplexer offers a nominal rejection of the transmitter signal at the RX port of 85 dB, with the Ku Band downlink spectrum assumed to span 10.7 to 11.45 GHz. The guard-band between the downlink spectrum’s edge and Uplink Band A is 1.3 GHz.

Observation 2: To accommodate a Full Duplex mode for Uplink Band A, operationally, we would require a guard band of approximately 700 MHz to 1 GHz separation between the Downlink and Uplink bands.

## Ku Band Downlink Bandpass Filter

To further reduce the uplink power leaking into the RX port, a typical bandpass filter can be employed. Figure 4 provides an example of a ceramic bandpass filter designed for the Ku Band downlink, covering 10.7 to 12.75 GHz.

Figure 4 Example of a Ceramic Bandpass Filter for Ku Band Downlink.

Figure 5 presents an example of bandpass filter rejection plotted against guard-band size. It demonstrates that with guard-bands of 0.6, 1, and 1.5 GHz between the right edge of the downlink and the left edge of the uplink carrier, the corresponding filter rejections are 22 dB, 40 dB, and 50 dB, respectively. The bandpass filter helps mitigate TX signal leakage into the RX port.

Figure 5 Filter Rejection Example

Observation 3 Ceramic surface mount Band Pass filter provides additional rejection from uplink carrier in Uplink Band A.

## Ku Band Uplink Band A: Half-Duplex Mode

Figure 6 illustrates the Ku Band uplink and downlink spectrum along with the terminal configuration for Uplink Band A in Half-duplex mode. The downlink spectrum spans from 10.7 to 12.75 GHz (ITU Regions 1 and 3), while the uplink spectrum covers 12.75 to 13.25 GHz. As the terminal can only transmit or receive at any given time, a diplexer is not required. This setup streamlines the terminal design, with no limitations imposed between the uplink and downlink spectra.

Figure 6: Uplink Band A Configuration Half-duplex Mode.

Observation 4: For a Half Duplex operation, no stringent band pass filter is necessary, and consequently the Uplink and Downlink bands could potentially be placed very close to the Downlink/Uplink boundary.

## Ku Band Uplink Band A: Operational Example

The following Figure 7 illustrates several real operational examples of bands in Ku Band’s Downlink and in Uplink Band A. The examples are from ITU Region 2. The carrier bandwidth ranges from 10-74 MHz

Figure 7 Examples of Operational Downlink and Uplink Bands in in ITU Region 2

As it can be observed from Figure 7, the separation between the Uplink and Downlink range approximately from 500-1000 MHz

Therefore, even in the proprietary system used today, there is a large Tx-Rx separation between the Downlink and Uplink bands.

Considering the operational scenario illustrated, the same approach could be used in defining the Tx-Rx separation in Ku Band Phase 2 work.

Furthermore, in operational scenarios, Downlink and Uplink bands are never deployed nor planned adjacent to the boundary as illustrated in Figure 7

Observation 5: In the current operational scenarios, there already is a Tx-Rx separation of approximately 500-1000MHz.

Observation 6: Downlink and Uplink Bands are never deployed nor planned adjacent to the boundary.

# Conclusion

In this analysis we have made the following observations:

Observation 1: for Downlink and Uplink Band B band combination, the 1 GHz separation between the Downlink and Uplink Band B is a natural guard band.

Observation 2: To accommodate a Full Duplex mode for Uplink Band A, operationally, we would require a guard band of approximately 700 MHz to 1 GHz separation between the Downlink and Uplink bands.

Observation 3 Ceramic surface mount Band Pass filter provides additional rejection from uplink carrier in Uplink Band A.

Observation 4: For a Half Duplex operation, no stringent band pass filter is necessary, and consequently the Uplink and Downlink bands could potentially be placed very close to the Downlink/Uplink boundary.

Observation 5: In the current operational scenarios, there already is a Tx-Rx separation of approximately 500-1000MHz.

Observation 6: Downlink and Uplink Bands are never deployed nor planned adjacent to the boundary.

Furthermore, we can draw the following conclusions from our analysis

For the Half Duplex scenario, we do not envisage large Tx-Rx separation, as there will not be interference between the Uplink and Downlink carrier signals.

For the Full Duplex scenario, initial assessment indicated a guard band of approximately 1 GHz between the Uplink and Downlink carrier signals. The determination of the actual guard band required will be assessed in the WID.

In the current operational scenarios, there already is a 500-1000 MHz separation between the Downlink and Uplink bands.

We propose to use The analysis presented in this contribution as the basis of the normative work for Ku Band Phase 2.