3GPP TSG RAN Meeting #110						RP-253489

Baltimore, USA, December 8-11, 2025

Agenda Item:	9.9.4.10

Source:	Qualcomm Incorporated

Title:	Release independence for 6L

Document for:	Approval

1	Introduction

Introduction of requirements for 6Rx with 4L/6L has been a part of Rel-19 package under the UE RF enhancements WI [1]. Introduction of some of the requirements in a release independent way has been discussed in RAN4 for several meetings but the discussion was not concluded. RAN2 confirmed the feasibility of introducing release independence for these requirements in [2].

In this paper we discuss the release independence handling of the requirements for 6L and propose to introduce the 6Rx/6L capability from Rel-18.

2	Discussion

2.1 Release Independent Introduction of Requirements and Features

3GPP has introduced the release independence concept such that certain features or functionalities introduced later can be implemented from a previous release. This mechanism enables faster time to market and simpler implementation since it is not necessary for a UE to support all other features/functionalities defined up to the current release. The mechanism has been extensively used for new frequency bands (usually made available from the 1st release of a certain RAT), band combinations, higher order MIMO, etc.

The release independence mechanism has in general two components: the signaling defined in RAN2 which is introduced such that a feature can be advertised and configured from a previous release and the RAN4 requirements referenced in TS 38.307 indicating the requirements which should be met by a UE implementing the feature from an earlier release.

For the RAN2 signaling part, the RAN2 CRs with “magic” sentence “implementable from an earlier release” are captured in Annex C of TS 38.331 to indicate that the feature can be implemented from an earlier release. For example, CRs with signalling changes for BCS4/5, NS value extension and FGB2 BW classes for FR2 are listed there.

Observation 1: Release independent handling of features is possible through RAN2 signaling and RAN4 requirement referencing in 38.307.

Observation 2: The release independence mechanism is important to enable faster time to market and simpler inter-dependence of features.

2.2 Release independence handling of 6Rx/6L

RAN4 has discussed introducing the requirements for 6Rx/6L and/or 6Rx/4L for FWA/handheld devices in a release independent manner from Rel-18. This issue was discussed for several meetings, however consensus could not be reached. It should be noted that RAN2 has already confirmed the feasibility of introducing the capability signaling in a release independent way from Rel-18 [2] after it was asked by RAN4.

Observation 3: RAN2 has confirmed the feasibility of introducing the 6Rx/6L capability from Rel-18.

Furthermore, another important aspect is that no other signaling changes are needed to enable this feature from a previous release. The RAN1/2 components (signaling mechanism for MIMO configuration,etc) needed to configure the UE with this feature are already defined from Rel-18. As such, release independence for 6Rx/6L can be enable only through the “magic” sentence presented in Section 2.1.

Observation 4: No other changes besides the RAN2 “magic” sentence are needed to enable release independence for 6Rx/6L.

In RAN4#117 and RAN#110 several operators have expressed interest in having this feature early implementable to address commercial deployment needs [3], [4].

Proposal 1: Introduce 6Rx/6L (including 6L and 4L) requirements and corresponding capability signaling for FWA from Rel-18 to address commercial deployment needs.

2.4 RAN4 CR to TS 38.307

RAN4 can enable 6Rx/6L feature release-independent from Rel-18 by submitting CR to Rel-19 version of TS 38.307. An example of CR to Rel-19 TS 38.307 is drafted as below.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| -------- Skip the unchanged context -------- |  |  |  |
| RF requirements for 6Rx UE | Rel-18 | Table B.4.x-1 | Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements |
| Demodulation requirements for 6Rx UE | Rel-18 | Table B.3.y-1 | Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |

-------- Skip the unchanged context --------

B.3.y	Common PDSCH demodulation and CQI requirements with 6Rx

The requirements and test cases listed in Table B.3.y-1 are specified in Rel-P version of TS 38.101-4 [5].

Table B.3.y-1: UE PDSCH demodulation requirements with 6Rx

| Section / Clause | Description |
| --- | --- |
| 5.2.x | PDSCH demodulation requirements for 6Rx TDD |
| 6.2.x | CQI requirements with 6Rx |

-------- Skip the unchanged context --------

B.4.x-1	Common UE RF requirements for 6Rx

The requirements and test cases listed in Table B.4.x-1 are specified in REL-P version of TS 38.101-1 [2].

Table B.4.x-1: Common UE RF requirements for 6Rx for single band in FR1

| Clause | Description |
| --- | --- |
| 6.2.4 | Configured output power |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

Observation 5: RAN4 can enable 6Rx/6L feature release independent from Rel-18 by submitting an CR to Rel-19 TS 38.307. The above CR draft can be used as a reference. The specific CR content could be further discussed at RAN4#116bis meeting.

3	Conclusion

In this paper we briefly discussed the mechanism of introducing new features or requirements in a release independent manner and this can be easily applied to the 6Rx/6L requirements. We made the following observations and proposal;

Observation 1: Release independent handling of features is possible through RAN2 signaling and RAN4 requirement referencing in 38.307.

Observation 2: The release independence mechanism is important to enable faster time to market and simpler inter-dependence of features.

Observation 3: RAN2 has confirmed the feasibility of introducing the 6Rx/6L capability from Rel-18.

Observation 4: No other changes besides the RAN2 “magic” sentence are needed to enable release independence for 6Rx/6L.

Observation 5: RAN4 can enable 6Rx/6L feature release independent from Rel-18 by submitting an CR to Rel-19 TS 38.307. The above CR draft can be used as a reference. The specific CR content could be further discussed at RAN4#116bis meeting.

Proposal 1: Introduce 6Rx/6L (including 6L and 4L) requirements for FWA from Rel-18 to address commercial deployment needs.

References

[1]	RP-251816, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, Hisilicon, TSG RAN#108.

[2]	R4-2520009, “Reply LS on Release Independence of 6Rx”, TSG-RAN4#117,

[3]	R4-2509968, “On Release Independence of 6Rx”, T-Mobile USA, BT, SK Telecom. TELUS, Verizon, Deutsche Telekom, Bell Mobility, Boost Mobile, Southern Linc, Telecom Italia, Telstra, TSG-RAN4#116

[4] 	RP-25xxxx, Operator paper on release independence