3GPP TSG RAN meeting #110									RP-253500

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.2

Source:	PML

Title:	Views on 6G RAN Architecture for Data Framework

Document for:	Discussion

- Introduction and outcome of RAN#109 [1]

3GPP TSG RAN #109 , agreements summarized in RP-252909: Summary of Discussion on 6G Requirements for architecture and migration[2] ，the 6G RAN architecture shall support enhanced service awareness in RAN, which will be officially incorporated into TR 38.914[3].

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks. New services - The design of the 6G RAN architecture shall allow the collection and transport of new service data (e.g. AIML data, sensing data) efficiently to fulfil new service requirements. - The design of the 6G RAN architecture shall support the generation, transport, and/or visibility of new service data (e.g., AIML data, sensing data) efficiently to fulfil new service requirements.

Towards the service awareness , 6G RAN needs to be capable of deeply understanding, dynamically perceiving, and intelligently adapting to diverse service requirements, translating this awareness into precise network resource configuration and optimized decision-making, thereby enabling more efficient service deployment and operational observability.

As described in the agenda for RAN #110 meeting [4], this contribution paper analyzes the urgent need for deep analysis and extraction of data features arising from diverse service types, demonstrates the necessity of data knowledge graph technology within the 6G data framework design requirements, and proposes directional recommendations for the architecture and functional design for 6G RAN study, etc.

- Data Feature for Service Awareness RAN

Service Awareness at the RAN level is primarily manifested through three-tiered capabilities:

Understanding service requirements: Deeply parsing the QoE/QoS requirements of different services (e.g., "time to first token" for AI inference, ultra-low latency for industrial control, high bandwidth for immersive communications, etc.)

Perceiving service status: Real-time monitoring and analysis of service flow characteristics, network resource utilization, usage patterns, and variation trends.

Intelligent resource adaptation: Based on the perception results from the first two capabilities, automatically discovering, allocating, coordinating, and aggregating heterogeneous resources (spectrum, computing power, cache, etc.)

Observation 1: 6G Data framework and AI-native RAN should support cross-domain data collection, integrating application-layer service characteristics (e.g., traffic type, QoE metrics) with radio channel state and load information. Through AI models and Data Feature, it will enable:

-	Automatic service and data type identification and classification.

-	Service-characteristic-based PDU session mapping and dynamic DRB (Data Radio Bearer) configuration.

-	Predictive resource reservation and interference coordination.

Proposal 1: pCR for 38.914 on requirements for architecture and migration for 6G to note data feature for service awareness： The 6G RAN architecture shall support enhanced service awareness in RAN（e.g. Data Feature based Service Requirements Understanding, Service Status Perception, and Intelligent Resource Adaptation）.

- Data Knowledge Graph enabling Data Framework for 6G Service Awareness RAN

  - Use Case: AI and Data Knowledge Graph driven Intelligent Wireless Resource Optimization

The use case represents a key future evolution direction, focusing on the intelligent optimization needs of RAN-side wireless resources (such as spectrum, power, time slots, RBs, etc.). The objective is to dynamically adjust wireless parameters (e.g., nMCS, nRBSize, transmission power) through AI technology to adapt to terminal mobility, service types, and dynamic changes in the wireless environment, thereby achieving core goals of enhanced network throughput, reduced service latency, and balanced user experience.

Observation 2: Traditional RAN-side wireless resource optimization methods suffer from significant limitations: multi-source data (wireless environment data, service data, network configuration data, etc.) are stored and processed in isolation without structured correlation, leading to data feature extraction that relies on manually predefined rules or single-dimensional statistical analysis. This makes it difficult to dynamically identify implicit correlations among multiple factors. Moreover, centralized processing tends to cause latency accumulation, while pure edge processing lacks global views, failing to balance low latency responsiveness and comprehensive optimization decisions.

Proposal 2: The 6GR study of data collection/management framework needs to consider data classification and knowledge graph-based analysis of intrinsic data relationships.

  - Data knowledge Graphs for High-efficiency Feature Extraction

As a novel approach to knowledge representation and knowledge management, knowledge graphs have begun to emerge and play an increasingly critical role across numerous vertical application domains. The network data knowledge graph, constructed based on mobile communication industry data, represents an industry-specific knowledge graph for the mobile communications vertical. Its construction process demands exceptionally high standards of depth, breadth, and accuracy in mobile communication domain knowledge.

It primarily leverages network data knowledge graphs for knowledge representation, correlation analysis, and deep mining, providing effective knowledge rules and knowledge computing support for 6G network intelligence. The network architecture, terminal types, terminal behaviors, data service requirements, and system resources in 6G networks are characterized by high dynamics, strong timeliness, and mutual coupling. Mobile communication data faces multiple challenges, including difficulty in acquiring dispersed data, diverse types with complex structures, and intricate correlations that are hard to mine. Knowledge graphs can effectively clarify various relationships between data fields and communication network metrics, enabling further deep mining on this basis—such as quantifying correlation strength and characterizing feature attributes of data fields and metrics. This systematically organizes collected, scattered, chaotic, coupled, and heterogeneously-sourced data, achieving deep processing from data to information to knowledge, and providing rich raw materials for subsequent network intelligence.

In data feature extraction scenarios, this structured knowledge representation demonstrates targeted advantages:
Relying on the "entity-relationship-attribute" knowledge graph structure, it can rapidly integrate dispersed multi-source heterogeneous data, directly locate core features strongly correlated with network metrics, and avoid ineffective processing of redundant data inherent in traditional extraction—adapting to the strong timeliness demands of network data. Faced with dynamic changes in network status and service requirements, the knowledge graph can update association rules efficiently and automatically adjust feature extraction logic to fit new scenarios without manual intervention, solving the lag issues of traditional fixed-rule extraction. By utilizing pre-existing association templates accumulated from mobile communication domain knowledge, it can directly reuse them in new scenarios, reducing development and adaptation costs for feature extraction. It also supports incremental updates, iterating features only on changed data without full reconstruction, further enhancing processing efficiency. Through built-in industry rules and reasoning capabilities, it can derive potential correlations from existing features, compensating for limitations of single-source data collection and making efficient extracted features more comprehensive to provide more robust support for network intelligent decision-making.

Observation 3: Data knowledge Graph transforms scattered, coupled, and highly dynamic network data into a structured knowledge representation enabling efficient updates and reasoning, providing efficient and precise feature extraction and decision support for 6G network intelligence.

Proposal 3: 6G RAN to standardize data knowledge graph for data feature extraction/classification in data and AIML framework.

- RAN Architecture for High-efficiency Data Feature Extraction

  - Logical Architecture

Figure 4-1 Logical network arch for data framework

As shown in Figure 4-1 , based on the distributed architecture of "L-KGM on the RAN-NDU side + G-KGM on the DPF side," the data knowledge graph provides end-to-end knowledge support for this scenario: When an AI application initiates an optimization request, it transmits core KPIs to the G-KGM on the DPF side. The G-KGM then filters structured knowledge subgraphs strongly correlated with the KPIs(e.g. Filter the knowledge graph nodes within 5 hops of the KPI node.) (including critical fields and basic correlation logic) from the global knowledge graph and pushes them to the L-KGM on the RAN-NDU side. Upon receiving the knowledge subgraphs, the L-KGM completes weight calculation and dynamic updates locally at the RAN, delivering precise decision-making basis for the AI application. When the wireless environment, service type, or terminal status changes, the L-KGM iterates the local subgraphs efficiently, triggering dynamic AI policy adjustments. Simultaneously, it periodically feeds the updated subgraph results back to the G-KGM to support collaborative optimization of the global knowledge graph, forming the core loop of "global knowledge graph empowerment → local iterative updates → cross-domain collaborative closed loop."

This solution effectively addresses the issues of data isolation, incomplete data feature sets, and insufficient low latency capabilities in traditional optimization methods, significantly enhancing the accuracy and response speed of AI application decisions for wireless resource optimization, thereby contributing to improved network resource utilization and user experience balance.

Observation 4: Adopting the distributed data knowledge graph architecture, it enhances the accuracy and latency performance of AI wireless resource optimization through a closed loop of global filtering and pushing, local iterative updating, and periodic feedback coordination.

Proposal 4: RAN and SA to define distributed data functionalities based on "new functional entities + enhancement of existing network elements" that encompasses data collection, classification, feature, and security. Embedded with global + local dual knowledge graph modules, which provide a unified data management framework that satisfies traditional services while adapting to new demands of "beyond connectivity" services such as AI and sensing.

  - Justification for Distributed Data knowledge Graphs Architecture

From the perspective of service evolution: Traditional communication services impose rigid requirements on data processing stability and end-to-end transmission reliability, while emerging "beyond connectivity" services such as AI inference and sensing fusion further demand core capabilities including low latency data processing response, dynamic adaptation, and precise correlation support. Centralized graph architectures suffer from latency accumulation caused by cross-domain data transmission and computing overload at core nodes, making it difficult to simultaneously satisfy the dual demands of both traditional and emerging services.

From the perspective of technical implementation logic: RAN-side AI applications use RAN as the data collection source, and L-KGM interfaces with the DA module in close proximity to complete data processing, avoiding the drawbacks of full-link latency superposition in "collection-transmission-computation" inherent in centralized architectures and ensuring low latency response capability for local services. The CN-side DPF, serving as the network's global control node, needs to integrate data from both RAN and core network domains to achieve large-scale knowledge graph iteration and cross-domain collaborative optimization. Therefore, the "local + global" distributed deployment forms a collaborative closed loop of "lightweight processing at the source + global-scale optimization," ensuring both the immediacy of local services and the consistency of global data and knowledge graphs.

From the perspective of resource utilization: Distributed deployment can fully reuse existing hardware resources and data processing foundations at RAN-NDU and DPF, avoiding resource overload and single-point failure risks at core nodes in centralized architectures, thereby achieving balanced allocation of network resources.

Advantages : Compared with traditional centralized data knowledge graph architectures, the distributed deployment scheme proposed herein offers significant differentiated advantages:

1）Efficient data processing capability meets emerging service demands: L-KGM is deployed on the RAN-NDU side, directly interfacing with locally collected data to complete weight calculation, knowledge graph updates, and feature push without cross-domain transmission, effectively shortening the data processing chain and meeting the demand for low latency decision support in emerging services like AI applications. In contrast, centralized architectures require data to be transmitted over long distances to core nodes for processing, making link latency incompatible with low latency requirements.

2）Significantly optimized network resource utilization efficiency: L-KGM focuses only on lightweight processing of local data, uploading only full-cycle data to G-KGM, which substantially reduces bandwidth consumption by invalid data transmissions. G-KGM concentrates on global knowledge graph iteration and cross-domain collaboration without participating in local low latency computation, avoiding waste of computing resources and achieving precise allocation of bandwidth and computing power.

3）Service adaptation flexibility aligns with network evolution trends: When new AI applications emerge or network scenarios change, G-KGM can quickly complete knowledge sub-graph matching and field filtering, synchronizing dynamic update commands to L-KGM, which then coordinates with DA to adjust collection strategies. This enables rapid response to service changes without global configuration, offering greater flexibility than the unified configuration adjustments required in centralized architectures.

4）Knowledge Graph iteration accuracy is effectively guaranteed: G-KGM integrates multi-domain data for global knowledge graph optimization, avoiding graph distortion caused by data bias in a single region and providing more accurate correlation fields and weight support for AI applications. Meanwhile, the closed-loop mechanism of "global update-local synchronization" ensures consistency between both side graphs.

Observation 5: The distributed data knowledge graph deployment architecture for data feature extraction separates local low latency processing from global optimization, balancing low latency, resource efficiency, high flexibility, and precise iteration.

Proposal 5: Holistically consider the architecture design for data feature extraction and classification from both 6G RAN and 6GC perspectives, with a focus on data knowledge graph standardization.

  - Critical Architecture Challenges to focus on

The periodic full data uploads and knowledge graph update command interactions between L-KGM and G-KGM may impose certain bandwidth pressure, which will become more prominent especially in concurrent multi-AI application scenarios.

Under distributed deployment, G-KGM dynamically adjusts KPI correlation fields based on global knowledge graph iteration results, placing demands on the dynamic adaptation capability of existing data collection and transmission architecture. Rapid response of field adjustments and stable adaptation of transmission links must be ensured.

The G-KGM deployed on CN-side DPF needs to undertake tasks including global knowledge sub-graph matching, large-scale knowledge graph iteration updates, and cross-domain data integration, requiring higher computing power and storage capacity. As a data processing hub on the core network side, DPF possesses the advantage of centralized computing deployment and can meet global graph processing and storage requirements by deploying high-performance computing units and distributed storage devices. The L-KGM on RAN-NDU side focuses on real-time weight calculation of local data, dynamic graph updates, and collection status verification, which are lightweight computing requirements. As the core node for RAN-side data processing, RAN-NDU already possesses edge computing support, requiring no additional hardware resources—only software upgrades to expand graph computing modules are needed to meet requirements. The core network L-KGM has the same function as RAN-NDU-side L-KGM, focusing on lightweight processing of core network-side local data. It can reuse DPF's surplus computing resources and deploy independent modules through virtualization technology, eliminating the need for additional hardware nodes and achieving efficient reuse of computing resources.

Regarding bilateral knowledge graph synchronization consistency, it is necessary to avoid timing deviations or data inconsistency issues between G-KGM global updates and L-KGM local updates.

Observation 6: The RAN architecture faces four major challenges: 1) Bandwidth Pressure: L-KGM/G-KGM full synchronization and real-time updates consume significant bandwidth in multi-AI concurrent scenarios. 2) Dynamic Adaptation: G-KGM's KPI field adjustments demand rapid response and stable adaptation from the data collection/transmission architecture. 3) Hierarchical Computing: CN-side G-KGM requires DPF high-performance hardware; RAN-NDU-side L-KGM lightweight via software upgrade; Core network L-KGM can virtualize DPF resources. 4) Consistency Risk: Temporal misalignment and data inconsistency between G-KGM global updates and L-KGM local updates must be avoided.

Proposal 6: Data framework study should consider new plane (e.g. Data/Info Plane) to enable more efficient utilization of transmission network and wireless resources, meeting the low-latency, high-reliability transmission requirements for real-time feature data. Simultaneously, the Control plane, User plane, and Data plane of the transmission network interface between RAN and 6GC should be optimized or redefined (e.g. GTP-D for group/multi-cast data transfer), while also accounting for collaboration requirements among these three planes.

  - Data functionality defined for 6G RAN

Define functional boundaries for the dedicated data plane as shown in Figure 4-2: the RAN-native NDU (Network Data Unit) module performs data acquisition, categorization, and persistence at the RAN edge, whereas the DPF (Data Plane Function) manages corresponding functions within the core network domain, thereby establishing an end-to-end telemetry chain. The dual-layer knowledge graph architecture (L-KGM/G-KGM) provides robust support for AI applications, facilitated by the DA (Data Agent) and NEF+ (Enhanced Network Exposure Function) interfaces that ensure high-efficiency data exchange both internally and externally.

Figure 4-2 RAN data plane and functionalities

Observation 7: RAN-native dedicated data processing function modules can undertake primary processing of the data plane. The function module should be responsible for RAN-side data collection, classification, and storage, where data feature classification and extraction needs to cover raw data, metric data, and topic data, while the data knowledge graph should possess update capability.

Proposal 7: For the standardization of data feature extraction and classification in RAN, the following dimensions should be considered: 1) Protocol Layer Data Model Standardization, 6G RAN needs to unify L1/L2/L3 three-layer feature dataset specifications. 2) For data knowledge graphs, standardization of knowledge graph storage and query should be considered.

- Dynamic Data Knowledge Graph Refresh

6G exhibits highly dynamic and complex coupling characteristics. Traditional static knowledge graphs cause lag in recommended feature data or decision-making basis due to their inability to capture real-time state changes. Dynamic knowledge graphs, by updating edge weights and node states in real-time, can reflect instantaneous network topology and real-time operational status. Compared with static knowledge graphs and weights, they can provide more temporal data support and gains for intelligent decisions such as network optimization and resource scheduling. However, implementing dynamic knowledge graphs faces multiple technical challenges: legacy network algorithms are rarely designed based on dynamic knowledge graphs; real-time collection and streaming processing of massive state data places extremely high requirements on system architecture; and efficient updating of knowledge graph weights requires resolving efficiency bottlenecks.

Observation 8: Dynamic data knowledge graphs can provide real-time temporal support for intelligent decision-making, but face three major technical challenges: algorithm design, massive data processing architecture, and weight update efficiency.

Proposal 8: It is recommended that RAN and SA jointly initiate, at the earliest opportunity, work on data plane architecture design, protocol design, algorithm performance and overhead evaluation related to dynamic data knowledge graphs.

- Text Proposal for TR 38.914

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN (e.g. Data Feature based Service Requirements Understanding, Service Status Perception, and Intelligent Resource Adaptation).

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- Conclusions

Observation 1: 6G Data framework and AI-native RAN should support cross-domain data collection, integrating application-layer service characteristics (e.g., traffic type, QoE metrics) with radio channel state and load information. Through AI models and Data Feature, it will enable:

-	Automatic service and data type identification and classification.

-	Service-characteristic-based PDU session mapping and dynamic DRB (Data Radio Bearer) configuration.

-	Predictive resource reservation and interference coordination.

Proposal 1: pCR for 38.914 on requirements for architecture and migration for 6G to note data feature for service awareness： The 6G RAN architecture shall support enhanced service awareness in RAN（e.g. Data Feature based Service Requirements Understanding, Service Status Perception, and Intelligent Resource Adaptation）.

Observation 2: Traditional RAN-side wireless resource optimization methods suffer from significant limitations: multi-source data (wireless environment data, service data, network configuration data, etc.) are stored and processed in isolation without structured correlation, leading to data feature extraction that relies on manually predefined rules or single-dimensional statistical analysis. This makes it difficult to dynamically identify implicit correlations among multiple factors. Moreover, centralized processing tends to cause latency accumulation, while pure edge processing lacks global views, failing to balance low latency responsiveness and comprehensive optimization decisions.

Proposal 2: The 6GR study of data collection/management framework needs to consider data classification and knowledge graph-based analysis of intrinsic data relationships.

Observation 3: Data knowledge Graph transforms scattered, coupled, and highly dynamic network data into a structured knowledge representation enabling efficient updates and reasoning, providing efficient and precise feature extraction and decision support for 6G network intelligence.

Proposal 3: 6G RAN to standardize data knowledge graph for data feature extraction/classification in data and AIML framework.

Observation 4: Adopting the distributed data knowledge graph architecture, it enhances the accuracy and latency performance of AI wireless resource optimization through a closed loop of global filtering and pushing, local iterative updating, and periodic feedback coordination.

Proposal 4: RAN and SA to define distributed data functionalities based on "new functional entities + enhancement of existing network elements" that encompasses data collection, classification, feature, and security. Embedded with global + local dual knowledge graph modules, which provide a unified data management framework that satisfies traditional services while adapting to new demands of "beyond connectivity" services such as AI and sensing.

Observation 5: The distributed data knowledge graph deployment architecture for data feature extraction separates local low latency processing from global optimization, balancing low latency, resource efficiency, high flexibility, and precise iteration.

Proposal 5: Holistically consider the architecture design for data feature extraction and classification from both 6G RAN and 6GC perspectives, with a focus on data knowledge graph standardization.

Observation 6: The RAN architecture faces four major challenges: 1) Bandwidth Pressure: L-KGM/G-KGM full synchronization and real-time updates consume significant bandwidth in multi-AI concurrent scenarios. 2) Dynamic Adaptation: G-KGM's KPI field adjustments demand rapid response and stable adaptation from the data collection/transmission architecture. 3) Hierarchical Computing: CN-side G-KGM requires DPF high-performance hardware; RAN-NDU-side L-KGM lightweight via software upgrade; Core network L-KGM can virtualize DPF resources. 4) Consistency Risk: Temporal misalignment and data inconsistency between G-KGM global updates and L-KGM local updates must be avoided.

Proposal 6: Data framework study should consider new plane (e.g. Data/Info Plane) to enable more efficient utilization of transmission network and wireless resources, meeting the low-latency, high-reliability transmission requirements for real-time feature data. Simultaneously, the Control plane, User plane, and Data plane of the transmission network interface between RAN and 6GC should be optimized or redefined (e.g. GTP-D for group/multi-cast data transfer), while also accounting for collaboration requirements among these three planes.

Observation 7: RAN-native dedicated data processing function modules can undertake primary processing of the data plane. The function module should be responsible for RAN-side data collection, classification, and storage, where data feature classification and extraction needs to cover raw data, metric data, and topic data, while the data knowledge graph should possess update capability.

Proposal 7: For the standardization of data feature extraction and classification in RAN, the following dimensions should be considered: 1) Protocol Layer Data Model Standardization, 6G RAN needs to unify L1/L2/L3 three-layer feature dataset specifications. 2) For data knowledge graphs, standardization of knowledge graph storage and query should be considered.

Observation 8: Dynamic data knowledge graphs can provide real-time temporal support for intelligent decision-making, but face three major technical challenges: algorithm design, massive data processing architecture, and weight update efficiency.

Proposal 8: It is recommended that RAN and SA jointly initiate, at the earliest opportunity, work on data plane architecture design, protocol design, algorithm performance and overhead evaluation related to dynamic data knowledge graphs.

- References

- RP-252971, DRAFT Meeting Report for TSG RAN meeting #109, Sep. 2025

- RP-252909, Summary of Discussion on 6G Requirements for architecture and migration

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements; V0.2.0 (2025-09)

- RP-252970, Agenda for RAN #110, Dec. 2025