3GPP TSG-RAN5 Meeting #109	R5-255917

# Dallas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3634 | rev | 1 | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Tables related to SL DRX configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is a editorial issue at Table 4.6.1A-3. - The values of IE SL-DestinationIndex are incorrect at Table 4.6.6-9A. - The value of IE sl-drx-RetransmissionTimer-r17 is missing at Table 4.6.6-9C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Tables 4.6.1A-3, 4.6.6-9A and 4.6.6-9C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1A, 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

–	RRCReconfigurationSidelink

Table 4.6.1A-3: RRCReconfigurationSidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier-r16 |  | RRC-TransactionIdentifier |  | RX |
|  |  | (0..3) |  | TX |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 |  | Not present |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { |  | 1 entry |  | SL_DRB |
| SLRB-Config-r16[1] SEQUENCE { |  |  | entry 1 |  |
| slrb-PC5-ConfigIndex-r16 |  | Not checked |  | TX |
|  |  | 1 |  | RX |
| sl-SDAP-ConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  | 1 entry |  |  |
| SL-PQFI-r16[1] |  | 1 | entry 1 |  |
| } |  |  |  |  |
| sl-MappedQoS-FlowsToReleaseList-r16 |  | Not present |  |  |
| sl-SDAP-Header-r16 |  | absent |  |  |
| } |  |  |  |  |
| sl-PDCP-ConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-PDCP-ConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-PDCP-SN-Size-r16 |  | len12bits |  |  |
| sl-OutOfOrderDelivery-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RLC-ConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-RLC-ConfigPC5-r16 CHOICE { |  |  |  | RX |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-SN-FieldLengthAM-r16 |  | size12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-MAC-LogicalChannelConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-LogicalChannelIdentity-r16 |  | LogicalChannelIdentity with condition DRB1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| slrb-ConfigToReleaseList-r16 |  | Not present |  |  |
| sl-MeasConfig-r16 |  | Not present |  |  |
| sl-MeasConfig-r16 CHOICE { |  |  |  | SL_MEAS |
| setup |  | Set according to parameter given in test case |  | TX |
| setup SEQUENCE { |  |  |  | RX |
| sl-MeasObjectToRemoveList-r16 |  | Not present |  |  |
| sl-MeasObjectToAddModList-r16 |  | SL-MeasObjectList- |  |  |
| sl-ReportConfigToRemoveList-r16 |  | Not present |  |  |
| sl-ReportConfigToAddModList-r16 |  | SL-ReportConfigList |  |  |
| sl-MeasIdToRemoveList-r16 |  | Not present |  |  |
| sl-MeasIdToAddModList-r16 |  | SL-MeasIdList |  |  |
| sl-QuantityConfig-r16 |  | SL-QuantityConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-CSI-RS-Config-r16 |  | Not present |  |  |
| sl-CSI-RS-Config-r16 CHOICE { |  |  |  | SL_CSI |
| setup |  | Set according to parameter given in test case |  | TX |
| setup SEQUENCE { |  |  |  | RX |
| sl-CSI-RS-FreqAllocation-r16 CHOICE { |  |  |  |  |
| sl-OneAntennaPort-r16 |  | 000000000001 |  |  |
| } |  |  |  |  |
| sl-CSI-RS-FirstSymbol-r16 |  | 6 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-ResetConfig-r16 |  | Not present |  |  |
| sl-LatencyBoundCSI-Report-r16 |  | Not present |  |  |
|  |  | 160 |  | SL_CSI |
| lateNonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| nonCriticalExtension |  | Not checked |  | TX and not (L2RelayUE or L2RemoteUE or SL_DRX) |
|  |  | Not present |  | RX and not (L2RelayUE or L2RemoteUE or SL_DRX) |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE, SL_DRX |
| sl-DRX-ConfigUC-PC5-r17 |  | Not checked |  | TX and not SL_DRX |
|  |  | Not present |  | RX and not SL_DRX |
| sl-DRX-ConfigUC-PC5-r17 CHOICE { |  |  |  | SL_DRX |
| setup |  | SL-DRX-ConfigUC |  |  |
| } |  |  |  |  |
| sl-LatencyBoundIUC-Report-r17 |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| sl-RLC-ChannelToReleaseListPC5-r17 |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| sl-RLC-ChannelToAddModListPC5-r17 |  | Not checked |  | TX |
| sl-RLC-ChannelToAddModListPC5-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfigPC5-r17 { |  | 2 entries |  | RX |
| SL-RLC-ChannelConfigPC5-r17[1] |  | SL-RLC-ChannelConfigPC5 with condition PC5RelaySRB2 | entry 1 |  |
| SL-RLC-ChannelConfigPC5-r17[2] |  | SL-RLC-ChannelConfigPC5 with condition PC5RelayDRB1 | entry 2 |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_DRB | To provide peer UE SL DRB related configuration via PC5 RRC |
| SL_MEAS | To provide peer UE SL RSRP measurement and reporting related configuration via PC5 RRC |
| SL_CSI | To provide peer UE SL CSI reporting related configuration via PC5 RRC |
| TX | UE transmits and NR-SS-UE receives. |
| RX | UE receives and NR-SS-UE transmits. |
| SL_DRX | To configure sidelink DRX configuration |

< Unchanged section skipped >

–	SL-DRX-Config

Table 4.6.6-9A: SL-DRX-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-Config-r17 ::= SEQUENCE { |  |  |  |  |
| sl-DRX-ConfigGC-BC-r17 |  | Not Present |  |  |
| sl-DRX-ConfigGC-BC-r17 |  | SL-DRX-ConfigGC-BC with condition SL_DRX_GC_BC |  | SL_DRX_GC_BC |
| sl-DRX-ConfigUC-ToReleaseList-r17 |  | Not Present |  |  |
| sl-DRX-ConfigUC-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIndex-r16 { |  | 1 entry |  | SL_DRX_UC_Release |
| SL-DestinationIndex-r16[1] |  | 0 | entry 1 The first destination UE reported in sl-TxResourceReqList in SidelinkUEInformationNR |  |
| } |  |  |  |  |
| sl-DRX-ConfigUC-ToAddModList-r17 |  | Not Present |  |  |
| sl-DRX-ConfigUC-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DRX-ConfigUC-Info-r17 { |  | 1 entry |  | SL_DRX_UC_AddMod |
| SL-DRX-ConfigUC-Info-r17[1] SEQUENCE { |  |  | entry 1 |  |
| sl-DestinationIndex-r17 |  | 0 | The first destination UE reported in sl-TxResourceReqList in SidelinkUEInformationNR |  |
| sl-DRX-ConfigUC-r17 |  | sl-DRX-ConfigUC |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_DRX_GC_BC | To configure one or multiple sidelink DRX configurations for groupcast and broadcast communication. |
| SL_DRX_UC_Release | To configure the sidelink DRX configurations for corresponding unicast destinations to remove. |
| SL_DRX_UC_AddMod | To configure the sidelink DRX configurations for corresponding unicast destinations to add and/or modify. |

< Unchanged section skipped >

#### –	SL-DRX-ConfigUC

Table 4.6.6-9C: SL-DRX-ConfigUC

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-ConfigUC-r17 ::= SEQUENCE { |  |  |  |  |
| sl-drx-onDurationTimer-r17 CHOICE { |  |  |  |  |
| milliSeconds |  | ms6 |  |  |
| } |  |  |  |  |
| sl-drx-InactivityTimer-r17 |  | ms1280 |  |  |
| sl-drx-HARQ-RTT-Timer1-r17 |  | Not Present |  |  |
| sl-drx-HARQ-RTT-Timer2-r17 |  | Not Present |  |  |
| sl-drx-RetransmissionTimer-r17 |  | sl16 |  |  |
| sl-drx-CycleStartOffset-r17 CHOICE { |  |  |  |  |
| ms10240 |  | 0 |  |  |
| } |  |  |  |  |
| sl-drx-SlotOffset |  | 0 |  |  |
| } |  |  |  |  |

<End of Changed Section>