3GPP TSG RAN Meeting #71	RP-160433

# Göteborg, Sweden, March 7 - 10, 2016

# Source:	Intel Corporation

# Title:	New WID: High-Performance eMTC (HeMTC)

# Document for:	Approval

Agenda Item:	10.1.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title:	High-Performance eMTC (HeMTC)

## Acronym:

## Unique identifier:

NOTE:	If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI. Please tick (X) the applicable box(es) in the table below:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

## 1	3GPP Work Area

| X | Radio Access |
| --- | --- |
|  | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification

This work item is a …

|  | Study Item (go to 2.1) |
| --- | --- |
| X | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

NOTE:	Core, Performance and Testing parts of RAN WIs are usually Building Blocks.
If you are in doubt, please contact MCC.

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 650033 | Further LTE Physical Layer Enhancements for MTC |  |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is …

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1	Stage 1

| Source of external requirements (if any) |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2	Stage 2

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify:

Go to §3.

#### 2.3.3	Stage 3

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) or external document | Clause | Remarks |

If no identified source of stage 2 information, justify:

Go to §3.

#### 2.3.4	Test spec

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5	Other

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4	Work task

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification

It is foreseen that Machine Type of Communications will experience an extremely large growth in the near future; billions of connected devices are expected in 2020. MTC can span a large variety of use cases, ranging from  (1) very low data rate type of applications (e.g. smart metering), (2) medium data rate type of application (e.g. wearable), and (3) higher data rate applications (e.g. industry control and ultra-low latency /ultra-reliable applications). While the latter family (3) of use case might require the use of new air interface such as 5G, the first two families (1) and (2) of uses cases can be optimally supported by solutions based on LTE. It is clear that the use of cellular technologies particularly optimized for typical MTC use cases represents a significant opportunity for new revenue generation for mobile operators. Since Release 12, 3GPP has introduced several solutions with the aim to satisfying different use cases. In Release 12, the WI in [“Low cost & enhanced coverage MTC UE for LTE”] specifies the introduction of a new UE category (Cat-0) which can be supported with reduced complexity compared to those UE supporting the lowest LTE category (Cat-1). In Rel-13, new solutions commonly referred to as ‘Bandwidth-Reduced Low Complexity (BL)’ or ‘Cat-M1’ UE (defined as part of Rel-13 WI on eMTC), and ‘NB-IoT’ UE (to be defined as part of the Rel-13 WI on “Narrowband IoT”) have been (are being) standardized with the aim of not only reducing even further the complexity but also enhancing the coverage and increasing the battery life.

NB-IoT and BL UEs are specifically designed to address low data rate use cases from less than 200kbps for NB-IoT UE to data rates in the order of 300kbps for BL UE when it operated in HD-FDD, and 800kpbs when it operates in FD-FDD.  However, there are other MTC/IoT use cases that require higher data rates,  in the order of several Mbps, involving voice call or larger file transfer for applications related to consumer and medical, smart building and industry such as wearables, health monitoring, alarm systems, vending machines, etc. For instance, it would not be possible to support a wide variety of VoLTE codecs for Cat-M1 UEs without significantly sacrificing the coverage. While such use cases demand the support of delay sensitive services together with higher data rates relative to Rel-13 BL UE, the requirements on coverage enhancement relative to LTE when operating in high data rate/near real time modes and on battery lifetime may be relaxed compared to Rel-13 BL devices.

The aim of this WI is to introduce a new (backward compatible) feature that extends the Rel-13 eMTC design to address (delay sensitive) use cases such as VoLTE with higher data rate requirements compared to those addressed by Rel-13 eMTC or Rel-13 NB-IoT while maximally harvesting the power consumption and complexity reduction features enabled by using the existing Rel-13 eMTC solution.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this Rel-14 Work Item is to specify a new feature for a UE targeting MTC/IoT applications as an evolution of the existing Rel-13 BL (i.e., Cat-M1) devices to support high data rate MTC/IoT use cases in an optimized way. The solution should aim maximal re-use of the design specified as in the Rel-13 eMTC WI.

The following detailed objectives should be addressed as part of the HeMTC WI:

- Specify enhancements to increase peak data rates in downlink (DL) / uplink (UL) and to improve support of delay sensitive services (such as VoLTE) compared to Cat-M1 with target peak data rates for unicast traffic of the order of several Mbps, not less than 3 Mbps/ 2 Mbps for DL/UL respectively assuming FD-FDD as reference [RAN1/RAN2].

  - Increase the values of maximum transport block size (TBS) for both DL and UL beyond those for Cat-M1 [RAN1].

  - Support HARQ-ACK bundling/multiplexing for HD-FDD to enhance peak data rates [RAN1].

  - Support scalable UE bandwidth at RF and baseband within the maximum supported LTE system bandwidth [RAN1/RAN2].

    - Determine the maximum supported bandwidth considering power consumption, complexity, data rates, and latency [RAN1].

    - Enhance Rel-13 eMTC solution to enable scalable bandwidth support in RRC connected mode depending on data rate and latency requirements (e.g. for VoLTE) to effectively trade-off performance against power consumption and device complexity [RAN1/RAN2].

  - Note: Coverage enhancement targets compared to Rel-13 BL UEs could be traded off with the support of high performance and delay sensitive services.

- Specify protocol changes to Rel-13 eMTC solution to enable the above new HeMTC features [RAN 2].

- Specify UE RF core requirements for the above new HeMTC features if necessary [RAN 4].

- Specify RRM core requirements for the above new HeMTC features [RAN 4].

  - Introduction of core requirements to enhance the mobility support with respect to Rel-13 eMTC.

Note: The WI should aim to maximize the support of power consumption and complexity reduction features defined for Rel-13 BL devices, including operation with 1 Rx antenna.

Note: Support for all duplex modes of operations (i.e. Half-duplex FDD, FDD and TDD).

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part

RAN4 to specify the following performance requirements:

  - UE and Base Station demodulation requirements

  - UE RRM performance requirements

### 4.3	RAN time budget proposal

NOTE:	For Wis/Sis under RAN WG5 leadership this section is not filled out. Otherwise:
For a not yet approved WI/SI the rapporteur has to fill out the last row of the table(s) below up to the target date of the WI/SI (if necessary add further tables): Indicate the number of time units (1 TU ~ 2h), i.e. one value for each session/field. If no time unit is needed, leave the field empty.
For WI/SI already approved in the past, the tables below will no longer be updated in the WI/SI description (i.e. the tables reflect the status of the initial approval). But changes can be proposed in the status report of the WI/SI.

| RAN #71 Q2/2016 RAN #72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84bis | 84bis | 93bis | 93bis | 89bis | 91bis | 78bis | 78bis | 78bis | 78bis | 85 | 85 | 94 | 94 | 90 | 92 | 79 | 79 | 79 | 79 |
| 2 |  | 0.5 |  |  |  |  |  |  |  | 3 |  | 1 |  |  |  | 0.5 | 0.5 |  |  |

| RAN #72 Q3/2016 RAN #73 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 86 | 86 | 95 | 95 | 91 | 93 | 80 | 80 | 80 | 80 |
| 3 |  | 1.5 |  |  |  | 0.5 | 0.5 |  |  |

| RAN #73 Q4/2016 RAN #74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 86bis | 86bis | 95bis | 95bis | 91bis | 93bis | 80bis | 80bis | 80bis | 80bis | 87 | 87 | 96 | 96 | 92 | 94 | 81 | 81 | 81 | 81 |
| 2 |  | 1 |  |  |  | 0.5 | 0.5 |  |  |  |  |  |  |  |  |  |  | 0.5 | 0.5 |

| RAN #74 Q1/2017 RAN #75 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 88 | 88 | 97 | 97 | 89 | 95 | 82 | 82 | 82 | 82 |
|  |  |  |  |  |  |  |  | 0.5 | 0.5 |

L: LTE, U: UMTS, J: Joint, RD: RRM/demodulation

NOTE:	In case further explanation of the time budget proposal is needed, then please explain this below.

## 5	Service Aspects

## 6	MMI-Aspects

## 7	Charging Aspects

## 8	Security Aspects

## 9	Impacts

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  |  |
| Don’t know |  |  |  | X | X |

## 10	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
|  | High-Performance eMTC (HeMTC) | RAN1 | RAN 2 |  | RAN #74 |  |

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |
| 36.201 |  |  | RAN#74 | Core part |
| 36.211 |  |  | RAN#74 | Core part |
| 36.212 |  |  | RAN#74 | Core part |
| 36.213 |  |  | RAN#74 | Core part |
| 36.214 |  |  | RAN#74 | Core part |
| 36.300 |  |  | RAN#74 | Core part |
| 36.302 |  |  | RAN#74 | Core part |
| 36.306 |  |  | RAN#74 | Core part |
| 36.321 |  |  | RAN#74 | Core part |
| 36.331 |  |  | RAN#74 | Core part |
| 36.101 |  |  | RAN#74 | Core part |
| 36.104 |  |  | RAN#74 | Core part |
| 36.133 |  |  | RAN#74 | Core part |
| 36.101 |  |  | RAN#75 | Performance part |
| 36.104 |  |  | RAN#75 | Performance part |
| 36.133 |  |  | RAN#75 | Performance part |

The work in the WGs will assume to reuse the existing specifications listed above. During the final approval process at RAN, RAN can decide to approve the provided CRs and/or to copy them into (additional) new specifications.

NOTE:	If this is a RAN WID including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Comments for each spec.
By default a new specs can only be new for one of both parts.

NOTE:	If this is a RAN WID including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Comments for each spec.
If an existing spec is affected by both (Core part and Perf. Part), then it has to be listed twice with appropriate approval dates.

## 11	Work item rapporteur(s)

Chatterjee, Debdeep

Company:	Intel Corporation

Email: debdeep.chatterjee@intel.com

## 12	Work item leadership

3GPP RAN WG1

NOTE:	If this is a RAN WID including Core and Perf. Part, then this WG specifies the WG leading the Core part.
RAN WG4 is by default leading the Perf. Part.

## 13	Supporting Individual Members

Supporting IM name

Intel Corporation

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"