3GPP TSG RAN meeting #74	RP-162200

Vienna, Austria, December5 - 8, 2016

Title:	On remaining work of LTE UE TRP and TRS requirements

Source: 	OPPO, Xiaomi, Coolpad, vivo

Agenda item:	10.5.4

Document for:	Discussion

1	Introduction

In RAN4 #80bis and RAN4 #81 meetings, there were lots of discussions on LTE UE TRP/TRS measurements results and framework improvement [1~11]. The measurement results summary and analysis has been provided in [12]. However, there are still substantial amount of remaining work on this WI, which will be discussed in this paper.

2	Discussion

On defining LTE UE TRP/TRS requirements, it is utterly important that measurements results are from multiple sources. It is preferable that data from both UE vendors and operators are collected and analysed, and based on which, a reasonable requirement can be specified.  As in [12], it is seen that for only Band 1, Band 3, Band 7 and Band 20, there are reasonable amount of measurement results available for analysis.  However, in the agreed LTE TRP TRS work plan [13], up to 21 LTE bands are listed for study. Therefore it is observed that more data collections are required to cover the LTE bands planned in this WI as in [13].

Observation 1：Further data collections are required to cover more LTE bands

In RAN4 #80 meeting [13], CA and non-CA cases were agreed to be studied in the framework improvement scope. As recognized in [11] and [13] that, inter-band CA capable UEs may have different requirements than non-CA capable UEs. However, till now, no sufficient studies and analysis have been carried out on it. It is noted that although some measurement results [12] distinguish generally between CA and non-CA cases, from UE vendors’ point of view, it is much more meaningful to categorize inter-band CA combinations properly and in details for data collection and analysis. Specifically, RF design for CA combinations across low, middle and high bands are much more challenging than RF design for CA combinations among just similar bands, which may incur different TRS and/or TRP requirements given reasonable industrial design (ID), time-to-market and/or cost constrains.

Therefore, more efforts are still required to improve the framework, e.g., finer categorization of CA cases into the at least following possible cases:

- Intra-band contiguous

- Low band + high band

- Low band + middle band

- Middle band + high band

- Low band + middle band + high band

Observation2：More efforts are required to improve the framework, e.g., finer categorization of CA cases

With these observations, we see more time is required to complete this WI. So we support the proposal from

rapporteurs [14] that this WI shall be extended for 3 months.

3	Proposal

Proposal：Improvement of the framework e.g. finer categorization of CA cases should be targeted for next 3 months.

4	References

[1]	R4-167348	BHH test results for LTE, Verizon UK Ltd.

[2]	R4-167737	Measured TRP/TRS results for some Sony phones, Sony Mobile Communications

[3]	R4-167739	AAU measured right phantom hand LTE TRS, Sony Mobile Communications

[4]	R4-167402	LTE OTA Measurement results of BHH TRP/TRS, OPPO

[5]	R4-168097	TRP & TRS data for LTE handphantom, Qualcomm Incorporated

[6]	R4-168867	Additional measurement results of BHH LTETRP/TRS, NTT DOCOMO INC.

[7]	R4-1609802	Measurement results of BHH and tablet TRP/TRS for Band 21, NTT DOCOMO INC.

[8]	R4-1610277	LTE smartphone TRP and TRS measurements for Bands 3, 7 and 20, ORANGE

[9]	R4-1610513	LTE handset TRP/TRS measurements, Intel Corporation

[10]	R4-168312		Discussion on PDA Hand/Wide Hand impacts on LTE UE TRP and TRS requirements, OPPO, CATR, CMCC, China Telecom, Xiaomi, Coolpad, Vivo, MediaTek, Qualcomm, ZTE, Intel.

[11]	R4-1610781	WF on LTE TRP and TRS framework improvement, OPPO, Motorola mobility, Xiaomi, Vivo, Coolpad, CATR

[12]	R4-1610780	LTE TRP and TRS BHH measurement results summary for B1, B3, B7, B20, Vodafone Group Plc, Telecom Italia

[13]    R4-167201	Way forward on LTE TRP TRS framework improvement and work plan

[14]    RP-162060	Status Report for LTE UE TRP and TRS and UTRA Hand Phantom related UE TRP and TRS Requirements

[15]	RP-161323	Revised WID: LTE UE TRP and TRS and UTRA Hand Phantom related UE TRP and TRS Requirements