3GPP TSG RAN#74	TDoc RP-162261

December 5-8, 2016	Agenda Item 9.1

Vienna, Austria	Revision of: NN-YYTTTT

Source: MediaTek

Document for: discussion

# Scope of 5G NR Phase 1

# Introduction

With 5G NR normative work poised to start at RAN#75/Mar2017, preliminary discussions are needed at RAN#74 on the scope of the normative work aiming at Rel-15 timeframe.

This contribution provides MediaTek views on essential aspects to be covered in Phase 1 primarily from the point of view of RAN1 and RAN2. Additional considerations on 5GC connectivity and Phase 2 (study) are also introduced.

# RAN1

| Item | Description | Standalone/Anchor NR specific |
| --- | --- | --- |
| Numerology | - Sub6 15kHz subcarrier spacing (5, 10, 20, 25MHz UE BW) 60kHz subcarrier spacing (20, 40, 80, 100MHz UE BW) - Super6 60kHz subcarrier spacing (20, 40, 80, 100MHz UE BW) 240kHz subcarrier spacing (80, 160, 320, 400MHz UE BW) | No |
| Frame structure | - Forward compatibility with dynamic TDD: 4 slot types (DL-only, UL-only, DL-major, UL-major) - Minimal scheduling unit for eMBB: 0.5ms for 15kHz subcarrier spacing 0.125ms for 60kHz subcarrier spacing 0.125ms for 240kHz subcarrier spacing - Flexible timing relationship: no self-contained operation - Forward compatibility for eMBB+URLLC radio resource multiplexing - DL: semi-static/dynamic multiplexing - UL: semi-static multiplexing - Bandwidth extension - Single carrier - Carrier aggregation within NR - UE-specific bandwidth adaptation in both DL and UL for single carrier operation | No |
| MIMO | Beam management, reference signal design for beam management, sharing of the beam management reference signals with initial access reference signals | No |
| Initial access (NR Anchor only i.e. SA NR or NSA eLTE) | - “Narrowband” anchor-based initial access procedure for eMBB/URLLC - SS block transmission with new SS design with 40ms periodicity - “Narrowband” anchor-based DL RRM measurements - Synchronization Signal design, RACH signal design considering performance and complexity trade-off | No |
| Channel coding | - LDPC for data, Polar for control | No |
| Waveform | - DL: OFDM - UL: OFDM + DFT-S-OFDM | No |
| Modulation | - QPSK~256QAM (no 1024QAM) | No |
| DL Control | - Slot-type/configuration signalling | No |
| UL Control | - Preferred: 2-step RACH for latency reduction | Yes |

# RAN2

| User Plane |  |  |
| --- | --- | --- |
| Item | Description | Standalone/Anchor NR specific |
| MAC, RLC, PDCP | Support necessary functions for - High throughput; and - Low latency; and - RLC TM/UM/AM - Header compression (PDCP) - Security (PDCP) - Centralized or distributed RAN (“CU/DU split”) | No |
| QoS flow / DRB / Logical Channel / Numerology mapping | - Full QoS support in L2 (QoS flow / DRB mapping) - Forward compatibility: one MAC entity for multiple numerologies however restrict to a single numerology per UE in Rel-15 | No |
| NR-LTE tight interworking | EPS3 scenario (mobility via LTE) | N/A |
|  | SA NR Scenario (or NSA with NR Anchor) - Intra NGS inter RAT (NR<>eLTE) and inter EPS/NGS inter RAT NR<>LTE - Support MCG bearer and SCG bearer | Yes |
| Others: DRX, HARQ | - Per UE DRX, no per service DRX (most stringent DRX used) - Asynchronous HARQ | No |
| Others: RACH | - 4-msg random access procedure supporting DL and UL beam-forming - Preferred: 2-step RACH for latency reduction | Yes |

| Control Plane |  |  |
| --- | --- | --- |
| Item | Description | Standalone/Anchor NR specific |
| System Information | - On-demand unicast SI - Wide-specific SI instead of cell-specific SI | No |
| Access Control | - Generic access control | No |
| UE capability | - Take into account both NR and LTE and enable HW implementation flexibility to avoid over-dimensioning | No |
| LTE-NR Mobility | Sub6 - Full mobility support - Inter RAT LTE-NR handover through CN or via Xn – Hard HO / Make-before-break HO | Yes |
|  | Super6 - Idle mode mobility supported (UE autonomous) - No connected mode mobility - Inter-RAT LTE-NR seamless handover through multi-connectivity “soft HO” | Yes |
| RRC Security (@PDCP) | Ciphering and integrity protection | No |
| RRC Paging | Basic | Yes |
| RRC Inactive | Preferred: Data transmission without state transition | Yes |

| Mobility with beam-forming |  |  |
| --- | --- | --- |
| Item | Description | Standalone/Anchor NR specific |
| Connected mode mobility with beam-forming | - UE cell evaluation based on aggregated beam measurement - “0”ms interruption | No |
| RLM/RLF | - Support recovery with cell with multiple beams | No |
| Idle mode mobility with beam-forming | - Cell evaluation based on control beam measurement - No UL RRM | No |

# 5GC Connectivity

Besides the necessary work in RAN1 and RAN2 to support the definition of a new radio access (“NR” now known as “5G-RA”) in both standalone and non-standalone configurations, work is also necessary in RAN2 and RAN3 to support connectivity to the 5G Core in Phase 1 (NG2/NG3).

# NR Phase 2 considerations

With an aggressive schedule for NR Standardization in Phase 1, any non-essential aspect of NR should be deemed as a potential candidate for being deferred to Phase 2. We therefore recommend TSG RAN to identify the essential aspects of NR (without which NR cannot work), distinctively from the non-essential ones (e.g. nice-to-have). The latter could be discussed on a case-by-case basis i.e. whether to be covered in Phase 1 or to be deferred to Phase 2.

In addition, items that were put on hold for consideration at RAN#75/Mar2017 (see RP-161914) are as such candidates for Phase 2 – we recommend discussions as to which should be considered for Phase 2 (study) or not.

# Conclusions

With an aggressive schedule for NR Standardization in Phase 1, any non-essential aspect of NR should be deemed as a potential candidate for being deferred to Phase 2. We therefore recommend TSG RAN to identify the essential aspects of NR (without which NR cannot work), distinctively from the non-essential ones (e.g. nice-to-have). The latter should be discussed on a case-by-case basis i.e. whether to be covered in Phase 1 or to be deferred to Phase 2 whilst being especially mindful about the WG workload.

The timeline endorsed at TSG#72 ought to be followed.

Waveforms above  40GHz, mMTC, [Flexible duplex of paired spectrum], Interworking with non-3GPP systems, Wireless relay, Satellite communication, Air-to-ground and light air craft  communications, Extreme long distance coverage, Sidelink, V2V and V2X, Multimedia Broadcast/Multicast Service, Shared spectrum and unlicensed spectrum, [Location/positioning functionality], Public warning/emergency alert, New SON functionality