3GPP TSG RAN Meeting #74                                               	RP-162449

Vienna, Austria, 5th –8th December 2016

Source: 	Thales, SES S.A., DISH Network, Fraunhofer IIS

Title:	Satellite specifics for the forward compatibility of NextGen/New Radio phase 1 with the integration of satellite capabilities

Agenda Item:	9.2.2 Study on New Radio (NR) Access Technology [FS_NR_newRAT]

Document for:	Discussion

# Introduction

This paper aims at providing a preliminary list of satellite specifics to be taken into account in the specifications of NextGen/New Radio phase 1 foundational architecture and protocols to ensure forward compatibility with the future integration of satellite capabilities as part of phase 2.

# Specifics of satellite capabilities in 5G

The NextGen/NR architecture and protocols specified in phase 1 shall be designed with the necessary flexibility to take into account the specifics of satellite capabilities in 5G (preliminary analysis):

## Satellite as radio access network

The specification of the NR architecture, protocols and radio interfaces (for both eMBB and URLLC) shall take into account:

- Flat fading but limited multi path propagation channel, typically Rician type (see  R1-1612794);

- Propagation delay (earth-space-earth) which ranges up to 250 – 300 ms for GEO satellite systems, up to 50 – 100 ms for MEO satellite systems, and up to 20 - 30ms for LEO satellite systems;

- Larger delay variation due to large cells (several hundreds of km of diameter);

- Doppler shift and spread effect for moving cells in case of LEO and MEO satellite constellations;

- Broadcast/multicast capabilities.

This requires flexibility especially in the design of phase 1 features:

- Channel bandwidth, Modcod capabilities, guard time interval, interleaving, pilot density for synchronisation;

- RACH (Contention based, Two steps RACH, UL grant free access), ARQ/HARQ (ACK less), Power control / Channel State Information  / modulation and coding control loop;

- UL access grants with increased distribution delay;

- Definition and scheduling of QoS classes also exploiting broadcast/multicast channels;

- Adaptation of protocols for variable and long delay, e.g. ARQ/HARQ and/or Performance Enhancement proxies;

- Mobility management: Service/session continuity, hand-over, paging/tracking areas, network discovery and selection;

- Traffic splitting/steering between several access networks.

## Satellite as transport network (RAN – CN interface)

The specification of the NextGen architecture and protocols shall take into account different network topologies and deployment contexts such as:

- RAN backhaul on board a moving platform like an airplane, train, or a ferry or cruise ship leading to possible relocation of the anchor points;

- RAN backhaul via a transport network aggregating several parallel transport network links among which a satellite link (e.g. in underserved areas).

This requires flexibility especially in the design of phase 1 features

- Initial access;

- Traffic splitting/steering between several transport network links each featuring different latency, bandwidth, Jitter, Error rate and/or broadcast/multicast capability;

- Definition and scheduling of QoS classes;

- Service/session continuity: Dynamic Reselection of user path for a group of UEs or individual UEs;

- Adaptation of protocols to variable and long delays and/or Performance Enhancement proxies.

# Recommendations

3GPP and relevant working groups are invited to take into account satellite specifics when designing the NextGen/New Radio architecture, protocols and radio interfaces.