3GPP TSG RAN#74	RP-162465

December 5-8, 2016	Agenda Item 9.1

Vienna, Austria	Revision of: RP-162261

Source: MediaTek

Document for: discussion

Scope of 5G NR Phase 1

Introduction

With 5G NR normative work poised to start at RAN#75/Mar2017, preliminary discussions are needed at RAN#74 on the scope of the normative work aiming at Rel-15 timeframe.

This contribution provides MediaTek views on essential aspects to be covered in Phase 1 primarily from the point of view of RAN1 and RAN2. Additional considerations on 5GC connectivity and Phase 2 (study) are also introduced.

RAN1

| Item | Description | Standalone/Anchor NR specific |
| --- | --- | --- |
| Numerology | - Sub6 15kHz subcarrier spacing (5, 10, 20, 25MHz UE BW) 60kHz subcarrier spacing (20, 40, 80, 100MHz UE BW) - Super6 60kHz subcarrier spacing (20, 40, 80, 100MHz UE BW) 240kHz subcarrier spacing (80, 160, 320, 400MHz UE BW) - Normal CP for 15KHz, 60KHz and 240KHz subcarrier spacing - Extended CP for 60KHz subcarrier spacing only | No |
| Frame structure | - Forward compatibility with dynamic TDD: 4 slot types (DL-only, UL-only, DL-major, UL-major) - Minimal scheduling unit for eMBB: 0.5ms for 15kHz subcarrier spacing 0.125ms for 60kHz subcarrier spacing 0.125ms for 240kHz subcarrier spacing - Flexible timing relationship: no self-contained operation - Forward compatibility for eMBB+URLLC radio resource multiplexing - DL: semi-static/dynamic multiplexing - UL: semi-static multiplexing - Bandwidth extension - Single carrier - Carrier aggregation within NR - UE-specific bandwidth adaptation in both DL and UL for single carrier operation | No |
| MIMO | - Beam management, reference signal design for beam management, sharing of the beam management reference signals with initial access reference signals - Reference signals - UE-specific demodulation reference signals for up to [8] antenna ports - Reference signals for channel state information measurement for up to [32] antenna ports | No |
| Initial access (NR Anchor only i.e. SA NR or NSA eLTE) | - “Narrowband” anchor-based initial access procedure for eMBB/URLLC - SS block transmission with new SS design with 40ms periodicity - “Narrowband” anchor-based DL RRM measurements - Synchronization Signal design, RACH signal design considering performance and complexity trade-off | No |
| Mobility | - Reference signals for DL RRM measurement (no UL RRM) |  |
| Channel coding | - LDPC for data, Polar for control | No |
| Waveform | - DL: OFDM - UL: OFDM + DFT-S-OFDM | No |
| Modulation | - Control (DL/UL): QPSK - Data (UL/DL): QPSK~256QAM (no 1024QAM) | No |
| DL Control | - Slot-type/configuration signaling - Single-stage DCI with forward compatibility to 2-stage DCI - Support scheduling for unicast/multicast/broadcast message or data | No |
| UL Control | - Preferred: 2-step RACH for latency reduction - UCI on PUCCH with short and long duration - UCI on PUSCH | Yes |

RAN2

| User Plane |  |  |
| --- | --- | --- |
| Item | Description | Standalone/Anchor NR specific |
| MAC, RLC, PDCP | Support necessary functions for - High throughput; and - Low latency; and - RLC TM/UM/AM - Header compression (PDCP) - Security (PDCP) - Centralized or distributed RAN (“CU/DU split”) | No |
| QoS flow / DRB / Logical Channel / Numerology mapping | - Full QoS support in L2 (QoS flow / DRB mapping) - Forward compatibility: one MAC entity for multiple numerologies however restrict to a single numerology per UE (per carrier/per band) in Rel-15 | No |
| NR-LTE tight interworking | EPS3 scenario: Dual Connectivity with X2-like Xn interface | N/A |
|  | - NGS4, NGS7 scenarios:Support MCG bearer and SCG bearer - No split bearers | No |
| Others: DRX, HARQ | - Per UE DRX, no per service DRX (most stringent DRX used) - Asynchronous HARQ | No |
| Others: RACH | - 4-msg random access procedure supporting DL and UL beam-forming - Preferred: 2-step RACH for latency reduction | Yes |

| Control Plane |  |  |
| --- | --- | --- |
| Item | Description | Standalone/Anchor NR specific |
| System Information | - On-demand unicast SI - Area-specific SI instead of cell-specific SI | No |
| Access Control | - Generic access control | No |
| UE capability | - Take into account both NR and LTE and enable HW implementation flexibility to avoid over-dimensioning | No |
| LTENR Mobility | Sub6 - Full inter-RAT mobility support (inter-system EPS/NGS and intra NGS) - Inter RAT LTE-NR HO through CN or via Xn – Hard HO / Make-before-break HO | Yes |
|  | Super6 - Idle mode mobility supported (UE autonomous) - No handover, connected mode inter-cell mobility only by LTE-NR tight interworking | Yes |
| NR Sub6 NR Sub6 Mobility | - Full intra-RAT mobility support | Yes |
| NR Sub6 NR Super6 mobility | - Idle mode mobility supported (UE autonomous) - No handover, connected mode inter-cell mobility only by LTE-NR tight interworking | Yes |
| NR Super6 NR Super6 mobility | - Idle mode mobility supported (UE autonomous) - No handover, connected mode inter-cell mobility only by LTE-NR tight interworking | Yes |
| RRC Security (@PDCP) | Ciphering and integrity protection | No |
| RRC Paging | Basic | Yes |
| RRC Inactive | Preferred: Data transmission without state transition | No |

| Mobility with beam-forming |  |  |
| --- | --- | --- |
| Item | Description | Standalone/Anchor NR specific |
| Connected mode mobility with beam-forming | - UE cell evaluation based on aggregated beam measurement - “0”ms interruption | No |
| RLM/RLF | - Support recovery with cell with multiple beams | No |
| Idle mode mobility with beam-forming | - Cell evaluation based on control beam measurement - No UL RRM | No |

5GC Connectivity

Besides the necessary work in RAN1 and RAN2 to support the definition of a new radio access (“NR” now known as “5G-RA”) in both standalone and non-standalone configurations, work is also necessary in RAN2 and RAN3 to support connectivity to the 5G Core in Phase 1 (NG2/NG3).

NR Phase 2 considerations

With an aggressive schedule for NR Standardization in Phase 1, any non-essential aspect of NR should be deemed as a potential candidate for being deferred to Phase 2. We therefore recommend TSG RAN to identify the essential aspects of NR (without which NR cannot work i.e. without which applicable requirements in TR38.913 cannot be met), distinctively from the non-essential ones (e.g. nice-to-have). The latter could be discussed on a case-by-case basis i.e. whether to be covered in Phase 1 or to be deferred to Phase 2.

In addition, items that were put on hold for consideration at RAN#75/Mar2017 (see RP-161914) are as such candidates for Phase 2 – we recommend discussions as to which should be considered for Phase 2 (study) or not.

Conclusions

With an aggressive schedule for NR Standardization in Phase 1, any non-essential aspect of NR should be deemed as a potential candidate for being deferred to Phase 2. We therefore recommend TSG RAN to identify the essential aspects of NR (without which NR cannot work i.e. without which applicable requirements in TR38.913 cannot be met), distinctively from the non-essential ones (e.g. nice-to-have). The latter should be discussed on a case-by-case basis i.e. whether to be covered in Phase 1 or to be deferred to Phase 2 or beyond whilst being especially mindful about the WG workload.

The timeline endorsed at TSG#72 ought to be followed.

E.g. 5, 10, 20MHz

Waveforms above  40GHz, mMTC, [Flexible duplex of paired spectrum], Interworking with non-3GPP systems, Wireless relay, Satellite communication, Air-to-ground and light air craft  communications, Extreme long distance coverage, Sidelink, V2V and V2X, Multimedia Broadcast/Multicast Service, Shared spectrum and unlicensed spectrum, [Location/positioning functionality], Public warning/emergency alert, New SON functionality