3GPP TSG RAN meeting #84										RP-191112

Newport Beach, USA, June 3-6, 2019

## Status Report to TSG

Agenda item:			9.3.11

| WI / SI Name | Study on 7 – 24 GHz frequency range for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_7to24GHz_NR |  |  |  |  |
| Unique ID | 820060 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190373 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2019 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 15 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 4 |
| --- | --- | --- |
| Rapporteur | Name | Michal Szydelko |
|  | Company | Huawei |
|  | Email | michal.szydelko@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#90 (Athens, Greece, 25 February – 1 March 2019)

The study item work has been triggered during RAN4#90 meeting. Number of contributions were submitted to the General part (including regulatory survey), UE-, and BS-related agendas in [1-18].

During RAN4#90 meeting, it was agreed to request for an internal TR 38.8xx series technical report for this SI (TR 38.820 was assigned by MCC afterwards). Draft skeleton was endorsed in [18], including General part, UE-, and BS-specific part.

Work plan was approved in [17], as extracted below:

| 3GPP meeting | Dates | Location | Proposed workplan |
| --- | --- | --- | --- |
| 3GPP RAN4 #90 | 25 Feb - 1 Mar 2019 | Athens | - Work kick-off in RAN4 - Formulation of the workplan - Discussion on TR skeleton / agreement - Inputs to the selection of proxy/example frequencies - Decision on the internal/external TR |
| 3GPP RAN #83 | 18 - 21 Mar 2019 | Shenzhen | Status report SID updates on the TR (internal vs. external) |
| 3GPP RAN4 #90-Bis | 8 - 12 Apr 2019 | Xi'an | - TR: further extensions of the skeleton structure - Selection of proxy/example frequencies: final decision - Regulatory survey: collection of inputs NOTE: no TPs to TR until RAN92bis/ RAN#93 - System parameters: discussion on expected modifications in case of future bands, and related TR scope - RF technology aspects: collect inputs - e.g. PA properties, antenna properties, receiver properties in particular noise factor, filter capabilities, noise figure, phase noise, etc. - BS specifications and requirements: continue discussion - Expected impacted on specs and requirements - Comparison with existing FR1 and FR2 requirements - Further discuss need for co-location requirements - UE specifications and requirements: continue discussion - Expected impacted on specs and requirements, e.g. comparison with existing FR1 and FR2 requirements - BS RF architecture: - Discussion on the potential BS architecture variants feasible (conducted, hybrid, OTA) - Consider antenna array sized - UE RF architecture: Discussion on the potential UE architecture variants feasible (conducted, hybrid, OTA) - EMC: discussion on EMC requirements impact |
| 3GPP RAN4 #91 | 13 - 17 May 2019 | Reno, Nevada | - System parameters: continue discussion - RF technology aspects: continue discussion - Deployment scenarios: trigger discussion on possible scenarios, considering technology capabilities - BS specifications and requirements: continue discussion - UE specifications and requirements: continue discussion - BS RF architecture: continue discussion on requirements classification - UE RF architecture: continue discussion on requirements classification - EMC: conclusion on requirements impact |
| 3GPP RAN #84 | 3 - 6 Jun 2019 | Newport Beach California | Status report |
| 3GPP RAN4 #92 | 26 - 30 Aug 2019 | Ljubljana | - System parameters: continue discussion - RF technology aspects: agree what and how to capture in TR - Key parameters: Begin consideration of which key parameters may be concluded on and ranges - E.g. ACLR, Noise Figure, Phase noise and need for CPE compensation, etc. - Deployment scenarios: continue discussion - BS specifications and requirements: continue discussion - Conclude on need for co-location requirements - UE specifications and requirements: continue discussion - BS RF architecture: continue discussion on requirements classification - UE RF architecture: continue discussion on requirements classification - FR terminology: Start discussion of how to deal with range - E.g. whether evaluated sub-ranges can be merged, whether new FR are needed, etc. - TR: preparation of the TR submission to RAN#85 (for information) |
| 3GPP RAN #85 | 16 - 19 Sep 2019 | Newport Beach California | Status report TR for information |
| 3GPP RAN4 #92-Bis | 14 - 18 Oct 2019 | China | - Regulatory survey: - Continue collection of inputs - Decide how to capture the regional regulatory aspects in TR - System parameters: completion - RF technology aspects: capture technology characteristics in TR - Key parameters: continue discussion - Deployment scenarios: completion - BS specifications and requirements: capture likely impact in TR, completion - UE specifications and requirements: capture likely impact in TR, completion - BS RF architecture: continue discussion on requirements classification - UE RF architecture: continue discussion on requirements classification - Useful frequency ranges for Mobile use: re-open discussion and decide what to capture in TR (if not already done) |
| 3GPP RAN4 #93 | 18 - 22 Nov 2019 | Reno, Nevada | - Regulatory survey: completion - Key parameters: Capture conclusions in TR, completion - BS RF architecture: completion - UE RF architecture: completion - Completion of all outstanding and remaining issues - Useful frequency ranges for Mobile use: capture in TR, completion - TR: completion - TR updates based on WRC-19 (28.10- 22.11.2019) conclusions |
| 3GPP RAN #86 | 9 - 12 Dec 2019 | Sitges | Status report TR for approval |

RAN4#90bis (Xi’an, China, 8 – 12 April, 2019)

Documents [19 - 48] were submitted to RAN4#90bis. Based on the discussion papers submitted, the following way forward documents were agreed during RAN4#90bis meeting:

- WF on General aspects for 7 -24GHz SI [47]: the following areas of the general part of the SI were addressed:

- FR1/FR2 differentiators: WF captured the required study for the FR1/FR2 differentiators, in order to understand the consequences of potential FR1/FR2 extensions in RAN4 and other WGs. For that purpose, multiple NR technology items which differ among FR1 and FR2 were identified as valid inputs to be considered during further, e.g.:

  - Conducted vs. OTA interface,

  - Set of SCS and CHBW supported,

  - Broadcast channels coverage (beam-sweeping vs. static),

  - Set of supported modulation and coding schemes,

  - Duplex mode,

  - Single/multi-band support,

  - Consideration of deployment scenarios,

  - Consideration of UE types and usage conditions,

  - For BS only:

    - BS types,

    - Presence or lack of certain BS requirements,

    - Concept for BS receiver requirements,

    - Requirement values common in some cases across an FR.

- Deployment scenarios: interested companies were encouraged to provide further inputs on the potential 7 - 24 GHz deployment scenarios.

- Example frequency ranges: the example frequency ranges and example frequencies were selected based on the analysis of inputs from multiple companies. The frequency ranges for the future RF technology analyses were considered as still very flexible (one boundary spanning 10 – 13 GHz and the other boundary spanning 16 – 18 GHz). Those ranges were clarified not to be linked with any potential FR1/FR2 extensions in future RAN4 WI; based on the RF technology analyses those ranges are still subject to adjustments:

  - Example range 1: 7.125 – [10-13] GHz, example frequency: 10 GHz

  - Example range 2: [10-13] – [16-18] GHz, example frequency: 15 GHz

  - Example range 3: [16-18] – 24.25 GHz, example frequency: 20 GHz

- WF for UE requirements type study within the 7-24GHz [46]: high level WF captured the need to study the range for conducted/OTA requirements, study UE RF (e.g. UE architecture, ACLR impact on system performance, antenna size, PA performance, filtering, testability, etc.), and to provide RF component performance data.

Furthermore, based on the online discussion, it was also captured in the meeting minutes that the common understanding is that the system performance impact is not precluded from study scope.

- WF on BS study for 7 - 24 GHz [48]: the following items were captured in the WF:

- BS classes,

- BS types,

- Power amplifiers,

- Phase noise,

- RF filtering capabilities,

- Rx requirements (including out of band blocking requirement),

- Noise Figure,

- Co-location requirements,

- EMC requirements,

- single-band operation,

- TDD OFF power requirement.

Furthermore, in this WF it was captured that it is sufficient to consider single-band operation only for the study item phase.

For the BS classes for 7 – 24 GHz range, it was captured that the current set of BS classes can be supported (subject to the spectrum scenarios considerations), i.e. Wide Area, Medium Range, Local Area.

Skeleton of the TR 38.820 was updated in [20].

RAN4#91 (Reno, Nevada, US, 13 – 17 May, 2019)

Documents [49 – 102] were submitted to RAN4#91.

The 7-24 GHz SI adhoc was held with the focus on the BS part. Adhoc meeting minutes were agreed in [91].

Based on the discussion papers submitted, the following way forward documents were agreed during RAN4#91 meeting:

- WF on FR1/FR2 differences [101]: this WF captured differences between FR1 and FR2 requirements for different areas of the 3GPP specifications in the different RAN working groups. The aim is to further study those aspects and to capture in the TR the common understanding on the potential impact of the FR1/FR2 extensions to RAN4 and other RAN working groups.

- WF on the deployment scenarios for 7-24 GHz [99]: based on the submitted contributions the initial set of deployment scenario applicable to 7 – 24 GHz range was proposed, including eMBB, V2X, IAB and the optional HST:

  - Indoor hotspot

  - Dense urban

  - Urban macro

  - V2X

    - Highway scenario

    - Urban Grid for Connected Car

  - High speed train (optional)

  - IAB

The above set of deployment scenarios is subject to the RF technology feasibility studies (both BS and UE). There was no intention to introduce prioritization to the analyses of deployment scenarios. It is possible that not all of the listed deployment scenarios will be considered in the conclusion of the SI.

The following aspects are to be further studied with the aim to provide feasible range of values:

- Aggregated system bandwidth, also considering CA and DC deployments,

- BS antenna arrays sizes and UE antenna assumptions,

- Relationship of requirement ranges on emissions, sensitivity etc., coverage, feasible array size, leading to a conclusion on what drives the feasibility of covering the scenarios.

- WF on BS requirements sets for 7-24 GHz range [93]: this WF captured the required study to decide on the BS requirement sets for WA, MR and LA BS. This WF included potential considerations on the output power limits.

It was agreed that for the NR BS requirements section in TR 38.820, a summary tables will be created for Tx and Rx requirements capturing at least:

- A summary of the conclusion relating to the requirement during the SI,

- An indication of what further work is expected during a WI.

- WF on BS PA and filter aspects for 7-24GHz [94]: this WF was related to the RF technology aspects, including PA and filtering for the NR BS in 7 – 24 GHz range.

Furthermore, progress in the two following areas was achieved:

- BS EMC requirements: According to the 7-24 work-plan, the BS EMC topic for 7 – 24 GHz range was concluded this meeting, with TPs to the TR for the Emissions and Immunity requirements in [92] and [86], respectively. Basically, the conclusions from the NR WI are reused for the 7-24 GHz range.

With this, the BS EMC requirements discussion in considered completed for this SI.

- TR 38.820: based on multiple discussion items during the online discussion, it was proposed to clarify in the TR scope, that the intention of the SI is the feasibility analysis, and not precise specification of the RF requirement values, nor system parameter values. Still, the feasible ranges of some key requirements could be addressed in the TR. Related TP to TR was agreed in [98].

Skeleton of the TR 38.820 was updated and extended to version 0.0.2 in [102].

The following TPs were agreed for the TR 38.820:

- TP to TR 38.820: Addition of BS requirement sets in subclause 8.2 [95]

- TP to TR 38.820: Addition of example frequencies in subclause 6.2 [97]

- TP to TR 38.820: Addition of NR terminology in clause 3 [63]

- TP to TR 38.820: Addition of PA trends in subclause 6.3.1 [96]

- TP to TR 38.820: BS EMC emission requirements for 7-24 GHz [92]

- TP to TR 38.820: BS EMC immunity requirements for 7-24 GHz [86]

- TP to TR 38.820: Inclusion of NF Background Information for BS requirements [100]

- TP to TR 38.820: single-band operation of NR BS [85]

- TP to TR38.820 on Scope [98]

#### 2.4.2	Remaining Open issues

The following open issues are to be resolved:

- TR skeleton

  - Conclusion on TR structure and selection of the information required to conclude the SI

- Regulatory survey:

  - Collection of inputs

  - Concluding remarks

- System parameters

  - Conclusions on the expected modifications in case of future bands from 7-24 GHz range

- RF technology conclusions on the following aspects:

  - PA properties,

  - antenna properties,

  - receiver properties in particular noise factor,

  - filter capabilities,

  - noise figure,

  - phase noise, etc.

- BS specifications and BS requirements

  - Expected impacted on BS specifications and requirements

  - Conclusions on the comparison of existing FR1 and FR2 requirements

  - Conclusion on the co-location requirements

- UE specifications and UE requirements

  - Expected impacted on UE specifications and requirements, e.g. comparison with existing FR1 and FR2 requirements

- BS RF architecture

  - Conclusions on the potential BS architecture variants feasible (e.g. conducted, hybrid, OTA)

  - Conclusions on the antenna array sizes

- UE RF architecture

  - Conclusions on the potential UE architecture variants feasible (e.g. conducted, hybrid, OTA)

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1]	R4-1900824	On requirement sets considerations in relation to FR1-FR2 intermediate frequency support, Ericsson, RAN4#90

[2]	R4-1900825	On the topic of co-location requirements for FR1-FR2 intermediate frequencies, Ericsson, RAN4#90

[3]	R4-1900826	On FR1-FR2 intermediate frequency support and OTA out-of-band receiver blocking considerations, Ericsson, RAN4#90

[4]	R4-1900827	On antenna aspects related to FR1 to FR2 intermediate frequency support, Ericsson, RAN4#90

[5]	R4-1900832	7-24 GHz technical aspects and considerations, Ericsson, RAN4#90

[6]	R4-1900980	Overview of FR1 and FR2 downlink requirements, Ericsson, RAN4#90

[7]	R4-1900981	Overview of FR1 and FR2 uplink requirements, Ericsson, RAN4#90

[8]	R4-1901013	7-24GHz SI overview, Ericsson, RAN4#90

[9]	R4-1901491	Recommendations on 7 - 24 GHz frequency range, Nokia, Nokia Shanghai Bell, RAN4#90

[10]	R4-1901683	Study on 7 -24GHz frequency range, Ericsson GmbH, Eurolab, RAN4#90

[11]	R4-1901750	Discussion on the general issues for the 7-24GHz SI, Huawei, RAN4#90

[12]	R4-1901751	Draft skeleton for the 7-24 GHz SI technical report, Huawei, RAN4#90

[13]	R4-1901752	Summary of operator’s interest in the 7-24 GHz frequency range, Huawei, RAN4#90

[14]	R4-1901753	Work plan for the 7-24 GHz study item, Huawei, RAN4#90

[15]	R4-1901754	Inputs to the regulatory survey on the 7-24GHz SI: U.S. market, Huawei, RAN4#90

[16]	R4-1902010	Design Choices for UEs serving 7-24G, Qualcomm Incorporated, RAN4#90

[17]	R4-1902509	Work plan for the 7-24 GHz study item, Huawei, RAN4#90

[18]	R4-1902510	Draft skeleton for the 7-24 GHz SI technical report, Huawei, RAN4#90

[19]	R4-1903058	Views on FR1 boundary frequency in the 7-24 GHz frequency range, Apple Inc., RAN4#90bis

[20]	R4-1903328	TR 38.820: updated TR skeleton for 7-24GHz SI, Huawei, RAN4#90bis

[21]	R4-1903329	EMC emission requirements for 7-24GHz range, Huawei, RAN4#90bis

[22]	R4-1903342	EMC immunity requirements for 7-24GHz range, Huawei, RAN4#90bis

[23]	R4-1903343	BS RF general aspects for 7-24GHz SI, Huawei, RAN4#90bis

[24]	R4-1903455	TP to TR 38.820: Addition of example frequencies in subclause 6.2, Ericsson, RAN4#90bis

[25]	R4-1903464	7-24 GHz PA considerations and dependencies, Ericsson, RAN4#90bis

[26]	R4-1903465	TP to TR: 7-24 GHz power amplifier trends, Ericsson, RAN4#90bis

[27]	R4-1903466	TP to TR: 7-24 GHz Phase Noise trends, Ericsson, RAN4#90bis

[28]	R4-1903484	On BS types for the 7-24GHz range, Ericsson, RAN4#90bis

[29]	R4-1903485	TP to TR 38.820: Impact to BS transmitter requirements, Ericsson, RAN4#90bis

[30]	R4-1903486	TP to TR 38.820: Impact to BS receiver requirements, Ericsson, RAN4#90bis

[31]	R4-1903487	7-24 GHz Filter Aspects, Ericsson, RAN4#90bis

[32]	R4-1903513	7.125 - 24.25GHz Frequency Range from a UE Perspective, Skyworks Solutions Inc., RAN4#90bis

[33]	R4-1903564	on EMC requirements for 7-24GHz, ZTE Corporation, RAN4#90bis

[34]	R4-1903565	on out-of-band blocking test for 7-24GHz, ZTE Corporation, RAN4#90bis

[35]	R4-1903988	Overview on system parameters for 7-24GHz, ZTE Corporation, RAN4#90bis

[36]	R4-1903990	Overview on TX requirement for 7-24GHz, ZTE Corporation, RAN4#90bis

[37]	R4-1903991	Overview on RX requirement for 7-24GHz, ZTE Corporation, RAN4#90bis

[38]	R4-1903994	On performance of existing RF connector and probe technology, LG Electronics Finland, RAN4#90bis

[39]	R4-1904070	7-24 GHz Receiver Aspects, Ericsson, RAN4#90bis

[40]	R4-1904127	7-24GHz - discuss implications of FR1 and FR2 architectures, Huawei, RAN4#90bis

[41]	R4-1904128	7-24GHz - discuss ACIR requirements and potential frequency ranges of interest, Huawei, RAN4#90bis

[42]	R4-1904129	7-24GHz - discuss TX IMD and co-location requirements, Huawei, RAN4#90bis

[43]	R4-1904130	[7-24GHz] discussion on specification structure (BS), Huawei, RAN4#90bis

[44]	R4-1904319	BS RF technology aspects for 7-24 GHz, Nokia, Nokia Shanghai Bell, RAN4#90bis

[45]	R4-1904481	Discussion On 7-24GHz UE RF requirement, Huawei, HiSilicon, RAN4#90bis

[46]	R4-1905197	WF for UE requirements type study within the 7-24GHz, Huawei, HiSilicon, RAN4#90bis

[47]	R4-1905198	WF on General aspects for 7 -24GHz SI, Huawei, RAN4#90bis

[48]	R4-1905242	WF on BS study for 7 -24 GHz, Ericsson, Huawei, RAN4#90bis

[49]	R4-1905537	on 7-24GHz ancillary equipment RE, ZTE Corporation, RAN4#91

[50]	R4-1905538	on 7-24GHz OOB requirement, ZTE Corporation, RAN4#91

[51]	R4-1905834	Discussion on EMC requirements for 7-24 GHz, Ericsson, RAN4#91

[52]	R4-1905846	[7-24GHz] Applicable FR1 UE Technologies, Skyworks Solutions Inc., RAN4#91

[53]	R4-1905911	7-24GHz - BS types and classes and deployment scenarios, Huawei, RAN4#91

[54]	R4-1905912	7-24GHz - FR1 and FR2 definitions, Huawei, RAN4#91

[55]	R4-1905913	7 to 24GHz Discussion on hybrid AAS Conducted requirements, Huawei, RAN4#91

[56]	R4-1905914	TP to TR 38.820 – capturing BS classes, Huawei, RAN4#91

[57]	R4-1906073	Proposals for moving forward on BS types, Ericsson, RAN4#91

[58]	R4-1906074	TP to TR 38.820: Impact to BS requirements, Ericsson, RAN4#91

[59]	R4-1906075	On frequency ranges and FR extension, Ericsson, RAN4#91

[60]	R4-1906076	further elaboration on 7-24 GHz filtering, Ericsson, RAN4#91

[61]	R4-1906162	On co-location scenarios and technical background for the frequency range 7 to 24 GHz, Ericsson, RAN4#91

[62]	R4-1906168	TP to TR 38.820: Addition of power scaling and AAS dependency approaches in sub-clause 6.3, Ericsson, RAN4#91

[63]	R4-1906172	TP to TR 38.820: Addition of NR terminology in clause 3, Ericsson, RAN4#91

[64]	R4-1906175	TP to TR 38.820: Correction of TR structure in clause 8, Ericsson, RAN4#91

[65]	R4-1906178	On OTA out-of-band blocking requirement for the frequency region 7 to 24 GHz, Ericsson, RAN4#91

[66]	R4-1906180	Further elaboration on 7-24 GHz PA dependencies for different example frequencies, Ericsson, RAN4#91

[67]	R4-1906183	TP to TR 38.820: Addition of PA parameter dependency for different example frequencies in subclause 6.3, Ericsson, RAN4#91

[68]	R4-1906184	Elaboration on 7-24 GHz PA power scaling and AAS dependency, Ericsson, RAN4#91

[69]	R4-1906185	Further elaboration on 7-24 GHz phase noise for different example frequencies, Ericsson, RAN4#91

[70]	R4-1906727	Analysis of system parameters for the study of 7-24 GHz, Huawei, HiSilicon, RAN4#91

[71]	R4-1906781	Phase noise for 7-24GHz, ZTE Corporation, RAN4#91

[72]	R4-1906787	PA characteristic for 7-24GHz, ZTE Corporation, RAN4#91

[73]	R4-1906800	Further discussion on BS types for 7-24GHz, ZTE Corporation, RAN4#91

[74]	R4-1906843	On performance of RF filters for 12GHz range, LG Electronics Finland, RAN4#91

[75]	R4-1906846	Views on the 7 – 24 GHz Frequency range for handheld mobile devices, Sony, RAN4#91

[76]	R4-1906884	BS RF technology aspects for 7-24 GHz, Nokia, Nokia Shanghai Bell, RAN4#91

[77]	R4-1907006	Propagation in frequency range 7-24GHz, Huawei, HiSilicon, RAN4#91

[78]	R4-1907015	TR 38.820: updated TR skeleton for 7-24GHz SI, Huawei, RAN4#91

[79]	R4-1907016	Status update on the 7-24GHz SI work progress, Huawei, RAN4#91

[80]	R4-1907017	ECC CPG PT A group discussion on the IMT spectrum allocations in 7-24 GHz range, Huawei, RAN4#91

[81]	R4-1907018	Spectrum allocation in Europe: initial analysis of applications in 7 - 24 GHz range, Huawei, RAN4#91

[82]	R4-1907019	TP to TR 38.820: example frequency ranges selection, Huawei, RAN4#91

[83]	R4-1907020	Analysis of the deployment scenarios for 7-24GHz frequency range, Huawei, RAN4#91

[84]	R4-1907021	TP to TR 38.820: deployment scenarios for 7-24GHz frequency range, Huawei, RAN4#91

[85]	R4-1907022	TP to TR 38.820: single-band operation of NR BS, Huawei, RAN4#91

[86]	R4-1907024	TP to TR 38.820: BS EMC immunity requirements for 7-24 GHz, Huawei, RAN4#91

[87]	R4-1907025	Discussion on the RI testing in 7-24 GHz range, Huawei, RAN4#91

[88]	R4-1907069	on 7-24GHz UE RF requirement, Huawei, HiSilicon, RAN4#91

[89]	R4-1907139	Applicability of conducted requirement types to frequencies in the range of 7-24 GHz, Apple Inc., RAN4#91

[90]	R4-1907170	Channel Models for 7-24GHz, Qualcomm Incorporated, RAN4#91

[91]	R4-1907791	Ad-hoc meeting minutes for 7-24GHz SI, Huawei, RAN4#91

[92]	R4-1907792	TP to TR 38.820: BS EMC emission requirements for 7-24 GHz, Huawei, RAN4#91

[93]	R4-1907793	WF on BS requirement sets for 7-24 GHz range, Ericsson, RAN4#91

[94]	R4-1907794	WF on BS PA and filter aspects for 7-24GHz, Huawei, Ericsson, RAN4#91

[95]	R4-1907795	TP to TR 38.820: Addition of BS requirement sets in subclause 8.2, Ericsson, RAN4#91

[96]	R4-1907796	TP to TR 38.820: Addition of PA trends in subclause 6.3.1, Ericsson, RAN4#91

[97]	R4-1907798	TP to TR 38.820: Addition of example frequencies in subclause 6.2, Ericsson, RAN4#91

[98]	R4-1907799	TP to TR38.820 on Scope, Huawei, HiSilicon, RAN4#91

[99]	R4-1907800	WF on the deployment scenarios for 7-24GHz, Huawei, RAN4#91

[100]	R4-1907802	TP to TR 38.820: Inclusion of NF Background Information for BS requirements, Ericsson, RAN4#91

[101]	R4-1907803	WF on FR1/FR2 differences, Huawei, Ericson, RAN4#91

[102]	R4-1907856	TR 38.820: updated TR skeleton for 7-24GHz SI, Huawei, RAN4#91

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion level swith colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template