3GPP TSG RAN Meeting #84  	RP-191184

# Newport Beach, USA, June 3-6, 2019

Agenda item:	9.3.3

Source: 	Samsung

Title: 	Survey update on regulatory requirements up to 114.25 GHz in Korea

Document for:	Discussion

Introduction

In RAN#82 meeting, a new study item on NR beyond 52.6GHz was agreed with the following for the scope of the work:

- Identify target spectrum ranges up to 114.25 GHz

  - Survey on global spectrum availability and regulatory requirements (including channelization and licensing regimes)

    - For 60 GHz bands, TR 38.805 can be a reference.

- Identify potential use cases and deployment scenarios

- Identify NR design requirements and considerations on top of regulatory requirements

Based on work plan of SI, we provide the update survey result of regulatory requirements between 52.6 GHz and 114.25 GHz in Korea.

Discussion

The followings are regulatory requirements above 52.6 GHz including available spectrum band and related regulatory requirements and revision marks are update based on the current TR.

Table 4.2.4.3-1 shows summary of the spectrum allocations for South Korea for frequency between 52.6GHz and 114.25GHz, based on [1]. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur.

Table 4.2.4.3-1: South Korea spectrum allocation between 52.6 GHz and 114.25 GHz

| Frequency [GHz] | Allocations | Notes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) |  |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) |  |
| 55.78-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) |  |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) |  |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE INTER-SATELLITE RADIOLOCATION SPACE RESEARCH (passive) |  |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION |  |
| 64-65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile |  |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH |  |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) |  |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Space research (space-to-Earth) |  |
| 77.5-78 | AMATEUR AMATEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) |  |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) |  |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) |  |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) |  |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY |  |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION |  |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy |  |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION |  |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |
| 100-102 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 102-105 | FIXED MOBILE RADIO ASTRONOMY |  |
| 105-109.5 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 109.5-111.8 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 111.8-114.25 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) |  |

Table 4.2.4.3-2 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in South Korea based on [2], [3], [4], [5], [6], and [7].

Table 4.2.4.3-2: South Korea regulatory requirements for frequency between 52.6 GHz and 114.25 GHz

| Frequency band [GHz] |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 52.6 – 54.25 | All emission is prohibited | No requirement | No requirement | No requirement | All emission is prohibited |
|  | 57 – 66 | Max Tx power 500mW (beamformed antenna) Max Tx power 100mW (omni-direction antenna). Max PSD 13dBm/MHz Max EIRP 43dBm Max antenna gain 16dBi [Note 1] | No requirement. Note 3 | Occupied BW shall be within the range of 57-66 GHz | Other than fixed point-to-point | Unlicensed |
|  | 57 – 66 | Max Tx power 500mW (beamformed antenna) Max Tx power 100mW (omni-direction antenna). Max PSD 13dBm/MHz Max EIRP 57dBm [Note 1] | No requirement. Note 3 | Occupied BW shall be within the range of 57-66 GHz | Fixed point-to-point | Unlicensed |
|  | 71 – 76 | Max EIRP 85dBm Max Tx power 3W Max PSD 150mW/100MHz Min antenna gain 43dBi, antenna half beamwidth less than 1.2 degrees. [Note 2] | Permits use under condition that do not cause interference to Astronomy stations that are already in use. | Max bandwidth 5GHz. Occupied BW shall be within the range of 71-76 GHz | For licensed fixed point-to-point wireless communications For amateur radio station (see Note 4 for channelization) | Frequency error should be less than ±150×10-6. Note 4. |
|  | 76 – 77 | Max Tx power 10mW EIRP below 55dBm [Note 5] | Function for interference avoidance shall be required. (No Specific technical requirement is described) | Occupied BW shall be within the range of 76-77 GHz | Automotive crash prevention radar | Unlicensed |
|  | 76 – 81 | PSD below -3 dBm/MHz EIRP below 34dBm/50MHz Fixed antenna orientation with down tilt angle. The half power beamwidth of the antenna array must be smaller than 8 degrees Unwanted emission outside the 76-81GHz shall be at least attenuated 20dB more than the PSD | No requirement | Occupied BW shall be within the range of 76-81 GHz | For outdoor level detection radars. For amateur radio station (see Note 4 for channelization) |  |
|  | 81 – 86 | Max EIRP 55dBW Max Tx power 3W Max PSD 150mW/100MHz Min antenna gain 43dBi, antenna half beamwidth less than 1.2 degrees. [Note 2] | Permits use under condition that do not cause interference to Astronomy stations that are already in use. | Max bandwidth 5GHz. Occupied BW shall be within the range of 81-86 GHz | For licensed fixed point-to-point wireless communications | Frequency error should be less than ±150×10-6. |
|  | 86 – 92 | No requirement | No requirement | No requirement | No requirement | All emissions are prohibited |
|  | 100-102 | No requirement | No requirement | No requirement | No requirement | All emissions are prohibited |
|  | 109-5-111.8 | No requirement | No requirement | No requirement | No requirement | All emissions are prohibited |
| Note 1: Antenna absolute gain shall be less than 16dBi. If antenna gain greater than 16dBi may be employed than conducted output power shall be reduced as much as the increased antenna gain. A user identification code shall be used in each device to prevent malfunction of other devices and to prevent malfunction caused by signals from other devices. However, it does not apply to fixed point-to-point communication. Unwanted emission shall not exceed emission level given in followin Table. Spectrum Range; Unwanted Emission Limit; Measurement Bandwidth. Less than 1GHz; -36dBm; 100kHz. 1GHz to 40GHz; -30dBm; 1MHz. More than 40GHz; -10dBm; 1MHz. Additionally, Tx or Rx spurious emission for Standby state (not operating) shall not exceed emission level given in following Table. Spectrum Range; Unwanted Emission Limit; Measurement Bandwidth. Less than 1GHz; -54dBm; 100kHz. More than 1GHz; -47dBm; 1MHz. |
| Note 2: The mean power of emissions must be attenuated below the mean output power of the transmitter in any 1 MHz band by A, where A = max{56, 11 + 0.4*(S-50) + 10*log10(B) } dBc and A = attenuation (in decibels) below the mean output power level, S = percent removed from the center frequency of the transmitter bandwidth and between 50 and 250 percent, and B = occupied bandwidth in MHz. Attenuation to an absolute power of less than -13dBm/1MHz is not required. Spurious emission must be below the mean output of the transmission in any 1 MHz band by A = 43 + 10*log10(P), where A = attenuation (in decibels) below the mean output power level, and P = average transmit power. |  |  |  |  |  |  |
| Note 3: For equipment with more than 27dBm EIRP operating in 57-58GHz, the following instruction shall be indicated in user the instruction manual. “User who would like to use this equipment within 300 m from Astronomical antenna have to obtain consent from astronomical observatory” |  |  |  |  |  |  |
| Note 4: From 75.5 to 81GHz, 75.75, 77.75, 78.5, and 80 GHz are assigned for the carrier frequencies of amateur radio station. |  |  |  |  |  |  |
| Note 5: The unwanted emission outside the 76-77GHz shall not exceed emission level given in following table. Spectrum Range; Unwanted Emission Limit; Measurement Bandwidth. Less than 1GHz; -26dBm * or 0dBm **; 100kHz. More than 1GHz; -26dBm * or 0dBm **; 1MHz. * In case that Conducted power level shall be less than 10mW ** In case that EIRP shall be less than 55dBm. |

- References

[1] 	Republic of Korea Communications Agency. Republic of Korea Frequency Allocations Status. May 2018.

[2]	Republic of Korea National Radio Research Agency. National Radio Research Agency Publication 2014-12. July 02, 2014.

[3]	Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-3. July 02, 2014.

[4]	Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-4. July 02, 2014.

[5]   No. 2018-71 	Ministry of Science and ICT notice

[6]	No. 2018-26 	National Radio Research Agency notice

[7]	No. 2018-36 	Ministry of Science and ICT notice