3GPP TSG-RAN4 Meeting #91	R4-1906179

# Reno, US,  13th- 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0020 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.843: Addition of MU evaluation for testing output power, ACLR and OBUE in RC test method in subclause 10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR adds the MU evalaution for testing output power, ACLR and OBUE in RC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of subclause 10.4.1.4A for output power - Addition of subclause 10.4.2.4A for ACLR - Addition of subclause 10.4.3.4A for OBUE - Addition of subclause 10.4.4.4A for SEM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, RC cannot be used for output power, ACLR,OBUE and SEM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.4	Measurement uncertainty for In-band TRP requirements

### 10.4.1	OTA Base station output power requirement

#### 10.4.1.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.4.1.2	In-door anechoic chamber

##### 10.4.1.2.1	General

This method measures the base station output power in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.4.1.2.1-1.

NOTE: Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

Figure 10.4.1.2.1-1: In-door Anechoic Chamber measurement system setup for OTA base station output power.

##### 10.4.1.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

##### 10.4.1.2.3	Procedure

1)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

2)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

3)	Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model

4)	Measure the appropriate test parameter.

5)	Repeat the above steps 3)~5) at a number of discrete directions around the sphere according to the chosen measurement grid, see subclause 10.8.3.

6)	Calculate TRPs from EIRP, as shown in subclause 10.8.

The appropriate test parameter in step 5 is PR_AAS_desired, D, measured mean power within the desired signal channel bandwidth for each carrier arriving at the measurement equipment connector at D in figure 10.4.1.2.1-1, and  calculation of EIRP EIRPd using the following equitation:

EIRPd = PR_AAS_desired, D+ LTX_cal, A→D

NOTE: 	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS, then measure and sum the EIRP on both polarizations. If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations and calculate total radiated transmit power as the sum over both polarizations.

##### 10.4.1.2.4	MU assessment

###### 10.4.1.2.4.1	MU Budget

Table 10.4.1.2.4.1-1: In-door Anechoic Chamber uncertainty assessment condition for OTA BS output power EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | E1-2 |
| 3 | Quality of quiet zone | E1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E1-5 |
| 6 | Phase curvature | E1-6 |
| 7 | Uncertainty of the RF power measurement equipment | E1-7, F.1 |
| 8 | Impedance mismatch in the receiving chain | E1-8 |
| 9 | Random uncertainty | E1-9 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | E1-12 |
| 13 | Quality of quiet zone | E1-3 |
| 14 | Polarization mismatch for reference antenna | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature | E1-6 |
| 17 | Uncertainty of the Network Analyzer | E1-13, F.1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E1-17 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E1-18 |

###### 10.4.1.2.4.2	MU Value

Table 10.4.1.2.4.2-1: Indoor Anechoic Chamber uncertainty assessment for EIRP measurement of OTA base station output power

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.44 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.87 | 1.06 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,3dB

#### 10.4.1.3	CATR

##### 10.4.1.3.1	General

The Compact Antenna Test Range (CATR) uses the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  The sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.4.1.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The DUT transmits a wave front that will illuminate the range antenna reflector, which will then reflect the transmitted energy into the feed antenna.  The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

Figure 10.4.1.3.1-1: CATR measurement system setup for OTA BS output power

Test Method limitations and scope

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account.

##### 10.4.1.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration should be carried out using the same procedure as in 10.2.3.3.2.

##### 10.4.1.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage.  Align the coordinates system of the DUT with that of the test system.

2)	Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model Measure the appropriate test parameter

3)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

4)	Measure the appropriate test parameter.

5)	Repeat the above steps 3)~5) at a number of discrete directions around the sphere according to the chosen measurement grid, see subclause 10.8.3.

6)	Calculate TRPs from power density, as shown in subclause 10.8.3.

The appropriate test parameter in step 5 is PR_AAS_desired, B, measured mean power within the desired signal channel bandwidth for each carrier arriving at the measurement equipment connector B in figure 10.4.1.3.1-1, and  calculation of power powerd using the following equitation:

PowerD = PR_AAS_desired, B+ LTX_cal, A→B

NOTE: If the test facility only supports single polarization, then measure power with the test facility's test antenna/probe polarization matched to the AAS BS, then measure and sum the power on both polarizations. If the test facility supports dual polarization then measure total power for two orthogonal polarizations and calculate total radiated transmit power as the sum over both polarizations.

##### 10.4.1.3.4	MU assessment

###### 10.4.1.3.4.1	MU Budget

Table 10.4.1.3.4.1-1: Compact antenna test range uncertainty contributions for OTA Base station output power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | RF power measurement equipment | F.1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-14 |
|  | Test system frequency flatness | E2-16 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | E2-6, F.1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | E |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.4.1.3.4.2	MU Value

Table 10.4.1.3.4.2-1: Compact antenna test range uncertainty assessment for OTA Base station output power measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.594 | 0.669 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.16 | 1.31 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,4 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,5dB

#### 10.4.1.4	Near Field

##### 10.4.1.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA Base Station output power measurements, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the declared beam.

##### 10.4.1.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2

##### 10.4.1.4.3	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1)	Configure the beam of the AAS BS according to the required conditions for the TRP test

2)	Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model e.g. E-TM1.1.

3)	Measure the power density at a number of points (Theta; Phi) in the beam according to the chosen measurement grid. Refer to subclause 10.8 for details about measurement grids for TRP measurements

4)	Calculate TRP from the measured power density as in subclause 10.8

##### 10.4.1.4.4	MU assessment

###### 10.4.1.4.4.1	MU Budget

Table 10.4.1.4.4.1-1: Near field test range uncertainty contributors in power density pattern measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2: Near Field AAS power density pattern measurement |  |
| 1 | Axes Intersection | E3-1 |
| 2 | Axes Orthogonality | E3-2 |
| 3 | Horizontal Pointing | E3-3 |
| 4 | Probe Vertical Position | E3-4 |
| 5 | Probe H/V pointing | E3-5 |
| 6 | Measurement Distance | E3-6 |
| 7 | Amplitude and Phase Drift | E3-7 |
| 8 | Amplitude and Phase Noise | E3-8 |
| 9 | Leakage and Crosstalk | E3-9 |
| 10 | Amplitude Non-Linearity | E3-10 |
| 11 | Amplitude and Phase Shift in rotary joints | E3-11 |
| 12 | Channel Balance Amplitude and Phase | E3-12 |
| 13 | Probe Polarization Amplitude and Phase | E3-13 |
| 14 | Probe Pattern Knowledge | E3-14 |
| 15 | Multiple Reflections | E3-15 |
| 16 | Room Scattering | E3-16 |
| 17 | DUT support Scattering | E3-17 |
| 18 | Positioning | E3-21 |
| 19 | Probe Array Uniformity | E3-22 |
| 20 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | E3-23 |
| 21 | Insertion loss of receiver chain | E3-24 |
| 22 | Uncertainty of the absolute gain of the probe antenna | E3-25 |
| 23 | Measurement Equipment (spectrum Analyzer) | E3-26, F.1 |
| 24 | Measurement repeatability - Positioning Repeatability | E3-27 |
| 25 | Test System Frequency Flatness | E3-36 |
|  | Stage 1: Calibration measurement |  |
| 26 | Uncertainty of network analyser | E3.28, F.1 |
| 27 | Mismatch of receiver chain | E3-29 |
| 28 | Insertion loss of receiver chain | E3-30 |
| 29 | Mismatch in the connection of the calibration antenna | E3-31 |
| 30 | Influence of the calibration antenna feed cable | E3-32 |
| 31 | Influence of the probe antenna cable | E3-33 |
| 32 | Uncertainty of the absolute gain of the calibration antenna | F.1 |
| 33 | Short term repeatability | E3-35 |

Note: 	Refer to TR 37.842 for description of each uncertainty term

###### 10.4.1.4.4.2	MU Value

Table 10.4.1.4.4.2-1: Near field test range uncertainty assessment for power density pattern measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 19 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 21 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 22 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 23 | Measurement Equipment (Spectrum Analyzer) | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 24 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| 25 | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1.00 | 1 | [0.25] | [0.25] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 26 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 27 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 28 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 29 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 30 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 32 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 33 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | [0.59] | [0.70] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | [1.15] | [1.37] |

MU for OTA BS Output Power is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP summation error (subclause 10.8):

For OTA BS Output Power measured in Near Field setup the MU is:

=1.87dB]

=2dB]

#### 10.4.1.4A	Reverberation chamber

##### 10.4.1.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.1.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.1.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.1.4A.4	MU assessment

###### 10.4.1.4.4A.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.1.4.4A.2	MU Value

Table 10.4.1.4.4A.2-1: Reverberation chamber uncertainty assessment for output power

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.1.5	Summary

Table 10.4.1.5-1: Test system specific measurement uncertainty values for the OTA BS output power test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,1 | 1,3 |
| Compact Antenna Test Range | 1,4 | 1,5 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.4 | 1.46 |
| Common maximum accepted test system uncertainty | 1.4 | 1.5 |

### 10.4.2	OTA Adjacent channel leakage power ratio

#### 10.4.2.1	General

#### 10.4.2.2	In-door anechoic chamber

##### 10.4.2.2.1	General

This method measures the ACLR in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.2.3.2.1-1.

NOTE: 	Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

##### 10.4.2.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

Calibration shall be performed individually for both the wanted signal and adjacent channel frequencies.

##### 10.4.2.2.3	Procedure

Reference procedure in subclause 10.4.1.2.3.

The appropriate parameters in step 5 are:

PR_AAS_desired, D: Measured mean power within the desired signal channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

PR_AAS_emission, D: Measured mean emission power in the neighbouring channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

Calculation of Powers Powerd and Powere using following formula:

Powerd = PR_AAS_desired, D+ LTX_cal, A→D

Powere = PR_AAS_emission, D+ LTX_cal, A→D

After calculation of TRP from Power as shown in subclause 10.8 calculate ACLR.

##### 10.4.2.2.4	MU assessment

###### 10.4.2.2.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.2.4.1.

Table 10.4.2.2.4.1-1: Void

###### 10.4.2.2.4.2	MU Value

Table 10.4.2.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA BS ACLR EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.444 | 0.538 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.871 | 1.055 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,3dB

#### 10.4.2.3	CATR

##### 10.4.2.3.1	General

The CATR is described in section 10.4.1.3.1.

##### 10.4.2.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration of the CATR test setup should be carried out in the same manner as described for the EVM procedure in subclause 10.2.6.3.2.

##### 10.4.2.3.3	Procedure

Stage 2 - Measurement:

1)	Align DUT with boresight of the range antenna.

2)	Configure carrier at a power level according to the manufacturer's declared rated output power and test configuration.

3)	Measure wanted and adjacent channel power for the frequency offsets both side of carrier frequency considering both polarizations of the range antenna. In multiple carrier case only offset frequencies below the lowest and above the offsets highest carrier frequency used shall be measured for ACLR; offsets in between carriers may be subject to CACLR according to the rules in 37.105..

4)	Repeat step 4 for additional points for all necessary points needed for full TRP for the wanted signal and adjacent channel emissions.

5)	Calculate ACLR or CACLR from the wanted signal TRP and the adjacent channel emissions TRP,

The appropriate parameters in step 5 are:

PR_AAS_desired, D: Measured mean power within the desired signal channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

PR_AAS_emission, D: Measured mean emission power in the neighbouring channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

Calculation of Powers Powerd and Powere using following formula:

Powerd = PR_AAS_desired, D+ LTX_cal, A→D

Powere = PR_AAS_emission, D+ LTX_cal, A→D

After calculation of TRP from power as shown in subclause 10.8, calculate ACLR.

##### 10.4.2.3.4	MU assessment

###### 10.4.2.3.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.3.4.1.

Table 10.4.2.3.4.1-1: VoidTable 10.4.2.3.4.1-2: Void

###### 10.4.2.3.4.2	MU Value

Table 10.4.2.3.4.2-1: Compact antenna test range uncertainty assessment for absolute ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | [0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.069 | 1.349 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.10 | 2.64 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

Table 10.4.2.3.4.2-2: CATR uncertainty assessment for relative ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.14 | Gaussian | 1 | 1 | 0.14 | 0.14 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.37 | 0.46 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.73 | 0.89 |

The agreed summation error (SE) of 0,75 dB is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,0 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,2dB

#### 10.4.2.4	Near Field

##### 10.4.2.4.1	Description

The system setup is depicted in section 10.2.3.4.1. In case of OTA ACLR measurement, the NF to FF transform is not needed since ACLR is based on TRP.

##### 10.4.2.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done by following the procedure in 10.2.3.4.2.

##### 10.4.2.4.3	Procedure

The procedure for relative ACLR OTA measurement consists of the following steps:

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1.	Configure TX branch and carrier according to the required test configuration

2.	Measure in Near Field ACLR and CALCR for the frequency offsets both side of carrier frequency for both polarizations. In multiple carrier case only offset frequencies below the lowest and above the offsets highest carrier frequency used shall be measured.

a.	Power is measured Near Field – no NF to FF transform is applied to

b.	Full sphere power is measured with using a defined sampling grid (refer to subclause 10.8 for proper measurement grids)

3.	Calculate TRP by using the formula in [2]

##### 10.4.2.4.4	MU assessment

###### 10.4.2.4.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.4.4.1.

Table 10.4.2.4.4.1-1: Void

###### 10.4.2.4.4.2	MU Value

Table 10.4.2.4.4.2-2: Near Field uncertainty assessment for relative ACLR measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 19 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 21 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 22 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 23 | MU of TE derived from conducted specification | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 24 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| 25 | Test System Frequency Flatness | [0.25] | [0.25] | Gaussian | 1.00 | 1 | [0.25] | [0.25] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 26 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 27 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 28 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 29 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 30 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 32 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 33 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | [0.59] | [0.7] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | [1.16] | [1.38] |

MU for OTA ACLR is TBD

TRP summation error is reported in Subclause 10.8

#### 10.4.2.4A	Reverberation chamber

##### 10.4.2.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.2.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.2.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.2.4A.4	MU assessment

###### 10.4.2.4.4A.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.2.4.4A.2	MU Value

Table 10.4.2.4.4A.2-1: Reverberation chamber uncertainty assessment for relative ACLR measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

Table 10.4.2.4.4A.2-2: Reverberation chamber uncertainty assessment for absolute ACLR measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.2.5	Summary

Table 10.4.2.5-1: Test system specific measurement uncertainty values for the OTA absolute ACLR

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,1 | 1,3 |
| Compact Antenna Test Range | 2,2 | 2,7 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 2,2 | 2,7 |

Table 10.4.2.5-2: Test system specific measurement uncertainty values for the OTA relative ACLR

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber |  |  |
| Compact Antenna Test Range | 1,0 | 1,2 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 1,0 | 1,2 |

### 10.4.3	OTA Operating band unwanted emission

#### 10.4.3.1	General

#### 10.4.3.2	In-door anechoic chamber

##### 10.4.3.2.1	General

This method measures the base station output power in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.2.3.2.1-1.

NOTE: 	Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

##### 10.4.3.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured  (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.3.2.3	Procedure

Reference procedure in subclause 10.4.1.2.3.

The appropriate The appropriate parameters in step 5 is the mean power for OBUE test for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_OBUE, D, and calculation of EIRP EIRPe using following formula:

EIRPe = PR_AAS_OBUE_D+ LTX_cal, A→D

##### 10.4.3.2.4	MU assessment

###### 10.4.3.2.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.2.4.1.

Table 10.4.3.2.4.1-1: Void

###### 10.4.3.2.4.2	MU Value

Table 10.4.3.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA OBUE EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.444 | 0.538 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.871 | 1.055 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1.1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1.3dB

#### 10.4.3.3	CATR

##### 10.4.3.3.1	General

The CATR is described in subclause 10.4.1.3.1.

##### 10.4.3.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration should be carried out using the same procedure as in 10.2.3.3.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured. (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.3.3.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate The appropriate parameters in step 5 is the mean power for OBUE test for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_OBUE, B, and calculation of power powere using following formula:

Epowere = PR_AAS_OBUE_B+ LTX_cal, A→B

##### 10.4.3.3.4	MU assessment

###### 10.4.3.3.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.3.4.1.

Table 10.4.3.3.4.1-1: Void

###### 10.4.3.3.4.2	MU Value

Table 10.4.3.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA operating band unwanted emissions measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.83 | 0.93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.63 | 1.83 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

#### 10.4.3.4	Near Field

##### 10.4.3.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA OBUE measurement, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the power density for OBUE.

##### 10.4.3.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2

##### 10.4.3.4.3	Procedure

Stage 2 - Measurement:

Refer to section 10.4.1.4.3. The measured power in step 3 is the power density for OBUE test for each carrier arriving at the measurement equipment connector.

##### 10.4.3.4.4	MU assessment

###### 10.4.3.4.4.1	MU Budget

Refer to clause 10.4.1.4.4.1 for MU budget.

###### 10.4.3.4.4.2	MU Value

Refer to clause 10.4.1.4.4.2 for MU value per point measurement.

MU for OTA OBUE is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP summation error (Subclause 10.8):

For OTA OBUE measured in Near Field setup the MU is:

=1.4dB

=1.6dB

#### 10.4.3.4A	Reverberation chamber

##### 10.4.3.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.3.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.3.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.3.4A.4	MU assessment

###### 10.4.3.4.4A.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.3.4.4A.2	MU Value

Table 10.4.3.4.4A.2-1: Reverberation chamber uncertainty assessment for OBUE

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.3.5	Summary

Table 10.4.3.5-1: Test system specific measurement uncertainty values for the OTA absolute ACLR

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,1 | 1,3 |
| Compact Antenna Test Range | 1,8 | 2,0 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 1,8 | 2,0 |

### 10.4.4	OTA Spectrum emission mask

#### 10.4.4.1	General

#### 10.4.4.2	In-door anechoic chamber

##### 10.4.4.2.1	General

This method measures the base station output power in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.2.3.2.1-1.

NOTE: 	Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

##### 10.4.4.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured. (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.4.2.3	Procedure

Reference procedure in subclause 10.4.1.2.3.

The appropriate The appropriate parameters in step 5 is the mean power for SEM test at each carrier arriving at the measurement equipment connector, denoted by PR_AAS_SEM, D, and calculation of EIRP EIRPe using following formula:

EIRPe = PR_AAS_SEM_D+ LTX_cal, A→D

##### 10.4.4.2.4	MU assessment

###### 10.4.4.2.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.2.4.1.

Table 10.4.4.2.4.1-1: Void

###### 10.4.4.2.4.2	MU Value

Table 10.4.4.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA SEM EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.493 | 0.560 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.966 | 1.098 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1.3dB

#### 10.4.4.3	CATR

##### 10.4.4.3.1	General

The CATR is described in subclause 10.4.1.3.1.

##### 10.4.4.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration should be carried out using the same procedure as in 10.2.3.3.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.4.3.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate The appropriate parameters in step 5 is the mean power for SEM test at each carrier arriving at the measurement equipment connector, denoted by PR_AAS_SEM, B, and calculation of power powere using following formula:

powere = PR_AAS_SEM_B+ LTX_cal, A→B

##### 10.4.4.3.4	MU assessment

###### 10.4.4.3.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.3.4.1.

Table 10.4.4.3.4.1-1: Void

###### 10.4.4.3.4.2	MU Value

Table 10.4.4.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA SEM

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0,83 | 0,93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1,63 | 1,83 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

#### 10.4.4.4	Near Field

##### 10.4.4.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA Spectrum Emission Mask measurement, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the power density for SEM.

##### 10.4.4.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2

##### 10.4.4.4.3	Procedure

Stage 2 - Measurement:

Refer to section 10.4.1.4.3. The measured power in step 3 is the power density for SEM test for each carrier arriving at the measurement equipment connector.

##### 10.4.4.4.4	MU assessment

###### 10.4.4.4.4.1	MU Budget

Refer to clause 10.4.1.4.4.1 for MU budget.

###### 10.4.4.4.4.2	MU Value

Refer to clause 10.4.1.4.4.2 for MU value per point measurement.

MU for OTA Spectrum Emission Mask is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP summation error (Subclause 10.8):

For OTA Spectrum Emission Mask measured in Near Field setup the MU is:

=1.4dB

=1.6dB]

#### 10.4.4.4A	Reverberation chamber

##### 10.4.4.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.4.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.4.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.4.4A.4	MU assessment

###### 10.4.4.4.4A.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.4.4.4A.2	MU Value

Table 10.4.4.4.4A.2-1: Reverberation chamber uncertainty assessment for SEM

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.4.5	Summary

Table 10.4.4.5-1: Test system specific measurement uncertainty values for the OTA SEM

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,2 | 1,3 |
| Compact Antenna Test Range | 1,8 | 2,0 |
| Near Field chamber | [1.4] | [1.6] |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 1,8 | 2,0 |