3GPP TSG RAN meeting #85										RP-191923

## Newport Beach, USA, September 16-20, 2019

## Status Report to TSG

Agenda item:			10.2.4

| WI / SI Name | LTE-based 5G terrestrial broadcast |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_terr_bcast |  |  |  |  |
| Unique ID | 830076 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190732 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2020 | Performance part: 06/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 60% | Performance Part: 0% | Testing part: |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Alberto RICO ALVARINO |
|  | Company | Qualcomm Incorporated |
|  | Email | albertor@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 endorsed the Skeleton of TR 36.276 in R1-1908844

RAN1 discussed aspects related to the new numerology for rooftop reception for PMCH, and made the following agreements:

Agreement:

The numerology of Tcp=300us/Tu=2.7ms should be used for PMCH to support rooftop reception.

Agreement:

An RS pattern with Df=3 is supported

- FFS: Dt and other values for Df

Agreement:

The effect of CFO in PMCH reception should be considered

Agreement:

Companies are encouraged to evaluate the performance of rooftop reception with tone interleaving

Agreement:

TBS mapping of the new numerology for rooftop reception (300/2700) is achieved by applying scaling factors to the legacy TBS determination procedure.

- FFS: Select from the following alternatives:

  - PRB scaling

  - Scaling by using the TBS table for a larger number of layers

  - TBS scaling

RAN1 discussed aspects related to the necessity and detailed enhancements to the physical channels and signals in the CAS, and made the following agreements:

Agreement:

The 99th percentile should be used for the target location availability of the CAS

Agreement:

PDCCH reception in CAS is enhanced by specifying CFI indicated in MBMS-dedicated MIB and one of the following:

- Aggregation level 16

- Repetition of aggregation level 8, e.g. time/frequency domain repetition

- Note: Only one of the above options can be supported

Agreement:

Enhance PBCH reception with the provision of additional REs for PBCH, e.g., PBCH repetition.

- FFS: Details for PBCH repetition

RAN1 discussed aspects related to the new numerology for support of mobility of up to 250km/h, and made the following agreements:

Agreement:

The RS pattern of Df=2 and Dt=2 should be specified for the new numerology to support mobility of up to 250km/h

#### 2.1.2	Remaining Open issues

Study/specify remaining issues for the following objectives:

- Specify new numerology(ies) for PMCH for support of rooftop reception in MPMT and HPHT-1 scenarios as described in TR 36.776

- Specify, if found necessary, enhancements to the physical channels and signals in the CAS

- Define a new numerology with 100us CP and 400us core symbol duration for support of mobility of up to 250km/h

- Specify demodulation requirements for [a subset of] CAS and PMCH with 15 kHz, 7.5 kHz, 2.5 kHz and 1.25 kHz subcarrier spacings for fixed rooftop and mobile environments at 700 MHz carrier frequency

- Specify signalling related to the features above

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 4.	References

R1-1908844	Skeleton TR for 36.276	Qualcomm Incorporated

R1-1908093	On new numerologies for PMCH to support rooftop reception	Huawei, HiSilicon

R1-1908096	New numerology for PMCH to support rooftop reception in LTE-based 5G Terrestrial Broadcast			Shanghai Jiao Tong University

R1-1908841	Support of longer numerologies for rooftop reception	Qualcomm Incorporated

R1-1909004	FFT Window Positioning Strategies for the PMCH	EBU/BBC/IRT

R1-1908102	PBCH repetition for enhancement in the CAS	Shanghai Jiao Tong University

R1-1908105	Network Simulations Regarding the Performance of the CAS	ABS

R1-1908334	CAS Network Simulations Incorporating Fading and Time Variation	EBU/BBC/IRT

R1-1908712	Potential enhancement for CAS	Huawei, HiSilicon

R1-1908842	Analysis of CAS reception	Qualcomm Incorporated

R1-1909680	Summary of offline proposals	Qualcomm Incorporated

R1-1908095	On the new numerology for PMCH to support mobility	Huawei, HiSilicon

R1-1908097   On the new numerology for PMCH to support mobility based on designed RS pattern(s)	Shanghai Jiao Tong University

R1-1908843	Numerology with 100us CP	Qualcomm Incorporated

R1-1908094	Simulation results for CAS evaluation	Huawei, HiSilicon