Draft ETSI TR 103 439 V0.0.3 (2020-10)

Intelligent Transport Systems (ITS);

Multi-Channel Operation Study;

Release 2

European Standard

Reference

DEN/ITS-00279

Keywords

Multi-Channel Operation (MCO), ITS

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2020.

All rights reserved.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	7

Foreword	7

Modal verbs terminology	7

Executive summary	7

Introduction	7

Scope	8

1	References	8

1.1	Normative references	8

1.2	Informative references	8

2	Definition of terms, symbols and abbreviations	16

2.1	Terms	16

2.2	Symbols	17

2.3	Abbreviations	18

3	Background	21

3.1	Introduction	21

3.2	European developments	21

4	Environmental aspects relevant to MCO	27

4.1	Introduction	27

4.2	Regulation	27

4.2.1	Introduction	27

4.2.2	European C-ITS regulatory aspects	27

4.2.2.1	Introduction	27

4.2.2.2	Technology Neutrality	27

4.2.2.3	C-ITS interoperability	27

4.2.2.4	C-ITS Backward Compatibility	28

4.2.2.5	Security services related to EU C-ITS certification and security policies [i.65]	28

4.2.3	European C-ITS spectrum regulation	28

4.3	European Business requirements	30

4.4	Release-1 Basic set of Application release consequences	30

4.4.1	Introduction	30

4.4.2	BSA Communication considerations	31

4.5	Release-1 specifications being affected	31

4.5.1	Introduction	31

4.5.2	Traffic classes	31

4.5.3	Channel load measurement	32

4.5.4	Channel assignment	32

4.6	Functional Safety and Safety of the Intended functionality	33

4.6.1	Introduction	33

4.6.2	Impact of Safety of The Intended Functionality (SOTIF)	33

4.6.3	Functional Safety	34

5	Functional MCO considerations and requirements	36

5.1	Introduction	36

5.2	Release-1 Extended Services	36

5.2.1	Introduction	36

5.2.2	Extended Service dissemination	36

5.2.3	Extended Service MCO communication requirements	36

5.3	System MCO relevant aspects	36

5.3.1	Introduction	36

5.3.2	Service Announcement (SA)	37

5.3.2.1	Introduction	37

5.3.2.2	SAEM information dissemination	37

5.3.2.3	SAEM MCO communication requirements	38

5.3.3	Position Accuracy Improvement (PAI)	38

5.3.3.1	Introduction	38

5.3.3.2	PAI information Dissemination	39

5.3.3.3	PAI MCO communication requirements	39

5.3.4	Privacy	40

5.3.5	Security	41

5.3.5.1	Introduction	41

5.3.5.2	Security information dissemination	42

5.3.5.3	Security MCO communication requirements	42

5.4	Collective Perception (CP)	43

5.4.1	Introduction	43

5.4.2	CP information dissemination	43

5.4.3	CP MCO communication requirements	44

5.5	Vulnerable Road User (VRU)	45

5.5.1	Introduction	45

5.5.2	VRU information dissemination	45

5.5.3	VRU communication requirements	46

5.6	Maneuver Coordination (MC)	47

5.6.1	Introduction	47

5.6.2	MC information dissemination	48

5.6.3	MC MCO communication requirements	49

5.7	Basic CACC and Platooning	49

5.7.1	Introduction	49

5.7.2	CACC information dissemination	50

5.7.3	Platooning information dissemination	50

5.7.4	CACC and Platooning MCO communication requirements	52

5.8	Intersection Related Safety	53

5.8.1	Introduction	53

5.8.2	IS information dissemination	53

5.8.3	AS information dissemination	54

5.8.4	Urban Rail Safety (URS) information dissemination	54

5.8.5	Intersection Related Safety MCO communication requirements	56

5.9	High definition sensor sharing	57

5.9.1	Introduction	57

5.9.2	High definition Sensor Sharing information dissemination	57

5.9.3	High definition Sensor Sharing MCO communication requirements	57

5.10	Non-Safety ITS applications and services	57

5.10.1	Introduction	57

5.10.2	Non-Safety ITS applications and services information dissemination	58

5.10.3	Non-Safety ITS applications and services MCO communication requirements	58

5.11	Testing and Validation services	58

5.11.1	Introduction	58

5.11.2	Testing and Validation services information dissemination	58

5.11.3	Testing and Validation services MCO communication requirements	58

5.12	Other MCO Functional considerations	58

5.12.1	MCO Functional Scope	58

5.12.2	Generic MCO communication considerations	59

6	Technical capabilities and limitations	61

6.1	Introduction	61

6.2	Transceiver configurations	61

6.3	Multi-channel Interference effects	61

6.3.1	Overview	61

6.3.2	Unwanted emissions	62

6.3.2.1	Overview	62

6.3.2.2	Adjacent and second adjacent channel leakage	62

6.3.2.3	Spurious emissions	62

6.3.3	Blocking and selectivity	62

6.3.4	Combined unwanted emission and selectivity effects	63

6.3.5	Other effect	64

6.3.5.1	Overloading	64

6.3.5.2	Intermodulation [i.121]	64

6.3.5.3	CSMA/CA energy detection threshold	65

6.3.6	Summary MCO interference effects	66

7	Multi-Channel Concepts	67

7.1	Introduction	67

7.2	Survey and taxonomy of MCO concepts	67

7.2.1	Introduction	67

7.2.2	Channel usage concepts for MCO	67

7.2.3	Service and channel association policies for MCO	68

7.2.4	Supplementary functional support for MCO	69

7.3	Analyses of the MCO approaches	70

7.3.1	Introduction	70

7.3.2	Placeholder - Requirements on MCO approaches -	70

7.3.3	Criteria	70

7.3.4	Evaluation of MCO approaches	70

8	Simulations and Verifications	72

8.1	Introduction	72

8.2	Key performance indicators	72

8.2.1	Packet Reception Ratio (PRR)	72

8.2.2	Range (maximum distance with PRR=0.9)	72

8.2.3	Inter-Packet Gap (IPG)	72

8.2.4	Channel busy Ratio (CBR)	73

8.3	Simulation scenarios	73

8.4	Simulation settings	74

8.4.1	Introduction	74

8.4.2	Traffic generation	74

8.4.3	Access layers settings	75

8.4.3.1	Physical layer settings	75

8.4.3.2	Decentralized congestion control	75

DCC is applied as in ETSI EN 302 663 [i.25]. Specifically, based on the CBR, the minimum generation interval is calculated as	75

8.4.4	Physical layer modelling	75

8.4.4.1	Propagation	75

8.4.4.2	MCO interference modelling	76

8.4.4.3	Error evaluation	76

8.5	Simulation results	77

8.5.1	Balanced data traffic in adjacent channels	77

8.5.1.1	Introduction	77

8.5.1.2	Scenarios and preliminary considerations	77

8.5.1.3	PRR versus distance	78

8.5.1.4	Range and IPG	80

8.5.1.5	Deriving conclusions in the balanced traffic case	82

8.6	Summary	82

9	The C-ITS MCO Concept	83

9.1	Introduction	83

Annex A: Multi-channel interference effects ITS-G5	84

A.1	Overview	84

A.2	Sensitivity	84

A.3	Adjacent channel rejection	84

A.4	TX spectrum mask	86

A.5	Combined unwanted emission and selectivity effects	87

A.6	Conclusion ITS-G5	88

Annex B: Multi-channel interference effects LTE-V2X	89

B.1	Overview	89

B.2	Sensitivity	89

B.3	Adjacent channel rejection	89

B.4	TX spectrum mask	89

B.5	Combined unwanted emission and selectivity effects	89

B.6	Conclusions LTE-V2X	89

Annex C: MCO solutions and algorithms	90

C.1	ETSI TS 102 724	90

C.2	CARHet	90

C.2.1	Introduction	90

C.2.2	Channel access strategy	90

C.2.3	Channel usage strategy	90

C.3	LTE-V2X RB allocation	91

C.3.1	Introduction	91

C.3.2	Channel Access	91

C.3.3	ProSe Per Packet Priority (PPPP) for V2X communication	91

C.4	SAMCO (Service-Actuated Multi-Channel Operation)	92

C.4.1	Introduction	92

C.4.2	Channel access strategy	92

C.4.3	Channel usage possibilities	92

C.5	IEEE 1609	93

C.5.1	Introduction	93

C.5.2	Channel access strategy	93

C.5.3	Channel usage strategy	94

Annex D: Single connection interfered by traffic in the adjacent channel: additional results	95

D.1	Additional results in the balanced traffic case	95

D.2	Inter-packet gap detailed statistics	95

D.3	Results assuming a different channel model	97

D.4	Results assuming messages with a larger size	98

D.5	Results in urban scenarios	100

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Technical Report (TR) has been produced by {ETSI Technical Committee Intelligent Transportation Systems (ITS).

This report is based on input provided by various European projects, AutoNet 2030, CODIS, Truck Platooning Challenge, ENSEMBLE, TIMON, HIGHTS, Intercor, C-ROADS, VRUITS (improving the safety and mobility of Vulnerable Road Users through ITS applications), PROSPECTS (Proactive Safety for Pedestrians and Cyclists), XCYCLE (Advanced measures to reduce cyclists' fatalities and increase comfort in the interaction with motorised vehicles), SafetyCube (Safety CaUsation, Benefits and Efficiency) and SENIORS (Safety Enhanced Innovations for Older Road userS). This report considers input from liaising with SDO’s such as SAE, IEEE, CEN/ISO and 3GPP.

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Executive summary

This unnumbered clause, if present, appears after the "Modal verbs terminology" and before the "Introduction". It is an optional informative element and shall not contain requirements.

The "Executive summary" is used, if required, to summarize the ETSI deliverable. It contains enough information for the readers to become acquainted with the full document without reading it. It is usually one page or shorter.

# Introduction

The current report considers multi-channel operation (MCO) technology agnostic concepts for the realisation of single technology solutions. The present report does not consider mixed technology MCO concepts since multiple technology coexistence solutions are needed to realise such concepts, which are currently not available.

# Scope

The present technical report provides an overview of the requirements for multi-channel operation (MCO) in C-ITS and the technical capabilities and limitations of C-ITS with respect to MCO. It further proposes a concept for MCO that supports a multitude of C-ITS applications, services and functions. Finally, the report derives recommendations for the standardisation to update published and develop new ETSI specifications related to MCO.

# References

## Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference/.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

As this document is a report and no normative requirements are set, no normative references are applicable.

The following referenced documents are necessary for the application of the present document.

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document, but they assist the user with regard to a particular subject area.

[i.1]	Spectrum Allocation National Telecommunication & Information Administration (NTIA) USA: “https://www.ntia.doc.gov/page/land-mobile-spectrum-planning-options-chapter-2-spectrum-requirements”

[i.2]	ETSI TR 102 492-1 V1.1.1: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Intelligent Transport Systems (ITS); Part 1: Technical characteristics for pan European harmonized communications equipment operating in the 5 GHz frequency range and intended for critical road-safety applications; System Reference Document.”

[i.3]	ETSI TR 102 492-2 V1.1.1: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Intelligent Transport Systems (ITS); Part 2: Technical characteristics for pan European harmonized communications equipment operating in the 5 GHz frequency range intended for road safety and traffic management, and for non-safety related ITS applications; System Reference Document.”

[i.4]	ETSI TR 102 638 V1.1.1: “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Definitions.”

[i.5]	EC Decision 2008/671/EC: “Decision on the harmonised use of radio spectrum in the 5 875-5 905 MHz frequency band for safety-related applications of Intelligent Transport Systems (ITS) .”: https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX%3A32008D0671

[i.6]	ETSI EN 302 637-2 V1.4.0: “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 2: Specification of Cooperative Awareness Basic Service.”

[i.7]	ETSI EN 302 637-3 V1.3.1: “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 3: Specifications of Decentralized Environmental Notification Basic Service.”

[i.8]	ETSI EN 302 665 V1.1.1: “Intelligent Transport Systems (ITS); Communications Architecture.”

[i.9]	ISO/TS 19321:2015: “Intelligent Transport Systems (ITS); Cooperative ITS — Dictionary of in-vehicle information (IVI) data structures.”

[i.10]	ISO/TS 19091:2017: “Intelligent transport systems — Cooperative ITS — Using V2I and I2V communications for applications related to signalized intersections.”

[i.11]	CVIS EU project: “http://www.ecomove-project.eu/links/cvis/”

[i.12]	Safespot EU project: “http://www.safespot-eu.org”

[i.13]	simTD: “http://www.simtd.de”

[i.14]	SCOOP@F: “https://connectedautomateddriving.eu/project/scoopf/”

[i.15]	Directive 2010/40/EU of the European Parliament and of the Council of 7 July 2010 on the framework for the deployment of Intelligent Transport Systems in the field of road transport and for interfaces with other modes of transport.

[i.16]	ETSI TS 103 324 (V0.0.16), “Intelligent Transport System (ITS); Vehicular Communications; Basic Set of Applications; Specification of the Collective Perception Service”.

[i.17]	ETSI TR 103 562 V2.1.1 (2019-12), “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Analysis of the Collective Perception Service (CPS); Release 2”.

[i.18]	ETSI TR 103 578 (V0.0.4), “Intelligent Transport Systems (ITS); Vehicular Communications; Informative Report for the Maneuver Coordination Service”.

[i.19]	TransAID Consortium, “Definition of V2X message sets”, Public Deliverable D5.1, Aug. 2019. Online available: https://www.transaid.eu/deliverables/

[i.20]	TransAID Consortium, “V2X-Based Cooperative Sensing and Driving in Transition Areas”, Public Deliverable D5.2, March 2020. Online available: https://www.transaid.eu/deliverables/

[i.21]	ETSI EN 302 571 (V2.1.1), “Intelligent Transport Systems (ITS); Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

[i.22]	ETSI TS 103 574 V1.1.1, “Intelligent Transport Systems (ITS); Congestion Control Mechanisms for C-V2X PC5 interface; Access layer part”

[i.23]	Wireless World Initiative New Radio – WINNER+, “D5.3: WINNER+ Final Channel Models”, version 1.0, June 2010

[i.24]	ETSI TR 103 766, “Intelligent Transport Systems (ITS); Pre-standardization study on co-channel co-existence between IEEE- and 3GPP-based ITS technologies in the 5 855 MHz-5 925 MHz band; TR on co-channel co-existence between ITS-G5 and LTE-V2X.”

[i.25]	ETSI EN 302 663, “Intelligent Transport Systems (ITS); ITS-G5 Access layer specification for Intelligent Transport Systems operating in the 5 GHz frequency band,” v1.3.0, May 2019

[i.26]	ETSI EN 303 613, “Intelligent Transport Systems (ITS); LTE-V2X Access layer specification for Intelligent Transport Systems operating in the 5 GHz frequency band,” v1.1.0, May 2019

[i.27]	ETSI TR 103 257-1, “Intelligent Transport Systems (ITS); Access Layer; Part 1: Channel Models for the 5,9 GHz frequency band,” v1.1.1, May 2019

[i.28]	ETSI TR 101 607: “Intelligent Transport Systems (ITS); Cooperative ITS (C-ITS); Release-1”

[i.29]	IEEE 1609.4-2016: “IEEE Standard for Wireless Access in Vehicular Environments (WAVE) -- Multi-Channel Operation”

[i.30]	Mate Boban, Andreas Festag, “Service-actuated multi-channel operation for vehicular communications”, in Computer Communications, Volume 93, 2016, Pages 17-26, ISSN 0140-3664.

[i.31]	M. Sepulcre and J. Gozalvez, "Heterogeneous V2V Communications in Multi-Link and Multi-RAT Vehicular Networks," in IEEE Transactions on Mobile Computing (Early Access), Sept. 2019. DOI: 10.1109/TMC.2019.2939803

[i.32]	CCAM: https://ec.europa.eu/transport/themes/its/c-its_en

[i.33]	ETSI EN 301 893: Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN; Harmonized EN covering essential requirements of article 3.2 of the R&TTE Directive

[i.34]	Declaration of Amsterdam: https://www.rijksoverheid.nl/documenten/rapporten/2016/04/29/declaration-of-amsterdam-cooperation-in-the-field-of-connected-and-automated-driving

[i.35]	ENSEMBLE: https://platooningensemble.eu

[i.36]	ETSI TR 103 298 (Drafting stage: DTR/ITS-00156): “Intelligent Transport Systems (ITS); Platooning; Pre-standardization study.”

[i.37]	ETSI TR 103 299 V2.1.1: “Intelligent Transport Systems (ITS); Cooperative Adaptive Cruise Control (CACC); Pre-standardization study.”

[i.38]	ETSI TR 103 300-1 V2.1.1: “Intelligent Transport System (ITS); Vulnerable Road Users (VRU) awareness; Part 1: Use Cases definition; Release 2.”

[i.39]	ETSI TS 103 300-2 (Drafting stage: DTR/ITS-00186): “Intelligent Transport System (ITS); Vulnerable Road Users (VRU) awareness; Part 2: Functional Architecture and Requirements definition; Release 2.”

[i.40]	ETSI TS103 300-3 (Drafting stage: DTR/ITS-00183): “Intelligent Transport System (ITS); Vulnerable Road Users (VRU) awareness; Part 3: Specification of VRU awareness basic service; Release 2.”

[i.41]	IMAGinE: https://h2020-imagine.eu

[i.42]	ETSI TS 103 561 (Drafting stage: DTR/ITS-00184): “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Maneuver Coordination Service.”

[i.43]	AutoNet2030: http://www.autonet2030.eu

[i.44]	ICT4CART: http://www.maven-its.eu

[i.45]	AEF: https://www.aef-online.org/home.html

[i.46]	ETSI TS 103 301 V1.2.1: “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Facilities layer protocols and communication requirements for infrastructure services.”

[i.47]	ETSI EN 302 890-2 (Drafting stage: DTR/ITS-0010013): “Intelligent Transport Systems (ITS); Facilities layer function; Part 2: Position and time facility specification.”

[i.48]	5GAA Releases White Paper on C-V2X Use Cases: Methodology, Examples and Service Level Requirements: https://5gaa.org/news/5gaa-releases-white-paper-on-c-v2x-use-cases-methodology-examples-and-service-level-requirements/

[i.49]	AETA: https://eata.be/relevant-links/projects/

[i.50]	5GAA: https://5gaa.org

[i.51]	Car2Car-CC: https://www.car-2-car.org

[i.52]	ACEA: https://www.acea.be

[i.53]	C-ROADS: https://www.c-roads.eu/platform.html

[i.54]	Intercore: https://intercor-project.eu

[i.55]	CODECS: https://www.codecs-project.eu/index.php?id=5

[i.56]	Regulation (EU) 2015/2120 of the European Parliament and of the Council of 25 November 2015 laying down measures concerning open internet access and amending Directive 2002/22/EC: https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX%3A32015R2120

[i.57]	Directive 2002/22/EC: universal service and users' rights relating to electronic communications networks and services (Universal Service Directive): https://eur-lex.europa.eu/legal-content/EN/ALL/?uri=CELEX%3A32002L0022

[i.58]	Regulation (EU) No 531/2012 of the European Parliament and of the Council of 13 June 2012 on roaming on public mobile communications networks within the Union Text with EEA relevance: https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=celex%3A32012R0531

[i.59]	“Technology and Neutrality….” By Peter Alexiadis and Miranda Cole : https://www.scribd.com/document/400542082/2915-file-WB-alexiadis-pdf

[i.60]	“Technology Neutrality in Internet, telecoms and data protection” by Winston Maxwell https://www.hlmediacomms.com/2014/11/17/technology-neutrality-in-internet-telecoms-and-data-protection-regulation/

[i.61]	EU antitrust and competition laws: https://ec.europa.eu/competition/antitrust/overview_en.html, and https://www.slaughterandmay.com/media/64569/an-overview-of-the-eu-competition-rules.pdf

[i.62]	OSI Model: https://en.Wikipediapedia.org/Wikipedia/OSI_model

[i.63]	EU regulation COM(2016) 766: https://ec.europa.eu/transport/sites/transport/files/com20160766_en.pdf

[i.64]	GDPR (EU) 2016/679 Regulation: https://eur-lex.europa.eu/eli/reg/2016/679/oj and information: https://eugdpr.org

[i.65]	EU ITS certification and security policies: https://ec.europa.eu/transport/themes/its/c-its_en

[i.66]	TransAID - Transition Areas for Infrastructure-Assisted Driving: https://www.transaid.eu/

[i.67]	ETSI EN 302 890-1 V1.2.1: “Intelligent Transport Systems (ITS); Facilities layer function; Part 1: Services Announcement (SA) specification”

[i.68]	ISO TS 16460: 2016: “Intelligent transport systems| Communications access for land mobiles (CALM) | Communication protocol messages for global usage. Technical Specification”

[i.69]	ISO EN 22418: 2020: “Intelligent transport systems – Fast service announcement protocol (FSAP) for general purposes in ITS”

[i.70]	IEEE 1609.3-2016: “IEEE Standard for Wireless Access in Vehicular Environments (WAVE) - Networking Services”

[i.71]	ETSI EN 302 931 V1.1.1: “Intelligent Transport Systems (ITS); Vehicular Communications; Geographical Area Definition”

[i.72]	ETSI TS 103 248 V1.3.1: “Intelligent Transport Systems (ITS); GeoNetworking; Port Numbers for the Basic Transport Protocol (BTP)”

[i.73]	ETSI TS 102 965 V1.5.1: “Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration”

[i.74]	ETSI TS 103 097 V1.3.1: “Intelligent Transport Systems (ITS); Security; Security header and certificate formats”

[i.75]	ETSI EN 302 636-3 V1.2.1: “Intelligent Transport Systems (ITS); Vehicular Communications; GeoNetworking; Part 3: Network Architecture”

[i.76]	C-ITS platform, final report, 2016: https://ec.europa.eu/transport/sites/transport/files/themes/its/doc/c-its-platform-final-report-january-2016.pdf

[i.77]	C-ITS platform, Phase II report 2017: https://ec.europa.eu/transport/sites/transport/files/2017-09-c-its-platform-final-report.pdf

[i.78]	ETSI TR 103 415 V1.1.1: “Pre-standardization study on pseudonym change management”

[i.79]	ETSI TS 102 941 V1.3.1: “Trust and Privacy Management”

[i.80]	iKoPA Consortium, “Data Protection Impact Assessment of Mobility-Related Communications” Public Deliverable D3.3, February 2019, Online available: https://ikopa.de/en/results/

[i.81]	TransAID Consortium, “V2X-Based Cooperative Sensing and Driving in Transition Areas”, Public Deliverable D5.2, March 2020.

[i.82]	C-ITS platform phase I report: https://ec.europa.eu/transport/themes/its/c-its_en

[i.83]	Amsterdam Group: https://amsterdamgroup.mett.nl/default.aspx

[i.84]	ECC DEC (08)01: ECC Decision “The harmonised use of the 5875-5935 MHz frequency band for Intelligent Transport Systems (ITS)”, approved 14 March 2008, Amended 6 March 2020:  https://www.ecodocdb.dk/download/b470d271-048b/ECCDEC0801.PDF

[i.85]	ECC REC (08)01: ECC Recommendation “Use of the band 5855-5875 MHz for Intelligent Transport Systems (ITS)”, Amended 6 March 2020: https://www.ecodocdb.dk/download/798c1836-20c6/REC0801.pdf

[i.86]	EC implementation decision C(2020)6773/F1: ”Commission Decision on the harmonised use of radio spectrum in the 5875-5905 MHz frequency band of safety-related applications of Intelligent Transport Systems (ITS) and repealing Decision 2008/671/EC”, 2020. (will be updated in 2020)

[i.87]	EC Implementation Decision 2019/1345: ” COMMISSION IMPLEMENTING DECISION (EU) 2019/1345 of 2 August 2019 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices“

[i.88]	HIGHTS: https://ec.europa.eu/inea/en/horizon-2020/projects/h2020-transport/intelligent-transport-systems/hights

[i.89]	PROPART: http://propart-project.eu

[i.90]	VRUITS: https://cordis.europa.eu/project/id/321586

[i.91]	5GAA “Study of spectrum needs for safety related intelligent transport systems day 1 and advanced use cases”:

[i.92]	ETSI TS 102 940: “Intelligent Transport Systems (ITS); Security; ITS communications security architecture and security management”

[i.93]	SAE J2735 “Dedicated Short Range Communications (DSRC) Message Set Dictionary”

[i.94]	Drive-C2X EU project “DRIVE C2X\nDRIVing implementation and Evaluation of C2X communication technology in Europe”: https://cordis.europa.eu/project/id/270410

[i.95]	TIMON EU project: https://timon-project.eu

[i.96]	ETSI TS 103 724: “Intelligent Transportation Systems (ITS): Facilities layer function; Protected Zone Beacon message”

[i.97]	ETSI TS 102 724: “Intelligent Transportation Systems (ITS): Harmonized Channel Specifications for Intelligent Transport Systems operating in the 5 GHz frequency band”

[i.98]	ETSI TS 102 687: “Intelligent Transportation Systems (ITS): Decentralized Congestion Control Mechanisms for Intelligent Transport Systems operating in the 5 GHz range; Access layer part”

[i.99]	ETSI EN 302 636-4-1: “Intelligent transport Systems ((TS); Vehicular Communications; GeoNetworking; Part4: Geographical addressing and forwarding for point-to-point and point-to multipoint communications; Sub-part 1: Media-independent functionalities for ITS-G5”

[i.100]	ETSI EN 302 636-4-2: “Intelligent transport Systems ((TS); Vehicular Communications; GeoNetworking; Part4: Geographical addressing and forwarding for point-to-point and point-to multipoint communications; Sub-part 2: Media-dependent functionalities for ITS-G5”

[i.101]	ETSI TS 103 613: “Intelligent transport Systems ((TS); Access layer specification for Intelligent Transport Systems using LTE Vehicle to everything communication in the 5.9 GHz Frequency band”

[i.102]	ISO 26262, Functional Safety: “ Road vehicles – Functional Saferty” is a series of standards: https://www.iso.org/obp/ui/#iso:std:iso:26262:-7:ed-2:v1:en

[i.103]	IEC 61508: “Functional safety of electrical/electronic/programmable electronic safety-related systems - Part 1-7”: https://www.iec.ch/functionalsafety/standards/page2.htm

[i.104]	ISO 26262-3, Functional Safety: “Road vehicles — Functional safety — Part 3: Concept phase”

[i.105]	ISO PAS 21448, SOTIF: “Road Vehicles – Safety of the intended Functionality.”: https://www.iso.org/standard/70939.html

[i.106]	ECC Decision (09)01: " Harmonised use of the 63.72-65.88 GHz frequency band for Intelligent Transport Systems (ITS)”; https://www.ecodocdb.dk/download/09d84da1-2776/ECCDEC0901.PDF

[i.107]	RTCM 1005, 1077, 1087, 1097: https://rtcm.myshopify.com/collections/differential-global-navigation-satellite-dgnss-standards

[i.108]	SAE, Vehicle Automation levels, SAE J3016: https://www.sae.org/news/2019/01/sae-updates-j3016-automated-driving-graphic

[i.109]	ENSEMBLE “V1 Functional specification for intelligent infrastructure”: https://platooningensemble.eu/storage/uploads/documents/2019/02/11/ENSEMBLE-D2.6-functional-specification-for-intelligent-infrastructure_disclaimer.pdf

[i.110]	ETSI Drafting Rules (EDRs), only accessable for members: https://portal.etsi.org/Portals/0/TBpages/edithelp/Docs/40_directives_apr_2019_part2%20(EDR).pdf

[i.111]	ETSI TS 136 331: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (3GPP TS 36.331 version 15.3.0 Release-15)”

[i.112]	ETSI TS 136 213: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (3GPP TS 36.213 version 15.2.0 Release-15)”

[i.113]	ECC Decision 09(01):” Harmonised use of the 63.72-65.88 GHz frequency band for Intelligent Transport Systems (ITS)”

[i.114]	EC Decision 2019/1343: “COMMISSION IMPLEMENTING DECISION (EU) 2019/1345 of 2 August 2019 amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices”, Band No 77.

[i.115]	Multi-channel Usage in Day 2 and beyond EU V2X Systems: http://leinmueller.de/lib/exe/fetch.php/publications/lsbbs15mco.pdf

[i.116]	C. Campolo and A. Molinaro, "Multichannel communications in vehicular Ad Hoc networks: a survey," in IEEE Communications Magazine, vol. 51, no. 5, pp. 158-169, May 2013, doi: 10.1109/MCOM.2013.6515061.

[i.117]	Hybrid Communication CODECS workshop: https://www.codecs-project.eu/index.php?id=27&tx_ttnews%5Btt_news%5D=72&cHash=e729c8e7668ca8f3145b237b3dd96d70

[i.118]	ETSI TR 103 576-2: “Intelligent Transport Systems (ITS); Pre-standardization study on ITS architecture; Part 2: Interoperability among heterogeneous ITS systems and backward compatibility”

[i.119]	IEEE 802.11bd: ”P802.11bd - Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer

[i.120]	ECC Report 68: “Compatibility studies in the band 5725-2875MHz between fixed wireless access (FWA) system and other systems”, June 2005

[i.121]	ECC Report 310: “Evaluation of receiver parameters and the future role of receiver characteristics in spectrum management, including in sharing and compatibility studies“, January 2020.

[i.122]	ERC Recommendation 74-01:” ECC-REC 74-01: ERC Recommendation of 1998 on unwanted Emissions in the Spurious Domain, latest amendment on 29 May 2019”

[i.123]	IEEE802.11-2016:” IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications”

[i.124]	ETSI TS 103 574: “Intelligent Transport Systems (ITS); Congestion Control Mechanisms for the C-V2X PC5 interface; Access layer part”

[i.125]	ETSI TS 103 723: “Intelligent Transport Systems (ITS); Profile for LTE-V2X Direct Communication”

[i.126]	Van  Aerde,  M.  and  H.  Rakha,  “Multivariate  calibration  of  single  regime  speed-flow-density   relationships,”   in   Proceedings   of   the   6th   1995   Vehicle   Navigation   and   Information  Systems  Conference  (VNIS), 1995, Seattle,  WA,  USA

[i.127]	ETSI TS 136 201: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); LTE physical layer; General description”

[i.128] 	ETSI TR 103 580V1.1.1: ”Urban Rail ITS and Road ITS applications in the 5,9 GHz band; Investigations for the shared use of spectrum”, August 2019”

[i.129]	SAE J2945/0: “Dedicated Short Range Communication (DSRC) Systems Engineering Process Guidance for J2945/x Documents and Common Design Concepts™”

[i.130]	C2C-CC “Multi-Channel Operation (MCO); Part 1; Functional requirements”: https://www.car-2-car.org/documents/general-documents/

[i.131]	C2C-CC position paper “Position Paper on Road Safety and Road Efficiency Spectrum Needs in the 5.9 GHz for C-ITS and Cooperative Automated Driving”, 28.2.2020, https://www.car-2-car.org/fileadmin/documents/General_Documents/C2CCC_TR_2050_Spectrum_Needs.pdf

# Definition of terms, symbols and abbreviations

## Terms

For the purposes of the present document, the following terms apply:

Ad-Hoc: Operating mode in which an RLAN device establishes a temporary wireless connection with other RLAN devices without a controlling network infrastructure [i.33].

Active State: is the state in which the ITS-S or the road user is actively participating in Road Traffic.

Awareness in road traffic: Road users and roadside infrastructure are informed about each other’s dynamic state. Also see ETSI EN 302 637-2 [i.6]

Broadcast: An unidirectional point-to-multipoint mode of transmission, and a single sender transmits protocol units to all recipients.

C-ITS: Cooperative Intelligent Transport Systems (C-ITS) refers to ITSs, where the cooperation between two or more ITS sub-systems (personal, vehicle, roadside and/or central) enables and provides ITS services to serve the C-ITS Methodology.

C-ITS Methodology: Sharing transport (traffic situation) related information among traffic stakeholders, openly, to realize common traffic safety and traffic efficiency related benefit for all.

C-ITS-S: The Cooperative Intelligent Transportation Systems-Station is an ITS stations which implements the C-ITS Methodology and related C-ITS Domain trust model.

C-ITS-U: any person, group, organisation or automated system

Constellation: An ITS Constellation (ITS-C) is a group of ITS-Ss that communicate with each other. ITS-Ss mostly have different positions and be part of and active in different constellations but in general not necessary in all as not all ITS-Cs need to overlap.

Note: For example, ITS-Cs in Australia do not overlap with ITS-Cs in China.

Domain: The common area of interest and/or ownership

Data Provider: Role that a C-ITS-S has when it communicates service information.

Data User: Role that a C-ITS-S has when it consumes service information received.

ITS: “Intelligent Transportation Systems are Technical solution, platforms and applications that improves the quality of transportation, or achieves other outcomes based on applications that monitor, manage or enhance transportation systems. e.g. Real-time parking management, Electronic toll collection, Emergency vehicle notification systems, Automated road speed enforcement, Speed alerts, RFID in freight transportation, Variable speed limits, E-Call, Dynamic traffic light sequence, Collision avoidance systems and Cooperative-ITS as defined in clause 4.2.2.3.

ITS-G5: access technology to be used in frequency bands dedicated for European Intelligent Transport Systems as
defined in ETSI EN 302 663 [i.25].

ITS-S: The Intelligent Transportation System-Station is a generic ITS station able to support any selected ITS application.

LTE-V2X: access technology to be used in frequency bands dedicated for European Intelligent Transport Systems
as defined in ETSI TS 103 613 [i.101].

Multicast: An unidirectional point-to-multipoint mode of transmission, and a single sender transmits protocol units to a group of recipients.

Object Class: this identifies the different type of objects such as vehicles and pedestrians (see TS 103 324 ([i.16])

Offloading: Transmission of Data in a channel other than originally intended.

Oth: Overload threshold

Roadside ITS-S: ITS station in a roadside ITS sub-system

See Through of Passing: is a use case by which a proceeding vehicle provided raw sensor data such as a video signal about what is visible in front of the proceeding vehicle to following vehicles such that the following vehicles can take automation or safety related measures.

Service: A service resides in the facility layer according to the ETSI EN 302 665 [i.8]

Service Provider: A legal identify, organisation of business providing subscribers to have access to a provided service or function.

Short-Range: several meters to several 100 meters. Compared to Near-Field: several centimetres to several meters.

Trust Model: identifies the specific mechanisms that are necessary to respond to a specific threat profile. A trust model must include implicit or explicit validation of an entity's identity or the characteristics necessary for a particular event or transaction to occur.

Technology agnostic: Being unbiased towards the use of specific technologies

Unicast: A unidirectional point-to-point mode of transmission.  One sender transmits protocol units to one other recipient.

V2X: vehicle to vehicle (V2V), vehicle to infrastructure (V2I) and / or infrastructure to vehicle (I2V), or vehicle to network (V2N) and / or network to vehicle (N2V) communication

## Symbols

For the purposes of the present document, the following symbols apply:

Ton 	duration of a transmission

Tsys	ITS Station Time

ttarget	Target time gab

dSTEP	Discretization interval (granularity) for the calculation of the performance metrics

CBR	Measured channel busy ratio

TCBR-ITSG5	Duration of the time interval for the calculation of the channel busy ratio

Tbusy	Period of time within TCBR-ITSG5 when the strength of received signals exceeds a given threshold

Tgen-DCC	Minimum inter-packet generation interval set by DCC

tpack	Packet duration

dji	Euclidean distance between node i and node j

PjiMCO  	Interference due to MCO perceived by node i from node j

Pjir  	Power received at the position of node i from node j within the channel used by node j

PTX_int_eff	Effective Interference TX power, sum of Pi_ACL and Pi_ACS

Pi_ACL	Interference power from ACL effect at the transmitter

Pi_ACS	Interference power from the ACS effect at the transmitter

Ljiadj 	Portion of power interfering in the channel used by node i from a signal in the channel used by node j

## Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

AC	Access Category

ACC	Adaptive Cruise Control

ACLR	Adjacent Channel Leakage Ratio

ACR	Adjacent Channel Rejection

ACS	Adjacent Channel Selectivity

AETA	European Automotive and Telecoms Alliance

AEF	Agriculture industry Electronics Foundation

AI	Artificial Intelligence

AID	Application Identifier

AIFS	Arbitration InterFrame Space

AS	Agriculture Safety

ASIL	Automotive Safety Integrity Level

BSA	Basic Set of Applications

BTP	Basic Transport Protocol

CA	Cooperative Awareness

CAS	Cooperative Awareness Service

CAM	Cooperative Awareness Message

CACC	Cooperate Adaptive Cruise Control

CBR	Channel Busy Ratio

CBTC	Communications Based Train Control

CC	Congestion Control

CCAD	Connected and Cooperative Automated Driving

CCAM	Connected and Cooperative Automated Mobility

CCH	Control Channel, Channel. SCH 0.

C-ITS	Cooperative Intelligent Transportation Systems

C-ITS-S	Cooperative Intelligent Transportation Systems Station

C-ITS-U	Cooperative Intelligent Transportation Systems User

CIA	CAM Information Aggregation

CP	Collective Perception

CPM	Collective Perception Message

CPS	Collective Perception Service

CSMA	Carrier-Sense Multiple Access

CUS	Certificate Updating Service

CUM	Certificate Updating Message

C2C-CC	Car 2 Car Communication Consortium

DA	Data Age

DCC	Decentralized Congestion Control

DEN	Decentralized Environmental Notification

DP	Diagnostic Port

DSC	Dynamic Stability Control

DSCO	Detected Safety-Critical Object

EC	European Commission

ECC	European Cooperation Centre

EDCA	Enhanced Distributed Channel Access

EDRs	ETSI Drafting Rules

EED	End-to-End Delay

EIRP	Equivalent Isotropic Radiated Power

ERS	Empty Road Segment

EU	European

FSR	Functional Safety Requirements

GCM	GNSS Correction Message

GDPR	General Data Protection Regulation

GNSS	Global Navigation Satellite System

GPC	GNSS Positioning Correction

HFC 	High Frequency Container

ICDF	Inverse Cumulative Distribution Function

IEC 	International Electrotechnical Commission

IPG	Inter-Packet Gap

ICT	Information and Communication Technology

IEEE	Institute of Electrical and Electronics Engineers

IS	Intersection Safety

ISA	Integral Safety Awareness

ITS	Intelligent Transportation Systems

IVI	In-Vehicle Information

ISO	International Organisation for Standardisation

KPI	Key Performance Indicator

LCA-B	Lane Change Assist at Bus

LDC	Low Duty Cycle

LFC 	Low Frequency Container

LOS	Line of Sight

MAC	Media Access Control

MAP	Map

MAPEM	MAP Extended Message

MC	Maneuver Coordination

MCO	Multi-Channel Operation

MCM	Maneuver Coordination Message

MCS	Maneuver Coordination Service

MIB 	Management Information Base

MLME	MAC Layer Management Entity

NC	Not Connected

NLOS	Non-Line of Sight

NR	New Radio

NTIA	National Telecommunication & Information Administration

OSI Model	Open Systems Interconnection Model

PA	Position Accuracy

PAI	Position Accuracy Improvement

PAS	Position Augmentation Service

PCM	Platooning Control Massage

PDU	Protocol Data Unit

PD	Path prediction

PH	Path history

PHY	Physical Layer (device or protocol)

PoTi	Position and Time

PPPP	Proximity Services Per-Packet Priority

PRR	Packet Reception Ratio

PSD	Power Spectrum Density

PSD	Platform Screen Doors (Urban Rail)

PTW	Power Two Wheeler

PZB	Protected Zone Beacon

PZBM	Protected Zone Beacon Message

QM	Quality Management (As ASIL level, only standard QM is required)

RLAN	Radio Local Area Network

RRM	Roadside Ranging Message

RRR	Packet Reception Ratio

RRRM	RRR Mulit-Channel

RRRS	RRR Single-Channel

RRS	Roadside Ranging augmentation service

RSU	Road Site Unit

RTCM	Radio Technical Commission for Maritime Services

RTK	Real Time Kinematic

SA	Service Announcement

SAE	Society of Automotive Engineering

SAEM	Service Announcement Essential Message

SAMCO	Service-Actuated Multi-Channel Operation

SAS	Service Announcement Service

SCH	Service CHannel

SDO	Standardisation Developments Organisation]

SHB	Single Hop Broadcasting

SINR	Signal to Interference and Noise Ratio

SOTIF	Safety Of the Intended Function

SP	Service provider

SPAT	Signal Phase and Timing

SPATEM	SPAT Extended Message

SRD	Short Range Device

SRM	Signal Request Message

SSM	Signal Status Message

SSP	Service Specific Permission

STF	Special Task Force

TC	Traffic Class

TCP	Transmission Control Protocol

TFEU	Treaty on the Functioning of the European Union

TIM	Traffic Incident Management

TV	Target Vehicle

UC	Unified Communication

URS	Urban Rail Safety

VAM	VRU Awareness Message

VAS	VRU Awareness Service

VBS	VRU Basic Service

VRU	Vulnerable Road User

V2X	Vehicle two anything

WAVE	Wireless Access in Vehicular Environments

WLAN	Wide Local Area Network

WSA	WAVE Service Advertisement

ZC	Zone Controller

5GAA	5G automotive association

# Background

## Introduction

As results of the development of regional differentiated legislations, political and business interests around the world and despite harmonisation have been considered as an important component, harmonised but separated ITS realisation approaches have been developed for the realisation of Cooperative ITS (C-ITS) services and applications in the different regions of the world. The following clauses provide an overview of what is existing.

## European developments

In the late 90s, it was recognized that information exchange among vehicles and with road infrastructure can help improve road safety, road efficiency and support transport automation. In Europe in-depth spectrum analyses were based on findings by the National Telecommunication & Information Administration (NTIA) in the USA [i.1], which had allocated 85 MHz for C-ITS. In Europe this led to investigations performed by ETSI TC ERM in the period of 2004-2006, leading to two reports, i.e. the ETSI TR 102 492-1 [i.2] in 2005 and ETSI TR 102 492-2 [i.3] in 2006. In the ETSI TR 102 492-1 [i.2] an initial set of safety related applications was identified (see Table 1:), which was included in the ETSI TR 102 638 [i.4] that defines a Basic Set of Application (BSA) ETSI TR 102 638 [i.4] and ETSI TR 102 492-1 and -2 were the basis for the realisation of the European spectrum regulation most importantly covered in the EC Decision 2008/671/EC [i.5] (see Figure 1) from 2008 and the amended version EC implementation decision C(2020)6773/F1 [i.86].

| Application | Description |
| --- | --- |
| Cooperative Collision Warning | Cooperative collision warning collects surrounding vehicle locations and dynamics and warns the driver when a collision is likely. |
| Work Zone Warning | Work zone safety warning refers to the detection of a vehicle in an active work zone area and the indication of a warning to its driver. |
| Approaching Emergency Vehicle Warning | This application provides the driver a warning to yield the right of way to an approaching emergency vehicle. |
| Traffic Signal Violation Warning | Traffic signal violation warning uses infrastructure-to-vehicle communication to warn the driver to stop at the legally prescribed location if the traffic signal indicates a stop and it is predicted that the driver will be in violation. |
| Emergency Vehicle Signal Pre-emption | This application allows an emergency vehicle to request right of way from traffic signals in its direction of travel. |
| In-Vehicle Signage | The in-vehicle signage application provides the driver with information that is typically conveyed by traffic signs. |
| Road Condition Warning | Road condition warning is used to provide warning messages to nearby vehicles when the road surface is icy, or when traction is otherwise reduced. |
| Low Bridge Warning | Low bridge warning is used to provide warning messages especially to commercial vehicles when they are approaching a bridge of low height. |
| Highway/Rail Collision Warning | Railroad collision avoidance aids in preventing collisions between vehicles and trains on intersecting paths. |
| Wrong Way Driver Warning | This application warns drivers that a vehicle is driving or about to drive against the flow of traffic. |
| Emergency Electronic Brake Lights | When a vehicle brakes hard, the Emergency Electronic Brake light application sends a message to other vehicles following behind. |
| Left Turn Assistant | The Left Turn Assistant application provides information to drivers about oncoming traffic to help them make a left turn at a signalized intersection without a phasing left turn arrow. |
| Curve Speed Warning | Curve speed warning aids the driver in negotiating curves at appropriate speeds. |
| Vehicle-Based Road Condition Warning | This in-vehicle application will detect marginal road conditions using on-board systems and sensors (e.g. stability control, ABS), and transmit a road condition warning, if required, to other vehicles via broadcast. |
| Low Parking Structure Warning | This application provides drivers with information concerning the clearance height of a parking structure. |
| Lane Change Warning | This application provides a warning to the driver if an intended lane change may cause a crash with a nearby vehicle. |
| Highway Merge Assistant | This application warns a vehicle on a highway on-ramp if another vehicle is in its merge path (and possibly in its blind spot). |
| Cooperative Glare Reduction | This application uses C2C-C to allow a vehicle to automatically switch from high-beams to low-beams when trailing another vehicle. |
| Intelligent Intersection Control | Alerts driver to other vehicles at intersections. |

Table 1: TR 102 492-1 List of Release-1 safety-related applications

Figure 1:  TR 102 492-2 proposed spectrum allocation

The ETSI TR 102 638 identifies the initial cooperative C-ITS applications which have been functionally as well as technically tested in various R&D projects in Europe. To support these applications, information exchange is realized based on two key basic message services: the Cooperative Awareness service (CA) as specified in ETSI EN 302 637-2 [i.6], which provides awareness of neighbouring C-ITS-S, and the Decentralized Environmental Notification service (DEN), specified in the ETSI EN 302 637-3 [i.7], enabling the notification of safety-related situations detected by a transmitting C-ITS-S.

To accommodate safety related and non-safety related ITS information exchange European regulation EC Decision 2008/671/EC [i.5]/ EC implementation decision C(2020)6773/F1[i.86] identified 2 different allocations.

- 5 safety related channels between 5875-5925 MHz designated to facilitate the required information exchange and

- 2 non safety related channels in 5855-5875 MHz allocated for non-safety

Spectrum aspects are further discussed in clause 4.2.3.

The initial CA and DEN services were realized and tested in a single channel (5895-5905 MHz). The tests have shown that a single channel is sufficient for the initial services, at least before a high technology penetration is reached. During the development and testing of the CA and DEN two other services not identified in the TR 102 638 were additionally developed: the basic In-Vehicle Signage application, supported by the In-Vehicle Information (IVI) service (ISO TS 19321), and the traffic light information applications supported by the Signal Phase and Timing (SPAT) and map (MAP) services (ISO TS 19091 [i.10]). European projects have proven the correct operation of these services. European R&D projects such as CVIS [i.11], Safespot [i.12], simTD [i.13], Drive-C2X [i.94], Preserve and Scoop@F [i.14] showed that the information exchange required for the operation of these Release-1 [i.28] services already utilize the capacity of a single channel and that additional applications require the use of additional spectrum (channels).

Starting from 2017, Release 2 related applications and services are being developed. Besides C-ITS, the new services also include Automation application and services such as those identified by the CCAM platform [i.32] from the EU commission.

Four distinguish C-ITS levels, named information, active safety, integral safety and passive safety phases, as shown in Figure 2 are recognized.

- The “Information Phase” covers normal driving conditions during which general information is provided to drivers or to the automated system. This corresponds to the normal data exchange, to be considered as part of the ITS related infotainment system and not further discussed in this document.

- The “Active Safety phase” occurs during normal driving mode, when the driver and its C-ITS-system is informed or warned of events in their proximity. All applications as defined for Day-1 or as identified in the TR 102 492-1 [ER-8] are C-ITS Active Safety related.

- The “Integral Safety phase” corresponds to the period in which the vehicle can intervene or take reversible preventive actions. This is the period before possible impact, in which Automation aspects have a key role and are seen as C-ITS Integral Safety.

- The “Passive Safety phase” comes at last in order to reduce the accident severity and includes the application of non-reversible countermeasures. When needed, this includes Rescue Facilities. This is an after-crash information exchange and is not considered in this document, except for possibly (the application has not been defined yet) an E-CALL information exchange based on the C-ITS system.

Figure 2:  Phases of the vehicular safety system (REF)

In parallel with the innovation of C-ITS driver assistance applications, which are developing further in safety related vehicle Automation, road guidance (Informatization of traffic) is developing itself from static navigation towards lane level guidance and headway advice. These aspects are identified in the Declaration of Amsterdam [i.34] (see Figure 3) to lead to common insight and a single safety road traffic automation approach.

Figure 3:  Merge of C-ITS and Vehicular Automation as agreed

in the EU “Declaration of Amsterdam” [i.34].

Many innovative projects are looking at beyond Day-1 applications. Just finished or currently active are for example: VRUITS [i.90], AutoNet [i.43], HIGHTS [i.88], TIMON [i.95], TransAID [i.66], RoadArt and there are new ones upcoming. There is quite a grow of applications and new possibilities are getting recognized. As a consequence, several application lists have been proposed.

Among the others, the project HIGHTS [i.88] has defined a list of C-ITS applications (see Table 2). This list was derived from the C-ITS platform phase I report [i.82], the Amsterdam Group [i.83], C2C-CC [i.51], ACEA [i.52][i.52], 5GAA [i.50], EATA [i.49] and the European projects C-ROADS [i.53], InterCor [i.54], CODECS [i.55] and country specific overviews.

| Group | Use-Case |
| --- | --- |
| Road Works Warning | Short Term Mobile; Short Term Static; Long Term Static; Driving RWW; Roadside RWW and Alarming Winter road works Vehicle |
| Traffic Flow | In Vehicle Signage Navigation; In Vehicle Signage Local; Dynamic Speed; Dynamic Sign Information; Network Flow Optimization; Shockwave Damping; Efficient traffic flow Urban/HighWay; Lane-Merge Assistance / Cooperative Merging; Complex Lane Marking; Regulatory / contextual speed limits notification; Traffic light optimal speed advisory; Zone access control for urban areas; Enhanced route guidance and navigation and Public Transport Vehicle Approaching |
| Intersections Specific | Energy Efficient Intersection Service; Green Light Optimal Speed Advice; Green Wave; Stopping Behaviour Optimization; Red Light Violation Warning; Cross-Traffic Left-Turn Assist, Intersection Movement Assist, Priority Request Business Transport; Priority Request Publik Transport; Priority Request Emergency; Priority Request Group of Cyclists; Virtual VRI in Traffic centre and Intersection Obstacle indication |
| Vulnerable Road Users (VRU) | Bicycle Safety Awareness; Bicycle Priority; Bicycle Approaching Indication; Trian Awareness; Pedestrian Awareness; Motorcycle Awareness and Motorcycle Approaching Indication |
| Autonomous Driving | Basic ACC, Basic CACC; Advanced CACC; Basic Platooning; Advanced Platooning; Automation level 4 and 5 Vehicles; Coop. Adaptive Cruise Control; Merging Assistant; Speed Harmonisation; Automation level road assignment Static and Dynamic and Automation assist in Tunnels; Tele-operated Driving Support and Tele-operated driving for automated parking; Obstructed View Assist |
| Traffic Information | Virtual VMS and Traffic Information Service |
| Traffic Safety Avoidance | Traffic Jam Ahead Warning; Hazardous Location Warning; Emergency Vehicle Warning; Emergency Brake Light; Slow Vehicle Warning; Stationary Vehicle Warning; Overtaking Warning; Intention Sharing and Overtaking Assistance |
| Incident Management | Automatic Incident Detection; Incident Warning |
| Navigation | Intermodal Route Planner; Navigation; Rerouting; Eco Route Planner; Laser scanner based landmark positioning Aid; Parking Assist; Point of interest notification; Automatic access control and parking access; ITS local electronic commerce; Media downloading; Multimodality support; Information on AFV fuelling & charging stations; Loading zone management and Fleet management |
| Vehicle Oriented Non-Safety | EV Charging Point Planner; Smart Parking Assistant; Pay How You Drive; Probe Vehicle Data and IMMA Interface |
| Vehicle Oriented Awareness | Behaviour (awareness); Road Status (awareness) holes in the road etc; Driver Status (awareness) and Vehicle Status (awareness); High-Definition Sensor Sharing; See Through for Passing |
| Tolling | Basic Road Tolling; REQ-2014-0431R03-Use_cases_of_Electronic_Toll_Collection(ETC)_service; ITS-G5 based Tolling and LTE based Tolling |
| E-Tachograph | E-Tachograph |
| Railway | Railway-Road Crossing |
| General Basic | The HIGHTS project [i.88] could be neutral place to develop some basic requirements on positioning and communication to fulfil the basic; Vehicle software / data provisioning and update; Functional testing new applications and Vehicle and RSU data calibration |

Table 2: Safety related applications

Roadmaps from C2C-CC [i.51] and C-ROADs [i.53] roadmaps and the 5GAA [i.50] spectrum requirements paper identify the list of applications in Table 2 as the main applications which can be expected for an ITS release-2. From this list, about 80% of the applications benefit from safety related Ad-Hoc short-range Active or Integral Safety information exchange. From the provided list, we can identify a number of key applications and services, which are expected to have a relative high impact on the safety spectrum usage. The order of this list is defined based on the matureness of the specifications and expected timing these applications and services may become operational.

- Extension of the Release-1 basic applications by the introduction of new warning applications and use of existing for Automated driving leading to updates of CAM (ETSI EN 302 637-2 [i.6]), DENM (ETSI EN 302 637-3 [i.7]), SPATEM (ISO TS 19091 [i.10]), MAPEM ISO TS 19091 [i.10]) and IVI (ISO TS 19321 [i.9]).

- Collective Perception (CP): Cooperative Awareness, letting neighbours know the dynamic state of each C-ITS-S was fundamental as a first step towards safety improvement; however, it has been recognized that for Integral Safety Awareness (ISA) more information, such as that including of non-C-ITS-connected road users is desired. The exchange of knowledge of the surrounding is normally referred as Collective Perception. The projects IMAGinE [i.41], ICT4CART [i.42] and TransAID [i.66] are projects which are realizing innovations in this field. In addition, white papers of C2C-CC [i.51] and ETSI TC IT's are available on this aspect and ETSI TS 103 324 [i.16] on the Collective Perception Service (CPS) plays a major role.

- Vulnerable Road Users (VRU): The project VRUITS [i.90] was one of the early EU projects with focus on VRU applications, remarking the importance of such application as VRUs are victims in many of the accidents. This project has been followed by many other initiatives leading to applications and requirements gathering at ETSI standardisation in an ETSI Special Task Force (STF), which is realizing the VRU specifications ETSI TR 103 300-1 [i.38], ETSI TS 103 300-2 [i.39] and ETSI TS 103 300-3 [i.40].

- Position augmentation: Indirectly, the above-mentioned applications and applications require additional support by system services, which therefore need improvements. One of the crucial aspects is the support for position augmentation services. Augmentation services have been addressed in different EU projects such as HIGHTS [i.88], Propart [i.89], leading to standardisation in the ETSI TS 103 301 [i.46] and ETSI EN 302 890-2 [i.47].

- Maneuver Coordination (MC): CCAD as part of CCAM [i.32] introduces Automation Use cases such as lane change and overtake. More of such Use cases related to automation have been also recognized. Initial research such as executed in the AutoNet2030 [i.43] or TransAID [i.66] EU projects focusses on these aspects as well as projects further introduced in the coming years all leading to standardisations such as of Maneuver Coordination Service (MCS) ETSI TR 103 578 [i.18] and ETSI TS 103 561 [i.42].

- Truck Platooning: Among the multiple platooning projects, an example is the EU project Ensemble [i.35], which is working on a first version of Platooning. In addition to several deliverables which are publicly available, at ETSI a study including the results of this and other projects is being included in ETSI TR 103 298 [i.36] and the related basic CACC specifications in ETSI TR 103 299 [i.37]. The Truck manufactures are expecting to use multiple ITS-G5 channels.

- Beside the identified safety related information exchange in previous points, there is additional information exchange required for a group of specific applications:

  - New Intersection safety applications are being introduced by C-ROADS  [i.53]. applications realized in road infrastructure ICT systems to increase safety specifically based on infrastructure sensors. Additionally, applications such as weather forecast (short term, slippery road) and road conditions (sensors in/at the road and sensors in vehicles);

  - Traffic Prioritisation is a separate group of applications related to public transportation and emergency services;

  - For Agriculture additional specific applications are being defined by the Agriculture organisations such as the AEF [i.45] and,

  - Urban Rail requires information exchange between vehicles and the road infrastructure to satisfy urban rail specific safety requirements.

- High-Definition Sensor Sharing, See Through of Passing, use cases involved in raw data exchange such as video-frame exchange requiring high bandwidth information exchange.

- Non-safety-oriented services such as software updates, Vehicle Health Monitoring, Security Credentials and System validation (new applications and new systems needs to be tested in life situations while not interfering operating C-ITS-Ss.

- In case multiple channels are used in which channel and under what conditions to do application, equipment and systems testing while not interfering with operational applications and equipment.

The depicted main applications and services having impact are those also recognized in the C2C-CC paper “Multi Channel Operation Functional Requirements” [i.125] and in the 5GAA paper “5GAA Releases White Paper on C-V2X Applications: Methodology, Examples and Service Level Requirements” [i.48] and “Study of spectrum needs for safety related intelligent transport systems day 1 and advanced Applications” [i.91]. Those applications which do not make use of ad-hoc communication are not here considered, as they do not have ad-hoc MCO communication requirements.

In the following clauses especially these applications and services will be analysed for their MCO requirements.

# Environmental aspects relevant to MCO

## Introduction

To come to the realization of an implementable MCO concept not only system, functional and technical requirement but also pre-conditional aspects such as regulatory and business aspects need to be considered. In the following subclauses, relevant MCO influencing aspects are recognized and consequences identified.

## Regulation

## Introduction

Regulations is country or regional dependent and therefore needs to be handled as such. The following clauses include relevant workable general and spectrum regulations to be considered for MCO

## European C-ITS regulatory aspects

## Introduction

European regulation provides mandatory and optional obligations to C-ITS realisation. This concerns not only direct functional or technical regulations but also general regulations such as those for open markets. In the following clauses those regulations are included which could have effect on the realisation of an MCO concept and its requirements.

## Technology Neutrality

The European Regulation 2015/2120 [i.56] is laying down principles concerning open internet access, amending directive 2002/22/EC [i.57], Regulation No 531/2012 [i.58] and the EU antitrust and competition laws [i.61] on roaming on public mobile communication networks in the EU which needs to be considered. It states that any EU regulation needs to be “Technology Neutral” to ensure that EU regulation and related other legal EU directives or statements enable an open market according to antitrust and competition laws [i.61].  The term “Technology Neutrality” therefore needs to be judged at regulatory level and does not directly mean technically neutral at a technical level.

NOTE: In spectrum regulation, spectrum allocation can be technology specific such as for 3G/4G/5G. There the technology neutrality (open market) is realized by having auctions for the specific spectrum such that all can bid. For other pieces of the spectrum this can be assigned for a specific functional purpose or device group such as Radar, RLAN, SRD and ITS bands. As none of the solutions is specified, they are “Technology Neutral”

At ETSI the Technology Neutrality is ensured by the ETSI Drafting Rules (EDRs) [i.110]. The standards are technically specific as it should be able to compare technical solutions for their compliance and or interoperability. Standards should not include implementation solutions other than example or as specific case. MCO specifications must follow the drafting rules, be technology agnostic and include functional and technical requirements to enable compliance and or interoperability.

Technology Neutrality as stated in the EU regulations is in the first place a regulation to be followed while writing EU regulations and should be used as check in Business development processes.

## C-ITS interoperability

The EU CCAM [i.32] refers to the European Directive 2010/40/EU [i.15] of the European Parliament and of the Council, which states in action (Article 3) that interoperability should be achieved, with the meaning “the capacity of systems and the underlaying business processes to exchange data and to share information and knowledge”. About specifications, the Directive states that Europe shall use C-ITS specifications which ensures compatibility, interoperability and continuity. Furthermore EU CCAM [i.32] refers to the EU COM(2016) 766 [i.63] regulation “A European strategy on Cooperative Intelligent Transport Systems, a milestone towards cooperative, connected and automated mobility”, A regulation specific for C-ITS which identifies that interoperability has to be realised at all levels and interact with each other, across European borders and transport modes. Interoperability must be effectively be reached following the principles reflected in Annex II of the European Directive 2010/40/EU [i.15].

MCO concepts should not break the interoperability of C-ITS.

## C-ITS Backward Compatibility

The EU ITS regulation Directive 2010/40/EU [i.15] identifies in article 4, Compatibility as “the ability of a device or system to work with another device or system without modification”. In Annex II of this Directive the support as ensuring, where appropriate, the capability for new C-ITS systems to work with already operating C-ITS systems that share a common purpose, without hindering the development of new applications, services, functions, or technologies.

MCO concepts for Release 2 and beyond should not break the C-ITS backward compatibility with previous releases, i.e.  Release 1.

## S

## European C-ITS spectrum regulation

The legal basis of the operation of an C-ITS system in the spectrum band 5855 MHz to 5905 MHz is the ECC decision (08)01 [i.84] and the ECC recommendation (08)01 [i.85] including the EC implementation decision C(2020)6773/F1 [i.86] which will be updated in 2020. The overall spectrum is 50 MHz split into 5 channels of 10 MHz each. The lower 2 channels from 5855 MHz to 5875 MHz are for traffic efficiency application whereas the band 5875 MHz to 5905 MHz is allocated to traffic safety applications. The lower 2 channels overlap with the Short-Range device (SRD) allocation in the band 5725 MHz to 5875 MHz (see Figure 4),

In the updated regulation in 2020 an additional 20 MHz have been made available. The additional band is spit into a 10 MHz band fully available for safety regulation (5905 MHz to 5915 MHz) and a shared band between safety related road C-ITS and rail ITS (Urban Rail) (5915 MHz to 5925 MHz) where rail ITS has a priority. In Figure 5 the spectrum mask is depicted for an C-ITS system with (black line and green line) and without mitigation techniques (black line) to protect tolling operations in the band 5795 MHz to 5815 MHz.

Figure 4: Regulated ITS spectrum

Figure 5: Regulatory spectrum mask in ECC DEC(08)01[i.84] and ECC REC(08)01[i.85] in the year 2020 in dBm/MHz e.i.r.p.

The maximum allowed TX power density is 23 dBm/MHz Equivalent Isotropic radiated Power (EIRP) and maximum TX power of 33 dBm EIRP.

In the regulation the average duty cycle over 1 hour is assumed to have an average of 1% [i.84]  and a message length of 1 ms with a peak limitation to 3% in 1 second. This allows for the transmission of typical messages like CAM on a regular pattern and some additional event driven emergency messages for a limited time (like DENM).

In the European spectrum regulation specific considerations and rules have been included to protect the proper operation of road tolling systems and Urban Rail system in the band 5915 MHz to 5925 MHz. The tolling system in the band 5795 MHz to 5815 MHz has to be protected by a significantly reduced spurious emission level of a C-ITS system in that band or equivalent mechanisms like the reduction of the duty cycle. The sharing mechanisms between Urban Rail systems in the band 5915 MHz to 5925 MHz are under development in ETSI. In this band the Urban Rail system has priority over any kind of road C-ITS system.

Since the band 5855 MHz to 5875 MHz is harmonized as part of the Short Range Devices (SRD) Regulation EC decision 2019/1345 [i.87] specific spectrum polite rules need to be implemented allowing other SRDs to access the spectrum. For an ITS-G5 system these spectrum polite rules are inherently available by the CSMA/CA medium access protocol which uses a listen-before-talk mechanism with an energy threshold of -65dBm. In the shared spectrum in the band 5855 MHz to 5975 MHz this mechanism allows for the smooth operation of other system in this licensed exempt band.

In the band 5875 MHz to 5925 MHz C-ITS systems are operated on a co-primary allocation together with fixed satellite uplinks. These uplinks are operated at very limited number of positions in Europe. In the very close vicinity of these satellite base stations operating in the band 5875 MHz to 5925 MHz a range degradation of the C-ITS systems might occur. More detailed investigations (simulations, measurements) are needed in the future to evaluate the effect in real life situations.

In Europe the CEPT has allocated the frequency band 63.72 – 65.88 GHz for safety related and traffic efficiency ITS applications in ECC decision (09)01[i.113] from 5 July 2019. In this allocation ITS is considered as part of the mobile services. In this 2.16 GHz of spectrum ITS devices can operate with a maximum EIRP of 40dBm. No spectrum density limits are defined in the decision.

The band is also harmonized in the EC Decision 2019/1343[i.114] from 2 August 2019 as band number 77 of the SRD decision using the same conditions.

The band has to be shared with other applications, which are mainly used by fixed links and broadband SRDs. The band can be used for future extensions of the ITS services with the main focus onto very short-range high data-rate communication. An initial specification effort is under way as part of the IEEE802.11bd [i.119] activities.

In the scope of the MCO development covered in this document this spectrum band will not be further considered.

## E

## Release-1 Basic set of Application release consequences

## Introduction

According to European regulations and arguments presented in previous clauses such as in 4.3, for the realisation of a MCO concept the Release-1 set of specifications needs to be the starting point and its consequences considered. Release-1 includes all Basic Set of Applications (BSA) identified in the ETSI TR 102 638 [i.4] but also includes initial Vehicle Signage (IVS) applications supported by message standard IVI (ISO TS 19321 [i.9]) and Traffic light priority applications supported by message standards SPAT/MAP (ISO TS 19091 [i.10] and SAE J2735  [i.93] as supported by the C2C-CC V1.5 and aligned to C-ROADs profiles. The following clauses reflect the communication behaviour of this message exchange as identified in many European Projects and stakeholder consortia evaluations. The evaluations consider only the information exchange for these applications and services in the single 10 MHz band (180) in the safety related 5.9 GHz spectrum.

## BSA Communication considerations

The Basis set of Applications (BSA) as specified in the ETSI TR 102 638 [i.4] technical report was the starting point of many functional and technical developments and evaluations in Europe. Car industry oriented projects such as simTD [i.13] and general oriented EU projects such as CVIS [i.11] and Safespot [i.12] were important to allow different stakeholders to work together and realize interoperable standards and specifications. All ETSI TR 102 638 related applications were based on the exchange of 2 basic message facility services, CAM [i.6] and DENM [i.7] message sets providing Cooperative Awareness and Decentralized Notifications.

The basic operation of these message sets in the single 10 MHz channel in the 5.9 GHz safety related spectrum was identified and led to the specification of a set of communication parameters, system conditions, standards and white papers. All communication settings are depicted from released standards and made interoperable in the first ETSI ITS Release-1 ETSI TR 101 607 [i.28] and C2C-CC Basic System Profile (BSP) [i.51], and ETSI LTE-V2X Profile ETSI TS 103 723 [i.125].

For the purpose of creating an MCO concept the following Release-1 aspects need to be considered.

- Release-1 identified communication parameters are based on expected C-ITS penetration in the coming 5-10 years of about 60-70% (mainly CAM’s and DENM’s) market penetration level. For higher penetration levels a single channel for the release-1 services may not be sufficient.

- Current single channel congestion control mechanisms intend to keep headroom to enable the transmission of high priority messages. For example, a restrictive congestion level is conservatively set to 60% when Ton is max 1 ms in ETSI TS 102 687 [i.98], and more CR limits are set to higher priority (i.e. PPPP) messages in ETSI TS 103 574 [i.124].

- It is further assumed that DENM’s are rarely transmitted so that CAM transmission is very dominant in the channel. Aspects at least confirmed by the applications knowledge and large-scale tests at that time.

- Later as proposed by authorities based on simulations and expectations the transmission in the same control channel of IVI, SPAT and MAP was agreed under the condition that the size and transmission frequencies will stay low and that the transmission priority should be lower than CAM and DENM. These aspects have not been verified on large-scale but assumed not to be problem during the initial years of deployment.

If the C-ITS penetration reaches near 100%, not all the CAMs and even DENMs can be successfully transmitted, and therefore off-loading or other mitigation mechanisms may need to be considered.

## Release-1 specifications being affected

## Introduction

Different Release-1 specifications can influence the design of an MCO concept. They will need to be taken into account and evolved when needed.

## Traffic classes

Packets generated by applications and services are classified into TC (Traffic Classes) at the Facilities layer depending on their priority. The TC is included in a field of the Common Header of all transmitted GeoNetworking packets as specified in the ETSI EN 302 636-4-1 [i.99]. The Common Header is defined within the media-independent functionalities, which are common to all C-ITS access technologies for short-range wireless communication to be used for GeoNetworking (e.g. in ETSI TS 102 636-4-2 [i.100] for ITS-G5 and ETSI TS 103 613 [i.101] for LTE-V2X). The TC field has 8 bits that include:

- TC ID (6 bits): as specified in the media-dependent part of GeoNetworking corresponding to the interface over which the packet will be transmitted, e.g. in ETSI TS 102 636-4-2 [i.100] for ITS-G5 and ETSI TS 103 613 [i.101] for LTE-V2X.

- Channel Offload (1 bit): indicates whether the packet may be off-loaded to another channel than the one specified in the TC ID.

- SCF (store-carry-forward, 1 bit): indicates whether the packet shall be buffered when no suitable neighbour exists.

According to the C2C-CC Basic System Profile 1.5.1 [i.51], it is not required to off-load packets to another channel, and therefore the Channel Off-load bit of the TC field is set to pGnChannelOffLoad=0. It is intended to enable future use of according features and could be particularly useful in a potential MCO concept.

In ITS-G5, each TC ID is mapped into an Access Category (AC). ACs are used by ITS-G5 to prioritize between different types of packets using the four different Enhanced Distributed Channel Access (EDCA) queue defined in [i.25] for each frequency channel. ITS-G5 is able to prioritize between different ACs using different listening periods (arbitration inter-frame space, AIFS) and Contention Window (CW) settings for each AC [i.25]. The TC IDs are also mapped into transmission parameters for ITS-G5, such as the maximum transmission power or the data rate (modulation and coding scheme) [i.25].

In LTE-V2X, each TC ID is mapped into a ProSe Per-Packet Priority (PPPP) [i.101]. Packets are prioritized according to its PPPP value in LTE-V2X as defined in ETSI TS 136 331 [i.111] and ETSI TS 136 213  [i.112]. Each PPPP is mapped to certain transmission parameters, such as modulation and coding scheme. Higher priority PPPPs can use a higher amount of radio resources, as defined in ETSI TS 103 574 [i.22], following the congestion control mechanism defined for LTE-V2X.

The MCO concept should be able to differentiate the priority of different applications and services. For example, packets generated by high priority applications and services could be transmitted in the CCH, where more C-ITS-S may be listening to, while lower priority applications and services could be off-loaded to a SCH. This could be done by reusing the already existing TCs or extending them for a more granular and accurate classification.

## Channel load measurement

Both ITS-G5 and LTE-V2X make use of congestion control protocols based on the periodic measurement of the channel busy ratio (CBR) metric. The CBR is a channel load metric used to control and distribute the channel access among C-ITS-S. CBR is an estimate of how much a single channel is used based on listening on surrounding radio transmitters. It is defined in ETSI EN 302 571 [i.21] for ITS-G5 and in ETSI TS 103 574 [i.22] for LTE-V2X. For ITS-G5, the CBR is calculated according to clause 4.2.10 in [i.21], as the ratio of time when the received signal strength is higher than -85 dB over a period of 100 ms. The DCC_ACC component provides the local CBR value to the DCC algorithm ETSI TS 102 687 [i.98]. For LTE-V2X, the CBR is calculated according to clause 5.2 in [i.22], as the  fraction of sub-channels whose S-RSSI exceeds a threshold -94 dBm for PSSCH, or the fraction of resources  (2 PRB pair) whose S-RSSI exceed a threshold of -94 dBm for PSCCH with non-adjacent control and data resources. It is also measured over 100 ms.

Updated CBR information is needed to be able to satisfy the congestion control limits in both ITS-G5 and LTE-V2X and may also be needed to decide the channel(s) to be used. However, a C-ITS-S may only have updated CBR information about the channel(s) it is using. It is not clear how a C-ITS-S could obtain the CBR of a channel it is not listening to. That information could be needed by the C-ITS-S to transmit a packet.

In addition, an MCO concept could potentially require knowing the CBR of the different channels in order to decide the channel used to transmit each packet. That would also require updated CBR information of all the channels.

## Channel assignment

ETSI TS 102 724 [i.97] specifies the channel usage in the ITS G5A and ITS G5B bands for multichannel operation support, including control and service channels operation for ITS G5, ITS G5 transmit and receive policies, channel selection and configuration, and ITS G5 adjacent channel interference considerations, among others. According to ETSI TS 102 724 [i.97], the Channel-Configuration entity located at the Access layer manages the transmission to and reception from multiple channels. The usage of the ITS-G5 channels is under control of the Decentralized Congestion Control (DCC). This specification defines the access layer requirements for different DCC profiles (DP1 to DP32) for each channel (CCH, SCH1, SCH2, …). The message received by the access layer will be transmitted using the first possible channel in the usage order defined in ETSI TS 102 724 [i.97] depending on the congestion status and the DCC profile of the message.

While ETSI TS 102 724 [i.97] is an active specification, the adoption and current usage of the DCC profiles and the multichannel operation concept defined in ETSI TS 102 724 [i.97] are not clear:

- DPs were adopted and mapped to TCs in the C2C-CC Basic System Profile 1.2.0 (September 2017), but their definition and usage are not present in the current version, 1.5.1 (July 2020) [i.51].

- Current C2C-CC Basic System Profile [i.51] includes in Requirement RS_BSP_435 that the access layer shall be compliant with ETSI TS 102 724, but it might refer to the channel policies.

- The access layer requirements defined in ETSI TS 102 724 [i.97] for the different DCC profiles are not compliant with the DCC Adaptive strategy defined in ETSI TS 102 637. These requirements are restricted to 3 states of the DCC Reactive strategy, but ETSI TS 102 637 leaves the door open to have more states when the Reactive strategy is used.

- ETSI TS 102 724 is referred by EN 302 637-2 for the adaptation of the T_GenCam_Dcc parameter in order to reduce the CAM generation according to the channel usage requirements of DCC. It can be misleading as it gives the impression that the 32 DPs should be enforced.

- The access layer requirements defined in ETSI TS 102 724 [i.97] for the different DCC profiles collide with the message transmission policies being defined in ETSI TS 103 141 for the DCC Facilities entity.

A revision of ETSI TS 102 724 [i.97] might be needed in consideration of potential problems regarding congestion control with MCO.

## F

# Functional MCO considerations and requirements

## Introduction

In the following clauses the application and service communication requirements are identified from the most spectrum demanding applications as identified in clause 3.2 list of applications and services having highest spectrum performance impact.

## Release-1 Extended Services

### Introduction

In clause BSA Communication requirements 4.4.2, the single channel CCH/SCH0 requirements of the Day-1 applications are addressed.  As identified in 5GAA [i.50], C2C-CC [i.51] and C-ROADs [i.53] reports (see clause 3.2) for Day-2, extended warning applications are identified and recognized requiring more effective use of the existing message sets such as DENM (ETSI EN 302 637-3 [i.7]), SPATEM (ISO TS 19091 [i.10]), MAPEM (ISO TS 19091 [i.10]) and IVI (ISO TS 19321 [i.9]), but also extension of these sets. In the following clauses the related extensions and MCO communication requirements identified.

### Extended Service dissemination

The increase of events generating safety applications, extended data characterisation and higher accuracy requirement will increase the size and amount of the DENM (ETSI EN 302 637-3 [i.7]), SPATEM (ISO TS 19091 [i.10]), MAPEM (ISO TS 19091 [i.10]) and IVI (ISO TS 19321 [i.9]). One example is the MAPEM (ISO TS 19091 [i.10]), currently being a single layer map will grow significantly into a multi-layer and more dynamic map. In general, the extended use of warning applications more towards automated actions in vehicles for example will lead only to more precise parameters such as the position accuracy and a like. This could also have effect on the transmission rules and require higher levels of trust and confidence. All of this will lead to more data transmissions while penetration of the amount of C-ITS-Ss is expected to grow. These aspects are currently being investigated but have not result in any change of specifications and no test results are available.

### Extended Service MCO communication requirements

Based on the ongoing investigations into extended warning services the dissemination requirements will increase the size and amount of transmitted message and may lead to a situation in which the congestion state in the CCH/SCH0 channel is earlier reached. Fur such cases it could be required to use off-load mechanisms to satisfy the applications and therefore should be considered. So far there is one case recognize which could require another approach. The expected Multi-Layer MAPEM (ISO TS 19091 [i.10]) will use a significant larger packet size to exchange the information and as the CCH/SCH0 will be occupied it is recommended to use another channel or be exchanged by another mechanism. For this case as MAPEMs (ISO TS 19091 [i.10]) for dynamic use will be more time critical compared to the static MAPEM (ISO TS 19091 [i.10]) of Release-1 this differentiation is also relevant.

## System MCO relevant aspects

### Introduction

To allow applications to exchange information they need to be supported by some additional services. The minimum set would at least consist of security as this is needed to realize trust between the different equipment participating in S-ITS. Additionally, in C-ITS there are services such as the Service Announcement (SA) service as defined in the ETSI EN 302 890-1 [i.67] which enables the possibility of making C-ITS-S aware of available C-ITS services, and the Position and Time (PoTi) service as defined in the ETSI EN 302 890-2 [i.47] which includes the support for some position accuracy improvement services making use of information exchange. This also include privacy considerations.

In the following clauses those system services which require information exchange between C-ITS-Ss are identified.

### Service Announcement (SA)

#### Introduction

When there is an interest of notifying C-ITS-Ss that there is a specific service available, such service can be broadcasted by a C-ITS-S. To inform other C-ITS-Ss of the availability of a specific service a C-ITS-S can send a Service Announcement Essential Message (SAEM) as specified in the ETSI EN 302 890-1 [i.67]. Announcements can be used for safety and non-safety services and could lead to communication via any protocol or network, whether this is via Hybrid Communication as defined in the C-ITS platform phase I report [i.82], via multiple technologies or via various channels in a single technology setups, via DAB+, or via internet protocols. The Service Announcement Service (SAS) as specified in the ETSI EN 302 890-1 [i.67] is an awareness message and enables C-ITS applications to select the appropriate communication setup suited for the application. The SAS as specified in the ETSI EN 302 890-1 [i.67] has a direct link to the base SAS standards ISO TS 16460 [i.68] and ISO TS 22418 [i.69].

The SAS provides a C-ITS service provider or just a C-ITS application the possibility to setup the communication for the application activation in other stations. A single, group, or all C-ITS-Ss present in the C-ITS constellation could be addressed. The ETSI EN 302 890-1 [i.67] defines the interfaces for both the transmission and the reception application for SAEM, and related security requirements. The SAS can be used between applications in the C-ITS-S and between C-ITS-Ss or a combination of both, see ETSI EN 302 890-1 [i.67].

#### SAEM information dissemination

The distribution of services can target a geographical area in which the service is relevant. A SAEM can be disseminated outside of the service relevance area to prepare the consuming part of the application. They could also be planned to be along a predictable journey.

For MCO, the SAEM should include all necessary information for ITS application to be able to access other ITS applications on different channels and/or technologies. Currently, the SAEM includes information about the communication technology used for the application relevant information exchange (only IEEE-802.11 based technologies are supported for now).  From a protocol point of view the EN 302 890-1 [i.67] supports WAVE Short Message Protocol, GeoNetworking, Basic Transport Protocol, and IPv6 (based on TCP according to IEEE 1609.3 [i.70]).

In case BTP is used, the BTP port number should be as specified in ETSI TS 103 248 [i.72]. The SAEM should be send as broadcast with a repetition between 0 and 255 (number of times the SAEM is transmitted per 5 s). The SAEM could be sent via ITS-G5 according to CPS_001 or LTE-V2X according to CPS_004 from ETSI TS 103 301 [i.46]. Other protocols or repetition intervals information should be specified for MCO based on technology and channel and if the service is safety related or non-safety related.

For the realisation of advertising, SAS is an awareness service and makes use of multicasting protocols. To allow all participating C-ITS-Ss to be aware of the advertised application, the SAEMs must be distributed on a predefined channel and by predefined communication protocols. As in Europe safety and non-safety related applications are spectrum wise separated, it is relevant in the European case to also distinguish safety and non-safety related SAEMs. Safety related SAEMs must therefore be distributed in one of the safety related channels and non-safety SEAMs in a non-safety channel in the European case.

Service Providers (SPs) and C-ITS-Ss applications can announce their applications or capabilities via SAEM unicast or broadcast transmissions. It has no sense to announce awareness applications or services which by nature are broadcast services for which the initiative is with multiple SPs or C-ITS-Ss such as the CA, CP, MC or event messages as DEN services. The SAS service can be used for the announcement of specific services for which there is a single controlled entity having the control over the service such as for such as for the IVI, SPAT, MAP, Preemption (SSM and SRM), Certificates provisioning and System Services e.g. TIM and RTCM services. For these services it is not a requirement to use SAS, but the SP or community having interest in specific C-ITS applications may decidedeside individual or commonly (interoperable) the approach. The SAS service would very well fit to applications such as Platooning, GLOSA, Green Wave, CACC or Traffic Light Information.

SAS could be used as MCO mechanism to direct services to a channel which is available and capable to support the applications data exchange requirements. In that case some MCO general rules may need to be set.

#### SAEM MCO communication requirements

As clarified in the previous clause it needs to be agreed where and with what protocol stations can expect SAEMs being distributed as it is awareness information to all interested C-ITS-Ss. The use of SAS is limited to the non-awareness-oriented services as identified in 5.3.2.2. Two SAEM use cases can be identified.

- SAEMs of interest to all C-ITS-Ss. A generic SAEM for which is required to fix all without exception requires fixed communication related requirements such as channel, protocol and included underlaying technical specifications.

- SEAMs which are of interest to subscribers and where a SP or a specific C-ITS-S is responsible for. For such SEAMs, it is up to the SP or owner of the C-ITS-S to specify all communication related requirements such as channel, protocol and included underlaying technical specifications.

Based on spectrum regulations in Europe there is the additional restriction in Europe that safety related SAEMs can be transmitted in a safety related channel but that non-safety related SAEMs are restricted to the non-safety related channel.

As it has no sense to use SAS for awareness services, SAEMs are mostly transmitted by Road Site Units (RSUs) under the transmission rules as specified in the SAS specification ETSI EN 302 890-1 [i.67]. Based on this it is not expected to generate a big load on any channel. The communication requirements are given by Table 4.

Table 4: SAS communication requirements [i.130]

Note: % channel load is the allocated % of the channel load where 100% represents the maximum channel load before the channel congestion state is reached. There are no messages and therefor assumed similar to CAM.

### P

### P

### S

## Collective Perception (CP)

### Introduction

The Cooperative Awareness Service (CA service) lets other road users know about the dynamic state of C-ITS-S itself. This was the first step bringing awareness. It has been recognized that for further Integral Safety Awareness (ISA) more information is needed.

Collective Perception Message (CPM) provides information of objects based on detection by sensor systems such as camera’s and infra-red sensors connected to the C-ITS-S. The projects IMAGinE [i.41], ICT4CART [i.42] and TransAID [i.66] are realizing innovations in this field which are reflected in the technical report ETSI TR 103 562 [i.17] which outlines the potential functional possibilities and in the ETSI TS 103 324 [i.16] which specifies the Collective Perception (CP) service. The CP service supports sharing the kinematic and attitude state information about objects in the surrounding of the C-ITS-S equipped road user. These objects for instance could be C-ITS-S non-equipped road users, static or other dynamic objects like roadblocks and empty road segments. These can be detected by local perception sensors such as camera’s and infrared sensors.

### CP information dissemination

Collective Perception (CP) will further increase awareness and may address the following use-cases:

- Create Awareness about non-connected road users of any kind which could be subdivided in:

  - Non-connected vehicles

  - Non-connected other road users (mainly VRU’s) 
Which could be still differently behaving, e.g. bikes, moppets, pedestrians, and road workers.

- Create Awareness about Detected Safety-Critical Objects (DSCOs): these objects are not a recognized as active road users but can be seen as object influencing road situations and related resulting traffic behaviour.

- Create Awareness via CAM Information Aggregation (CIA): CAMs received from connected road users which are possibly not received by other connected road users due to radio propagation of the surrounding environment can be aggregated in a CPM.

- Create Awareness about Empty Road Segments (ERSs): provide recognized areas which are not occupied by any of the known objects as referenced in previous categories.

Although Collective Perception is a single service it projects the behaviour of objects with different dynamics. While the Cooperate Awareness service describes only the behaviour of a single road user, the Collective Perception service provides information about the dynamic state of different behaving objects. Current specification identifies the rate being dominated by the object with most dynamic behaviour and information for those with a lower dynamic state, the information is less frequently included in the message.

In the TR 103 562 [i.17], analyses show that such single service may need more than one channel to share the information. To realize clearness about where and how to receive relevant CP information it would be better to limit CPMs to include only information for a specific application or group of applications. The following CPM use case classification is identified:

- Non-Connected Vehicles (NC-Veh, Extension of the vehicle awareness as shared by CAM)

- Non-Connected Vulnerable Road Users (NC-VRU, VRUs, a new use case implemented in VRU applications)

- Detected Safety-Critical Objects and Empty Road Segments (DSCOs+ERSs, additional information)

- CAM information Aggregation (special case on request of authorities, such as CIA and RSU initiatives).

The CPM is generated cyclically with a message size that depends on the number of detected objects and onboard sensors, among other parameters as specified in ETSI TS 103 324 [i.16]. CP messages can be fragmented when exceeding the maximum message size allowed by the lower layers, and each segment can be interpreted independently. Adaptive message generation rates are used to decrease the channel load while focusing on reporting changes in the dynamic road environment. How often a detected object is included in a CPM depends on the mobility of the object. The maximum message generation rate may be limited by the congestion state of the channel.

Simulation results in [i.17] show that CPM generation rates between 3Hz and 10Hz, and CPM sizes between 300 bytes and 1350 bytes can be expected. The CP message lifetime does not exceed 1s. The Traffic Class (TC) of the CPM is defined in ETSI EN 302 636-4-1 [i.99]. CPMs are signed using private keys associated to Authorization Tickets that contain SSPs of type BitmapSsp as specified in ETSI TS 103 097 [i.74].

CPMs bring awareness and at least for the analyses for Day-2 use, they are recognized as additional information. Therefore, they are currently assumed not requiring different Functional Safety and SOTIF levels, as needed for CAMs, and therefore no advanced Functional Safety and SOTIF requirements are applicable.

### CP MCO communication requirements

The size of the CPM very much depends on the scenario e.g. urban, sub urban or highway and whether there is dense traffic including pedestrians or not.

CPMs are generated periodically with a rate controlled by the CP service in the originating ITS-S. The generation frequency is determined by taking into account the dynamic behaviour of the detected object status, e.g. change of position, speed or direction, sending of CPMs for the same (perceived) object by another ITS-S.

Depending on the type of C-ITS-S they me send specific object class (see TS 103 324 ([i.16])

The size variation is very dynamic depending on the scenario, see Table 8 and the dynamic state perceived from the different detected objects. The studies presented in ETSI TR 103 562 [i.17] indicate that CPM transmission requires message segmentation (larger size then allowed by the underlaying used technologies) and is expected to exceeds the bandwidth of a single channel in future.

The minimum time elapsed between the start of consecutive CPM generation events shall be equal to or larger than T_GenCpm. T_GenCpm is limited to T_GenCpmMin ≤ T_GenCpm ≤T_GenCpmMax, where T_GenCpmMin and T_GenCpmMax are defined in Table 7.

In case of ITS-G5, T_GenCpm shall be managed according to the channel usage requirements of Decentralized Congestion Control (DCC) as specified in ETSI TS 102 724 [i.97]. The parameter T_GenCpm shall be provided by the management entity in the unit of milliseconds. If the management entity provides this parameter with a value above T_GenCpmMax, T_GenCpm shall be set to T_GenCpmMax and if the value is below T_GenCpmMin or if this parameter is not provided, the T_GenCpm shall be set to T_GenCpmMin. The parameter T_GenCpm represents the currently valid lower limit for the time elapsed between consecutive CPM generation events.

In case of LTE-V2X PC5, T_GenCpm shall be managed in accordance to the congestion control mechanism defined by the access layer in ETSI TS 103 574 [i.124].

| Parameter | Type | Meaning | Recommended value |
| --- | --- | --- | --- |
| T_GenCpmMin | Time in ms | The minimum time elapsed between the start of consecutive CPM generation events. For LF container [2000] ms shall be used (Optional Container added as part of the CPM every 2000 ms) | 100 |
| T_GenCpmMax | Time in ms | The maximum time elapsed between the start of consecutive CMP generation events | 1000 |

Table 7: Parameters for CPM generation TS 103 324 ([i.16], clause 6.1.3.1)

Table 8: Typical CPM requirements [i.130], [i.131]

## Vulnerable Road User (VRU)

### Introduction

The project VRUITS [i.90] was one of the early EU projects with focus on VRU applications, pointing out the importance of such application as VRUs are victims in many of road accidents. This project has been followed by many other initiatives coming to a number of conclusions from which a number are included in the ETSI TR 103 300-1 [i.38].

For the realisation of VRU awareness by other road users a number of aspects is relevant from which two are essential to realize a high enough confidence level such that the information is used.

- The position accuracy according to the HIGHTS project [i.88] needs to be better than 30 cm to be able to identify the difference of the different VRUs and whether it is on the road or on the side walk.

- The determination of the active state (participating in traffic or not) and its intensions.

Overcoming these challenges depends on availability of position accuracy improving possibilities the determination possibilities available often also depend on the possible implementation. There are three cases in which a C-ITS-S can be informed about the presents of an VRU.

- By its own sensors

- By an observing ITS-S which recognizes the presence of an VRU

- By the ITS-S of the VRU

The first case is obvious. In the second case for instance an RSU can be the observer and provide VRU information via the CPM as described in 5.4. In the last case VRU Awareness messages (VAMs) can be transmitted by the VRU C-ITS-S as identified in the ETSI TS 103 300-2 [i.39] and specified in ETSI TS 103 300-3 [i.40].

### VRU information dissemination

The objective of the VRU basic service is to enable the transmission and reception of VRU awareness messages (VAM) to enhance the protection of road users such as pedestrians, bicyclists, motorcyclists as well as animals that may pose a safety risk to other road users.

VRUs like other road users should only transmit safety related information in case it participates in the road traffic. VRU ITS-S should always send VAMs, while other ITS-S including infrastructure and vehicle ITS-S can use CPM to signal the identified presence of a VRU in the vicinity. The relevant use cases are depicted in ETSI TR 103 300-1 [i.38].

The following VRU profiles are specified in clause 6.1 of ETSI TS 103 300-2:

VRU Profile 1 - Pedestrian. Typical VRUs in this profile: pedestrians, i.e. road users not using a mechanical device for their trip. It includes for example pedestrians on a pavement, but also children, prams, disabled persons, blind persons guided by a dog, elderly persons, persons walking beside their bicycle.

VRU Profile 2 - Bicyclist. Typical VRUs in this profile: bicyclist and similar e.g. light vehicles riders, possibly with an electric engine. It includes bicyclists, but also wheelchair users, horses carrying a rider, skaters, e-scooters, personal transporter, etc.

VRU Profile 3 - Motorcyclist. Typical VRUs in this profile: motorcyclists, which are equipped with engines that allow them to move on the road. It includes users (driver and passengers, e.g. children and animals) of Powered Two Wheelers (PTW) such as mopeds (motorized scooters), motorcycles or sidecars.

VRU Profile 4 - Animals presenting a safety risk to other road users. Typical VRUs in this profile: dogs, wild animals, horses, cows, sheep, etc. Some of these VRUs might have their own ITS-S (e.g. dog in a city or a horse) but most of the VRUs in this profile will only be indirectly detected. Especially wild animals in rural areas and highway situations.

NOTE:	A VRU vehicle itself does not represent a VRU but only the combination with at least one person will create the VRU.

The details are specified in ETSI TS 103 300-3[i.40].

### VRU communication requirements

In case VRUs participate in traffic and are integral part of Road Safety focussed improvements, the use cases given in ETSI TR 103 300 -1[i.38] and the functional requirements given in ETSI TS 103 300-2[i.39] apply.

Based on the specified VBS operation and the given VAM format in TS 103 300 -3 [i.40] communication and resource requirements can be derived. As all C-ITS messages the VAM dissemination is managed in accordance with the relevant congestion control mechanisms depending on the used access layer.

The size of an individual VAM message depends on the actual situation, set of parameters to be transmitted and the use of security certificates and signatures. Some typical VAM sizes for VRU profile 1 are given in Table 9.

Parameters:

- High Frequency Container (HFC):

  - Heading, speed, longitudinal acceleration, lane position, environment

- Low Frequency Container (LFC):

  - Profile, size class, but not exterior lights

- Motion Prediction Container:

  - Path history (PH): 1, 5, 10 entries

  - Path prediction (PD): 5 entries

| Case No | Parameter | comment | Size |
| --- | --- | --- | --- |
| 1 | LFC + 10PH + 5PD | HFC including LFC and 10 points in the path history without GN header | 137 bytes |
| 2 | No LFC + 10PH + 5PD | HFC, no LFC and 10 points in the path history without GN header | 135 bytes |
| 3 | LFC + 5PH + 5PD | HFC including LFC and 5 points in the path history without GN header | 104 bytes |
| 4 | No LFC + 5PH + 5PD | HFC no LFC and 5 points in the path history without GN header | 103 bytes |
| 5 | LFC + 1PH + 5PD | HFC including LFC and 1 point in the path history without GN header | 78 bytes |
| 6 | No LFC + 1PH + 5PD | HFC no LFC and 1 point in the path history without GN header | 77 bytes |

Table 9: Some typical VAM sizes for VRU profile 1

The figures given in  Table 9 are without any additional overhead from security, GeoNetworking nor Access layer header. The security overhead is typically 100 bytes if the signing certificate is omitted, 230 bytes if the signing certificate is attached. This overhead represents typically the most significant overhead to be transmitted. Thus, the typical range for a VAM size is between 178 Bytes (case 6 with security) and 367bytes (case 1 with security and certificate).

The VAM generation time is mainly contingent upon the dynamic behaviour (e.g. speed, distance, orientation) of the individual VRU or cluster of VRUs (a group of VRUs identified by a single VAM). More details to the optional cluster operation of VRUs can be found in ETSI TS 103 300 -3 [i.40]The generation periodicity T_GenVam is bounded by the parameters given in Table 10. It has to be mentioned that the low frequency container of the VAM is not always present. It should be included at least every 2000 ms in case it is used by the individual VRU. In ETSI TS 103 300 -3[i.40] it can be seen that the time between two consecutively transmitted VAMs ranges from 100 ms up to 5,000 ms. It can be assumed that for a typical pedestrian VRU (VRU profile 1) the generation time ranges from 1,000 ms to 5.000 ms. Overall communication requirements can be found in Table 11.

| Parameter | Type | Meaning | Recommended value |
| --- | --- | --- | --- |
| T_GenVamMin | Time in ms | The minimum time elapsed between the start of consecutive VAM generation events. For LF container [2000] ms shall be used | 100 ms |
| T_GenVamMax | Time in ms | The maximum time elapsed between the start of consecutive VAM generation events | 5,000 ms |
| T_AssembleVAM | Time in ms | The time allocated for assembling a VAM packet in the facilities layer | 50 ms |

Table 10: Parameters for VAM generation ([i.40], clause 8)

Table 11: VAM generalised communication requirements [i.130]

## Maneuver Coordination (MC)

### Introduction

Maneuver Coordination (MC): CCAD as part of CCAM [i.32] introduces Automation use cases such as lane change and overtake. More of such use cases related to automation have been also recognized. Initial research such as executed in the AutoNet2030 [i.43] or TransAID [i.66] EU projects focusses on these aspects as well as projects further introduced in the coming years all leading to standardisations such as of Maneuver Coordination Service (MCS) ETSI TR 103 578 [i.18] and ETSI TS 103 561 [i.42].

The Maneuver Coordination Service ETSI TR 103 578 [i.18] is indented to support connected automated driving by enabling vehicle C-ITS-Ss to exchange information about their intended Maneuvers. This exchange enables that two or more vehicle C-ITS-Ss coordinate with each other to efficiently and safely perform a Maneuver (e.g. a lane change or lane merge). It also improves the prediction of the future locations of nearby vehicle C-ITS-S. Road side C-ITS-Ss can assist vehicle C-ITS-Ss by suggesting specific Maneuvers, such as a speed change a lane change [i.20].

In contrast to the Cooperative Awareness service, that allows the exchange of past trajectories (pathHistory data element), the Maneuver Coordination Service enables C-ITS-S vehicles to exchange information about the intended future trajectories, and coordinate in case of conflict.

### MC information dissemination

The Maneuver Coordination Message (MCM) is a broadcast message that is expected to include basic information about the transmitting C-ITS-S as well as additional information depending on the type of C-ITS-S. For vehicle C-ITS-Ss, the MCM is expected to include the intended (or planned) Maneuvers and one or more desired (or alternative) trajectories. Each trajectory is a spatial-temporal description of the vehicle C-ITS-Ss trajectory in the next 5 to 10s. While the planned trajectories are used by C-ITS-Ss to improve the prediction of future locations of nearby vehicle C-ITS-Ss and to detect conflicts, the desired trajectories are used to request a coordination between vehicle C-ITS-Ss. The message format has not been standardized yet, but MCMs that contain only a planned trajectory could have a size of around 300 bytes, and this size is increased up to around 600 bytes when a desired trajectory is added according to the TransAID EU project deliverable D5.1 “Definition of V2X message sets” [i.19]. The message generation rules have not been defined yet, but MPMs are expected to be generated continuously at a rate between 1Hz and 10Hz depending on the context as identified in the TransAID EU project deliverable 5.2, “V2X-Based Cooperative Sensing and Driving in Transition Areas” [i.20]. The idea behind the continuous message exchange is the early detection of the need of a Maneuver coordination. For roadside C-ITS-Ss, the MCMs are expected to include specific advices for specific vehicle C-ITS-Ss, to e.g. suggest a given speed or a lane change according to the TransAID EU project deliverable D5.1 “Definition of V2X message sets” [i.19]. Thus, the MCMs transmitted by roadside C-ITS-Ss are expected to be smaller in size (although they can include advices for multiple vehicles) and transmitted less frequently than those transmitted by vehicle C-ITS-S.

Besides the planned and intended trajectories as defined in the ETSI TR 103 578 [i.18] this specification also recognized the need for proposed trajectories. As for instance as presented in Figure 6 Lane Change Assist at Bus stop (LCA-B), Vehicle S wants to overtake the stationary Bus. Any Road user or other ITS-S equipped road safety stakeholder could analyse the road traffic situation based on received C-ITS information combined with other sensor data and advice or manage the road users how to act to realize an efficient and safe resolution of the situation.

Instructions on legal bases could be provided by IVI and DENM messages but also suggestions could be made how to behave. This could be done by providing proposed trajectory patterns to the road users in the relevance area.

Figure 6: LCA-B Situation description (ETSI TR 103 578 [i.18]).

By using one of these methods the situation of shared liability is avoided. The legal binding methods based on IVI and warning based on DENM are not covered here but are covered as part of Day-1 services operation.

Proposed trajectories are expected to be mostly used in specific use cases for instance at dangerous crossovers, schools and bus stops as in Figure 6. As the nature of use is similar to that of an intended trajectory it may be assumed that for the purpose of this report the communication requirements are similar.

The MC service specification TR 103 578 [i.18] currently support planned, intended and proposed trajectory. For the purpose of for instance emergency and public transport Mandatory trajectories could be an additional trajectory message. The mandatory trajectory could be used by police, ambulance, fire brigade and public transportation. For the purpose of the MCO concept this is considered.

### MC MCO communication requirements

It has been argued that MC is not specific safety related however the impact of actions related to MC information exchange has safety related effects and therefore MCM data exchange must be seen as safety related. As these are intended to be used for Automation related actions by the vehicle itself ASIL levels may apply. Something which can result in more stringent communication requirements. We can expect that higher guaranties of data delivery and mre robust operation of the communications will be required. Projects have not yet resulted in usable parameters, therefore only some assumptions can be made. Table 12 provides some expected requirements.

Table 12: MCM communication requirements [i.130], [i.131]

## Basic CACC and Platooning

### Introduction

There are 2 parallel developments ongoing.

- The extension of ACC towards Cooperative ACC (CACC) current specification covers automation levels 1 and 2 (SAE J3016 [i.108]) there where CCAD is expected to cover the levels 3 to 5. Current CACC communication requirements are captured in the ETSI TR 103 299 [i.37] report.

- Truck Platooning is of high interest to logistic companies and truck manufactures as it is expected lower employee costs, reduce gasoline consumption and decrease emission.  Therefore all European Truck manufacturers are cooperating in realizing platooning in the ENSEMBLE EU project [i.35] leading to an expected deployment by 2022-2025. Communication requirements are derived from the Ensemble deliverables and ETSI TR 103 298 [i.36] report and may be further developed in coming years.

Truck Platooning: Among the multiple platooning projects, an example is the EU project Ensemble [i.35], which is working on a first version of Platooning. In addition to several deliverables which are publicly available, at ETSI a study including the results of this and other projects is being included in ETSI TR 103 298 [i.36] and the related basic CACC specifications in ETSI TR 103 299 [i.37]. The Truck manufactures are expecting to use multiple channels.

### CACC information dissemination

Definition ETSI TR 103 299 [i.37]: “CACC is an in-vehicle driving assistance system that adjusts automatically the vehicle speed to keep a target time gap △ttarget with a target vehicle (TV) while keeping a minimum safety distance with it” (see Figure 7).

Figure 7: CACC is an in-vehicle driving assistance

As Identified by the use case descriptions in the ETSI TR 103 299 [i.37] report, only one-use case (UC007) requires some additional short-range data sharing besides SPAT/MAP messages received from the infrastructure. This information exchange requires only very limited bandwidth in a relatively small relevance area in a message broadcasting setup. The information may be included as part of the CAM (in a backward compatible way) or be an additional message type with small message size. It is assumed not using significant more bandwidth.

In principle current CACC functional requirements as depicted in ETSI TR 103 299 [i.37], do not require more accurate requirements compared to what is covered by the C2C-CC BSP [i.51] except for an higher CAM rate similar to the extended CAM rate presented as PAM’s in the platooning application realizing a maximum CAM rate of 30Hz in total, leading to a drastically increases MCO requirement on the CAM transmission as noted by the ETSI TR 103 299 [i.37] in 9.1.2.2. It will double the number of messages.

Increased lane level position accuracy will be required for advanced CACC and therefore may require more stringent system positioning accuracy leading to integrate additional position augmentation methods with the support of additional C-ITS information exchange, as identified in  clause 5.5.3.

### Platooning information dissemination

No clear definition is available but in the ENSEMBLE project [i.35] three platooning levels A, B and C are defined.

- Level A attributes

  - Longitudinal coordinated automated control for the whole velocity range from 0 to maximum cruise velocity (depending on country regulations). The longitudinal control remains the driver’s responsibility.

  - Maximum number of trucks of 7 is considered for platoon level A in ENSEMBLE.

  - A minimum time gap of 0.8 seconds @ maximum cruise velocity (depending on country regulations)

  - New members of a running platoon can only join from the rear.

  - Under adverse conditions like bad weather, slopes, etc.… the drivers have the responsibility to increase the time gap or disengage the platoon completely.

  - The driver is responsible for the dynamic drive task in case of system failures. The system needs to be fail-safe.

  - Interaction with platooning services and infrastructure is technically available.

- Level B, t.b.d.

- Level C, t.b.d.

The ENSEMBLE project recognized a number of communication layers including a Service, Strategic, Tactical, Operational and System Elements layer as defined in “V1 Functional specification for intelligent infrastructure” paper [i.109].

It clearly identifies different communication layers with different functional as well as none functional (Functional Safety (ISO TS 26262 [i.100]), Safety of the Intended Functionality (SOTIF, ISO PAS 21448 [i.103]), privacy and security related) requirements. Communication with infrastructure both on strategic as service level is recognized. For MCO these are not considered. Only the tactical and operational layer communication requirements are considered.

For platooning the CAM is extended with a Platooning Container to show the platooning capabilities and platooning status (this is also good for other vehicles so they can identify platoons when they meet any).

Platooning requires increased awareness from members of the platoon. Information which may also help other road users. This could be just small CAMs send with different generation rules (based on early analyses but not specified in the current released deliverables from ENSEMBLE the rate may go up to twice the normal maximum CAM rate in addition to the already standard CAMs sent). The definition of these CAM’s could be realized by extending the current CAM specification or defining a separate Platooning Awareness Message (PAM). For readability of the document and keeping standard CAM information exchange separate from the extended Platooning awareness, in the following PAM is used. It is not advised to combine this awareness information as part of the Platooning Control Message (PCM) because of the data nature and way of transmission. PAM’s are sent in multicast and PCM’s in unicast.

There are different ways the PAM’s can be transmitted:

- The PAM’s can be seen as an extension of the CAM and transmitted as such with a different generation rule and a different priority.

  - In case of no congestion on the channel all PAM’s are sent.

  - In case of congestion on the channel

    - The PAM’s can be negated and not transmitted and CACC deactivated. This possibly as it is so crowded that it has no sense to have the CACC activated.

    - The PAM’s can be automatically forwarded to a specific agreed channel or via the SA assigned one.

- All PAM’s can be transmitted statically on a specific channel and when congestion in this channel occurs:

  - All others are not transmitted and CACC is degraded.

  - All others are forwarded statically to a different channel or via a SA assigned one. This forwarding could be depending on congestion levels in the appropriate channel(s).

- ALL PAM’s are sent in channel selected by service announcement message (SAM). PAM’s may also be sent in another specific channel assigned fixed or by mains of Service Announcement (SAM, ETSI EN 302 890-1 [i.67]) assignments even for each platoon specified by the platoon leader. The assignment could for instance depend on the congestion ratio’s in the channels.

PAM’s have similar dynamic and safety impact risks as standard CAM’s and therefore it is assumed that for the transmission and use of these messages no additional Functional Safety or SOTIF requirements need to be realized.

PAM’s may be sent in a smaller relevance area then standard CAM’s, maybe dynamically depending on the size of the Platoon.

Based on the expected message exchange a 100% single channel (of the 60% DCC level as defined for Release-1 systems) occupation may be expected when there are only a few platoons are in the same areas. This channel occupation is only in the area where active platoons are present. Initially this will be only on the highway’s but indications showing that a certain level of platooning may also be expected in urban areas. As highways also go through urban areas, it is not clear but currently not assumed that other information exchange could exists in the same channel as PAM’s are transmitted. Although level A platooning on highways only is considered today, for an MCO concept also level B and C expectations need to be considered.

Platoons are controlled by means of a platoon control state machines and required Platooning Control Messages (PCM) exchange. A typical exchange flow is presented in Figure 8. PCM’s may be exchanged in a specific channel but also use a channel selected by the platoon leader announced via an Service Announcement (SAM, ETSI EN 302 890-1 [i.67]), similar or the same as for the PAM’s. PCM’s are sent in a unicast or broadcast transmission mode (in ENSEMBLE [i.35] unicast is specified). For the operation of the platoon the operation of the platoon state machine is essential and not properly working has safety relevant consequences. It is essential for this information exchange that the Safety of the Intended Functionality (SOTIF) and Functional Safety requirements identified. Which may lead to additional communication requirements. This is addressed in clause 5.5.3.

Figure 8: PCM information exchange (ENSEMBLE [i.35])

As Level B and C are not yet defined it is assumed that future developments require additional information exchange and therefore flexible extendibility of the channel usage must be encouraged. Mechanisms such as SAM assignment where the service can be found could be considered.

### CACC and Platooning MCO communication requirements

Although the content and negotiation methods used by CACC and Platooning may differ, the MCO requirements do overlap and therefore are here combined. As still many CACC and Platooning developments are expected in future current MCO communication requirements are only initial requirements.

More accuracy related to the derived requirements are not required for the definition of an MCO concept except for Functional Safety and SOTIF as these are system configuration dependent. It can be recognized that although the PAM’s provide a more refined dynamic status knowledge of the other vehicles in the platoon or CACC string, they can be seen as additional CAMs with the same ASIL QM level. Already by having more data a higher ASIL level can be supported. PAM’s could be just addition but smaller (less information) CAMs. They can also be used by normal traffic (not participating in the platoon) to improve the quality of the information. PCMs are Platooning specific and will be exchanged with an additional trust level (additional security). Depending on the Platooning level the ASIL level will increase. For higher ASIL levels additional measures will have to be realized such as sending the m PCMs on 2 channels. Table 13 provides the initial requirements not considering possible ASIL level requirements

Table 13: CAM, PAM, SAM and PCM related communication requirements [i.130], [i.131]

## Intersection Related Safety

### Introduction

Intersection safety covers all those applications increasing safety in especially urban environment and managed by reginal or local authorities. Coordination of what application are of importance are brought to European level by participation of main European cities C-ROADS [i.53] to specify the most relevant urban applications. As traffic controllers manage the traffic via the intersection traffic lights this is called Intersection Safety (IS). Agriculture (AEF, [i.45]) is interested in Agriculture Safety (AS) and Urban Rail is interested in Urban Rail Safety (URS). Only Train crossings as application is part of Intersection Safety the others are included here as they have some requirements of interest but have only a limited impact on MCO.

### IS information dissemination

Day-1 applications include the support for basic traffic light use cases making use of Signal Phase and Timing (SPAT) and related Map (MAP) messages services as specified in ISO TS 19091 [i.10] and related protocols in ETSI TS 103 301 [i.46]. Extension of these applications for Day-2 show the support for more complex road crossing use cases and support for emergency and public transport prioritisation via pre-emption message exchange. Also unequal crossings such as rail-road crossings use case descriptions and weather forecast (short term, slippery road) and road conditions (sensors in/at the road and sensors in vehicles) application requirements are currently being drafted by authorities themselves or in cooperation with C-ROADs [i.53].

All together these applications cannot be all operating in the CCH/SCH0. It is needed that most Day-2 application related message services operate in a different channel. As the MCO requirements are application and service depending these needs separate assessment.

- Extended SPAT and MAP services will lead to more complex messages and a higher rate of updates. As we can expect an increasing population of C-ITS-S in an urban area, a conservative handling of the congestion levels will lead to not sending or off-loading of lowest priority messages such as SPATEM and MAPEM. It is therefore advised to agree assigning a channel to which the off-loading needs to be done to accommodate the continuous robustness and interoperability of the application. This off-loading could be facilitated by SA services.

- Unequal crossings such as a rail-road crossing, are from a signalling point of view not much different from a standard SPAT/MAP message exchange realized for Day-1 SPAT/MAP standard road crossing use cases and has no additional needs other than what is addressed in previous bullet. It can send warnings via DENMs but this will not have additional requirements as well.

- Traffic prioritisation applications are typical Day-2 applications. We need to distinguish automated and not automated when equipped.

  - When not automated, depending on their role and state the C-ITS-S sends CAMs identifying the dynamic state and can send DENMs to warn traffic when it is in the emergency state additionally to the CAMs sent (the CAM should be sufficient as all the data is there. More advanced it could indicate its intent and send MCMs. In principle the transmission of these messages should be sufficient to make the road traffic system (via its roadside C-ITS-Ss) aware of existence of the prioritised vehicle. To ensure that priority is only given to the allowed authority vehicles a separate trusted information exchange is required as specified as pre-emption in the ISO TS 19091 [i.10] supported by communication protocols specified in the ETSI TS 103 301 [i.46]. This information exchange allows the traffic control system to make the required traffic light scheduling changes and inform other C-ITS-Ss about the presence of the priority vehicle not only in the C-ITS current priority relevance area but also ahead in time into next priority relevance areas. The prioritised information exchange may be realized on a separate channel. These applications lead to additional SPAT, MAP and Pre-emption message exchange only. Channel selection needs to be identified and ensure interoperable use.

In addition to non-automated vehicles, automated vehicles could make use of the Maneuver Coordination Services (MCS) in more detailed manor. In case there is a high penetration of automated vehicles the priority could be part of the negotiation and by that the MCM could also be used in a mandatory way (see clause 5.7). The question is how detailed the communication with the road infrastructure needs to be in that case.

Of cause as full penetration of equipped automated vehicles is far aware a mixture of C-ITS-S equipped and C-ITS-S non-equipped vehicle circumstances needs to be considered. Currently to no research on this topic can be referenced. With respect to MCO we therefore only can consider the information exchange as covered by SPAT, MAP, Pre-emption and MCM and can only indicate that this development will lead to additional SPAT, MAP, Pre-emption and MCM message exchange.

### AS information dissemination

For Agriculture Safety we need to distinguish the situation that Agriculture equipment is on the normal road and when it is on private territory. C-ITS development are realized by organisations such as the Agriculture organisations such as the AEF [i.45] in Europe.

- In case the Agriculture equipment is actively participating in normal road environment and when equipped with a C-ITS-S it should be interoperable with other vehicles and send CAMs based on its active state as other vehicles. Additional it could send warnings, for instance about its size or special behaviour. Something normally supported by the DENM but possibly some extension of DENM may be required. As no special additional information exchange is required it can be seen as a special vehicle and does not have additional MCO requirements as Day-1 services.

- In case the Agriculture equipment is on private territory no C-ITS requirements are applicable except when agriculture related information is exchanged in the safety relevant 5.9 GHz band. In that case those transmissions are not allowed to interfere the normal operation of C-ITS-Ss or have impact on the performance of C-ITS applications operating in this band. As agriculture applications are non-safety oriented, related information exchange should take place in the non-safety related bands or in other available spectra. Still C-ITS protocols can be used.
One of the applications to use C-ITS protocols is the possible use of C-ITS standardised PAI related services. It is currently expected that there are no significant or conflicting MCO requirements recognized.

### Urban Rail Safety (URS) information dissemination

Urban Rail ITS makes use of CBTC systems to exchange safety-related information among Urban Rail trains and their infrastructure. It does not have protocols as specified for C-ITS and coexistence needs to be established at the media (spectrum) level. This while most reliable coexistence could be reached when such systems would have decided to use the same protocols. All C-ITS protocols are available including the Protected Zone Beacon (PZB) services are defined in ETSI TS 103 724 [i.96] supporting the mechanism to ensure the protection of CBTC systems.

According EU spectrum regulation Urban Rail ITS has priority in channel SCH6 (184 IEEE counting) and road ITS in the other channels. According to this regulation only I2V (roadside) C-ITS-S equipment can transmit in this channel which limits the possible use of this channel for C-ITS until coexistence is defined. Roadside C-ITS-Ss have mostly a fixed location and therefore the possible interference can be easily managed. These criteria can also be achieved for C-ITS application, C-ITS functional and C-ITS technical testing, as testing needs to be done in a controlled environment in which interference is excluded.

Independent of the actual deployed technology the most efficient way of deploying Urban Rail ITS would be the smooth integration into the overall ITS concept including MCO. A sharing of the same resources would lead to a flexible deployment scenario of the two ITS systems. In ETSI TR 103 580 [i.128] three different deployment scenarios have been identified and the resource usage has been analysed for these scenarios:

- Single Zone Controller (ZC)

- Three zone Controllers

- Three Zone controllers including a platform screen doors (PSD).

In most cases the single ZC scenario will be valid, whereas the other two scenarios are only valid in rail station environments and track crossing scenarios.

In TR 103 580 V1.1.1 [i.128] the channel loads for the three scenarios have been analysed for 5MHz channels. In Table 14, Table 15 and Table 16 these values have been scaled to 10MHz channels to be comparable with the other applications and services presented in this document.

| CBTC Application services | Period in ms | Average packet length in Bytes | Maximum packet length in Bytes | Packets/s | Throughput in Bits/s average | Throughput in Bits/s max |
| --- | --- | --- | --- | --- | --- | --- |
| Location Report to one ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| Periodic Train Functional Status message | 300 | 500 | 1 000 | 3,3333 | 13 333 | 26 667 |
| On demand specific status message | 300 | 500 | 1 000 | 3,3333 | 13 333 | 26 667 |
| Movement of authority from ZC | 600 | 200 | 1 000 | 1,6666 | 2 667 | 13 333 |
| Information about Line from ZC | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Burst Traffic for Track data base update (File transfer) | 100 | 50 | 150 | 10 | 4 000 | 12 000 |
| Request for Health train status | 500 | 50 | 100 | 2 | 800 | 1 600 |
| Total |  |  |  |  | 52133 | 140267 |
| Channel load 10MHz channel (estimated): |  |  |  |  | 2,0% | 3,0% |

Table 14: Communication resources for Urban Rail ITS, single ZC case[i.132]

| CBTC Application services | Period in ms | Average packet length in Bytes | Maximum packet length in Bytes | Packets/s | Throughput in Bits/s average | Throughput in Bits/s max |
| --- | --- | --- | --- | --- | --- | --- |
| Location Report to one ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| Location Report to a second ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| location Report to a third ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| Periodic Train Functional Status message | 300 | 500 | 1 000 | 3,3333 | 13 333 | 26 667 |
| On demand specific status message | 300 | 500 | 1 000 | 3,3333 | 13 333 | 26 667 |
| Movement of authority from ZC | 600 | 200 | 1 000 | 1,6666 | 2 667 | 13 333 |
| Information about Line from ZC 1 | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Information about Line from ZC 2 | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Information about Line from ZC 3 | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Burst Traffic for Track data base update (File transfer) | 100 | 50 | 150 | 10 | 4 000 | 12 000 |
| Request for Health train status | 500 | 50 | 100 | 2 | 800 | 1 600 |
| Total |  |  |  |  | 88133 | 260267 |
| Channel load 10MHz channel (estimated ): |  |  |  |  | 3,26% | 7,16% |

Table 15: Communication resources for Urban Rail ITS, three ZC case [i.132]

| CBTC Application services | Period in ms | Average packet length in Bytes | Maximum packet length in Bytes | Packets/s | Throughput in Bits/s average | Throughput in Bits/s max |
| --- | --- | --- | --- | --- | --- | --- |
| Location Report to one ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| Location Report to a second ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| location Report to a third ZC | 200 | 200 | 800 | 5 | 8 000 | 32 000 |
| Periodic Train Functional Status message | 300 | 500 | 1000 | 3,3333 | 13 333 | 26 667 |
| On demand specific status message | 300 | 500 | 1000 | 3,3333 | 13 333 | 26 667 |
| Platform Screen Door monitoring and control approaching, in station and leaving station | 100 | 50 | 150 | 10 | 4 000 | 12 000 |
| Movement of authority from ZC | 600 | 200 | 1 000 | 1,6666 | 2 667 | 13 333 |
| Information about Line from ZC 1 | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Information about Line from ZC 2 | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Information about Line from ZC 3 | 400 | 500 | 1 400 | 2,50 | 10 000 | 28 000 |
| Platform Screen Door | 100 | 50 | 200 | 10 | 4 000 | 16 000 |
| Burst Traffic for Track data base update (File transfer) | 100 | 50 | 150 | 10 | 4 000 | 12 000 |
| Request for Heath train status | 500 | 50 | 100 | 2 | 800 | 1 600 |
| Total |  |  |  |  | 96133 | 288267 |
| Channel load 10MHz channel (estimated): |  |  |  |  | 3,70% | 7,30% |

Table 16: Communication resources for Urban Rail ITS, three ZC case including PSD [i.132]

It has to be taken into account that a typical Urban Rail ITS will deploy two redundant channels in order to increase the reliability of the communication. Taking these evaluation results the worst-case resource usage would be in the range of 14,6%. An average value will be around 6% for a redundant dual channel implementation.

### Intersection Related Safety MCO communication requirements

Infrastructure Safety applications may require extended use of channel capacity compared to Day-1 usage, for extended use it must be considered to not do this in the Day-1 used channel but use another channel where especially I2V communication could effectively be realized.

Agriculture equipment equipped with a C-ITS-S may only operate in the safety related channels for general road safety applications. As agriculture specific applications are not road safety related these agricultures specific applications ned to make use of the non-safety related channels for their information exchange. Non-safety related applications can use road C-ITS services in safety related bands for the realisation of non-safety related services.

Urban Rail may require the transmission of the PZB Message (PZBM) in the SCH6 (184 IEEE counting) channel by means of a C-ITS-S by Urban Rail stakeholders. Only C-ITS-S RSU’s and C-ITS-Ss in conditioned environments can transmit in SCH6 (184 IEEE counting) channel according to the European spectrum regulation identified in 4.2.3.

Table 17 provides the expected extended data exchange require for Day-2 and beyond IS applications.

Table 17: Intersection related communication requirements [i.130], [i.131]

## High definition sensor sharing

### Introduction

High-Definition Sensor Sharing, See Through of Passing, applications involved in raw data exchange such as video-frame exchange requiring high bandwidth information exchange and are expected to be required for advanced automation but are creating challenging requirements for current C-ITS systems.

### High definition Sensor Sharing information dissemination

High-Definition Sensor Sharing, See Through of Passing, applications and other type of date require bulk (large to very large size packages) or streaming (continues bandwidth access) type of data transfers leading to high demanding spectrum constrains for each C-ITS-S supporting such applications., according to the 5GAA spectrum requirements study [i.50] 500 MHz band would be needed.  Something which confirms earlier assessments as recognized in the Hybrid Communication CODECS workshop [i.117].

### High definition Sensor Sharing MCO communication requirements

When considering the ITS European Spectrum regulation in the 5.9 GHz band (ECC decision (08)01 [i.84]), and the related harmonized specification requirements as identified in the  ETSI EN 302 571 [i.21], it can be recognized that within this 5.9 GHz band the required bandwidth is not available. Additionally, such transfers violate the duty cycle requirement. There is however a 2.16 GHz band in the 64 GHz spectrum allocated for ITS safety related services according to ECC Decision (09)01 [i.106]. For the use of this spectrum no specifications have been defined which could be used to identify any MCO requirements it is therefore premature to take any high definition sensor sharing requirements in consideration.

## Non-Safety ITS applications and services

### Introduction

Applications and services such as software updates, Vehicle Health Monitoring, Security Credentials and System validation (new applications and new systems needs to be tested in life situations while not interfering operating C-ITS-Ss), are considered as non-safety related. All such applications or services need to make use of the non-safety related channels as defined in the ECC recommendation (08)01 [i.85].

### Non-Safety ITS applications and services information dissemination

When it concerns any private application or service it is up to the specific private entity to manage how and by which manor the information is exchanged. The information dissemination is however only permitted to spectrum and technologies outside the 5.9 GHz safety related channels. To inform possible users, whether those are human or automated applications and services could be announced. For this an agreed interoperable way needs to be agreed.

### Non-Safety ITS applications and services MCO communication requirements

Only in case desired by private or common entities a common non-safety related SAM exchange can be agreed, for which an interoperable specification will be needed. Operation of such SAS, as identified in clause 5.5.2, should be realized in a non-safety channel.

## Testing and Validation services

### Introduction

At the moment more than one channel is in use and with the expectation that all will be used it needs to  be considered how to realized validation and be able to do compliance and  system testing of C-ITS applications, services, C-ITS-Ss and technologies. Such activities need to be facilitated.

### Testing and Validation services information dissemination

Activities as described in clause 5.11.1 are realized in controlled environments such that the activities can’t be disturbed. In principle such activities are focussed on C-ITS aspects and therefore will comply to the related requirements, but more research related activities could go into more extreme use which should up to a certain level also be supported by any testing and validation supporting MCO mechanism.

### Testing and Validation services MCO communication requirements

Testing and Validation activities should be able to operate in one channel in a controlled environment. Such controlled environments are in most cased also restricted areas on for instance a test site. These activities could be realized in channel(s) where in a restricted area predictable almost no active C-ITS information exchange takes place.

## Other MCO Functional considerations

### MCO Functional Scope

A C-ITS application realizes one or more C-ITS safety or non-safety related use cases. C-ITS applications consuming part(s) realize their use cases based on the consumption of received information from at least one but most often from multiple data sourcing parts (see Figure 9). Any consuming part should be able to receive and interpret any available safety related data from any sourcing part in the area for which the received information is relevant. Any interested C-ITS user (C-ITS-U) implementing a specific standardised C-ITS application (also C-ITS services which have no active application at the sourcing side e.g. CAM See “Awareness based in Figure 9) must be realize C-ITS interoperable (see clause 4.2.2.4).

Figure 9: C-ITS Application partitioning into Sourcing and Consuming parts in the various C-ITS-Ss

Internet protocol point to point communication is based on knowing the IP addresses of both sourcing and consuming parts of the application by which the security method can be based on the same IP addresses, which enables trust between  the different acting parties and allows internet service providers to have full control over both the data sourcing and data consuming parts of the application and have business agreements with their customers). They are in control of the data exchanged and provisioning. These business models are technology neutrally realized as identified in clause 4.2.2.3 by means of offering the services via marketplaces and browsers.

C-ITS consuming parts of an application in one C-ITS-S are interested in safety relevant information from sourcing parts of the same application in other C-ITS-Ss (see Figure 9) in a dissemination area relevant for the application but not larger then 500 meters, an areas which  we visually can oversee. C-ITS makes use of direct communication Ad-Hoc, point to multi point communication to realise the data exchange, which does not make us Internet protocol addresses and IP-related security mechanisms. To realise a trust domain for C-ITS a different security system is realised based on security policies [i.65] and ETSI ITS security standards. This results in a different business model where information is shared directly between the users.

According to the nature of Cooperative-ITS, a single provider in general does not have control over the operation of both the coursing and consuming part of the safety related application. For this reason, the specifications of the interface between the sourcing and consuming part needs to be specified at all functional and technical layers into the details as far as required for the compliance to interoperability requirements. An MCO concept is an internal station entity from which its functions should support these same requirements.

### Generic MCO communication considerations

The amount of C-ITS Release-1 services is limited. First only CA and DEN later extended with SPAT and MAP are all supported in one channel. Release-2 developments show the need for a large number of different behaving awareness services or triggered by applications services. Service behaviour is not only caused by the differences of the applications and services themselves but also depends on the dynamics of the environment e.g. highway, sub-urban and urban and number of participants participating. Data exchange requirements may differ extremely from one moment to one other. To allow applications to operate robustness and MCO concept also the following aspects need to be considered (including generalized findings from previous sub-clauses):

- As some services are specifically sent by specific station types and the spectrum regulation recognized differences in channel usage, this aspect needs to be considered and could require to realise that I2V sourcing applications make use of specific channels while V2V or V2I sourcing of the same applications could happen on a different channel.

- It is important that applications have a good knowledge whether they are able to send messages and have knowledge of the communication possibilities statically extendable to dynamically.

- As applications have no knowledge of the existence of other applications, they do not know whether a channel in which it expects to transmit its messages is available or whether it is used by any other. A mechanism is needed which could have an overview and inform applications of the possibilities to transmit.

- Continuity of Service: Only the applications or awareness services know the urgency of data to be transmitted. In case there is sufficient bandwidth available prioritisation of the message would not really be needed but there are a few cases that this is required.

  - The time urgency of the message to be transmitted could be improved when the ordering of the transmitting messages could be managed by knowledge about the time window in which messages should have been transmitted. Prioritisation of messages compared to others would also allow to improve the situation.

  - Bandwidth limitation due to channel congestion leads to messages not being transmitted. Mechanisms to ensure this doesn’t happen are to be considered, such as informing applications of the communication possibilities or bringing message transmission flexibility offering lower layers to use other radio resources when available such as off-loading based on message prioritisation.

In both cases one or other form of message or application prioritisation should be considered. Such prioritisation could be generic based on generic rules or be based on a fixed list, supported by the C-ITS stakeholders.

- From the spectrum regulation we see that in one ITS channel (CH6) only I2V transmissions are allowed. It is therefore clear that road infrastructure specific application sourcing transmissions should be realized in this channel. It therefore could be considered to realize all extended SPATEM, MAPEM, IVI and pre-emption I2V transmissions are realized in this channel. Messages of these types by other C-ITS-Ss should be sent on a different channel (for instance CH5).

- From the explanation of the operation of SAS, it is suggested to allow safety related SAMs to be sent on any safety channel, while for general use it is suggested to at least select one safety related channel for general intended SAS supported applications. Similar to this for non-safety related applications in the non-safety related channel.

- Based on the use cases in which specific applications transmit more messages in an urban area then on highways and vice versa the transmission of opposite behaving applications could possibly use the same channel.

- The current tendency is that Functional safety related requirements are mostly avoided by using received information not as the main sensor information on which an automated decision is made by which the communication sub-system only needs to support the ASIL QM level. This level does not require additional communications related solutions. Although it can be recognized that higher ASIL levels will be applicable, first aspects to resolve are on the sensor data and confidence level approvements before such requirements can be identified and therefore can’t be considered at this moment.

# Technical capabilities and limitations

## Introduction

The following clauses present the identified technical capabilities and limitations that can influence the performance and operation of an MCO concept, and therefore must be taken into account. They include the spectrum needs, the transceiver configuration, multi-channel interference effects and some particularities of the access layer technologies.

## Transceiver configurations

The capabilities of an MCO solution are limited by the transceiver configurations of the C-ITS-Ss. A C-ITS-S can be equipped with one or more transceivers operating in the 5.9 GHz C-ITS band. Each transceiver may operate in one channel, or more channels simultaneously. All transceivers of a C-ITS-S may implement the same wireless communication technology or different ones (e.g. ITS-G5, IEEE 802.11bd, LTE-V2X or 5G NR V2X). All C-ITS-Ss in the same network may not be equipped with the same number of transceivers. ETSI TR 103 576-2 [i.118] presents different implementation options for achieving interoperability and backward compatibility with multiple communication interfaces.

The current report considers MCO technology agnostic concepts for the realisation of single technology solutions. As coexistence methods supporting different technologies are not yet defined, mixed technology possibilities cannot be considered. In the following Table 18 different C-ITS-S configurations are listed.

| Configuration Index | Number of channels that an ITS-S is capable of accessing simultaneously | Technology Configuration |
| --- | --- | --- |
| 1 | One | A |
| 2 | Two | A for both channels |
| 3 | Two | A for a channel, and Ax for the other channel |
| 4 | Multiple | A for all channels |
| 5 | Multiple | Any mixed combination of A and Ax |

Table 18: Transceivers setup

In the Table 18, “A” represents a radio technology (e.g. either ITS-G5 or LTE-V2X), and “Ax” represents a new iteration of the same radio technology (e.g. either IEEE 802.11bd for ITS-G5 or 5G NR-V2X for LTE-V2X). In the present document, each C-ITS-S is assumed to support one of the configurations in Table 18.

## Multi-channel Interference effects

### Overview

In a multi-channel operation of a wireless system, different intra-system interference effects have to be considered. These effects might lead to a significant performance degradation of the system In order to understand the possible mitigation techniques these effects will be presented in this section. Both transmitter and receiver parameters have an impact on the performance in an MCO environment.

In Figure 10 the basics of the interfering effects in an MCO operation are depicted with the focus onto the direct adjacent channel. Similar effects can be observed for the second adjacent channel, spurious emissions and any kind of blocking signal further away from the wanted channel.

Figure 10: Impact of an interfering transmitter and a victim receiver on the reception of wanted signals, see ECC Report 310 [i.121]

Note: Here the out-of-band emissions in the wanted signal bandwidth is the main factor.

### Unwanted emissions

#### Overview

Unwanted emissions are all kinds of emission of an interfering system which are not in the wanted emission band of this system. The unwanted emissions can be split into two main parts:

- Adjacent and second adjacent channel leakage or emissions and

- Spurious emissions

Unwanted emissions lead to interfering energy in the wanted band of the victim system. Unwanted emissions cannot be filtered by the victim system since the interfering energy is in the wanted band of the victim system. Unwanted emissions can be reduced by e.g. signal design and transmitter filtering at the interfering transmitter.

#### Adjacent and second adjacent channel leakage

The effects of the adjacent and second adjacent channel leakage of a transmitter is described by the transmitter mask as defined in EN302 571[i.21]. From the victim receiver side this effect cannot be mitigated by any kind of filtering. The leaked energy of the interfering transmitter is part of the wanted signal to be received and leads to an increased noise level.

#### Spurious emissions

The spurious domain frequency band starts at 250% of the carrier bandwidth above and below the centre frequency of the emission of the interfering systems. For a 10 MHz system this is the 3rd adjacent channel starting at 20 MHz separation form the band edge of the reference channel. The levels of spurious emissions are regulated in ECC-Rec 74-01[i.122]. For the MCO considerations the levels of spurious emission (-30 dBm/MHz EIRP) and the probability of occurrence are very low as compared to the adjacent channel effects. The real value of the interfering power in the spurious domain is significantly lower. Thus, they will not be considered further in this report.

### Blocking and selectivity

Blocking refers to the reduction of the receiver sensitivity, thus the degradation of its performance, in the presence of an off-channel interfering signal; the frequency offset of the interfering signal should generally cover a relatively large range of frequencies around the wanted signal. The reduction of the sensitivity of the receiver is called “desensitisation” [i.121].

For the 1st adjacent channel and the 2nd adjacent channel the capability of a receiver withstanding these kinds of interference are specified as adjacent channel rejection ACR or adjacent channel selectivity ACS.

The robustness of the receiver against interfering signals further away than the 1st or 2nd adjacent channel are typically specified as blocking rejection.

### Combined unwanted emission and selectivity effects

For the evaluation and simulation of the interfering effects in an MCO operation a combination of the two main effects (ACLR and ACS) into a single parameter will simplify the investigations. By combining and adding up the two interfering effects an interfering transmitter in an adjacent channel can be modelled as a simple interfering source transmitting a specific interference power. All relevant filtering effects (TX and RX) can be included into a single figure PTX_int_eff, which is the transmitted effective interference power seen in a reference channel at the position of the interfering transmitter. PTX_int_eff is dependent on the specified values for the TX power mask, the ACR or ACS, the transmit power of the interfering transmitter and the chosen adjacent channel. For the co-channel case, PTX_int_eff would be the actual TX power of the interfering transmitter.

PTX_int_eff = Pi_ACL + Pi_ACS

More general PTX_int_eff and the single components of the interference power are functions of offset frequency ∆f from the band edge of the wanted signal.

PTX_int_eff∆f mW =Pi_ACL∆f mW+Pi_ACS∆f mW

where ∆f is the frequency offset from the band edge. In order to calculate the effective interference power in a received bandwidth the PTX_int_eff∆f mW has to integrated over this bandwidth.

Based on PTX_int_eff the interfering power at the antenna of the victim receiver PRX_int_eff can be calculated by taking into account the actual pathloss PL between the interfering transmitter and the victim receiver.

PRX_int_eff[mW] = PTX_int_eff[mW] / PL or

PRX_int_eff[dBm] = PTX_int_eff[dBm] – PL[dB]

As described above this can also be expressed more general taking into account the frequency dependency of the interference effects:

PRX_int_eff∆f mW=PTX_int_eff∆f mW / PL  or

PRX_int_eff∆f dBm=PTX_int_eff∆f dBm- PL [dB]

where ∆f is the frequency offset from the band edge.

### Other effect

#### Overloading

Figure 11: Receiver blocking and overloading measurement ranges[i.121]

Blocking and overloading of a receiver are two different phenomena and should not be confused. Figure 11 shows the C(I) curve of an ideal receiver with a protection ratio (PR) of -40 dB and an overloading threshold (Oth) of -10 dBm.

Overloading occurs when the receiver front-end is fully overloaded by a strong off channel interfering signal. This results in the degradation of the PR of the receiver due to the “gain compression” and “noise increase” caused respectively by the third order and second-order nonlinearity of the receiver LNA. The receiver selectivity also affects the overloading threshold level. In such case the interfering signal level expressed in dBm is called the «Overloading threshold» of the receiver.

When the receiver front-end is fully overloaded the receiver may become “blind” and thus unable to receive anything at all in contrast to the blocking and unwanted emission effects where only the communication range will be reduced. Additionally, beyond the overloading threshold the receiver is interfered by the interfering signal independent of the wanted signal level, as explained in Figure 11.

#### Intermodulation [i.121]

The Intermodulation phenomenon arises from non-linearity of the amplifier in the receiving chain. The theoretical output signal of the amplifier can be described by a polynomial formula in the form:

where ax is the wanted output and bx2, cx3, etc.  are unwanted intermodulation products due to the mixing of two or more interfering signals. Intermodulation is the only parameter requiring two or more interfering signals.

When considering only two signals of frequencies f1 and f2, the amplifier will generate:

This means that two signals of frequencies 2f1 – f2 and 2f2 – f1 appear in the receiver: these are the third order intermodulation products. 2nd order products (and higher order even number products) do not appear within the receiver’s bandwidth and can be ignored. Higher order odd number products can also have an impact, but not as significant as 3rd order products.

Figure 12: Generation of intermodulation products[i.121]

A receiver operating at frequency f0 is interfered by third order intermodulation products when the following conditions are met:

or  or  or  or  or  or  or  or

and the strength of the signals A1 and A2 is above a given threshold. Note that some standards/specifications define one of the interfering signals (typically the signal close to the receiver) with a system specific modulation whereas the second one remains unmodulated.

#### CSMA/CA energy detection threshold

In the case of CSMA/CA based systems, such as ITS-G5, if the potential interfering system in the adjacent channel operates with a very small distance from the victim receiver antenna, it can happen that the generated interference by the interfering system reaches the level of the energy detection threshold. From the victim point of view this would lead to a positive sensing result and the transmission operation will be delayed. Here the adjacent channel interference will have a direct influence onto the transmission operation of the victim system. Furthermore, it could lead to an increased estimated channel load and thus a triggering of the DCC mechanism.

As an example, based on ITS-G5 in LOS transmission conditions between the interfering TX with TX power of PTX_I = 23 dBm in the adjacent channel and the victim systems antenna with 8 dBi antenna gain, this effect can happen in a distance up to around 15 m having in mind the energy detection threshold of -65 dBm in a 10 MHz channel.

Under very dense traffic conditions this effect might have to be taken into account especially when operating in DCC conditions.

### Summary MCO interference effects

In the scope of the definition of MCO the main effects to be taken into account are the adjacent channel leakage of the transmitter and the adjacent channel selectivity of the receiver. In addition, in CSMA/CA based systems, the energy detection process in the receiver have to be considered.

Other effects (e.g. overloading and intermodulation) of adjacent channel interference from any kind of device operation in the adjacent channels can influence the performance of the system. These effects are mainly related to very close proximity operation of the devices and thus need to be taken into account in the development process of the integrated device or of the multi-channel chips. These effects are very much related to the detailed architecture of the devices.

For further investigations in the scope of this report the focus will be put onto the ACL, ACS and energy detection threshold effects.

These effects are further investigated in Annex A (ITS-G5) and Annex B (LTE-V2X) for the different access layer technologies available.

# Multi-Channel Concepts

## Introduction

For C-ITS to operate efficiently, it is important that the communication requirements of the supported applications are satisfied. The Release-1 applications requirements are specified in the Release-1 specifications and can be supported by a single channel implementation. The Release 2 specifications are currently being developed and the Release 2 applications need additional communication requirements and extended and backward compatible resource allocation over multiple channels. Release 2 communication requirements and technical capabilities are captured in clauses 4, 5, and 6. These requirements and capabilities are for one-part basis for the definition of the MCO concept. For the other part these requirements can be recognized for use when defining the specification of MCO related service specification, application requirements and technical specifications.

In clause 7.2, various existing MCO concepts are reviewed and classified. In clause 7.3, criteria are established based on literature review and the requirements developed in clauses 4 and 5, and the technical capabilities and limitations discussed in clause 6. Using these criteria, a first evaluation of the different MCO concepts based is performed also in clause 7.3. Based on this evaluation, clause 7.4 presents the considered MCO approaches that have been chosen for further evaluation and elaboration.

## Survey and taxonomy of MCO concepts

### Introduction

In the following clauses, different existing approaches for MCO are reviewed and classified.

### Channel usage concepts for MCO

Existing channel usage concepts for MCO can be classified depending on how the different channels are used and how the load is distributed. They are classified in the present document in 3 categories: sequential filling, load balancing and elastic. Figure 13 illustrates this classification with 3 examples. In this figure, each dashed rectangle represents one channel. The height of each dashed rectangle represents the channel capacity. The green rectangles represent the channel load on each channel.

The concept of sequential filling organizes the channels in a predefined order and a given channel is not used until the prior order channels are not loaded enough. This is illustrated in Figure 13a where 5 channels are fully used, 1 channel is partly used and 1 channel is not used, as an example. This MCO concept is partially considered in ETSI TS 102 724 [i.97], which is described in detail in ANNEX C.1.

The concept of load balancing distributes the load among the different channels. This is illustrated in Figure 13b where the 7 channels are uniformly loaded. With this MCO concept, the channel load in the different channels is similar. This MCO concept is considered in CARHet [i.31] as described in ANNEX C.2 and LTE-V2X RB allocation [i.127][i.112] as described in ANNEX C.3. It is also partly considered by SAMCO [i.30], described in C.4, since it selects the channel with the lowest load but with some restrictions.

Elastic MCO concepts are those that are not designed to fill the different channels in an organized way. This concept is illustrated in Figure 13c, where the load in the different channels is not uniform. A particular implementation of this MCO concept is known as SAMCO [i.30] and is described in ANNEX C.4.

(a) Sequential filling 	(b) Load balancing	(c) Elastic

Figure 13: Channel usage concepts for MCO

### Service and channel association policies for MCO

Existing service and channel association policies for MCO can be classified depending on how each service (or service type) is associated to each channel for the transmission of messages. Two categories are identified in the present document: predefined association and flexible association. They are illustrated with examples in Figure 14 and Figure 22, where each colour represents the load generated by a different service. Each type of service-channel association can be in principle combined with each channel usage concepts for MCO described in clause 7.2.2.

The policy of a predefined association has common rules that define the channel to be used for each service. Following these rules, a C-ITS-S is able to identify the channel(s) where the messages of a given service are transmitted. Figure 14 illustrates the concept of predefined association when combined with the sequential filling, load balancing and elastic concepts. A particular application of the predefined association with sequential filling is defined in ETSI TS 102 724 [i.97] (see ANNEX C.1 for details). With this solution, the services are served in order and when a service does not fit completely in one channel, it starts using the next one. The combination of predefined association with load balancing is also possible but might require the use of one (or more) channels to accommodate the channel load surplus on the other channels to achieve a homogenous channel load distribution. This is represented in the rightmost channel in Figure 14b. The combination of predefined association with elastic load distribution is achieved when each service has associated a predefined channel. Depending on the channel load consumed by the services on each channel, the channel load on each channel might be different, as illustrated in Figure 14c. Additional mechanisms would be needed to avoid exceeding the maximum capacity. This policy is also considered in SAE J2945/0 [i.129] and the channel usage recommendation is provided to guide how to use the channels in 5.850 to 5.925 GHz frequency band in US for specific applications or services. For example, the Basic Safety Message (BSM) is recommended to be used in the channel 172, and the Road Side Alert (RSA) is recommended to be used in the channels 178 and 184.

(a) Sequential filling 	(b) Load balancing	(c) Elastic

Figure 14: Service and channel association policies for MCO: Predefined association

The policy of a flexible association allows C-ITS-S to select any channel(s) to transmit the messages of each service. This selection can be restricted by certain common rules that all C-ITS-S should follow, and that could result from the adoption of one of the concepts based on channel usage (i.e. sequential filling, load balancing and elastic). When flexible association is combined with the sequential filling concept, the channel selection is limited to follow the strict channel organization. This is shown with an example in Figure 15a. In this example, a new C-ITS-S will select the fourth channel to transmit. If the load in one of the previous channels decreases, it could then transmit using such channel. The combination of the flexible association and the load balancing concepts results in a uniform channel load distribution, but each channel accommodates different services and each service consumes a different amount of resources in each channel. This is illustrated in Figure 15b and is aligned to the concept defined in CARHet [i.31] (see ANNEX C.2 for details). Finally, the combination of the flexible association and the elastic concepts results in a dynamic and adaptable solution. This solution is depicted in Figure 15c and is less focused on having a certain distribution of the channel load. Instead, it can provide (nearly) full flexibility to C-ITS-S to select the channel(s) to use. This solution is aligned with SAMCO [i.30] (see ANNEX C.4 for details).

(a) Sequential filling 	(b) Load balancing	(c) Elastic

Figure 15: MCO concepts based on flexible service-channel association.

### Supplementary functional support for MCO

The deployment of certain MCO solutions require additional mechanisms for their correct operation. Some of them were defined in IEEE 1609.4 Standard for WAVE about Multi-Channel Operation [i.29] (see ANNEX C.6 for details).

Service announcements: in some cases, the transmission of service announcements might be required to inform nearby C-ITS-S that a given service is being provided in a given channel. This is particularly needed in the most dynamic and flexible solutions, where there is no predefined assignment and/or channel usage. Service announcements can therefore increase the flexibility but introduce signalling overhead.

Prioritization: the MCO concept should take into account the priority of the different services and applications. This prioritization is embedded in some of the concepts previously described (e.g. sequential filling) but should be also considered in the other solutions. This is particularly the case to address the situations where not all services and applications can be served (e.g. channel overload) or when not all nearby C-ITS-S can be served (they are using different channels).

Channel load information sharing: the limitations of C-ITS-S to locally measure the status of all the channels could require to cooperatively measure the channel load. C-ITS-S could exchange their channel load measurements to have an updated view of the status of all the channels. This mechanism was proposed and evaluated in [i.30] and [i.31].

Redundant transmissions: to overcome some limitations related to the fact that C-ITS-S may not be able to listen all the channels, the use of redundant transmissions could be useful. With this mechanism, a C-ITS-S could transmit the same message(s) in more than one channel. This mechanism is also particularly useful for the most dynamic and flexible MCO concepts previously described.

Guard channel: some channels might be used as guard channels to reduce adjacent channel interference.

## Analyses of the MCO approaches

### Introduction

To evaluate the MCO approaches reviewed in clauses 7.2, requirements on MCO approaches are discussed in clause 7.4.2 and criteria for evaluation are established in clause 7.4.3. The requirements are based on the ones found in previous clauses of this document. Criteria are defined based on these requirements as well as on literature review (principally Multi-channel Usage in Day 2 and beyond EU V2X from 2015 [i.115] and Multichannel communications in vehicular Ad Hoc networks [i.116]). A subjective evaluation of the MCO concepts is performed in clause 7.4.4.

### Placeholder - Requirements on MCO approaches -

### Criteria

Backwards compatibility: this criterion is defined whether or not an MCO concept works with the existing C-ITS systems (see clause 4.2.2.4).

Multi-channel interference effects: this criterion evaluates the potential negative effects produced by adjacent channel interferences on each MCO concept.

Channel efficiency: this criterion evaluates the channel efficiency of each MCO concept. The channel efficiency is here evaluated taking into account the ratio between the amount of data successfully delivered and the data transmitted.

Service dispersion on channels: this criterion is defined in the present document as the number of channels where a given service is present.

Service evolution: this criterion is defined in the present document as whether an MCO concept is capable of accommodating future services not envisaged yet.

Channel extendibility: this criterion is defined whether or not an MCO concept allows to be extendable with other channels not in scope of the current spectrum regulation.

C-ITS interoperability: this criterion is measured by the level of compliance with the definition of C-ITS interoperability as defined in clause 4.2.2.3.

### Evaluation of MCO approaches

      - Introduction

In the following clauses, the evaluation methodology of the MCO concepts is reviewed in clause 7.3.4.2. The resulting analysis is presented in clause 7.3.4.3. Additionally, a summary of the results is proposed in Table 20.

      - Evaluation methodology

The first evaluation of the proposed MCO concepts is based on an “expert review” analyse. The evaluation process is the following:

- Each expert is asked to evaluate each MCO concept described in clause 7.2 in respect to the criteria defined in clause 7.3.3.

- The evaluation of a MCO concept in respect to a specific criteria is performed by assigning a score between 1 and 5. A value of 1 means that the expert expects that the MCO concept would perform really poorly regarding this criteria. In the opposite, a value of 5 means that the expert believes that the MCO concepts would perform really well regarding the criteria. Table 19 summarizes the different values. An expert should be able to provide explanations on the reasoning behind the values attributed.

- Optional Assign a weight to each criteria defined in clause 7.3.3. This would allow to give more importance to some criteria in the evaluation as described in the following point. As the weights may influence significantly the results obtained, any weights repartition may be subject to controversy. Therefore, results using different weight repartitions, with clear explanations on the reasoning behind, could be proposed and analysed.

- Optional By multiplying the scores attributed to each criterion to their respective weight (see the previous point), and by summing the obtained results together for each MCO concept, an overall score would be obtained for each MCO concept. The overall score of each MCO concept could allow a first quick understanding of what are the expectations of the expert regarding the different MCO concepts.

- Collect and combine evaluations from the experts and analyse the obtained results.

This evaluation methodology has several advantages: it supports the extension of the evaluation process to a larger group of experts, it can reveal misunderstanding, and it provides measurable feedback and expectations on the MCO concepts. However, it may not be simple to estimate for an expert the influence of certain MCO concepts regarding some criteria. Furthermore, the evaluation is subjective. Therefore, this evaluation is only the first step of the realisation of the final MCO concept, additional evaluations, e.g., using simulations, are needed.

| Scoring system |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The MCO concept is expected to perform … regarding this criteria | Really poorly | Poorly | Passably | Well | Really well |
| Value | 1 | 2 | 3 | 4 | 5 |

Table 19: Scoring system for "Expert review" MCO evaluation

      - Results and analysis

TBD

| Criteria | MCO concepts |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weights* | Sequential filling |  | Load balancing |  | Elastic |  |
|  |  | Predefined | Flexible | Predefined | Flexible | Predefined | Flexible |
| Backwards compatibility |  |  |  |  |  |  |  |
| Multi-channel interference effects |  |  |  |  |  |  |  |
| Channel efficiency |  |  |  |  |  |  |  |
| Service dispersion on channels |  |  |  |  |  |  |  |
| Service evolution |  |  |  |  |  |  |  |
| Channel extendibility |  |  |  |  |  |  |  |
| C-ITS interoperability |  |  |  |  |  |  |  |
| Overall score* |  |  |  |  |  |  |  |

Table 20: TBD Summary of the evaluation of the different MCO concepts

# Simulations and Verifications

## Introduction

This clause includes the evaluation the impact of MCO usage in respect to interference on adjacent channels effects on the MCO performance. System level simulation results are presented with respect to the KPIs presented in clause 8.2, based on the scenarios detailed in clause 8.3 and under the settings specified in clause 8.4.

In this clause 8, ITS-G5 is considered as one of the main C-ITS wireless technologies.

The following sub-clauses include the ITS-G5 specific simulation and verification results based on known ITS-G5 settings specified in the C2C-CC BSP profiles. LTE V2X simulation may be included at a later stage or in an updated version of this report.

Results are provided without any reference to a specific service and should not be seen as performance assessment of services that have been already well investigated in other documents. Nonetheless, some settings like message size and generation interval are freely inspired by CAMs, which are the most known messages at the time of writing.

## Key performance indicators

### Packet Reception Ratio (PRR)

The PRR is obtained considering the transmitter-receiver distance within intervals of dSTEP m. The k-th interval is within (k-1)*dSTEP and k*dSTEP.

For one transmission packet with index n, Yn,k is the number of receiving vehicles that are located within the k-th distance interval from the transmitter, and Xn,k is the number of vehicles with successful reception among those Yn,k vehicles.

The average PRR for the k-th distance interval is calculated as (X1,k+X2,k+X3,k+....+XN,k) / (Y1,k+Y2,k+Y3,k+....+YN,k) where N denotes the number of transmitted messages.

The distance interval is set to dSTEP=100m.

### Range (maximum distance with PRR=0.9)

The Range is here defined as the maximum distance to have PRR above 90%, with PRR as defined in clause 8.2.1.

It is to note that Range is strictly related to PRR as it provides a synthetic number from the curve relating PRR and distance.

### Inter-Packet Gap (IPG)

The IPG (sometimes referred as update delay, see Figure 16) is the average time difference between two consecutive successful receptions of packets sent from the same transmitter to the same receiver that is located within a given distance from the transmitter. The IPG implicitly allows to evaluate the correlation between consecutive errors.

We evaluate the IPG in the form of complementary cumulative distribution function (ccdf) for all transmitter-receiver pairs within a communication range threshold of 200m.

Figure 16: Example of IPG

### Channel busy Ratio (CBR)

This metric is used to estimate the load in a single radio channel, obtained as the average of CBR values as detailed hereafter.

In ITS-G5, each vehicle calculates the CBR as a time-dependent value between zero and one, representing the fraction of time that a single radio channel is busy with transmissions. The CBR assessment shall be according to clause 4.2.10 in ETSI EN 302 571 [i.21], that uses the following equation:

CBR=TbusyTCBR-ITSG5

where Tbusy is the period of time in milliseconds when the strength of received signals over a period of TCBR-ITSG5 exceeds -85dBm [i.21] and TCBR-ITSG5 is equal to 100ms [i.21].

## Simulation scenarios

A straight road with 6 lanes, corresponding to 3 lanes in each direction of 4m width, is considered (see Figure 17 : Simulation scenario). The length of the highway is 8km.

Figure 17 : Simulation scenario

The following five vehicle densities with different average vehicle speed are considered.

- Scenario 1: 12.5 vehicles/km at 130km/h;

- Scenario 2: 25 vehicles/km at 120km/h;

- Scenario 3: 50 vehicles/km at 110km/h;

- Scenario 4: 75 vehicles/km at 100km/h;

- Scenario 5: 100 vehicles/km at 80km/h.

The rational for the selected speed derives from the Van Aerde model [i.126] with parameters c0=-0.0112, c1=2.268, c3=0.000174545, and max speed 140 km/h. The output of the referred model and the selected speed for each of the given densities is shown in Figure 18.

Figure 18 : Derivation of speed as a function of density in the adopted scenarios

The initial position of the vehicles is evenly distributed over the lanes and uniformly distributed over the road length (approximating a Poisson distribution). Each vehicle moves at a constant speed, randomly selected using a Gaussian distribution with the average indicated value and a standard deviation equal to one tenth of the average speed. Vehicles exiting in one direction will enter again the scenario in the same lane, at the opposite side (wrap around approach).

Results in urban scenarios are also shown in ANNEX D.5.

## Simulation settings

### Introduction

In this clause, the main settings and most relevant models are introduced.

### Traffic generation

Each vehicle is assumed to generate new messages of a fixed size every a given interval. Specifically, the adopted message size is 350 bytes. The generation interval is set 100ms, meaning a packet generation at 10Hz, unless DCC (clause 8.4.3.2) triggers a larger generation interval.

Given the objective to evaluate the effect of MCO interference, the derived conclusions are expected to have broader validity. To confirm this, a different size for the packet is considered in ANNEX D.4.

NOTE: 	The scope is here not to evaluate a given service but to investigate the impact of multi-channel interference. For this reason, a given size and a given periodicity are used causing variable vehicular density to produce variable channel load.

### Access layers settings

#### Physical layer settings

ITS-G5 nodes are assumed to transmit at either 23dBm or 33dBm EIRP, with 3dBi antenna gain at the receiver, and 6dB receiver noise figure.

The access category Best Effort (BE) is adopted, corresponding to AIFS equal to 110us, CW equal to 15. The sensing threshold, when a preamble is detected, is set to -85dBm, whereas it is set to -65dBm otherwise. The CBR is updated based on those signals that exceed -85dBm.

The modulation and coding scheme corresponding to QPSK modulation and ½ coding rate is assumed.

#### Decentralized congestion control

### DCC is applied as in ETSI EN 302 663 [i.25]. Specifically, based on the CBR, the minimum generation interval is calculated as

Tgen-DCC=min⁡( 1, tpackms∙4∙CBR-0.62CBR) [seconds]

where CBR is the measured CBR and tpack is the duration of the message.

### Physical layer modelling

#### Propagation

The adopted channel model is a modified version of the one detailed in the ECC REPORT 68 [i.120] for the rural scenarios, hereafter simply denoted as ECC68 rural. Specifically, the pathloss is calculated as follows:

PLdB(d)=20log10λ4μdif d≤dT020log10λ4μdT0-10n0log10ddT0if dT0<d≤dT120log10λ4μdT0-10n0log10dT1dT0-10n1log10ddT1otherwise

where d is the transmitter-receiver distance, λ is the wavelength of the signal, dT0=128m is the first breakpoint distance, n0=2.8 is the path loss factor beyond the first breakpoint distance, dT1=512m is the second breakpoint distance, n1=3.3 is the path loss factor beyond the second breakpoint distance.

The modification, compared to the original ECC Report 68, is in a smaller value for the breakpoint distance. The rational is that the original models in ECC Report 68 have been developed for the link between a device, with height 1.5m, and an access point, with height 10m to 25m (the addressed technologies were BWA systems, such as WiMax).

NOTE:	The adopted model has been selected as it appears in good agreement with the general experiences in highway scenarios. In particular, it has been preferred to WINNER+, B1, which is often used for similar evaluations, as the latter has been designed for urban scenarios and appears to significantly overestimate the path-loss in highway conditions. Results adopting the WINNER+, B1 model are shown in ANNEX D.4.

The path loss as a function of the transmitter-receiver is shown in Figure 26 in comparison with the WINNER+, B1 model, which has been designed for urban dense scenarios and is often used for V2X, and the free-space path-loss model.

Figure 19: Path loss vs. distance with ECC Report 68 rural model and references

Correlated log-normally distributed large-scale fading (shadowing) is considered, with 25m decorrelation distance.

Small-scale fading is implicitly included in the performance curves detailed in clause 8.4.4.3.

Results adopting the WINNER+, scenario B1, model are provided in ANNEX D.4.

#### MCO interference modelling

The interference from one channel to an adjacent channel is modelled following the calculations of clause 6.4.

In particular, given a transmission from a generic node j in one channel, the interference perceived at a generic node i in a different channel, at a distance djiis calculated as PjiMCO=Pjir∙Ljiadj, where Pjir is the power that is received in the position of node i in the same channel as the one used by node j and Ljiadj is the portion of power perceived in the channel where node i is receiving.

As motivated by clause 6.4, when an EIRP of 33dBm is adopted, Ljiadj equal to -33, -47, -53dB is assumed in the first, second, and third adjacent channel, respectively. When an EIRP of 23dBm is adopted, Ljiadj equal to -25.2, -40.8, and -45.3dB is assumed in the first, second, and third adjacent channel, respectively.

#### Error evaluation

Interference is modelled as additive, Gaussian and white, proportional to its duration and occupied bandwidth.

Thus, given one transmission, the average interference is calculated over the duration of the signal and the obtained value is added to the noise power. The average signal to noise and interference ratio (SINR) is then derived.

Given the average SINR, the correctness of a transmission is statistically derived from packet error rate (PER) vs. SINR curves obtained adopting link level simulations.

In particular, the curve reported in Figure 20 is used, where PER is 0.1 at SINR=3.1dB. This curve was obtained through link-level simulations, with the following assumptions: 1 transmitting antenna and 2 receiving antennas with 1x2 IRC (Hermitian noise covariance Matrix) equalizer, highway LOS fading model as per TS 103 257-1 [i.27] channel estimate based on preamble and feedback loop, perfect control channel decoding, perfect synchronization.

Figure 20: Adopted curve of packet error rate vs. signal to noise ratio (MCS 2, 350 bytes)

In Figure 21, the received power varying the transmitter- receiver distance is shown considering only the path loss, given an EIRP of either 23 or 33dBm and the path loss model detailed in clause 8.4.4.1. The power level corresponding to the CBR threshold (i.e., -85dBm) and that corresponding to a SINR of 3.1dB are also shown as references.

Figure 21: Received power vs. the transmitted receiver distance assuming path loss only

## Simulation results

### Balanced data traffic in adjacent channels

#### Introduction

Given the scenarios and settings detailed in clauses 8.3 and 8.4, here simulation results are provided assuming that half of the vehicles act in one channel and the other half in one of the other channels (1st, 2nd, or 3rd adjacent channel).

Results assuming a single channel, with either half or all of the vehicles are also provided.

#### Scenarios and preliminary considerations

In Table 20, preliminary considerations on the simulated scenarios are provided. Per each vehicle density and both EIRP values (23 and 33dBm), the following metrics are shown.

- The “in CBR-range” counts, for a generic vehicle, the average number of neighbouring vehicles in the same channel which are sensed above the CBR threshold of -85dBm during transmissions;

- The “estimated CBR with no collision” is derived by multiplying the average number of vehicles in CBR-range by the average number of packets per second and the duration of a packet transmission; given that it does not consider collisions, it is an upper bound to the average CBR if no traffic is present in adjacent channels;

- The “median CBR (simulated)”, which provides the minimum and maximum values of the median CBR obtained in the various cases; specifically, the minimum corresponds to the case with no traffic in the adjacent channels and the maximum to the other half of vehicles in the first adjacent channel.

The numbers reported in Table 21 show that scenarios 1 and 2 with 23dBm and scenario 1 with 33dBm are lightly loaded, with a median CBR that never goes above 0.15, and the data traffic becomes high in scenario 5 with EIRP=23 dBm and scenarios 3-5 with EIRP=33dBm. DCC, which adopts a threshold limit of 0.62 for the CBR, is triggered only in scenario 5 with EIRP=33dBm.

Comparing the estimated CBR with the minimum value of the median CBR it is also possible to have an idea of the collisions occurring in each scenario. As an example, in scenario 5 with EIRP=23 dBm and no interference from adjacent channels, the CBR is 0.343, which is much lower than the estimated 0.57, meaning that there are a relevant number of concurrent transmissions.

Finally, comparing the minimum and maximum values of the median CBR it can be noted that the impact of transmissions in the first adjacent channels is not negligible in terms of CBR, with an increase that mostly ranges between 5 and 10%.

|  | EIRP 23 dBm In CBR-range (same channel) | Estim. CBR single channel, no collisions | Median CBR (sim) | EIRP 33 dBm In CBR-range (same channel) | Estim. CBR single channel, no collisions | Median CBR (sim) |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario 1 (half of 12.5 v/km) | 6.9 vehicles | 0.07 | 0.046-0.051 | 13.9 vehicles | 0.14 | 0.087-0.091 |
| Scenario 2 (half of 25 v/km) | 13.8 vehicles | 0.14 | 0.087-0.098 | 27.8 vehicles | 0.29 | 0.169-0.179 |
| Scenario 3 (half of 50 v/km) | 27.7 vehicles | 0.28 | 0.177-0.194 | 55.7 vehicles | 0.57 | 0.333-0.339 |
| Scenario 4 (half of 75 v/km) | 41.4 vehicles | 0.43 | 0.264-0.288 | 83.5 vehicles | 0.85 | 0.486-0.506 |
| Scenario 5 (half of 100 v/km) | 55.4 vehicles | 0.57 | 0.343-0.376 | 111.4 vehicles | >1 | 0.585-0.606* (*DCC active) |

Table 21: Preliminary considerations about the scenarios

#### PRR versus distance

From Figure 22 to Figure 26, the PRR versus distance is shown in the five scenarios. In each figure, two plots are shown, referring to EIRP = 23 and 33dBm, respectively. In each plot, five curves are compared:

- Only half of the vehicles in one channel (the others are not equipped or not transmitting);

- Half of the vehicles in one channel and the other half in the 1st adjacent channel;

- Half of the vehicles in one channel and the other half in the 2nd adjacent channel;

- Half of the vehicles in one channel and the other half in the 3rd adjacent channel;

- All of the vehicles in one channel.

In Figure 22, the PRR versus distance is shown with reference to scenario 1. In this case the channel is lightly loaded. As observable, with an EIRP of 23dBm the presence of adjacent channel interference is almost irrelevant. With an EIRP of 33dBm, all results are similar, although it can be noted a small degradation when all the vehicles are transmitting in one channel (dashed curve).

Figure 22: PRR vs. distance in scenario 1

In Figure 23, the PRR versus distance is shown with reference to scenario 2. Results are still very similar when the EIRP is 23dBm, which corresponds to a CBR always below 0.1 in the main channel when the traffic is split into two channels (solid curves) and 0.175 when it is all in one channel (dashed curve). A small degradation can be observed if the traffic is split over two adjacent channels and a further degradation if all nodes transmit in one channel. The impact starts to be relevant in the case of EIRP equal to 33dBm, which corresponds to a CBR between 0.15 and 0.2 in the main channel when the traffic is split into two channels (solid curves) and 0.333 when it is all in one channel (dashed curve). In such a case, some degradation is observed using the 1st adjacent channel and 40% loss of range is noted when all the traffic is in one channel.

Figure 23: PRR vs. distance in scenario 2

In  Figure 24, Figure 25, and Figure 26, the PRR versus distance is shown with reference to scenarios 3, 4, and 5, respectively. All curves coherently show that the impact of an interference from the 2nd or 3rd adjacent channel is marginal, whereas the interference from the 1st adjacent channel has some impact. An exception is scenario 5 with EIRP=33 dBm, due to the fact that DCC is triggered in that case and thus less messages are sent when the 1st adjacent channel is used. It is also confirmed that in all scenarios it is always better in terms of PRR to distribute the traffic over the channels, even if the 1st adjacent channel is used, instead of having all the nodes using the same channel. Specifically, if PRR=0.9 is targeted, 20-25% reduction is observed in the worst cases adopting the 1st adjacent channel, whereas the reduction is always larger than 50% if all nodes use the same channel.

Figure 24: PRR vs. distance in scenario 3

Figure 25: PRR vs. distance in scenario 4

Figure 26: PRR vs. distance in scenario 5

#### Range and IPG

Hereafter, results under the same conditions are provided in terms of range and IPG.

Specifically, Figure 27 shows the range in all scenarios and under the various conditions. The values presented in Figure 27 correspond to those derived at PRR=0.9 within the curves of PRR versus distance and are also reported in the legends from Figure 22 to Figure 26.

The results basically summarize what observed from Figure 22 to Figure 26. Please note that in some cases the range is zero as the PRR is always below 0.9 and that in scenario 4 with EIRP=33dBm the results shown in Figure 27 are not reliable, since in that case the curves corresponding to PRR versus distance have a maximum near to 0.9 ( minor statistical variations cause large differences in terms of range).

Again, it can be observed that: i) all cases perform similarly under light data traffic conditions; ii) the impact of interference from the 2nd and 3rd channels is very limited; iii) the impact of the interference from the 1st adjacent channel has a non-negligible impact; and iv) it is better to distribute the traffic over two adjacent channels instead of letting all nodes using the same channel.

Figure 27: Range (maximum distance with PRR=0.9)

In Figure 28, the value of the IPG corresponding to its ccdf equal to 0.01 is shown. In other words, the reported values correspond to the IPG that is exceeded in the 1% of cases.

In terms of IPG, some effect is visible only in the case of all nodes using the same channel, meaning that the interference from adjacent channels is not causing major effects on the correlation among errors.

The detailed curves, including the entire ccdf of the IPG for all simulations, are reported in Annex D.

Figure 28: IPG exceeded with probability 0.01

#### Deriving conclusions in the balanced traffic case

The main observations following the results obtained in the balanced traffic case are hereafter summarized.

OBSERVATION: 	Interference from 2nd and farther adjacent channels can be neglected.

OBSERVATION: 	Interference from the 1st adjacent channel has little impact on CCA assessment but affects DCC measurements and overall performance (negligible only if the channels are lightly loaded); the range loss is minor if the channel is lightly loaded and can reach 20-25% when the channel is highly loaded.

OBSERVATION: 	Distributing the traffic over two adjacent channels is always better than all the traffic in just one; the difference is remarkable, also compared to the use of two adjacent channels.

## Summary

In this clause, the impact of interference from adjacent channels have been investigated through simulations in realistic scenarios. The objective was to derive an indication about the preferable traffic distributions from an interference point of view.

The shown results remark that distributing the traffic over multiple channels allows to overall improve the performance in terms of packet reception ratio and range. Whereas the use of the 2nd or 3rd adjacent channel has negligible impact, the use of the 1st adjacent channel is shown to cause some interference, reducing the reliability of the communications and altering the measurements for the DCC calculation (through an increase of the measured CBR). Still, the impact from the 1st channel is limited, and it is anyway preferable to distribute the data traffic over two adjacent channels than maintaining it in a single channel.

Obviously, these observations need to be considered jointly with the other limitations, such as, for example, the impossibility of some nodes to receive messages at the same time from all channels.

# The C-ITS MCO Concept

## Introduction

Annex A: Multi-channel interference effects ITS-G5

## Overview

The main technical characteristics and operational principles of the ETSI ITS-G5 access layer are taken from the IEEE802.11p or better 802.11-2016 [i.123] specification. These characteristics are adapted to the European requirements (mainly spectrum requirements) in the ETSI EN 302 663 [i.25].

The MAC layer operation (mainly CSMA/CA access) and the related parameters are identical to the specification in IEEE802.11-2016 [i.123] and have not been changed in the ETSI EN302 663 [i.25] access layer profile standard.

The main parameters which have been adapted are:

- Sensitivity

- TX mask or adjacent channel leakage ratio and

- Selectivity including blocking rejection

## Sensitivity

In table Table 22 the static receiver sensitivity of a ETISS ITS-G5 receiver is depicted as specified in EN302 663 V1.3.1 [i.25] and IEEE802.11-2016. The dynamic sensitivity is 3dB below this static sensitivity. In EN 302 663 [i.25] only a sensitivity value for the QPSK rate R = ½ is defined for the dynamic case in order to reduce the overall test effort. Real implementation will reach better values. In the context of this document we will use the values given in table Table 21  as reference values. The values in the IEEE802.11-2016[i.123] are less stringent by 6dB and no dynamic channel performance criterions are included.

| Transfer rate (Mbit/s) | Modulation | Coding rate | Minimum sensitivity for 10 MHz channel spacing (dBm) [i.25] | Minimum sensitivity for 10 MHz channel spacing (dBm) [i.123] |
| --- | --- | --- | --- | --- |
| 3 | BPSK | 1/2 | -91 | -85 |
| 4,5 | BPSK | 3/4 | -90 | -84 |
| 6 | QPSK | 1/2 | -88 | -82 |
| 9 | QPSK | 3/4 | -86 | -80 |
| 12 | 16-QAM | 1/2 | -83 | -77 |
| 18 | 16-QAM | 3/4 | -79 | -73 |
| 24 | 64-QAM | 2/3 | -75 | -69 |
| 27 | 64-QAM | 3/4 | -74 | -68 |

Table 22: Static receiver sensitivity in EN 302 663 [i.25] and IEEE802.11-2016[i.123]

## Adjacent channel rejection

In Table 23 the values for the adjacent channel rejection are given as defined in EN302 663 V1.3.1[i.25]. These values correspond to the enhanced rejection values as define in IEEE802.11-2016[i.123]

| Transfer rate (Mbit/s) | Modulation | Coding rate | Adjacent channel rejection (dB) | Alternate adjacent channel rejection (dB) |
| --- | --- | --- | --- | --- |
| 3 | BPSK | 1/2 | 28 | 42 |
| 4,5 | BPSK | 3/4 | 27 | 41 |
| 6 | QPSK | 1/2 | 25 | 39 |
| 9 | QPSK | 3/4 | 23 | 37 |
| 12 | 16-QAM | 1/2 | 20 | 34 |
| 18 | 16-QAM | 3/4 | 16 | 30 |
| 24 | 64-QAM | 2/3 | 12 | 26 |
| 27 | 64-QAM | 3/4 | 11 | 25 |

Table 23: Limits for receiver adjacent channel rejection and
alternate adjacent channel rejection[i.25]

Figure 29: Adjacent channel rejection EN302 571 V2.1.1[i.21] and EN 302 663 V1.3.1[i.25], QPSK with R = ½

In Figure 29 it can be seen that the enhanced selectivity figure in EN302 663V1.3.1[i.25] are significantly more stringent than the one originally specified in the harmonized standard EN302 571 V2.1.1[i.21].

The ACR is a modulation and coding scheme dependent parameter. For further investigations and the direct comparison with the adjacent channel leakage effect the adjacent channel selectivity (ACS) is a better value to be taken into account. From the ACR value given in the EN 302 663[i.25] the modulation and coding scheme independent ACS can be calculated as follows:

ACS = ACR + 3dB +SNR

Where SNR represents the required SNR value for a given modulation (for QPSK with R = ½ we assume 6dB). The 3dB are the margin taken in EN 302 663V1.3.1[i.25] for the ACR measurement procedure.

Figure 30: Adjacent channel selectivity ACS based on EN302 571 V2.1.1[i.21] and EN 302 663 V1.3.1[i.25], QPSK with R = ½

In Figure 30 the resulting ACS values are depicted for EN 302 571 V2.1.1[i.21] and EN 302 663 V1.3.1[i.25]. In Figure 30 the ACS value are depicted as attenuation value and thus they are negative.

The minimum ACS value for the first adjacent channel is -34dB. That means, for a 33dBm interfering transmitter in the adjacent channel the victim receiver will experience a worst-case interference level of -1dBm in 10MHz (plus path loss and other effects) from the selectivity effects of the RX. For a 23dBm TX level this value is -11dBm.

It has to be noted that the ACR values are defined for the complete 10MHz band and have to be reached at the band edges. In real systems, the ACR value will increase with the distance to the band edge. In order to have a more realistic selectivity performance the ACR value has to be interpolated between the specified frequency points (e.g. 10MHz to 20MHz) and then a linear integrated value for the ACR and ACS can be calculated.  Taking this effect into account, the resulting values are better than the specified values. The resulting equivalent interference power levels are given in Figure 31. It can be seen that the values are almost 3dB better than the worst-case values (-3,8dB versus -1dB).

Figure 31: Adjacent channel selectivity ACS: Interference power levels using linear integration

## TX spectrum mask

In the draft ETSI EN 302 571 V2.1.9[i.21]  from January 2020 the TX spectrum mask as depicted in Table 23 and

Figure 32 is specified. This mask is the same mask as defined in EN 302 571V2.1.0[i.21]. It specifies a spectrum mask in absolute values and a measurement bandwidth of 100 kHz. All C-ITS devices operating in the band 5855MHz to 5925MHz have to fulfil this mask independent of the transmit power. In real operation it can be assumed that a device with less than the maximum allowed 33dBm e.i.r.p. TX power will have a reduced adjacent channel power level. Nevertheless, the reduction will not be fully proportional to the TX power reduction and it is not specified in the EN.

| Frequency offset to carrier frequency (MHz) | Emission limits (dBm e.i.r.p.) | Measurement bandwidth |
| --- | --- | --- |
| ±5.0 | -13 | 100 kHz |
| 5.5 | -19 | 100 kHz |
| 10 | -27 | 100 kHz |
| 15 | -37 | 100 kHz |
| 25 | -40 | 100 kHz |

Table 24: Out-of-band emission limits in draft EN 302 571[i.21]

Figure 32: Transmitter spectrum mask draft EN 302 571[i.21]

Based on the figures in Table 24 the interfering power in the adjacent channel resulting from the adjacent channel leakage (ACL)/ out of band emission for a 33dBm interferer TX level will be around -2,5dBm in the 10MHz first adjacent channel and -18,2dBm in the second adjacent channel. These values are the integration over 10MHz channel bandwidth using the values in Table 23. For lower TX power the values will decrease see Figure 33.

Figure 33: ACL Pi_ACL based on draft EN 302 571[i.21]

## Combined unwanted emission and selectivity effects

The effective transmitted interference power of an interfering transmitter is depicted in Figure 34 for a TX power of 23dBm as presented in clause 6.3.4.

Figure 34: Example for ITS-G5: Effective interference power at the interfering TX PTX_int_eff as combination of ACLR and ACS effect

## Conclusion ITS-G5

Two parameters in the set of specifications will mainly govern the physical layer MCO behaviour of an ITS system:

- Selectivity (Adjacent channel selectivity)

- TX spectrum mask (Adjacent channel leakage or out off band emission)

If we consider the raw figure given in the specification it can be concluded that the main limiting factor for the MCO specification for a system using 33dBm TX power will be the ACL value taken from the TX mask of the system. In this case the interference created by the transmitter in the adjacent channel is 1,3dB higher than the effect resulting from the selectivity.

For lower TX powers this difference will even increase since the limits for the TX mask are given in absolute values whereas the selectivity limits are given in relative values relative to the interference level in the adjacent channel. Thus, a decreased TX power of the interferer in the adjacent channel will almost linearly decrease the interference effect from the selectivity effect, see Figure 31. In the worst-case the unwanted emission level of the same interfering transmitter will not decrease.

Annex B: Multi-channel interference effects LTE-V2X

## Overview

## Sensitivity

## Adjacent channel rejection

## TX spectrum mask

## Combined unwanted emission and selectivity effects

## Conclusions LTE-V2X

Annex C: MCO solutions and algorithms

## ETSI TS 102 724

ETSI TS 102 724 [i.97] defines the channel usage in the ITS G5A and ITS G5B bands for multichannel operation. It includes both control and service channels, different application types (e.g. traffic safety and efficiency), defines transmit and receive policies, channel selection and configuration. It introduces per-traffic-stream and per-channel rate control. To do so, it defines a Channel-Configuration entity at the Access layer. The usage of the ITS-G5 channels is under control of the DCC and makes use of DCC profiles (DP1 to DP32). The Control Channel (CCH) is basically dedicated to cooperative road safety and is the default channel for the transmission of DP1 and DP2 messages. The transmissions of messages using higher DPs on the CCH are allowed in the DCC state "RELAXED". The Service Channel 1 (SCH1) is the default channel for announcing and offering ITS services for safety & road efficiency under the DCC state ACTIVE and RESTRIC TIVE of the CCH. The Service Channel 2 (SCH2) is the second service channel on ITS G5A and is used as an alternate channel for traffic safety-related services.

## CARHet

## Introduction

In [i.31], the authors propose CARHet, a multi-link and multi-RAT V2V communications solution that exploits multiple channels in a decentralized way. The proposed solution is technology and application agnostic and takes into account the application requirements and the communication context conditions. While CARHet was evaluated in [i.31] considering different radio access technologies, the same concept could be applied to scenarios where all vehicles implement the same radio access technology but have multiple radio interfaces. The following subclauses describe CARHet’s channel access and usage strategies using the generic term “radio interface” and considering that each radio interface could implement the same or different radio access technology.

## Channel access strategy

CARHet assumes that all vehicles are equipped with as many radio interfaces as radio channels available. They consider that vehicles are able to simultaneously use different radio interfaces for data transmission and/or reception. Moreover, each radio interface is tuned (fixed) to a specific channel in a specific frequency band. The selection of a radio interface is therefore equivalent to the selection of a channel. The authors also assume that a vehicle can transmit data using one radio interface and simultaneously receive information through all its radio interfaces.

CARHet was applied in [i.31] assuming that N radio interfaces are implemented, each of them tuned (fixed) to one of the M radio channels. However, CARHet could be evolved to adapt to scenarios where all vehicles do not implement as many radio interfaces as radio channels, i.e. N<M for some vehicles, following the strategy described below.

## Channel usage strategy

Each vehicle dynamically selects the radio interface that it will use to transmit all its messages. Since each radio interface is associated to a specific channel, the selection of the radio interface is equivalent to the selection of the radio channel to be used for data transmission. To select the radio interface (and channel), CARHet first filters out the radio interfaces whose capabilities do not support the application requirements (e.g. in terms of communications range). CARHet then selects the radio interface that minimizes the maximum channel load experienced by the neighbouring vehicles. To this aim, vehicles periodically measure and exchange the channel load they sense on all available channels. Vehicles are assumed to be capable to receive information on all channels and this channel selection strategy increases the amount of information that can be exchanged in the vehicular network due to a decrease in the interferences and packet loses due to collisions. CARHet also defines additional mechanisms to avoid unnecessary oscillations on the selected channel and radio access technology due to minor variations in the context conditions.

In scenarios where all vehicles do not implement as many radio interfaces as radio channels, vehicles could inform each other if they are not able to transmit/receive in a given channel, so that nearby vehicles stop using it. This could be done in the process used to exchange channel load information (e.g. indicating that a given channel is overloaded, so that other vehicles avoid using it).

CARHet has been shown to be capable to adequately distribute the load among different channels, and ensure high and homogenous QoS levels across the network with a low computational and communication cost [i.31].

## LTE-V2X RB allocation

## Introduction

The LTE-V2X uses Single-Carrier Frequency-Division Multiple Access (SC-FDMA) and supports both 10- and 20-MHz channels as defined in ETSI TS 136 201 [i.127].  Each channel is divided into Resource Blocks (RBs), subchannels, and subframes as defined in ETSI TS 136 213 [i.112]. Each subframe has a duration of 1 ms and each RB consists of 12 subcarriers of 15 kHz (180 kHz wide in frequency). Subcarriers have 14 symbols in each subframe where 4 of these symbols are dedicated to the transmission of Demodulation Reference Signals (DMRSs) to address the doppler effect at high speed.  In LTE-V2X, a subchannel is also a group of the RBs in the same subframe. Each subchannel consists of 10 RBs, and the number of subchannels in the corresponding resource pool can be flexibly determined to support the required bandwidth, e.g. a value of 5 for 10 MHz channels and a value of 10 for 20 MHz channels.

## Channel Access

There are two methods of channel access in LTE-V2X: Semi Persistent Scheduling (SPS), and One-Shot Transmission.

In LTE-V2X mode 4, a transmitter uses sensing-based Semi-Persistence Scheduling (SPS) algorithm to reserve the channel and transmit the data. In this scheme, the transmitter senses the channel for one second of Sensing Window and reserves the selected subchannels for a number of R consecutive transmissions, where the reselection counter R is set randomly between five and fifteen.

To select a sub-channel, the transmitter makes a list, L1, of Candidate Subframe Resources (CSRs) in the selection window. The list L1 includes all the CSRs in the selection window except the ones that will be utilized. A CSR is recognized as utilized via a received SCI or average measured Reference Signal Received Power (RSRP) > ThSPS over the RBs, where ThSPS depends on the priority of the packet. Moreover, due to the half-duplex transmission mode, the transmitter must exclude all the CSRs of subframe F in the selection window if itself was transmitting during any previous subframe F-100*j (j∈ N, 1≤j≤10). After excluding the utilized CSRs, the L1 must include at least 20% of CSRs in the selection window. Otherwise, the transmitter will do the procedure with a 3 dB increment in ThSPS until 20% target is met.

After forming the L1 list, the transmitter creates another list of CSRs, L2, which is a subset of L1 and includes exactly 20% of the selection window CSRs with the lowest average Received Signal Strength Indicator (RSSI) over last ten intervals.

Finally, the transmitter randomly reserves one of the CSRs in L2 for its next R transmissions. This behavior helps avoid collisions between devices at the resource selection stage and gives more protection to devices using an SPS flow for transmission. In case the data cannot be fitted in the reserved subchannel(s), the transmitter must compete over the medium again. After each transmission, the reselection counter R is decremented by one. When it is equal to zero, additional resources must be selected and reserved with probability (1–P). Each vehicle can set-up P between zero and 0.8.

One-shot transmission is mainly used for critical event-based packet transmissions. However, it can be used for periodic packets as well. The resource selection procedure is exactly the same as SPS, but the transmitter does not reserve the selected resources for future transmission. This means that the reselection counter packet is zero in one shot transmission.

## ProSe Per Packet Priority (PPPP) for V2X communication

In LTE-V2X, each packet generated by the application layer is associated with a certain priority value, called ProSe Per Packet Priority (PPPP). PPPP is an inter number and can range from 1 to 8 where packets with lower PPPP value have higher priority. PPPP can be used to determine Packet Delay Budget (PDB), channel occupation ratio (CR) limit, and RSRP threshold for accessing the channel.

## SAMCO (Service-Actuated Multi-Channel Operation)

## Introduction

In “Service-actuated multi-channel operation for vehicular communications” [i.30] by Mate Boban and Andreas Festag, the authors propose the MCO concept “SAMCO” that provides a logic to control the prioritization of services and the timing of channel switching. The algorithm designed in SAMCO takes into account user preferences to decide on which channel(s) the transceiver(s) need(s) to be tuned. It manages services depending on their level of priority, and it has the objective to minimize the channel switching frequency.

The concepts of a data provider (DP) and data user (DU) roles are adopted to describe SAMCO entities. In short, DP is an entity (either a vehicle or an RSU) in charge of generating service advertisements and providing the service, while DU is the user/consumer of the service. While typically the DP transmits data to the DU, some services require the opposite directions of data traffic (e.g., a probe vehicle data service, where an RSU advertises the service and vehicles generate the data). Also, a bi-directional exchange between service provider and users is possible. The latter is relevant when a service implies a reliable exchange of data between two vehicles, typically via unicast, e.g., for a make-space operation when a vehicle joins a platoon. In general, an entity can assume the DP and DU roles both concurrently and interchangeably (i.e., a vehicle providing a service would assume an DP role, and likewise it could assume an SU role once it wants to consume a service).

The DP/DU concept represents a general framework for service provisioning in a vehicular network with MCO support, which covers a broad range of services for road safety, traffic efficiency and infotainment applications. In particular, the concept also includes specific services for vehicle automation and platooning, such as cooperative sensing and the exchange of manoeuvring commands.

## Channel access strategy

In their paper, the authors assumed that each C-ITS-S has two transceivers. The first one is continuously listening to the CCH while the second one can dynamically switch between SCHs. Referring to the IEEE 1609.4 document [i.29], the first transceiver would employ the continuous option while the second one would use the immediate one for the channel access strategy.

## Channel usage possibilities

SAMCO assumes that SPs use the Service Announcement service on the CCH to inform other C-ITS-Ss on which channel which service is providing information. Furthermore, all DPs communicate their critical safety messages on the CCH while other services publish their information on one of the SCH.

To decide on which SCH the second transceiver needs to be tuned, the DUs rely on applications and user requirements.

For the DPs, the SCH selection depends on the CBR observed on the different channels. It is not feasible with one transceiver to collect the CBR on all SCHs while keep transmitting information on one SCH at the same time. To have an idea of the channel load on all SCHs, C-ITS-Ss exchange their perceived CBR with SAMs. Based on the SAMs received, DPs select the SCH with the lowest channel load to start transmitting their non-critical safety messages. After determining the SCH with the lowest CBR, an DP tunes its transceiver to the channel and checks if the CBR observed matches the one received by SAMs.

If it is the case, the DP starts the transmission of its services to the SCH. Certain services might not need to provide the service on SCH before there are DUs that want to consume the service (e.g., platoon leader might announce, but not transmit a service before there are other vehicles in vicinity willing to join the platoon). Therefore, such services might require an DU to acknowledge consumption of the service. After one DU has acknowledged the consumption through a unicast message to DP, subsequent DUs consume the service without the need for acknowledgement. DUs are responsible to inform DPs of their message interests.

When the channel starts to be saturated, DPs stop publishing information of the services with the lower priority of all services publishing on the current SCH. If a DP stopped to publish information about all its services, it tries to reschedule its highest priority service stopped to another SCH.

## IEEE 1609

## Introduction

The IEEE 1609.4 Standard for WAVE about Multi-Channel Operation [i.29] specifies MAC sublayer functions and services to support multi-channel wireless connectivity between IEEE 802.11 WAVE devices. This standard defines an extension to the MAC sublayer management entity (MLME) specified in IEEE Std 802.11. It includes time synchronization and channel access features in support of channel coordination, as well as channel-specific access to other IEEE 802.11 services, MIB maintenance, and readdressing.

Channel coordination is about supporting data exchanges involving one or more switching devices with concurrent alternating operation on multiple channels.

The time synchronization and channel access features specified in the IEEE 1609.4 [i.29] standard are studied in C.5.2. Inputs and requirements for channel usage algorithm of the IEEE 1609.3 [i.70] standard are discussed in C.5.3.

## Channel access strategy

Each transceiver of the WAVE devices has different possible channel access options: continuous, alternating, and immediate channel access, as exemplified in Figure 35.

The descriptions of the different channel access options are:

- Continuous option: the designated channel is available for an indefinite duration.

- Alternating option: the transceiver is repetitively tuned to a channel for a time slot 0, and then is switched to another channel for a time slot 1. The duration of the time slots 0 and 1 is set to 50ms in the standard (see Annex H in [i.29]).

- Immediate option: Immediate access allows immediate communications access on a designated channel for an extended period without consideration for time slot boundaries.

Figure 35: IEEE 1609.4 Channel access examples: (a) continuous, (b) alternating, and (c) immediate

To support the different channel access options, WAVE devices shall synchronize to a common time base, especially when WAVE devices are switching channels on time slot boundaries such as in the alternating channel access option. The timing information used for synchronization may be derived by a timing management function from information received over the air from the other WAVE devices or may be obtained from a local source.

Each time slot includes an initial guard interval, which is used to account for radio switching effects and timing inaccuracies among different devices. During the guard interval, a switching device might not be available for communication, as its switching PHY could be in a transition state between channels. For a device switching channels on time slot boundaries, at the beginning of a guard interval the MAC transmit/receive activities on the previous channel may be suspended; at the end of the guard interval the transmit/receive activities on the next channel should be started, or resumed if they were suspended. To prevent multiple switching devices from attempting to transmit simultaneously at the end of a guard interval, a device switching to a channel during the guard interval should declare medium busy (specified in IEEE Std 802.11) during that guard interval so that its transmission attempts are subject to a random back-off when the guard interval ends. A device using alternating channel access should declare medium busy during each guard interval in which it switches. The length of the guard interval is set to 4 ms in [i.29].

## Channel usage strategy

The IEEE 1609.3 Standard for WAVE about Networking Services [i.70] does not provide an algorithm to assign channel to services. However, the standard defines roles and criteria to realize such algorithm.

For convenience, two WAVE device roles are defined. The provider role is assumed by a device transmitting WAVE Service Advertisements (WSAs) indicating its availability for data exchange on one or more channels. The user role is assumed by a device monitoring for received WSAs, with the potential to participate in a data exchange on any channel. A WAVE device may assume one, both, or neither role. WSA may be transmitted on any channel and during any time slot but are nominally sent on the CCH during time slot 0.

To elaborate an algorithm to assign channel to services, the following criteria should be considered by algorithm developers:

- Number of channels that can be simultaneously accessed by the device’s radio equipment

- Service requests

- Available services matching user service requests

- The relative priorities of the requested services

- The Channel Access of the application-service, i.e., the time slots during which it is active

In general, the access assignment algorithm may give precedence to requests with higher priority. Channel service requests with a higher priority may take precedence over lower priority assignments that would interfere with ongoing channel operation. For example, SCH access in support of a user service request may be assigned but without the requested immediate access.

Other factors may be considered, such as the following:

- The geographic proximity to the transmitter of the WSA

- Link quality associated with the transmitter of the WSA

- Application-service duration

After channel access has been assigned, certain events such as the following should cause the assignment to be ended:

- Application-service has been deleted

- User service request no longer matching an available application-service opportunity, i.e., there is no WAVE device offering the service wanted by the user

- Need to satisfy a service request with higher priority

- Loss of lower layer synchronization

- Other factors, such as poor Link Quality

Annex D: Single connection interfered by traffic in the adjacent channel: additional results

## Additional results in the balanced traffic case

In this Annex, additional results are reported for the case detailed in clause 8.5.1, where balanced traffic is supposed over two channels.

Similarly to clause 8.5.1, given the scenarios and settings detailed in clauses 8.3 and 8.4, here simulation results are provided assuming that half of the vehicles act in one channel and the other half in one of the other channels (1st, 2nd, or 3rd adjacent channel). Results assuming a single channel, with either half or all of the vehicles are also provided.

## Inter-packet gap detailed statistics

Hereafter, the ccdf of IPG of the simulations detailed in clause 8.5.1 are provided. Summary results from these curves were shown in clause 8.5.1.4, focusing only on the values corresponding to a ccdf equal to 0.1.

Figure 38: ccdf of IPG in scenario 1

Figure 39: ccdf of IPG in scenario 2

Figure 40: ccdf of IPG in scenario 3

Figure 41: ccdf of IPG in scenario 4

Figure 42: ccdf of IPG in scenario 5

## Results assuming a different channel model

In this clause, a different channel model is assumed, i.e., WINNER+, scenario B1 LOS [i.23]. Despite the fact that it was derived for urban microcells, it is one of the mostly adopted models also for highway scenarios. In this annex, it is used to verify that the derived conclusions are valid besides the specific channel model.

The WINNER+, Scenario B1 LOS model (or simply WINNER+ in this document), is a two-slope channel model with exponent 4 above approximately 20m, which is compared to the free space and the ECC 68 rural models in Figure 41.

Figure 41: Path loss vs. distance with different models

Given that the expected range with the WINNER+ B1 model is approximately one fourth of the one obtained with the ECC68 rural model, referring to a path loss value of 120dB, hereafter we consider scenarios with a higher density of vehicles compared to clause 8.5.1.

Specifically, a straight road with 6 lanes, corresponding to 3 lanes in each direction of 4m width, is considered. The length of the highway is here 2km. The following five vehicle densities with different average vehicle speed are considered (the average inter-vehicle distance is approximately 2.5s multiplied by the average vehicle speed):

- Scenario A: 35 vehicles/km at 250km/h;

- Scenario B: 61.5 vehicles/km at 140km/h;

- Scenario C: 166.5 vehicles/km at 50km/h;

- Scenario D: 333 vehicles/km at 25km/h;

- Scenario E: 500 vehicles/km at 15km/h.

The initial position and movements of the vehicles are generated as described in clause 8.3.

Selected results, in terms of PRR, are shown in Figure 42, Figure 43, and Figure 44.

Figure 42: PRR vs. distance in equivalent scenario 3 using the ECC68 rural model

Figure 43: PRR vs. distance in equivalent scenario 4 using the ECC68 rural model

Figure 44: Range (maximum distance with PRR=0.9) using the ECC68 rural model

As it can be observed looking at Figure 29, Figure 29, and Figure 32, despite the different model and taking into account a scale factor in the density due to the different range, the conclusions are very similar to those provided in clause 8.5.1. This implies that the derived observations are of wide validity and are not limited to the specific channel model that was adopted.

OBSERVATION: 	The observations provided in Clause 8.5.1.5 are not limited to a specific channel model.

## Results assuming messages with a larger size

In Figure 45, Figure 46, and Figure 47, results adopting the WINNER+ model and packets of 550 bytes are shown.

Figure 45: PRR vs. distance in scenario 3 with WINNER+ and packets of 550 bytes

Figure 46: PRR vs. distance in scenario 4 with WINNER+ and packets of 550 bytes

Figure 47: Range (maximum distance with PRR=0.9) with WINNER+ and packets of 550 bytes

As expected given the longer length of the packets, the PRR observed in Figure 45, Figure 46, and Figure 47 (550 bytes) is lower than the one shown in Figure 42, Figure 43, and Figure 44, respectively (350 bytes). However, the observations provided in clause 8.5.1 remain valid. This implies that the derived observations are of wide validity and are not constrained to the specific packet size that was adopted.

OBSERVATION: 	The observations provided in clause 8.5.1.5 are not limited to a specific packet size.

## Results in urban scenarios

In this clause, results in urban scenarios are shown.

In the urban scenario vehicles are moving along perpendicular roads separated by blocks of buildings. This topology is known as a Manhattan grid. Non-Line of Sight (NLOS) links between vehicles around the corners are considered.

The scenario is composed by 2x2 blocks, creating a grid of roads with intersections. Each block is formed by a building, with two lanes 3.5m wide all around it; the direction of movement around the building is clockwise defined. The composition of more than one block creates roads with 4 lanes per road, 2 per direction, as exemplified in Figure 48. Each block is 250m large and 433m long.

Two scenarios are considered with different number of vehicles and average speed on the road (the average inter-vehicle distance is approximately 2.5s multiplied by the average vehicle speed):

- Scenario 6: Urban A, with 65 vehicles per block at average 60 km/h;

- Scenario 7: Urban B, with 130 vehicles per block at average 30 km/h.

The initial position of the vehicles is evenly distributed over the lanes and uniformly distributed over the road length (approximating a Poisson distribution). Vehicles reaching an intersection will move forward with probability 0.5, turn left with probability 0.25, and turn right with probability 0.25 (uniformly distributed)); they will maintain the corresponding lane in the new road. Vehicles exiting the scenario will enter again the scenario on the same road, same lane, at the opposite side (wrap around approach).

Figure 48: urban scenarios

As channel propagation model, the WINNER+, scenario B1, either LOS or Manhattan NLOS, is used. Correlated log-normally distributed large-scale fading (shadowing) is considered, with 10m decorrelation distance.

In the urban scenarios, link-level curves relating the SINR to the PER were obtained with the same assumptions detailed in clause 8.4.4.3 but adopting the urban LOS and urban NLOS fading models detailed in TR 103 257-1 [i.27].

Results in the urban scenario are shown in Figure 49, Figure 50 and Figure 51.

Figure 49: PRR vs. distance in scenario 6 (urban A)

Figure 50: PRR vs. distance in scenario 7 (urban B)

Figure 51: Range (maximum distance with PRR=0.9) in the urban scenarios

Observing the results shown in Figure 49, Figure 50 and Figure 51, it appears clear that the NLOS conditions have a radical impact on the PRR. Still, the conclusions are that the interference from the 2nd or 3rd adjacent channel is negligible, whereas the interference from the 1st adjacent channel has some impact, even if minor in these scenarios. In all cases, distributing the data traffic over two channels allows to obtain better performance than having all the data traffic in a single channel.

OBSERVATION: 	The observations provided in Clause 8.5.1.5 are not limited to the highway scenario.

History

| Document history |  |  |
| --- | --- | --- |