3GPP TSG-RAN4 Meeting #98-e	R4-2101179

# Online, , 25th Jan 2021 - 5th Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0459 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_1BLTE_1BNR_2DL2UL |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarify the single uplink allowance of DC_3A_n3A due to potential emission issues and self-interference from Rel.16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to Table 5.5B.3-1 to clarify the single uplink allowance of DC_3A_n3A due to potential emission issues and self-interference from Rel.16. Change the NOTE 2 “Only single switched UL is supported in Rel.15” to “void” in the Rel.16 spec, since it is applicable for rel.15 only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reason of single uplink allowance for Rel.16 DC_3A_n3A remains unclear in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3B.1.3, 5.5B.3, 6.2B.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, an EN-DC configuration is a single operating band supporting E-UTRA and NR carriers, where E-UTRA configuration is indicated by using E-UTRA CA bandwidth class as defined in TS 36.101 [4] and NR configuration is indicated by using NR CA bandwidth class as defined in TS 38.101-1 [2].

Requirements for intra-band non-contiguous EN-DC are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.3-1.

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_2A_n2A | DC_2A_n2A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_3A_n3A | DC_3A_n3A |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  |  |  |
| DC_5A_n5A | DC_5A_n5A2 | 5, 10 | 5, 10, 15 |  | 20 | 0 |
| DC_7A_n7A3 | DC_7A_n7A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_41C_n41A | DC_41A_n41A | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
| DC_41D_n41A | DC_41A_n41A | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
| DC_48A_n48A4 | DC_48A_n48A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_48A_(n)48AA4 | DC_(n)48AA2 DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48A-48A_n48A4 | DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48C_n48A4 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48D_n48A4 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_66A_n66A | DC_66A_n66A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| DC_71A_n71A | DC_71A_n71A2 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

<< Second of changes >>

5.5B.3	Intra-band non-contiguous EN-DC

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A | Yesx |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48A_(n)48AA3 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| DC_71A_n71A3 | DC_71A_n71A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE x: Single UL allowed due to potential emission issues and self-interference. |  |  |

<< Third of changes >>

6.2B.1.2	Intra-band non-contiguous EN-DC

Table 6.2B.1.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A4 |  |  |  |  | 23 | +2/-3 |
| DC_3A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n5A4 |  |  |  |  | 23 | +2/-3 |
| DC_7A_n7A4 |  |  |  |  | 23 | +2/-3 |
| DC_48A_n48A4 |  |  |  |  | 23 | +2/-3 |
| DC_41A_n41A | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |
| DC_66A_n66A4 |  |  |  |  | 23 | +2/-3 |
| DC_71A_n71A4 |  |  |  |  | 23 | +2/-3 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Void NOTE 3: Power Class 3 is the default power class unless otherwise stated. NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [8] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	if the UE does not support a power class with higher maximum output power than power class 2; or

-	if the E-UTRA UL/DL configuration is not 2 or 4 or 5; or

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle/2 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [9] is provided and set to the maximum output power of the power class 2 or lower;

- apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in clause 6.2B.4;

<< End of changes >>