3GPP TSG-RAN5 Meeting #90-e	R5-210599

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1711 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction on numbering of several Tables in 38.508-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TTA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The numbering of several Tables are overlapped in section 4.9.6.2 and section 5.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected numbering of Tables in section 4.9.6.2 and section 5.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table numbering would be remain confused |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.6.2, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

4.9.6.2	Switch off / Power off procedure in RRC_INACTIVE

4.9.6.2.1	Procedure

Table 4.9.6.2.1-1: Switch off procedure in RRC_INACTIVE

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |  |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |  |
| 1a2 | UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest |  |
| 1a3 | SS transmit an RRCResume message. | <-- | NR RRC: RRCResume |  |
| - | EXCEPTION: Steps 1a4Aa1 to 1a4Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |  |
| 1a4Aa1 | The UE transmits an RRCResumeComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCResumeComplete 5GMM: SERVICE REQUEST |  |
| 1a4Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |  |
| 1a4Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |  |
| 1a4Aa4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |  |
| - | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed. | - | - |  |
| 1a4Aa5 | The UE transmits an RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete |  |
| 1a4Aa6 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |  |
| - | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration | - | - |  |
| 1a4 | Void | - | - |  |
| 1a5 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |  |
| 1b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

4.9.6.2.2	Specific Message contents

Table 4.9.6.2.2-1: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | Not checked |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<Unchanged sections skipped>

5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

For Tx test cases in which Power Class 3 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-1; For Tx test cases in which Power Class 2 UEs verifying Power Class 2 requirements, refer to Table 5.4.1-2; And for Tx test cases in which Power Class 2 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-3.

Table 5.4.1-1: P-Max-PC3

Derivation Path: Table 4.6.3-89 with condition FR1_RF_PC3

Table 5.4.1-2: P-Max-PC2

Derivation Path: Table 4.6.3-89 with condition FR1_RF_PC2

Table 5.4.1-3: P-Max-PC2_Testing_PC3

Derivation Path: Table 4.6.3-89 with condition FR1_RF_PC2_Testing_PC3

Table 5.4.1-4: PUSCH-Config

| Derivation Path: Clause 4.6.3 Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| maxRank | 2 |  | 2TX_UL_MIMO |
|  | 1 |  | ULFPTx_Mode1 or ULFPTx_Mode2 or ULFPTx_ModeFull |
| ul-FullPowerTransmission-r16 | Not present |  | 2TX_UL_MIMO |
|  | fullpowerMode1 |  | ULFPTx_Mode1 |
|  | fullpowerMode2 |  | ULFPTx_Mode2 |
|  | fullpower |  | ULFPTx_ModeFull |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

CSI-RS for Tracking

Table 5.4.1-5: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1000 |  | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 6 | l0 = 6 for CSI-RS resource 1 and 3 | TRS AND FR1 |
|  | 10 | l0 = 10 for CSI-RS resource 2 and 4 | TRS AND FR1 |
|  | 8 | l0 = 8 for CSI-RS resource 1 | TRS AND FR2 |
|  | 12 | l0 = 12 for CSI-RS resource 2 | TRS AND FR2 |
| nrofPorts | p1 |  | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

Table 5.4.1-6: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 0 | SCS 15kHz, CSI-RS resource 1 and 2 | TRS AND FR1_15kHz |
|  | 1 | SCS 15kHz, CSI-RS resource 3 and 4 | TRS AND FR1_15kHz |
| Slots20 | 1 | SCS 30kHz, CSI-RS resource 1 and 2 | TRS AND FR1_30kHz |
|  | 2 | SCS 30kHz, CSI-RS resource 3 and 4 | TRS AND FR1_30kHz |
| Slots40 | 2 | SCS 60kHz, FR1, CSI-RS resource 1 and 2 | TRS AND FR1_60kHz |
|  | 3 | SCS 60kHz, FR1, CSI-RS resource 3 and 4 | TRS AND FR1_60kHz |
| Slots80 | 40 | SCS 60kHz, FR2, CSI-RS resource 1 and 2 | TRS AND FR2_60kHz |
| Slots160 | 80 | SCS 120kHz, CSI-RS resource 1 and 2 | TRS AND FR2_120kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |
| FR2_60kHz | FR2 is used under the test. SCS is set to 60kHz. |
| FR2_120kHz | FR2 is used under the test. SCS is set to 120kHz. |
| TRS | Tracking-Reference Signal |

Table 5.4.1-7: CSI-MeasConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 2 entries in case of FR2 4 entries in case of FR1 | 2 entries in case of FR2 4 entries in case of FR1 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 | FR1 |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 | FR1 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList SetupRelease | Not present |  |  |
| } |  |  |  |

Table 5.4.1-8: NZP-CSI-RS-Resource for TRS

| Derivation Path: 38.508-1 Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| NZP-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
|  | 1 | CSI-RS resource 2 |  |
|  | 2 | CSI-RS resource 3 |  |
|  | 3 | CSI-RS resource 4 |  |
| CSI-RS-ResourceMapping | CSI-RS-ResourceMapping for TRS |  | Content conditioned by the CSI-RS resource # |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS |  | Content conditioned by the CSI-RS resource # |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| } |  |  |  |

Table 5.4.1-9: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries in case of FR2 4 entries in case of FR1 |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource 3 | FR1 |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource 4 | FR1 |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | true |  |  |
| } |  |  |  |

Table 5.4.1-10: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

<Unchanged sections skipped>

CSI-RS for beam refinement

NZP-CSI-RS-Resource

Table 5.4.2.0-29: NZP-CSI-RS-Resource

| Derivation Path: Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  | DEMOD_FR2 |
| nzp-CSI-RS-ResourceId | 5 | CSI-RS resource 6 | Beam refinement |
|  | 6 | CSI-RS resource 7 | Beam refinement |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-State #1 |  |  |
| } |  |  |  |

CSI-RS-ResourceMapping

Table 5.4.2.0-30: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  | DEMOD_FR2 |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | K0 = 0, row1 for resource 1 and 2 |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 8 | I0 = 8 for resource 1 |  |
|  | 9 | I0 = 9 for resource 2 |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-31: CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  | DEMOD_FR2 |
| Slots80 | 0 |  | SCS 60kHz |
| Slots160 | 0 |  | SCS 120kHz |
| } |  |  |  |

CSI-FrequencyOccupation

Table 5.4.2.0-32: CSI-FrequencyOccupation for beam refinement

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  | DEMOD_FR2 |
| nrofRBs | 68 |  | BW 100MHz SCS 120kHz |
| } |  |  |  |

NZP-CSI-RS-ResourceSet

Table 5.4.2.0-33: NZP-CSI-RS-ResourceSet for beam refinement

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  | DEMOD_FR2 |
| nzp_CSI_ResourceSetId | 2 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 5 | entry 1 CSI-RS resource 6 |  |
| NZP-CSI-RS-ResourceId[2] | 6 | entry 1 CSI-RS resource 7 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | Not present |  |  |
| } |  |  |  |

CSI-ResourceConfig

Table 5.4.2.0-34: CSI-ResourceConfig for beam refinement

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 2 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 2 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

CSI-RS for beam management

NZP-CSI-RS-Resource

Table 5.4.2.0-35: NZP-CSI-RS-Resource for beam management

| Derivation Path: Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  | PDCCH_FR2 |
| nzp-CSI-RS-ResourceId | 4 | CSI-RS resource 5 |  |
|  | 5 | CSI-RS resource 6 |  |
| resourceMapping | CSI-RS-ResourceMapping for beam management |  |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for beam management |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-State #1 |  |  |
| } |  |  |  |

CSI-RS-ResourceMapping

Table 5.4.2.0-36: CSI-RS-ResourceMapping for beam management

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  | PDCCH_FR2 |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | K0 = 0, row1 for resource 1 and 2 |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 8 | I0 = 8 for resource 1 |  |
|  | 9 | I0 = 9 for resource 2 |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation for beam management |  |  |
| } |  |  |  |

CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-37: CSI-ResourcePeriodicityAndOffset for beam management

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  | PDCCH_FR2 |
| Slots80 | 0 |  | SCS 60kHz |
| Slots160 | 0 |  | SCS 120kHz |
| } |  |  |  |

CSI-FrequencyOccupation

Table 5.4.2.0-38: CSI-FrequencyOccupation for beam management

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  | PDCCH_FR2 |
| nrofRBs | 68 |  | BW 100MHz SCS 120kHz |
| } |  |  |  |

NZP-CSI-RS-ResourceSet

Table 5.4.2.0-39: NZP-CSI-RS-ResourceSet for beam management

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  | PDCCH_FR2 |
| nzp_CSI_ResourceSetId | 1 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource 5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 1 CSI-RS resource 6 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | Not present |  |  |
| } |  |  |  |

CSI-ResourceConfig

Table 5.4.2.0-40: CSI-ResourceConfig for beam management

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 1 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 1 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

<Unchanged sections skipped>