3GPP TSG-RAN5 Meeting #90-e	R5-211375

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1715 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Table 4.6.3-185 SSB-MTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, If the SFNoffset of serving cell is odd number while the SFNoffset of inter frequency neighbour cell is even number, current SMTC(periodicity=20sf,Offset=0) will not include the SSB of inter frequency neighbour cell. 2. There are two configuration of periodicityAndOffset with same parameter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Modify the explanation for INTER-FREQ_ODD. If the SFNoffset of serving cell is odd number while the SFNoffset of inter frequency neighbour cell is even number, or SFNoffset of serving cell is even number while the SFNoffset of inter frequency neighbour cell is odd number, (SFNoffset of inter frequency neighbour cell - SFNoffset of serving cell) will equals to odd number, and SMTC(periodicity=20sf,Offset=10) is used to include the SSB of inter frequency neighbour cell. 2, Delete the unnecessary periodicityAndOffset. 3, Replace with invisible lines for param duration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-210624r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	SSB-MTC

Table 4.6.3-185: SSB-MTC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  |  |
|  | 10 |  | SIG AND INTER-FREQ_ODD |
| } |  |  |  |
| duration | sf2 |  | FR1 |
|  | sf3 |  | FR2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-FREQ_ODD | When the (SFNoffset of inter frequency neighbour cell - SFNoffset of serving cell) is odd number. SFNoffset is defined in TS 38.523-3 [23] Table 7.1.5.2-1 |

<End of modified section 1>