3GPP TSG-RAN WG5 Meeting #90-e	R5-211689

# Electronic Meeting, 22nd Feb – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0477 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.2A.1.1.5 UE maximum output power - EIRP and TRP for 6UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding new Tx test cases according to WP for RAN5 work item: 5GS_NR_LTE-UEConTest |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case - 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case for CA will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.1.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-211029 and the outcome of 1 revision to this document |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2A.1.1.5	UE maximum output power - EIRP and TRP for CA (6UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 3 and 4.

- Test Procedures for EIRP beam peak Extreme Conditions are FFS.

- Test Tolerances in the Test Requirement are FFS.

6.2A.1.1.5.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6UL CA.

NOTE: This test case can’t be performed due to lack of appropriate test points.

6.2A.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.5.4	Test description

6.2A.1.1.5.4.1	Initial condition

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.5.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.5.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.5.5-1.

Table 6.2A.1.1.5.4.1-1: Test Configuration Table

NOTE: No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

6.2A.1.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.5.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.5.5-1.

Table 6.2A.1.1.5.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257K | 23+TT | 43 | 22.4-TT |
| CA_n260K | 23+TT | 43 | 20.6-TT |
| CA_n261K | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.5.5-1a: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

Table 6.2A.1.1.5.5-1b: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

<< End of changes >>