3GPP TSG-RAN5 Meeting #90-e	R5-210788

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1140 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding additional TP for half Pi BPSK DMRS to NR ACLR test case for SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the affected 38.521-1 CR, the test configuration in MPR testing for SUL is modified to include the additional test points for Pi/2 BPSK DMRS. Hence, in NR ACLR testing for SUL there is a need to add additional test points for Pi/2 BPSK DMRS as per MPR testing for SUL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding additional test points for Pi/2 BPSK DMRS to NR ACLR for SUL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For Rel-16 UE supporting Pi/2 BPSK DMRS, NR ACLR for SUL would not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5C.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR 1138 TR 38.905 CR 0352 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5C.2.4	Adjacent channel leakage ratio for SUL

Adjacent channel leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

6.5C.2.4.1	NR ACLR for SUL

6.5C.2.4.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5C.2.4.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.2.4.1.3	Minimum conformance requirements

The minimum conformance requirements specified in Clause 6.5.2.4.1.3 applies to the UE that support SUL operating on the SUL bands.

The normative reference for this requirement is TS 38.101-1 clauses 6.5.2.4.1.

6.5C.2.4.1.4	Test description

Same test description as PC 3 with contiguous allocation specified in clause 6.5.2.4.1.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.2.4.1.4.1-1 use Table 6.5C.2.4.1.4-1

Table 6.5C.2.4.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |  |
|  |  | N/A | N/A | Modulation |  | RB allocation |  |
| 1 | Default |  |  | DFT-s-OFDM PI/2 BPSK |  | Inner_Full |  |
| 2 | Low |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |  |
| 3 | High |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |  |
| 4 | Default |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer_Full |  |
| 5 | Default |  |  | DFT-s-OFDM QPSK |  | Inner_Full |  |
| 6 | Low |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |  |
| 7 | High |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |  |
| 8 | Default |  |  | DFT-s-OFDM QPSK |  | Outer_Full |  |
| 9 | Default |  |  | DFT-s-OFDM 16 QAM |  | Inner_Full |  |
| 10 | Low |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |  |
| 11 | High |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |  |
| 12 | Default |  |  | DFT-s-OFDM 16 QAM |  | Outer_Full |  |
| 13 | Low |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |  |
| 14 | High |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |  |
| 15 | Default |  |  | DFT-s-OFDM 64 QAM |  | Outer_Full |  |
| 16 | Low |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |  |
| 17 | High |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |  |
| 18 | Default |  |  | DFT-s-OFDM 256 QAM |  | Outer_Full |  |
| 19 | Default |  |  | CP-OFDM QPSK |  | Inner_Full |  |
| 20 | Low |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |  |
| 21 | High |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |  |
| 22 | Default |  |  | CP-OFDM QPSK |  | Outer_Full |  |
| 23 | Default |  |  | CP-OFDM 16 QAM |  | Inner_Full |  |
| 24 | Low |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |  |
| 25 | High |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |  |
| 26 | Default |  |  | CP-OFDM 16 QAM |  | Outer_Full |  |
| 27 | Low |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |  |
| 28 | High |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |  |
| 29 | Default |  |  | CP-OFDM 64 QAM |  | Outer_Full |  |
| 30 | Low |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |  |
| 31 | High |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |  |
| 32 | Default |  |  | CP-OFDM 256 QAM |  | Outer_Full |  |
| 334 | Low |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Edge_1RB_Left |  |
| 344 | High |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Edge_1RB_Right |  |
| 354 | Default |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Outer Full |  |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. Note 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. Note 4: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.3-14 with condition SUL_SUL, Table 4.6.3-15 with condition SUL_NUL, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. Message contents in Table 6.5.2.4.1.4.3-3 in clause 6.5.2.4.1.4 only apply to Test ID 33-35 in Table 6.5C.2.4.1.4-1.

6.5C.2.4.1.5	Test requirement

The measured UE mean power in the channel bandwidth on SUL carrier, derived in step 3, shall fulfil requirements in Clause 6.2C.4.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in Table 6.5C.2.4.1.5-1.

Table 6.5C.2.4.1.5-1: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5C.2.4.1.5-2: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  |  | 30 + TT dB |
| NOTE 1: TT = 0.8 dB for f ≤ 4.0GHz, TT = 1.0 dB for 4.0GHz < f ≤ 6.0GHz, |  |  |  |

<End of Changes>