3GPP TSG RAN WG#91-e		RP-210317

Electronic Meeting, March 16 - 26, 2021

Source:	vivo

Title:	On IRC receiver performance in time selective interference

Agenda Item:	9.7.17

Document for:	Discussion

- Discussion

NR system is designed to support various application scenarios which can be configured very flexibly. For example, the time domain resource allocation (TDRA) is indicated dynamically in DCI, and the PDSCH/PUSCH can be configured as Type-A/Type-B with configurable number of DMRS symbols. The time domain position of DMRS depends on number of configured additional DMRS and the allocated TDRA. Few examples as shown in figures 1, 2 below. Furthermore, DMRS ports can be multiplexed in FDM manner, e.g. Type-1 DMRS (Comb structure), where the data RE may or may not be multiplexed on the DMRS symbol depending on indicated “value” in DCI.

In figure 1) UE1 in cell1 is allocated 6 symbols and UE2 in cell2 is allocated 3 symbols. In this scenario, two figures depict two cases where in the figure a) UE1 in cell1 is not able to observe interference from UE2 in cell2 on the DMRS symbol and in the figure b) UE1 in cell1 is able to observe interference from UE2 in cell2 on the DMRS symbol. On the other hand, if there are other UEs scheduled in cell2 on symbols not occupied by UE2, the interference measured on DMRS by UE1 still does not reflect the interference from other UEs.

Cell1, UE1

Cell2, UE2

Cell1, UE1

Cell2, UE2

- b)

Figure 1, type B scheduling in adjacent cells

And, in figure 2) UE1 in cell1 is scheduled such that data is multiplexed on DMRS symbols, whereas UE 2 in cell2 is scheduled such that there is no data multiplexed on DMRS symbols. In this case, UE1 does not observe interference on DMRS from UE2

Cell1, UE1, rank 1

DMRS value 0

Cell2, UE2, rank 2

DMRS value 2

Cell2, UE2, rank 1

DMRS value 4 or 5

Or

Data RE

DMRS RE

Blank RE

Figure 2, scheduling with and without data multiplexed on DMRS symbols

- Evaluations

In this section, preliminary evaluation results are provided to demonstrate IRC receiver performance degradation in case the interference in neighboring cell is misaligned. Following cases are simulated. Other assumptions can be found in Annex.

Scenario a: type-B PDSCH scheduling in cell1 and cell2

- Case1: UE1 in cell1 and UE2 in cell2 are scheduled with 6 symbols (fully overlapping with same DMRS configuration)

- Case2: UE1 in cell1 is scheduled with 6 symbols and UE2 in cell2 is scheduled with 3 symbols (partial overlapping, figure 1 a)

- Case3: UE1 in cell1 is scheduled with 6 symbols and UE2 in cell2 is scheduled with 3 symbols (partial overlapping with overlapping DMRS, figure 1 b)

Scenario b: type-A PDSCH scheduling in cell1 and cell2 (fully overlapping with same DMRS configuration)

- Case1: UE1 in cell1 is scheduled with rank 1 (DMRS port 0), UE2 in cell2 is scheduled with rank 2 (DMRS ports 0, 1)

- Case2: UE1 in cell1 is scheduled with rank 1 (DMRS port 0), UE2 in cell2 is scheduled with rank 2 (DMRS ports 0, 2)

- Case3: UE2 in cell1 is scheduled with rank 1 (DMRS port0), UE2 in cell2 is scheduled with rank 2 (DMRS ports 2, 3)

In the figure below, “full” corresponds to case1, “partial” corresponds to case2 and “non” corresponds to case3.

Scenario a)                                                              Scenario b)

Figure 3, link level evaluation results

Observation: from the evaluation results, it can be observed that when the interference measurement on DMRS does not reflect actual interference, the performance of IRC receiver degrades significantly.

- Conclusion

From the evaluations it can be observed that IRC receiver performance degrades significantly when the neighboring cell interference is not aligned.

- Annex

Simulaiton assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4 GHz |
| Channel model | TDL-A (delay spread:100 ns) as in 38.901 |
| UE speed | 3 km/h |
| BS antenna configuration | 4 Tx antenna with low correlation |
| UE antenna configuration | 4 Rx antenna with low correlation |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical MMSE filter |
| Receiver type | Adaptive MMSE-IRC receiver with 2PRBs granularity |
| Rank adaptation | disabled |
| AMC adaptation | enable |
| Codebook type | Type I |
| CSI-RS | Period :160 slots |
| UE assumption | One target UE with fixed 1 layer data, the other UE as interference |
| PDSCH allocation | Depend on the Cases listed in paper |