3GPP TSG RAN WG1 email discussion [5G-ACIA]

E-mail discussion, February 22 - 26, 2021

Source:	Qualcomm

Title:	Simulation Assumptions and URLLC Performance Evaluations for 5G-ACIA Performance Evaluation Round 2

Document for:	Discussion

# 1	Introduction

Following the agreements discussed during the first week of the 5G ACIA e-meeting in October 2020 [1] and the plan agreed in [3], simulations assessing the URLLC/IIOT performance have been performed.

The goal is to provide performance evaluations to 5G-ACIA for their LS on 3GPP NR Rel-16 URLLC [2], [3]. The target date for the completion of this activity is TSG RAN Meeting #91e in March 2021.

Simulation assumptions and performance metrics are agreed during the October meeting [1]. The first round of simulation results was presented and discussed in December 2020. At the end of the meeting, a summary with agreements as well as a table with simulations assumptions and models to be explicitly aligned during the second round of results was described [4].  Detailed description of the simulation parameters and models are given in section 2, with an emphasis on the parameters mentioned at the summary of the previous round. The Rel. 15 and Rel. 16 URLLC features used in the simulations are described in Section 3.

Simulation results obtained with the agreed simulation assumptions are presented in Section 4. The focus of this paper is FR2. Section 5 concludes the contribution.

# 2	Simulation Parameters and Models

In TR 38.824 ([5]), detailed simulation parameters were provided for system level and link-level analysis. Most parameters for the 5G-ACIA performance evaluation originated from TR 38.824 ([5]), as agreed in [1] and therefore what is mainly presented here is:

- the set of parameters that are different from the ones of TR 38.824

- parameters and assumptions for which alignment is required [4].

## 2.1	Performance Metrics

From 5G-ACIA LS [1], a set of performance metrics are provided. Among those performance criteria, the ones below are used:

- “URLLC Capacity, i.e. number of users satisfying reliability and latency requirements”, as defined in 38.824; hence, the number of UEs/cell satisfying the BLER Target requirement specified, assuming transport blocks (packets) are received prior to their packet expiration.

- Communication Service Availability (CSA): single CDF of CSA distribution of all UEs in factory hall

- Latency: single CDF of latency distribution of all UEs in factory hall

More details on the performance metrics, follow.

### 2.1.1 URLLC Capacity

It is defined as the maximum offered cell load, C, in terms of number of UEs per cell, at which (load) Y% of UEs in the cell operate with link reliability R under L latency bound.

The percentage X% of UE which do not satisfy the reliability R and latency L requirements are the UEs in outage.

X = 100 – Y									(1)

In these ACIA simulations, the assumption is to consider periodic traffic with period equal to 1 ms. Packets expire in 1 ms and therefore the latency requirement L, is equal to 1 ms as well.

The reliability requirement, R, is 10-4 (or 0.01%). The accepted percentage, X%, of UEs being in outage is 15%.  As an example, in a system with N UEs/cell in which at least 85% of the UEs in the cell have BLER lower than 0.01% and whose packets arrive within 1 msec, the “URLLC Capacity”, C, is equal to N UEs/cell.

### 2.1.2 Communication Service Availability (CSA)

It is defined ([6]) as the percentage of the amount of time the end-to-end service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver end-to-end service. The communication system is in the “available” state if the availability criteria for transmitted packets are met. The service is unavailable if the packets received at the target are impaired or they are received after their expiration. If the survival time is larger than zero, consecutive impairments and delays are ignored until the respective time has expired.

For the work here, it is assumed that the survival time is equal to the packet periodicity, which is 1 ms. Hence, the service is considered unavailable in case at least 2 consecutive errors occur, i.e., no packet reception is observed for at least 2 consecutive traffic periods. The target is that CSA is 99.999% of the time.

### 2.1.3 Latency

Latency is measured as the packet delay. It is measured both in DL and UL. In DL, the packet delay is the time difference between time instant 1 and time instant 2. Time instant 1 is the instant at which the DL packet arrives at the gNB application buffer. Time instant 2 is the instant at which the DL packet is successfully decoded at the UE and it is transmitted to the UE higher layers. In UL, the packet delay is measured in a similar way as in DL: the packet delay is the time difference between instants 1 and 2 where instant 1 is the instant of the packet arrival at the application layer of the UE buffer and instant 2 is the instant of successful decoding at the gNB. Between these 2 instants, there are few delays involved, such as gNB and UE processing delays, queuing delays at the gNB and frame alignment delays, as described in TR 37.910. All of them are considered here.

## 2.2	Simulation Parameters

### 2.2.1 Network Topology

A factory hall of 120m x 50m x 10m, which is then fully covered by 12 service areas of 50m x 10m. BS antenna height is 8m for InF-DH. The number of BS is 12. Each BS is consisted of 128 antenna elements, each antenna element having dual polarization. In the simulations up to 4 simultaneous DL transmissions to 4 different UEs are possible. For each scheduled UE, 32 antenna elements and up to 40 MHz bandwidth are allocated.

| Inter-BS distance | 20 m as in TR 38.824 (reproduced in Figure 1 below) for the factory automation use case. |
| --- | --- |
| BS antenna height | 8 meter |
| Channel model | InF-DH |

Figure 1. Base station layout for factory automation use case in TR 38.824.

### 2.2.2 Traffic model downlink and uplink

For both downlink and uplink, Option 1 is chosen, hence:

- Option-1: all UEs’ messages both in DL and UL arriving at NG-RAN node in the first transfer interval are uniformly random distributed within the TI time window.

In addition, “For DL-UL traffic arrival time relationship” the option 1 is considered.

- Option-1: DL and UL traffic arrival time instants are independent.

### 2.2.3 Number of UEs per cell

In the 5G-ACIA scenario, the UEs are randomly distributed within the respective service area, with up to 28 UEs per service area. Considering that there are a total of 12 service areas, the total number of UEs is up to 12x28=336 UEs.

After fine tuning, the cases of interest are the ones with 20 and 28 UEs per cell.

| Number of UEs per Cell | {20, 28} The UEs are randomly distributed in each cell. |
| --- | --- |

### 2.2.4 Other assumptions for FR 2

For FR2, only TDD is available. The same parameters are assumed in 5G-ACIA LS and TR 38.824, and summarized below.

| Duplex | TDD |
| --- | --- |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 160 MHz |
| SCS | 120 kHz |

### 2.2.5 TDD Frame and slot structure

To meet the latency requirements, the slot configuration #50 from the table 11.1.1-1 from TS 38.213 is selected. It is the slot configuration with the pattern in (symbols) DDFFUUU is repeated twice in a 14 symbols slot. The 3rd symbol in the slot configuration which initially was used as flexible symbol, it is used as DL symbol for PDSCH transmission. The fourth (initially) flexible symbol is used as gap in our simulations.

The slot format used is illustrated in Figure 2. Several reasons justify this choice. The first reason is that this slot configuration #50 satisfies the requirement for 1:1 DL:UL as agreed in [1]. Within each 14 symbols-slot, there are 6 DL symbols and 6 UL symbols. In addition, this configuration is the one which fits with the SPS and the CG configurations used, while keeping the UE and gNB feedback timing close to its minimum possible. For SPS, the PDSCH allocation is for a mini-slot of 2 DL symbols, as it can be seen in Figure 2. For CG, the PUSCH allocation is also a mini-slot allocation of 2 UL symbols. Since SPS and CG are configured, PDCCH (hence DCI) is used only for DL or UL allocations (PDSCH or PUSCH) for eventual (rare) retransmissions. Therefore, 1 symbol PDCCH is chosen. For UCI, one UL symbol PUCCH is used. For a given UE, since the traffic is only one packet per 1 ms, the DL - UL switching is determined by N1 and not just by the gap symbol in the slot format. In these simulations, N1 is assumed to be 20 symbols, N2 is assumed to be 42 symbols and gNB processing time to be 42 symbols.

Figure 2. Slot Format used in simulations.

### 2.2.6 Summary

Below a summary of simulation parameters and models can be seen.

| Parameters | 5G-ACIA LS |
| --- | --- |
| Factory hall size | 120x50 m |
| Room height | 10 m |
| Inter-BS/TRP distance | 20 m |
| BS/ antenna height | 8m for InF-DH |
| Layout – BS/TRP deployment | 12 BS |
| Channel model | InF-DH |
| Carrier frequency and simulation bandwidth | TDD 30 GHz: 160 MHz |
| TDD DL-UL configuration | Slot configuration #50 of Table 10.1.1-1 [TS 38.213] |
| Number of UEs per service area | 20, 28 (for DL) – 20, 24 (for UL) |
| UE distribution | All UEs randomly distributed within the respective service area. |
| Message size | 48 bytes |
| DL traffic model | DL traffic arrival with option-1 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 |
| CSA requirements | UC-#2: 99.9999% |
| Performance metrics | Percentage of UEs satisfying requirement, where requirement is PER:0.01% (10-4) |
| E2E latency & air interface latency | E2E latency: 1 ms for UC#2 |
| UE speed | Linear movement: 75 km/h |
| UE Tx Power | 23 [dBm] |
| BS Tx Power | 26 [dBm] (23 dBm per 80 MHz) |
| BS / UE Rx Noise Figure | 7 [dB] / 10 [dB] |

# 3	URLLC/IIoT Features

## 3.1 Features for FR2

For FR2, SCS of 120 kHz is assumed. This makes it easier to achieve the 1ms latency requirement, even though only TDD is possible for FR2.  Due to the limitation of TDD only for FR2, it is even more important to rely on DL SPS and UL CG to support the traffic pattern, thus eliminating the latency caused by PDCCH. Hence, Rel-15 features appear to be sufficient.

Thus, the following SPS, CG configurations are used:

- UL CG with one 2-symbol mini-slot configuration.

- DL SPS with one 2-symbol mini-slot configuration.

In addition, for downlink control, 1 symbol PDCCH is assumed and for uplink control 1 symbol PUCCH.

In terms of scheduling, FDM for both downlink and uplink have been used. In both directions, up to 4 UEs can be scheduled simultaneously and each UE can be granted one of the 4 orthogonal 40 MHz bands (in each direction).

# 4	Simulation Results

As mentioned above, for DL simulations, there are 20 and 28 UEs per BS, which means that there are 240 and 288 UEs in the whole system. The simulation time is equal to 100 seconds, resulting thus in 105 packets per UE in each direction, i.e. 105 packets, per UE in DL and 105 packets per UE in UL. The percentage of the UEs satisfying the PER requirement of 10-4 is ~86% in the case of 20 UEs/cell. This performance requirement is quite stringent to meet, even compared to the CSA requirement.

Figure 3 shows the number and percentage of UEs which satisfy the PER target (“reliability requirement”). In case the allowed number of UEs being in outage is 15%, then the “URLLC Capacity” according to the definition of TR 38.824 is equal to 20 UEs/cell. However, it is estimated that this number can be further increased by simply using another slot format configuration; the slot format currently used overprovisions PDCCH resulting to up to 33 % DL capacity loss, in the worst case, since 1 out of DL 3 symbols in the slot are reserved for PDCCH, which is needed only in case of (rare) retransmissions.

Figure 4 shows the DL packet latency CDF (left part) and the percentage of UEs meeting the CSA requirement of 99.9999%. Approximately ~75% of packets are successfully decoded in the physical layer in less than 250 μsec, which implies that the packet is successfully decoded with the 1st transmission.

On the right part of Figure 4, more than 95% of UEs satisfy the CSA requirement of 99.9999% in the case of 20 UEs/cell.

Figure 3. Number and Percentage of Downlink UEs satisfying the PER requirements of 10-2, 0.5*10-2, 10-3, 0.5*10-3, 10-4.

Figure 4. Downlink Packet Latency and percentage of UEs satisfying the CSA requirement of 99.9999%.

# 5	Conclusions

Baseline URLLC/IIOT performance analysis within FR 2 has been performed. Results show that with the agreed simulation assumptions and basic Rel. 15 URLLC/IIOT features, approximately 86 % of the UEs-in a simulation with 20 UEs/cell-can be served with PER less than 10-4 and more than 95 % (of those 20 UEs/cell) meet the CSA requirement of 99.9999%.

# 5	References

3GPP 5G-ACIA, “Agreements on URLLC Features and Simulation Assumptions for 5G-ACIA”.

RP-201279, “LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation (5G-ACIA-LS-2020-WI042; to: RAN, RAN1; cc: SA1, RAN2; contact: Bosch”, 5GCIA.

R1-2007186, “RAN1 LS to RAN plenary”.

“Summary of discussions in Week 2”, 3GPP-RAN 5G-ACIA Evaluation Week 2, December 14th-18th, Ericsson.

3GPP TR 38.824, “Study on Physical Layer Enhancements for NR Ultra-Reliable and low latency case (URLLC) (Release 16)”, v. 16.0.0 (2019-03).

3GPP TS 22.104, “Service Requirements for Cyber-physical control applications in vertical domains, Stage 1, Rel, 17”, v. 17.4.0 (2020-09).

RP-202069, “Way forward and RAN work for 5G ACIA requested simulations”, Ericsson.

RP-202097, “on 3GPP NR Rel-16 URLLC and IIoT performance evaluation” (to: 5G-ACIA; cc: RAN1, RAN2, SA1; contact: Ericsson).

# Appendix 1. Performance requirements in TS 22.104

The services requirements of motion control are copied below from TS 22.104 V17.4.0.

TS 22.104 V17.4.0, Table 5.2-1: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) (note 12a) | Service bit rate: user experienced data rate (note 12a) | Message size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) | Remarks |
| 99.999 % to 99.999 99 % | ~ 10 years | < transfer interval value | – | 50 | 500 μs | 500 μs | ≤ 75 km/h | ≤ 20 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99.999 9 % to 99.999 999 % | ~ 10 years | < transfer interval value | – | 40 | 1 ms | 1 ms | ≤ 75 km/h | ≤ 50 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99.999 9 % to 99.999 999 % | ~ 10 years | < transfer interval value | – | 20 | 2 ms | 2 ms | ≤ 75 km/h | ≤ 100 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| NOTE 1: One or more retransmissions of network layer packets may take place in order to satisfy the communication service availability requirement. NOTE 2: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 3: Length x width (x height). NOTE 12a: It applies to both UL and DL unless stated otherwise. NOTE 13: It applies to both linear movement and rotation unless stated otherwise. |  |  |  |  |  |  |  |  |  |  |

# Appendix 2. Activity plan

As it was agreed in [3]:

- Discussions are on the RAN1_NR reflector

  - Email activity only during short periods (< week) distributed across the time allocated to the activity

  - No email activity in weeks before/during/after RAN1 meetings or RAN defined inactive periods

Based on statements above, the following activity plan has been proposed:

- 12-16 October 2020

  - Discussion on which URLLC features to include in the evaluations and simulation assumptions

- 14-18 December 2020

  - First round of simulation results

- 22-26 February 2021

  - Second round of simulation results

- 8-12 March 2021

  - Finalization of the report to RAN#91