3GPP TSG RAN meeting #91-e								RP-210490

Electronic meeting, March 22 – 26, 2020

Agenda Item:	7

Source:	Moderator (Ericsson)

Title:	Report on evaluations for 5G-ACIA

Document for:	Discussion, Decision

# 1	Introduction

Based on the request from 5G-ACIA in LS RP-201279, RAN#89 agreed RP-202069 on providing evaluations for 5G-ACIA:

- Start an offline email-based activity to provide evaluation results for 5G-ACIA

- One company volunteers as moderator

  - Proposes a work plan to follow

  - Ericsson is willing do this

- Discussions are on the RAN1_NR reflector

  - Email activity only during short periods (< week) distributed across the time allocated to the activity

  - No email activity in weeks before/during/after RAN1 meetings or RAN defined inactive periods

  - All companies should strive to limit email activity as much as possible

  - Outcome of the offline discussion will directly go to RAN without need for discussion in RAN1 nor need for LS from RAN1 to RAN

- Target completion by RAN#91

- At RAN#91, RAN will decide on a response LS to 5G-ACIA

The moderator made the following proposal on a timeline:

- 12-16 October 2020

  - Discussion on which URLLC features to include in the evaluations and simulation assumptions

- 14-18 December 2020

  - First round of simulation results

- 22-26 February 2021

  - Second round of simulation results

- 8-12 March 2021

  - Finalization of the report to RAN#91

During week 1, the simulation assumptions were agreed as captures in the document below:

https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_90e/Inbox/Drafts/5G-ACIA  October/Agreements/Agreements week 1 5G-ACIA.docx

For week 2, companies provided the first round of simulation results. The summary is provided here:

https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_91e/Inbox/Drafts/5G-ACIA December/Final Summary/5G-ACIA Week 2 - Final summary.docx

For the third week, companies provided the second round of simulation results:

https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_91e/Inbox/Drafts/5G-ACIA February/Company Inputs/

The summary of discussions for week three can be found here:

https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_91e/Inbox/Drafts/5G-ACIA February/Final Summary/5G-ACIA evaluations week 3 Summary.docx

This contribution is the report of the evaluations for presentation at RAN#91e that can be sent to 5G-ACIA.

Big thanks go to the companies that participated in the simulation exercise: CATT, Ericsson, Huawei/HiSilicon, Intel, ITRI, Nokia/Nokia Shanghai Bell, Qualcomm, vivo and ZTE.

# 2	Features and assumptions

## 2.1	 Features investigated

### 2.1.1	Baseline

For the Rel-15 baseline, the following features were agreed:

- Rel-15 URLLC features included in the baseline are as follows, while it is up to each proponent to decide which Rel-15 features are used, and detail this when providing the results:

  - UE Processing capability 2

  - UL Configured grant

  - DL Semi-persistent scheduling

Regarding inclusion of Rel-16 features, the following was agreed:

- It is up to each proponent to decide on which Rel-16 features to provide simulations results for in addition to the Rel-15 baseline

For the exact features that each company has have simulated, please see the corresponding documents in the reference section.

### 2.1.2	Categorization of coordination schemes

The capacity of the system can be increased by coordinating between the BS/TRP. If coordination is applied, this can range from static partitioning of the resource to fully centralized joint scheduling of the BS/TRP. For the purpose of comparing simulation results, the level of coordination is divided into categories as follows:

- No coordination – no static partitioning or coordination of resources among BS/TRP

- Independent/Uncoordinated scheduling – static partitioning of resources among BS/TRP, but with uncoordinated scheduling

- Common/centralized scheduling among BS/TRP synchronized within the order of the cyclic prefix

The exact coordination schemes used are considered network implementation specific.

### 2.1.3	Performance metrics

As requested in the LS, the performance metrics used in the simulations are CSA, latency, percentage of UEs satisfying requirements and resource utilization. The CSA with a survival time of 1ms and latency CDF for each company's simulation results can be found in appendix A.

It should be noted that 3GPP has traditionally be using reliability as a performance metric for URLLC where the reliability for one UE is calculated as “1- PER” where PER=Nerror/NT where Nerror denotes the number of packets that are blocked from transmission or unsuccessfully decoded at the receiver within the latency deadline. The results summarized in this report are based on the CSA but some companies have included also reliability in their simulations. For the evaluated traffic model, the main difference between CSA with 1ms survival time and the traditionally used reliability is that for the former one packet error is tolerated and only two consecutive decoding errors are considered as a fault.

## 2.2	Simulation assumptions

The agreements on common simulation assumptions are shown in Table 1. Additional simulation parameters are taken from TR 38.824.

Table 1: Agreed simulation assumptions

| Parameters | 5G-ACIA LS | Agreement |
| --- | --- | --- |
| Factory hall size | 120x50 m | As in 5G-ACIA LS |
| Room height | 10 m | As in 5G-ACIA LS |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. | According to proposed layout below (see figure below table) |
| BS/TRP antenna height | 1.5 m for InF-SL and InF-DL 8m for InF-SH and InF-DH | As in 5G-ACIA LS |
| Layout – BS/TRP deployment | Depending on the number of TRPs | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| Channel model | UC-2: InF-DH > InD-DL > InF-SH > InF-SL | Mandatory: InF-DH Optional: InD-DL, InF-SH, InF-SL |
| Carrier frequency and simulation bandwidth | TDD 4 GHz: 100 MHz 30 GHz: 160 MHz | As in 5G-ACIA LS |
| TDD DL-UL configuration | Simulation company should report the used DL-UL configuration. | Companies should report the used DL-UL configuration. 1:1 DL-UL configuration is recommended. |
| Number of UEs per service area | Up to 50 per service area, e.g., 10, 20, 40, and 50 | As in 5G-ACIA LS |
| UE distribution | All UEs randomly distributed within the respective service area. | As in 5G-ACIA LS |
| Message size | 48 bytes | 48 bytes |
| DL traffic model | DL traffic arrival with option-1, option-2, and option-3. | 5G-ACIA Option 1 is mandatory. Companies are also encouraged to provide results for option 3 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 and option-2 | As in 5G-ACIA LS with Option 1 as mandatory |
| CSA requirements | UC-#2: 99.9999% | UC-#2: 99.9999% |
| Performance metrics | 1) CSA: single CDF of CSA distribution of all UEs in factory hall 2) Latency: single CDF of latency distribution of all UEs in factory hall 3) Percentage of UEs satisfying requirements 4) resource utilization | As in 5G-ACIA LS with 3) and 4) as low priority Note: For metric 2) it is clarified that a packet transmission cannot be performed after the latency deadline. The collected statistics cannot exceed the latency requirement. The packets exceeding the deadline are visible in the UE packet error statistics |
| E2E latency & air interface latency | E2E latency: 1 ms for UC#2 | E2E latency: 1 ms for UC#2 Air interface latency: 1ms |
| UE speed | Linear movement | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| BS antenna mount |  | Option 1 (1 sector per BS) from 38.824 is used |

The simulated layout is as follows:

Figure 1: Simulated layout

No synchronization errors have been considered in the simulations.

More detail on companies’ simulation assumptions can be found in each company’s input document which are attached to this report.

# 3	Simulation results

The per company simulation results are summarized in Appendix A. An overall summary is provided in this section. The simulations from different companies have been grouped according to the coordination category as defined in section 2.1.2 and the number of users per service area. As stated in section 2, companies may have simulated different features and coordination schemes with different simulation assumptions. This means that the results are not always directly comparable, but experience shows that even with identical simulation assumptions results will vary.

## 3.1	FR1

### 3.1.1	DL

Table 2: Simulation results for DL in FR1

| Coordination category | Number of users per service area | Number of samples | Percentage of UEs satisfying requirements | Resource utilization % range | Comments |
| --- | --- | --- | --- | --- | --- |
| A | 6.3 – 8.3 | 1 | 80 – 100 |  |  |
|  | 10 | 6 | 86 – 100 | 5.9 - 21.1 |  |
|  | 12 | 1 | 100 |  |  |
|  | 18 | 1 | 100 |  |  |
|  | 20 | 5 | 84 – 100 | 11.4 – 41.6 |  |
|  | 30 | 2 | 97 - 100 | 17.9 – 62.0 |  |
|  | 40 | 2 | 69 – 100 | 26.0 – 29.3 |  |
|  | 50 | 3 | 62 – 100 | 36.6 – 50.5 |  |
|  | 60 | 1 | 86 | 61.7 |  |
| B | 10 | 3 | 100 | 1.7 – 2.2 |  |
|  | 12 | 1 | 100 |  |  |
|  | 20 | 3 | 97.5 - 100 | 3.3 – 4.3 |  |
|  | 22 | 1 | 100 |  |  |
|  | 40 | 2 | 69 - 100 | 7.3 – 7.5 |  |
| C | 10 | 3 | 100 | 1.8 – 2.2 |  |
|  | 15 | 1 | 100 |  | HARQ with orthogonal ReTx |
|  | 20 | 4 | 100 | 3.7 – 4.7 |  |
|  | 22.7 | 1 | 100 |  | Single-layer SU transmission with cell coordination |
|  | 30 | 1 | 100 | 5.5 |  |
|  | 40 | 3 | 98.4 - 100 | 7.3 – 9.0 |  |
|  | 50 | 1 | 73 | 12.1 |  |
|  | 43.8 - 90.7 | 1 | 100 |  | MU transmission with cell coordination |

### 3.1.2	UL

Table 3: Simulation results for UL in FR1

| Coordination category | Number of users per service area | Number of samples | Percentage of UEs satisfying requirements | Resource utilization % range | Comments |
| --- | --- | --- | --- | --- | --- |
| A | 8.3 – 10.4 | 1 | 96-100 |  |  |
|  | 10 | 4 | 83 - 100 | 6.1 – 14.7 |  |
|  | 12 | 1 | 100 |  |  |
|  | 18 | 1 | 99.8 |  |  |
|  | 20 | 5 | 62 -100 | 3.7 – 29.4 |  |
|  | 30 | 1 | 100 | 22.6 |  |
|  | 40 | 3 | 35 – 99.8 | 29.3 – 58.8 |  |
|  | 50 | 3 | 26 – 100 | 36.6 – 43 |  |
|  | 60 | 1 | 88 | 49.3 |  |
| B | 10 | 3 | 96.7 - 100 | 1.8 |  |
|  | 12 | 1 | 100 |  |  |
|  | 20 | 2 | 94.2 - 100 | 3.7 |  |
|  | 22 | 1 | 100 |  |  |
|  | 40 | 2 | 78 - 84 | 6.6 – 7.3 |  |
| C | 10 | 4 | 98.3 – 100 | 1.8 |  |
|  | 15 | 1 | 100 |  |  |
|  | 20 | 4 | 97.5 - 100 | 3.7 |  |
|  | 22.7 | 1 | 100 |  | Single-layer SU transmission with cell coordination |
|  | 30 | 1 | 100 | 5.5 |  |
|  | 40 | 4 | 96.7 - 100 | 7.3 – 7.4 |  |
|  | 50 | 1 | 99 | 9.23 |  |

## 3.2	FR2

### 3.2.1	DL

Table 4: Simulation results for DL in FR2

| Coordination category | Number of users per service area | Number of samples | Percentage of UEs satisfying requirements | Resource utilization % range | Comments |
| --- | --- | --- | --- | --- | --- |
| A | 10 | 1 | 94.2 |  |  |
|  | 20 | 2 | 83 – 95.4 |  |  |
|  | 28 | 1 | 90.5 |  |  |
|  | 40 | 1 |  | 20.3 |  |
|  | 50 | 1 | 99.6 | 25.4 |  |
| B | 10 | 2 | 92.5 - 100 | 2.2 – 2.6 |  |
|  | 20 | 1 | 95.8 | 3.5 |  |
| C | 10 | 3 | 100 | 1.3 – 2.6 |  |
|  | 20 | 3 | 100 | 2.8 – 4.3 |  |
|  | 30 | 1 | 100 | 4 |  |
|  | 40 | 3 | 96.5 – 100 | 5.3 – 9.1 |  |
|  | 50 | 3 | 69 - 100 | 6.6 – 14.3 |  |
|  | 60 | 1 | 98.7 | 13.1 |  |
|  | 70 | 1 | 96.3 | 17.6 |  |

### 3.2.2	UL

Table 5: Simulation results for UL in FR2

| Coordination category | Number of users per service area | Number of samples | Percentage of UEs satisfying requirements | Resource utilization % range | Comments |
| --- | --- | --- | --- | --- | --- |
| A | 10 | 1 | 98.6 |  |  |
|  | 20 | 1 | 90.5 |  |  |
|  | 40 | 1 |  | 38.5 |  |
|  | 50 | 1 | 97.6 | 48.4 |  |
| B | 10 | 2 | 96.7 - 100 | 2.4 |  |
|  | 20 | 1 | 97.5 | 4.7 |  |
| C | 10 | 3 | 98.3 - 100 | 1.1 – 2.4 |  |
|  | 20 | 2 | 98.3 – 100 | 2.3 – 4.7 |  |
|  | 30 | 1 | 100 | 3.4 |  |
|  | 40 | 1 | 100 | 4.5 |  |
|  | 50 | 1 | 100 | 5.5 |  |
|  | 60 | 1 | 100 | 6.8 |  |
|  | 70 | 1 | 99.8 | 8 |  |

# 4 	Observations

The CSA for 1ms survival time and latency have evaluated. Companies performed simulations at 4GHz with 100 MHz bandwidth and at 30 GHz with 160 MHz bandwidth.

We make the following observations from the results:

- For both FR1 and FR2, the evaluations show that with the agreed simulation assumptions NR supports achieving close to 100 % of UEs satisfying the requirements for a relatively large number of users for both uncoordinated and coordinated deployments

- Simulation results from different companies vary due to different simulation assumptions

Coordination improves both communication service availability and resource utilization

# References

RP-201279, “LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation”, 5G-ACIA

RP-202069, “Way forward on RAN work for 5G ACIA requested simulations“, Ericsson

“Simulation Results for 5G-ACIA (Second round)”, CATT

“Simulation Results for 5G-ACIA (Second round)”, Ericsson

“Simulation results for 5G-ACIA in the second round”, Huawei, HiSilicon

“5G-ACIA LS – Phase 3 input”, Intel Corporation

“Results for 5G-ACIA Performance Evaluation Round 2”, ITRI

“Final round of simulation results for 5G-ACIA evaluation”, Nokia, Nokia Shanghai Bell

“ FR1 simulation results for 5G-ACIA  performance evaluation”, Qualcomm CDMA Technologies.

“Simulation Assumptions and URLLC Performance Evaluations for 5G-ACIA Performance Evaluation Round 1(FR2)”, Qualcomm CDMA Technologies

“5G-ACIA 2nd round URLLC evaluation results”, vivo

“5G-ACIA evaluations - 2nd round of simulation results”, ZTE

# Appendix A	Company simulation results

## A.1	CATT[3]

| Frequency Range / Category / Additional information | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A (Target BLER = 1e-3) | 10 | 100 |  |  |  |
|  | 20 | 100 |  |  |  |
| A (Target BLER = 1e-5) | 10 | 85.83 |  |  |  |

## A.2	Ericsson[4]

| Category / Additional information | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A | 10 | 90.71 |  |  |  |
|  | 20 | 84.05 |  |  |  |
| FR1 UL |  |  |  |  |  |
| A | 10 | 100 |  |  |  |
|  | 20 | 100 |  |  |  |
| FR2 DL |  |  |  |  |  |
| A | 10 | 94.17 |  |  |  |
|  | 20 | 83.17 |  |  |  |
| FR2 UL |  |  |  |  |  |
| A | 10 | 98.56 |  |  |  |
|  | 20 | 90.48 |  |  |  |

## A.3	Huawei/HiSilicon[5]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| B | 22 | 100 |  | 100% |  |
| A | 4.2 | 100 |  |  |  |
|  | 6.3 | 100 |  |  |  |
|  | 8.3 | 80 |  |  |  |
| C (Single-layer SU transmission with cell coordination) | 22.7 | 100 |  | 100% |  |
| C (MU transmission with cell coordination) | 43.8 | 100 |  |  |  |
|  | 45.3 | 100 |  |  |  |
|  | 75.0 | 100 |  |  |  |
|  | 83.3 | 100 |  |  |  |
|  | 43.8 | 100 |  |  |  |
|  | 90.7 | 100 |  |  |  |
| FR1 UL |  |  |  |  |  |
| B (Orthogonal frequency reuse among TRPs) | 22 | 100 |  | 100% |  |
| A | 4.2 | 100 |  |  |  |
|  | 6.3 | 100 |  |  |  |
|  | 8.3 | 100 |  |  |  |
|  | 10.4 | 96 |  |  |  |
| C Single-layer SU transmission with cell coordination | 22.7 | 100 |  | 100% |  |

## A.4	Intel[6]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A | 10 | 99.9 | 21.09 |  |  |
|  | 20 | 98 | 41.63 |  |  |
|  | 30 | 87 | 62.04 |  |  |
| FR1 UL |  |  |  |  |  |
| A | 10 | 100 | 14.71 |  |  |
|  | 20 | 99.9 | 29.41 |  |  |
|  | 40 | 80 | 58.82 |  |  |

## A.5	ITRI[7]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A | 10 | 91.67 | 7.326 |  |  |
|  | 20 | 87.08 | 14.652 |  |  |
|  | 40 | 68.75 | 29.303 |  |  |
|  | 50 | 62.17 | 36.629 |  |  |
| A (1 PRB) | 40 | 71.041 | 7.325 |  |  |
| B | 10 | 100 | 1.83131 |  |  |
|  | 20 | 100 | 3.66261 |  |  |
|  | 40 | 100 | 7.3253 |  |  |
| FR1 UL |  |  |  |  |  |
| A | 10 | 83.33 | 7.326 |  |  |
|  | 20 | 62.08 | 14.652 |  |  |
|  | 40 | 35 | 29.304 |  |  |
|  | 50 | 25.5 | 36.63 |  |  |
| A (1 PRB) | 40 | 56.875 | 7.3257 |  |  |
| B | 10 | 98.333 | 1.83141 |  |  |
|  | 20 | 94.1667 | 3.66284 |  |  |
|  | 40 | 84.167 | 7.32569 |  |  |

## A.6	Nokia, Nokia Shanghai Bell[8]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A | 50 | 100 | 50.5 |  |  |
|  | 60 | 86.5 | 61.7 |  |  |
| FR1 UL |  |  |  |  |  |
| A | 50 | 99.96 | 38.9 |  |  |
|  | 60 | 87.8 | 49.3 |  |  |
| FR2 DL |  |  |  |  |  |
| Nokia | 40 |  | 20.3 |  |  |
|  | 50 | 99.6 | 25.4 |  |  |
| FR2 UL |  |  |  |  |  |
| Nokia | 40 |  | 38.5 |  |  |
|  | 50 | 97.6 | 48.4 |  |  |

## A.7	Qualcomm[9][10]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A (One shot, packet survival time = 0) | 5 | 100 |  |  |  |
|  | 12 | 98.75 |  |  |  |
| A (One shot, packet survival time = 1) | 12 | 100 |  |  |  |
|  | 18 | 100 |  |  |  |
| B (One shot, packet survival time = 1) | 12 | 100 |  |  | Similar to the above |
| C ( HARQ, packet survival time = 0) | 15 | 100 |  |  |  |
|  | 20 | 100 |  |  |  |
| FR1 UL |  |  |  |  |  |
| A (One shot, packet survival time = 0) | 5 | 100 |  |  |  |
|  | 12 | 98 |  |  |  |
| A (One shot, packet survival time = 1) | 12 | 100 |  |  |  |
|  | 18 | 99.75 |  |  |  |
| B (One shot, packet survival time = 0) | 12 | 100 |  |  | Similar to the above |
| C ( HARQ, packet survival time = 0) | 15 | 100 |  |  |  |
| FR2 DL |  |  |  |  |  |
| A | 20 | 95.42 |  |  |  |
|  | 28 | 90.48 |  |  |  |

## A.8	vivo[11]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| A | 10 | 100 | 5.9 |  |  |
|  | 20 | 100 | 11.4 |  |  |
|  | 30 | 100 | 17.9 |  |  |
|  | 40 | 100 | 26.0 |  |  |
|  | 50 | 100 | 40.0 |  |  |
| C | 10 | 100 | 1.8 | 100% |  |
|  | 20 | 100 | 3.7 |  |  |
|  | 30 | 100 | 5.5 |  |  |
|  | 40 | 100 | 7.3 |  |  |
| FR1 UL |  |  |  |  |  |
| A | 10 | 100 | 6.1 |  |  |
|  | 20 | 100 | 13.1 |  |  |
|  | 30 | 100 | 22.6 |  |  |
|  | 40 | 99.8 | 30.0 |  |  |
|  | 50 | 99.7 | 43.0 |  |  |
| C | 10 | 100 | 1.8 | 100% |  |
|  | 20 | 100 | 3.7 |  |  |
|  | 30 | 100 | 5.5 |  |  |
|  | 40 | 100 | 7.3 |  |  |
| FR2 DL |  |  |  |  |  |
| C | 10 | 100 | 1.3 |  |  |
|  | 20 | 100 | 2.8 |  |  |
|  | 30 | 100 | 4.0 |  |  |
|  | 40 | 100 | 5.3 |  |  |
|  | 50 | 100 | 6.6 |  |  |
|  | 60 | 97.8 | 13.1 |  |  |
|  | 70 | 96.3 | 17.6 |  |  |
| FR2 UL |  |  |  |  |  |
| C | 10 | 100 | 1.1 |  |  |
|  | 20 | 100 | 2.3 |  |  |
|  | 30 | 100 | 3.4 |  |  |
|  | 40 | 100 | 4.5 |  |  |
|  | 50 | 100 | 5.5 |  |  |
|  | 60 | 100 | 6.8 |  |  |
|  | 70 | 99.8 | 8.0 |  |  |

## A.9	ZTE[12]

| Category / (Additional information) | Number of Users per service area | Percentage of UEs satisfying requirements | Resource utilization % | CSA CDF | Latency CDF |
| --- | --- | --- | --- | --- | --- |
| FR1 DL |  |  |  |  |  |
| B (1e-3 target BLER) | 10 | 100 | 1.72 | 100% |  |
|  | 20 | 100 | 3.34 | 100% |  |
|  | 40 | 68.75 | 7.48 | 100% |  |
| C (1e-3 target BLER) | 10 | 100 | 2.22 | 100% |  |
|  | 20 | 100 | 4.68 | 100% |  |
|  | 40 | 100 | 9.01 | 100% |  |
|  | 50 | 72.76 | 12.07 | 100% |  |
| B (1e-6 target BLER) | 10 | 100 | 2.21 | 100% |  |
|  | 20 | 97.5 | 4.29 |  |  |
| C (1e-6 target BLER) | 10 | 100 | 2.22 | 100% |  |
|  | 20 | 100 | 4.29 |  |  |
|  | 40 | 98.44 | 9.01 |  |  |
| FR1 UL |  |  |  |  |  |
| B (1e-3 target BLER) | 10 | 100 | 1.84 | 100% |  |
|  | 20 | 100 | 3.68 | 100% |  |
|  | 40 | 78.33 | 6.57 | 100% |  |
| C (1e-3 target BLER) | 10 | 100 | 1.84 | 100% |  |
|  | 20 | 100 | 3.68 | 100% |  |
|  | 40 | 100 | 7.35 | 100% |  |
|  | 50 | 98.67 | 9.23 | 100% |  |
| B (1e-6 target BLER) | 10 | 96.67 | 1.84 |  |  |
| C (1e-6 target BLER) | 10 | 98.33 | 1.84 | 100% |  |
|  | 20 | 97.5 | 3.68 | 100% |  |
|  | 40 | 96.67 | 7.35 |  |  |
| FR2 DL |  |  |  |  |  |

| B (1e-3 target BLER) | 10 | 100 | 2.24 | 100% |  |
| --- | --- | --- | --- | --- | --- |
|  | 20 | 95.83 | 3.54 |  |  |
| C (1e-3 target BLER) | 10 | 100 | 2.06 | 100% |  |
|  | 20 | 100 | 3.41 | 100% |  |
|  | 40 | 99.58 | 7.89 | 100% |  |
|  | 50 | 93.33 | 12.12 |  |  |
| B (1e-6 target BLER) | 10 | 92.50 | 2.57 |  |  |
| C (1e-6 target BLER) | 10 | 100 | 2.60 | 100% |  |
|  | 20 | 100 | 4.29 | 100% |  |
|  | 40 | 96.46 | 9.13 | 100% |  |
|  | 50 | 69.33 | 14.31 |  |  |
| FR2 UL |  |  |  |  |  |

| B (1e-3 target BLER) | 10 | 100 | 2.35 | 100% |  |
| --- | --- | --- | --- | --- | --- |
|  | 20 | 97.5 | 4.72 |  |  |
| C (1e-3 target BLER) | 10 | 100 | 2.35 | 100% |  |
|  | 20 | 98.33 | 4.72 |  |  |
| B (1e-6 target BLER) | 10 | 96.67 | 2.35 |  |  |
| C (1e-6 target BLER) | 10 | 98.33 | 2.35 |  |  |