3GPP TSG RAN meeting #91e										RP-210534

Electronic Meeting, March 16-26, 2021

Status Report to TSG

Agenda item:			10.7.1

| WI / SI Name | Additional enhancements for NB-IoT and LTE-MTC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NB_IOTenh4_LTE_eMTC6 |  |  |  |  |
| Unique ID | 860044 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201306 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 35% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG 1 |
| --- | --- | --- |
| Rapporteur | Name | Yubo YANG Emre YAVUZ |
|  | Company | Huawei Ericsson |
|  | Email | yangyubo1@huawei.com emre.yavuz@ericsson.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The TU is updated to align with the stage-3 freeze target of Rel-17 in 03/2022.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

In RAN1#104-e meeting, 31 contributions ([1-31]) were submitted, and the following agreements were achieved.

For NB-IoT 16-QAM:

Working Assumption

The previous working assumption on the following TBS indices for downlink is updated with following modifications:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS for I_SF > 7

Agreement

I_SF>7 is not supported in Rel-17.

Agreement

Confirm the following working assumption:

- The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Agreement

DL 16-QAM is applicable for NPDSCH scheduled from a DCI with CRC scrambled by C-RNTI.

- At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS.

- FFS: Applicability of 16-QAM for PUR.

Agreement

Repetition is not used for 16-QAM in uplink.

Agreement

UL 16-QAM is applicable for NPUSCH scheduled from a DCI with CRC scrambled by C-RNTI.

- At least C-RNTI from USS is supported, FFS if 16-QAM is applied to C-RNTI from CSS.

- FFS: Applicability of 16-QAM for PUR or EDT.

Agreement

The soft buffer size for Cat. NB2 UEs supporting 16QAM for downlink is 12800 bits.

Agreement

The following working assumption is confirmed with following modifications:

- For inband deployments, the downlink TBS entries between 11 (TBS of 2024 for I_SF=7) and [17] are used for 16QAM.

Agreement

Repetition of 2 is NOT supported for 16-QAM in downlink.

Agreement

On the breaking point between QPSK and 16QAM for NPUSCH, the UL TBS entries only between 14 and 21 are used for 16QAM if 16QAM is configured.

Agreement

16-QAM can be used for 3 and 6 subcarriers NPUSCH format 1

Agreement

The NPDSCH EPRE in symbols with NRS can be different and can be the same with the NPDSCH EPRE in symbols without CRS and NRS.

- FFS on signaling details

- FFS for the handling on whether the PCI is different or the same

For support of additional PDSCH scheduling delay for introduction of 14-HARQ processes for eMTC:

Agreement

The PDSCH scheduling delay for the PUCCH non-repetition case (i.e., PUCCH repetitions = 1):

- 2 BL/CE DL subframes.

- The PDSCH scheduling delay of 7 is expressed as:

  - 1 BL/CE DL subframe + 1 subframe + [3 subframes] + 1 subframe + 1 BL/CE DL subframe.

  - 1 subframe + [3 subframes] + 1 subframe + 2 BL/CE DL subframes.

Agreement

For the 14 HARQ processes feature, when PUCCH is used with 1 repetition and there is presence of non-BL/CE UL subframes (i.e., invalid UL subframes):

- The term surrounded by brackets in Solution 1 is resolved as 3 BL/CE UL subframes.

For support of a maximum DL TBS of 1736 bits:

Agreement

The number of soft channel bits is calculated based on the equation:

Working Assumption: N=8

Conclusion

“NOTE: It is RAN1 assumption that 1736 DL TBS feature is compatible with all other eMTC features applicable for HD-FDD Cat. M1 UEs in CE mode A. It is assumed that there’s no change to DCI formats, TBS tables and CQI tables.”

Agreement

The 1736 bits DL TBS feature is enabled by unicast RRC configuration.

Agreement

For a UE configured with “1736 bits DL TBS” and 64-QAM:

- If the UE is signaled with a TBS of up to and including 1736 bits, the UE shall apply the signaled TBS.

- If the UE is signaled with a TBS of greater than 1736 bits, the UE shall apply a TBS of 1736 bits.

2.1.2	Remaining Open issues

  - Confirmation of TBS tables and breaking points for DL, details on DCI design, and FFSs on application of 16-QAM. [NB-IoT]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT]

  - Other aspects to support 16-QAM in DL, including details of signaling of the power allocation for NPDSCH for 16-QAM. [NB-IoT]

  - Details to support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs, including DCI design, HARQ-ACK delay etc. [LTE-MTC]

  - Confirmation of the soft channel bits to support a maximum DL TBS of 1736 bits. [LTE-MTC]

2.2	RAN2

2.2.1	Agreements

Contributions [32]– [61] were submitted to RAN2#113-e meeting. The list of agreements made in the meeting is captured in [62].

NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

RAN2 discussed NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF and made the following agreements:

RAN2#113-e agreements: Neighbour cells measurement (detection and measurements) are performed only on the anchor carrier. The solution is optional R2-2102165 LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state LS_Out Rel-17 To: RAN4. NB_IOTenh4_LTE_eMTC6-Core

NB-IoT carrier selection based on the coverage level and associated carrier specific configuration

RAN2 discussed NB-IoT carrier selection based on the coverage level and associated carrier specific configuration and made the following agreements.

RAN2#113-e agreements: Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced.

RAN2 agreed to have an email discussion until the next meeting

[Post113-e][xxx][NBIOT/eMTC R17] Paging carrier selection (Huawei)

Scope: Details and pros and cons of the 2 options.

Intended outcome: Report to the next meeting.

Deadline: long

NB-IoT: 16-QAM for unicast in UL and DL

No discussion.

eMTC: 14-HARQ processes in DL, for HD-FDD Cat M1 UEs

No discussion.

2.2.2	Remaining Open issues

- For NB-IoT, support of NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

- For NB-IoT, support of NB-IoT carrier selection based on the coverage level and associated carrier specific configuration

- For NB-IoT, RAN2 aspects of support of 16-QAM.

- For eMTC, RAN2 aspects of support of additional PDSCH scheduling delay to support 14-HARQ processes in DL.

- For eMTC, RAN2 aspects of support of DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A.

2.3	RAN3

In RAN3#111-e meeting, 1 contribution [63] were submitted to provide the updated work plan and noted.

2.3.1	Agreements

There was no other discussion in RAN3.

2.3.2	Remaining Open issues

- Support of NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.)

2.4	RAN4

In RAN4#98-e meeting, 1 contribution [64] were submitted to provide the updated work plan.

2.4.1	Agreements

There was no discussion in RAN4.

2.4.2	Remaining Open issues

- For NB-IoT, specify 16-QAM for unicast in UL and DL, including to extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT]

- For NB-IoT, specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT]

- For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC]

- Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements. [NB-IoT][LTE-MTC]

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2101280	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC	Huawei, Ericsson

- R1-2100253	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2100507	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2100567	Discussion on UL and DL 16QAM for NB-IoT	ZTE

- R1-2100581	Consideration on CQI report and Repetition applicability for 16QAM in R17	MediaTek Inc.

- R1-2100762	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2101324	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

- R1-2101509	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2101698	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2101868	Feature lead summary #1 on 104-e-LTE-Rel17_NB_IoT_eMTC-01	Moderator (Huawei)

- R1-2102030	Feature lead summary #2 on 104-e-LTE-Rel17_NB_IoT_eMTC-01	Moderator (Huawei)

- R1-2100254	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

- R1-2100508	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

- R1-2100568	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC		ZTE

- R1-2101325	Design considerations to support 14-HARQ Feature for LTE-M	Sierra Wireless, S.A.

- R1-2101510	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

- R1-2101699	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson, AT&T, SoftBank, Telefónica, Verizon

- R1-2101845	Feature Lead Summary [104-e-LTE-Rel17_NB_IoT_eMTC-02] 1st check point	Moderator (Ericsson)

- R1-2101846	Feature Lead Summary [104-e-LTE-Rel17_NB_IoT_eMTC-02] 2nd check point		Moderator (Ericsson)

- R1-2101847	Feature Lead Summary [104-e-LTE-Rel17_NB_IoT_eMTC-02] 3rd check point	Moderator (Ericsson)

- R1-2100255	Support of a max DL TBS of 1736 bits in LTE-MTC	Huawei, HiSilicon

- R1-2100509	Support of a maximum DL TBS of 1736 bits for eMTC	Nokia, Nokia Shanghai Bell

- R1-2100569	DL TBS increase for eMTC	ZTE

- R1-2100869	Support of 1736 bit maximum DL TBS for eMTC	Sony

- R1-2101326	Design considerations to support DL TBS of 1736 bits for LTE-M	Sierra Wireless, S.A.

- R1-2101511	Support of larger TBS for eMTC	Qualcomm Incorporated

- R1-2101700	Support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson

- R1-2101908	Feature Lead Summary [104-e-LTE-Rel17_NB_IoT_eMTC-03] 1st check point	Moderator (Sony)

- R1-2100570	Channel quality report for 16QAM in NB-IoT	ZTE

- R1-2101278	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon

- R1-2101701	Compendium of 16-QAM simulation results in UL and DL for NB-IoT	Ericsson

- R2-2100324	Further considerations on measurement in connected mode	ZTE Corporation, Sanechips

- R2-2100325	draft LS on measurement in connected mode for NB-IoT	ZTE Corporation, Sanechips

- R2-2100326	Paging carriers configuration and selection	ZTE Corporation, Sanechips

- R2-2100512	Paging carrier selection procedure based on CEL	Nokia, Nokia Shanghai Bell

- R2-2100513	Analysis on Re-establishment time components and Solutions for Faster re-establishment	Nokia, Nokia Shanghai Bell

- R2-2100670	Further discussion on the corresponding measurement before RLF	Spreadtrum Communications

- R2-2100671	Further discussion on enhanced paging carrier selection and NPRACH carrier selection	Spreadtrum Communications

- R2-2101043	Neighbour cell measurements in RRC_CONNECTED	Huawei, HiSilicon

- R2-2101044	Paging carrier selection improvements	Huawei, HiSilicon

- R2-2101045	Summary of contributions on Paging carrier selection improvements	Huawei

- R2-2101046	Discussion on 16-QAM for NB-IoT	Huawei, HiSilicon

- R2-2101047	Support of 14 HARQ Processes in DL, for HD-FDD Cat M1 Ues	Huawei, HiSilicon

- R2-2101056	Impact on Static Devices	THALES

- R2-2101113	Neighbor cell measurements triggering before RLF	Lenovo, Motorola Mobility

- R2-2101156	Support for NB-IoT carrier selection based on the coverage level	Qualcomm Incorporated

- R2-2101157	Way forward for connected mode neighbour cell measurement in NB-IoT	Qualcomm Incorporated

- R2-2101329	On the solution for reduction of RLF detection time	Nokia Solutions & Networks (I)

- R2-2101395	NB-IoT carrier selection and configuration based on coverage level	Ericsson

- R2-2101396	Reducing time taken for reestablishment procedures in NB-IoT	Ericsson

- R2-2101397	Summary of NB-IoT AI 9.1.2 neighbor cell measurements before RLF	Ericsson

- R2-2101398	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R2-2101399	draft LS Measurements for Reducing time for RRC Reestablishment	Ericsson

- R2-2101552	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC	Ericsson

- R2-2101836	Measurement before radio link failure	MediaTek Inc.

- R2-2101839	Carrier selection enhancement	MediaTek Inc.

- R2-2102154	"Summary of Email Discussion [AT113-e][304][NBIOT/eMTC R17] 	Neighbour cell measurements before RLF"	Ericsson

- R2-2102155	Summary of [AT113-e][305][NBIOT R17] Paging carrier selection improvements	Huawei

- R2-2102156	[Draft] LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state	Ericsson

- R2-2102163	[Draft] LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state	Ericsson

- R2-2102165	LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state	RAN2

- R2-2102164	RAN2 agreements for Rel-17 NB-IoT and LTE-MTC	Ericsson (Rapporteur)

- R3-210795	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC	Huawei, Ericsson

- R4-2102233	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC  Huawei, Ericsson

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template