3GPP TSG RAN Meeting #91-e	RP-210548

Electronic Meeting, 16 –26, March, 2021

Title: 	About proposal to change configured output power for CA based UL Tx switching

Source: 	Huawei, HiSilicon

Agenda item:	9.4

Document for:	Discussion

Introduction

In RAN4#98-e meeting as well as the earlier RAN4 meetings, the CRs for modification of Pcmax for UL CA based Tx switching were proposed and discussed [1]. There was no conclusion. According to suggestion from RAN4 Chair in RAN4#98-e meeting, it is expected to raise and discuss this issue in RAN#91-e meeting.

In this paper, we would like to share our view on this issue.

Discussion

In Rel-16 FR1 RF enhancement WI, there is one objective to enable UL Tx switching between two carriers based on SUL, UL inter-band CA and EN-DC [2]. In the RAN4 meeting just before targeting completion date of WI, the proponent proposed a new proposal to enable additional 3dB power boosting for CA based UL Tx switching and brought the proposal directly to RAN#88-e meeting [3].

In our view, the proposal was new and not discussed almost during the whole WI stage. It was not aligned with the objective in WID to some extent, because in WID the UL Tx switching should be specified based on the framework of the inter-band uplink CA but the proposal tended to change “power class” concept for inter-band uplink CA. However, in order to timely complete the WI, we provided our compromise.

Finally, in RAN#88-e meeting, thanks to moderator and all the companies’ great efforts, the conclusions were reached and captured in [4]. The agreements in RP-201365 [4] are copied as follows:

For 38.101-1 CR

- Revise the 38.101-1 CR in RP-200879 to add the indication of 3dB power boosting for carrier 2 with 2Tx for transmission in Case 2, i.e., add the newly proposed text from ZTE.

- In the CR, the power boosting for carrier 2 with 2Tx is only applied to PC3 CA, i.e., not applied to PC2 CA (the same per BC power class, i.e. PC3 in this case is applied regardless of transmission in Case 1 or Case 2)

- The SAR issue due to this power boosting can be solved by UE implementation, e.g. P-MPR, with no spec change.

- No RAN1 impact due to this power boosting, and keep the RAN1 CR pack to this RAN plenary as approved.

- For RAN2, the capability to indicate support of power boosting for CA case, and the RRC signaling to indicate whether such power boosting for CA case is allowed will be specified in Q3, while keep the RAN2 CR pack to this RAN plenary as approved. The capability for 3dB power boosting is defined per band combination. No spec change for RAN2 RRC procedures and MAC procedures. Send the LS to RAN2 in this RAN plenary.

- No other RAN4 spec impact due to change introduced related to this power boosting and UL Tx switching between case 1 and case 2

  - No other RAN4 spec impact due to the change introduced related to this power boosting and UL Tx switching between case 1 and case 2 does not preclude companies’ maintenance Cat F CR, which can be agreed based on consensus.

For 38.101-3 CR

- Approve the 38.101-3 CR in RP-200880 without any additional change

We highlighted one bullet by yellow. Our understanding is that there will be no essential change allowed to support this new functionality after RAN#88-e meeting.

But in the following RAN4 meetings after RAN#88-e, companies started to further propose the new changes of Pcmax to support this 3dB power boosting. We copied part of changes in the CR [1] as below.

------------------------------------------------------------------ Part I ---------------------------------------------------------------------

------------------------ Part I -----------------------

------------------------ Part II -----------------------

------------------------ Part II -----------------------

------------------------ Part III -----------------------

------------------------ Part IV for Clause 6.2A.4 in TS 38.101-1 -----------------------

----------------------------------------------------------------- Part IV -------------------------------------------------------------------

Especially regarding change in Part IV, it introduces the new requirement for power control, which could not be viewed as maintenance and seems not aligned with the spirit of the agreement and compromise (highlighted by yellow above) captured in RP-201365.

Besides, the previous RAN plenary agreement is that the maintenance should be agreed based on the consensus.

- Observation 1: the proposed changes of Pcmax for UL CA based Tx switching cannot be viewed as maintenance work.

Furthermore, according to agreement “add the indication of 3dB power boosting for carrier 2 with 2Tx for transmission in Case 2” and “the power boosting for carrier 2 with 2Tx is only applied to PC3 CA, i.e., not applied to PC2 CA (the same per BC power class, i.e. PC3 in this case is applied regardless of transmission in Case 1 or Case 2)”, both UE and network know where and when the additional 3dB power boosting should apply. Thus there is no any mis-understanding between the network and UE.

For UE, in the existing UE RF requirement for this UL CA based Tx switching, there is one sentence in TS 38.101-1, which is read that “… NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power when the capability uplinkTxSwitchingPowerBoosting is present…”, so there has already been the basic UE core requirement specified for the additional 3dB power boosting functionality.

In our view the basic functionality and UE requirement to enable such 3dB power boosting are complete in the existing specifications.

- Observation 2: the basic functionality and UE requirement to enable 3dB additional power boosting for UL CA based Tx switching are complete in the existing RAN2 and RAN4 specifications.

Based on the above two observations, we propose not to continue discussion on the Pcmax proposal for UL CA based Tx switching in Rel-16 anymore. And we should keep the previous RAN plenary agreement for RAN4.

- Proposal: Keep the previous RAN plenary agreement, i.e., no other RAN4 spec impact due to change introduced related to this power boosting and UL Tx switching between case 1 and case 2. There is no need to further change Pcmax for UL CA based Tx switching.

Conclusion

In this paper, we provide the background for the proposal of modification of Pcmax for UL CA based Tx switching, and shared our view on how to handle it. We have the following observations and proposal:

- Observation 1: the proposed changes of Pcmax for UL CA based Tx switching cannot be viewed as maintenance work.

- Observation 2: the basic functionality and UE requirement to enable 3dB additional power boosting for UL CA based Tx switching are complete in the existing RAN2 and RAN4 specifications.

- Proposal: Keep the previous RAN plenary agreement, i.e., no other RAN4 spec impact due to change introduced related to this power boosting and UL Tx switching between case 1 and case 2. There is no need to further change Pcmax for UL CA based Tx switching.

References

- R4-210315, “Modification of Pcmax for UL CA with uplink Tx switching capability”, Ericsson, RAN4 #98-e, 25 January – 5 February, 2021.

- RP-193266, “WID revision: RF requirements for NR frequency range 1 (FR1)”.

- RP-201126, “Remaining issues on uplink Tx switching”, ZTE, Sanechips, RAN#88e, June 29- July 3, 2020.

- RP-201365, “Final Proposal of Switched_UL”, Nokia (moderator), RAN#88e, June 29 - July 3, 2020.