3GPP TSG RAN Meeting #91-e	   	                                                       RP-210601

Electronic Meeting, March 16 - 26, 2021

Source: 	ZTE, Sanechips

Title:	Views on WI scope for Rel-17 RAN slicing enhancement

Agenda item:	9.6.7

Document for:     Discussion

# Introduction

In the RAN2#113-e meeting, RAN2 has finished its study on RAN slicing enhancement with the technical report endorsed in TR 38.832 [1].

Basically, potential techniques for RAN slicing enhancement on the following items were discussed in RAN2.

- Slice based cell selection and reselection under network control

- Slice based RACH configuration

Although the study on RAN slicing enhancement is supposed to be finished in the RAN3#111-e, it has been proposed to extend the SI with one quarter in RAN3 and do down-selection during the extended SI. 0.5 TUs needed. And RAN3 also confirms that extending the RAN3 part of this SI would not impact the start of potential normative work in RAN2.

In this contribution, we provide our views on work item scope with respect to above items for RAN slicing enhancement in Rel-17, focusing on RAN2 part. Understanding on the SI extension in RAN3 is also shared.

# Discussion

## 2.1. Slice based cell (re)selection under network control

In the RAN2#103-e meeting, the following solutions have been recommended for normative work.

- To assist cell reselection, RAN can broadcast the supported slice info of the current cell and neighbor cells, and cell reselection priority per slice - adding the slice info (with similar information as agreed slice info in SI message) in RRC release message

Details for these solutions have been discussed in RAN2 SI phase and have been captured in the TR for broadcasting the supported slice info of the current cell and neighbor cell:

- Broadcast the part of the S-NSSAI, e.g. only the SST

- Define slice grouping and broadcast the supported slice group

- Broadcast the slice associated UAC information, e.g. the operator defined access category

The solutions mentioned above will also be considered to indicate the slice information when the slice specific priority is broadcast. On demand SI and SIB segmentation have also been discussed and captured in the TR as candidate solutions if more SIB payload is required in broadcasting the supported slice info as well as the slice specific reselection priority.

In addition, including similar information showing the supported slice, e.g. the SST, the slice group, the slice associated UAC information in RRCRelease message has also been considered in RAN2 SI phase to assist cell reselection and has already been captured in the TR.

With all these progress made in RAN2 SI phase, we understand the study on slice based cell reselection has been quite stable and clear and we recommend to support provisioning of the supported slice info of the current cell and neighbor cells as well as the cell reselection priority per slice via system information and RRCRelease message to assist cell reselection in the WI phase.

Proposal 1: Support the following enhancement in Rel-17 RAN slicing enhancement work item:

Provision of the supported slice info of the current cell and neighbor cell and cell reselection priority per slice via system information and RRCRelease message to assist cell reselection.

## 2.2. Slice based RACH configuration

For slice based RACH configuration, two techniques are recommended during the study item phase.

- RACH resource isolation per slice or slice group

  - The RACH resource refers to the transmission occasions of time-frequency domain and preambles. Providing ROs or preambles for specific slices would be helpful to ensure resources isolation between slices and reduce the RACH collisions for UE trying to access these slices.

- RACH prioritization per slice or slice group

  - The RACH parameters prioritization (i.e. scalingFactorBI and powerRampingStepHighPriority) can be configured per slice or slice group to prioritize random access procedure to certain slices so that UE fast access to these slices can be enabled.

As captured in the TR, the RACH resources and prioritization can be configured for a group of slices. Defining a new grouping mechanism or reusing UAC access category have been captured as candidate solutions for that with down selection and detailed configuration to be discussed and decided in the WI phase.

With the above progress made in RAN2 SI phase, we understand the study on slice based RACH configuration has been quite clear and stable, thus we recommend to support the RACH resource isolation per slice or slice group and RACH prioritization per slice or slice group in the WI phase.

Proposal 2: Support the following enhancement in Rel-17 RAN slicing enhancement work item:

- RACH resources (i.e. ROs and preambles) isolation per slice or slice group

- RACH prioritization (i.e. scalingFactorBI and powerRampingStepHighPriority) per slice or slice group

## 2.3 SI extension in RAN3

During the study phase, RAN3 investigated the scenarios and solutions on service continuity. However, the system impact of these methods on the OAM and core network requires further evaluation by SA5 and SA2 respectively. Therefore, the down-selection of candidate solution has not completed yet which is pending to the LS reply from SA5 and SA2.

SA5 analyzed the impact of the configuration based solution and replied with LS in SA5#135e. SA2 analyzed the impact of solutions described in TR38.832 in SA2#143e. Solutions with no core network and UE impact are feasible and RAN3 is encouraged to find alternative solutions without or limiting impacts on core network and UE. Those liaison letters are going to be discussed in the RAN3#112e meeting for solution down-selection.

Proposal 3: RAN3 work of FS_NR_Slice needs to be extended to the RAN3#112e and solution down-selection needs to be performed in RAN3#112e based on the reply LS from SA2 and SA5.

# Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: Support the following enhancement in Rel-17 RAN slicing enhancement work item:

Provision of the supported slice info of the current cell and neighbor cell and cell reselection priority per slice via system information and RRCRelease message to assist cell reselection.

Proposal 2: Support the following enhancement in Rel-17 RAN slicing enhancement work item:

- RACH resources (i.e. ROs and preambles) isolation per slice or slice group

- RACH prioritization (i.e. scalingFactorBI and powerRampingStepHighPriority) per slice or slice group

Proposal 3: RAN3 work of FS_NR_Slice needs to be extended to the RAN3#112e and solution down-selection needs to be performed in RAN3#112e based on the reply LS from SA2 and SA5.

# Reference

- TR 38.832 v0.5.0, Study on enhancement of Radio Access Network (RAN) slicing, China Mobile.