3GPP TSG RAN Meeting #91e									        	RP-210757

Electronic Meeting, March 16 - 26, 2021

## Status Report to TSG

Agenda item:			9.7.10

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB_enh |  |  |  |  |
| Unique ID | 860050 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201293 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: 06/2022 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 14% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #104-e

Agreement

Send LS response to RAN3 that both inter-donor multi-parent scenarios (Scenario 1 and Scenario 2) can be supported in Rel-17 with support for inter-donor resource coordination (e.g. DU H/S/NA and DL/UL resource configurations) in RAN3 specification.

- The reply LS to R1-210004 (RAN3) is endorsed in R1-2101880.

Agreement

Support indication/reporting of information between an IAB node and its parent node to assist in the determination of the applicability of a given multiplexing capability in case of simultaneous operation. The following solutions are considered (other solutions not precluded):

- Temporal applicability of a given multiplexing capability

- Time/frequency resource restrictions (e.g. access vs. backhaul links, DL vs. UL resources)

- Indications of conditions/reporting information required to realize the given multiplexing capability, (e.g. timing mode, power control, guard symbols, etc.)

FFS: channels/signals used for indicating/reporting information

Agreement

Further study whether/how to manage resources in the spatial domain. Candidate solutions are:

- Dynamic signaling between parent and child nodes for using/restricting/sharing antenna panels/beams

- Beam management / multi-panel enhancements for simultaneous operations

- Extension of H/S/NA resource indication to the spatial domain

Other solutions are not precluded.

Agreement

Regardless of simultaneous operation, the same cell-specific/semi-static signals and channels of the IAB-DU considered as hard time/frequency resources in Rel-16 are also considered as hard time/frequency resources in Rel-17.

- FFS: IAB-MT behavior in case of conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT (e.g., dedicated slot configurations)

Agreement

The following are considered to support at least inter-band inter-carrier scenarios in Rel-17:

- Solutions to address resource coordination/scheduling collision issues between parent nodes including TDD configurations and resource type indications at least in case of intra-donor CU multi-parent scenarios

  - Consider Rel-16 CA framework as starting point

  - Solutions for scheduling collision between two parent DUs due to indication of the resource availability for soft symbol(s) to the IAB-DU(s) by DCI format 2_5

  - Solutions for scheduling collision between two parent DUs due to indication of the slot format by DCI format 2_0

  - FFS: Whether or not separate solutions are required for resource coordination in case of inter-donor CU multi-parent scenarios

- Per-backhaul link (e.g. per child IAB-MT link) resource configurations in addition to per-DU resource configurations

- FFS: Enhancements to indication of soft resource availability from child node to parent node(s)

- FFS: Additional restrictions on simultaneous operation and/or multiplexing

- FFS: Whether the above solutions are also applicable for intra-band inter-carrier scenarios and whether additional solutions are required (e.g. RAN2 and RAN4 work related to adding band configuration and RRM requirements for intra-band inter-carrier NR-DC or updating related UE/MT capabilities for NR-DC so that they are applicable for intra-band inter-carrier NR-DC)

Agreement

Further consider until RAN1#104bis-e whether to support the extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/[S]/NA resource types, including the following aspects:

- Granularity for frequency domain resources within a carrier (starting point is a set of N RBs with FFS: value of N >=1)

- Relationship with Rel-16 DU resource type indications in case of coexistence between TDM and FDM operation, including time-granularity of switching between multiplexing options to ensure backwards compatibility with Rel-16 IAB nodes and avoid impact on access UEs and their RRC configurations at CU

- In case frequency-domain extension is supported for soft resources, enhancements for DCI format 2_5 to support dynamic indication of availability for soft frequency resources.

  - Alt. 1 Separate indication of time and frequency resources

    - FFS: different field, RNTI or different DCI

  - Alt. 2 Joint indication of time and frequency resources

    - FFS: backwards compatibility with Rel-16

- FFS: Extension of FDM across carriers

- FFS: Restrictions on band/minimum bandwidth for FDM operation (e.g. FR2 100MHz+ etc.)

Agreement

Case 7 timing is supported with symbol level alignment without explicit support for slot level alignment

Agreement

Switching between Case 1, Case 6, and Case 7 timing is supported.

- FFS whether Case 6 and Case 7 timing shall be restricted to certain resources, e.g. excluding resources used for access or TDM backhaul

- FFS details on switching including the switching conditions

- FFS relationship between switching timing modes with the usage/indication of different resource multiplexing modes

- FFS whether Rel-16 OTA synchronization shall be enhanced to support switching timing modes

Agreement

RAN1 to further study whether the legacy UL power control mechanism (including PHR) is sufficient for an IAB-node operating in an enhanced multiplexing mode.

- FFS: if not (i.e., the legacy mechanism is not sufficient), support an IAB-node indicating information to assist with its UL power control.

Agreement

Support an IAB-node indicating information to assist with the DL power control of its parent-node towards the IAB-node without mandating an expected behavior at the parent node.

- Note: At least the assistance information is for supporting the simultaneous operation within the IAB-node to avoid power imbalance

- FFS: type of assistance information (e.g., desired received power, power adjustment, preferred CSI-RS resource)

- FFS: whether this information is provided to the parent-node, the CU, or both.

- FFS: applicability of the assistance information (e.g. relation to beams or multiplexing modes)

- FFS: the channel carrying this assistance information

Conclusion

In Rel-17, RAN1 will not specify specific mechanisms for intra-IAB-node interference (self-interference) management.

- Self-interference can be handled by the implementation or via using the available techniques defined, or to be defined in Rel-17, that can commonly be used for other interference scenarios as well.

Agreement

RAN1 to select among the following options to support DU-to-DU measurement and report.

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM)

  - Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM)

  - Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements)

Agreement

RAN1 to decide whether to enhance interference mitigation through information exchange to support beam-management at the parent or child node in RAN1#104bis-e

  - FFS: reporting of desired beams for reception in DL or desired beams for transmission in UL by the IAB node for a given multiplexing mode

  - FFS: indicating applicable beams in DL or beams in UL for a given multiplexing mode.

#### 2.1.2	Remaining Open issues

30% of the items defined in the RAN1 WID objectives have been accomplished. The following items defined in the WID objectives require further work:

Duplexing enhancements:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #113-e

Agreements on outgoing LS:

LS on DAPS-like solution for service interruption reduction in Rel-17 IAB agreed in R2-2102364.

Agreements on enhancements to improve topology-wide fairness multi-hop latency and congestion mitigation

- RAN2 will not further discuss ways of evaluating success of any fairness mechanisms that may be introduced, beyond the already agreed definition of topology-wide fairness and its variants.

- Chair: On the agreed issues below, the agreement doesn’t mean that we have agreed that there need to be a solution for it in R17. Furthermore, liberal interpretation of the text is ok.

- ISSUES: eIAB work on topology-wide fairness will focus on the following issues

  - IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes)

  - IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement.

  - IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load)

- ISSUES: In the first instance, eIAB work on multi-hop latency will focus on the following issues:

  - IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency

  - IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs

  - IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16

  - IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free

  - IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel

- R2 has concluded that there is sufficient interest among companies to address the following two issues:

  - IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets

  - IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion.

- Both IC-1 and CI-7 are related to RAN3. RAN3 seems to also work on this, so to what extent R2 shall work on this is currently not clear.

Agreements on topology adaptation enhancements:

- RAN2 to discuss CHO and start with intra-donor CHO until RAN3 has made progress on inter-donor IAB-node migration.

- R2 confirm the intention Rel-16 CHO is / can be used for IAB-MT (FFS whether any modification is needed).

- R2 assumes that Rel-16 specification is the baseline for the configuration of default route, IP address(es) and target path for intra-donor CHO.

- RAN2 to support type-2/3 RLF indication (FFS specified behavior(s) TS impact, FFS details).

- Type-2 RLF indication may be used to trigger local rerouting

- Type-2 RLF indication may be used to trigger deactivation of IAB-supported in SIB

- Type-2 RLF indication may be used to trigger deactivation or reduction of SR and/or BSR transmissions

- Local rerouting can be triggered by indication of hop-by-hop flow control. Further details, e.g., on trigger information, trigger conditions, role of CU configuration, are FFS.

- RAN2 considers inter-donor-DU local rerouting to be in scope

#### 2.2.2	Remaining Open issues

10% of the items defined in the RAN2 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #111-e

Agreements on IAB-node procedure details:

- WA: NRDC is supported as a baseline procedure for the IAB-MT’s simultaneous connectivity to two IAB-donors; DAPS-like solution is not precluded

- Liaise RAN2 to discuss use cases, functionality, and protocol stack of DAPS-like solutions for IAB.

- RRC Reestablishment procedure of the migrating (top-level) IAB-MT is BL for inter-donor RLF recovery of a single-connected IAB-node

- For a single-connected IAB-MT:

  - The procedure for inter-donor migration of a (top-level) migrating IAB-MT supports:

    - reuse Xn handover procedure of the (top-level) migrating IAB-MT between two parent nodes connected to different IAB-donors, and

    - the migration of F1 transport path for the collocated and all descendent IAB-DUs (i.e. the anchor nodes for the logical F1 connection do not change)

- Inter-donor migration may terminate after top-level IAB-MT migration

- WA: Migration of collocated IAB-DU after the migration of the (top-level) migrating IAB-MT, is not precluded.

- If collocated IAB-DU is migrated, the Inter-donor migration procedure involves, among others:

  - the establishment of an F1-C association to the target donor, and

  - the context migration of the IAB-DU’s UEs and child IAB-MTs to the target CU.

- Xn signaling for IAB-MT’s migration may include information for the migration of F1 transport to the target path such as new IP addresses and/or default mappings; default mappings are used for F1-C and non-F1; exact XnAP procedure to be used is FFS

- For CU-based IP address allocation:

  - Xn: The following information is needed from source donor CU to target donor CU

    - information about IP address(es) requested for the IAB node (in RRC container)

  - Xn: The following information is needed from target donor CU to source donor CU:

    - IP address(es) allocated to IAB node (in RRC container)

  - F1: The target donor CU may obtain IP address(es) from the target donor DU (current Rel-16 procedure)

- FFS whether target donor may also explicitly signal IP addresses in the Xn message to the source donor-CU

- One common inter-donor topology transport mechanism should be defined for all scenarios where traffic between a donor and an IAB DU traverses the network under another donor; FFS whether it is possible to achieve a common signaling design for all scenarios

- For an MT with simultaneous connectivity to two IAB-donors, per-F1-U tunnel load balancing should be supported

- For an IAB-MT with simultaneous connectivity to two IAB-donors, it should be possible to keep its collocated IAB-DU, all UEs and descendent nodes at donor 1 while routing their F1-U connections via the top-levelmigrating IAB-MT’s link with donor 2.

- When the IAB-node performs RLF recovery via RRC Reestablishment at a new IAB-donor-CU, ongoing F1 transport connections of the IAB-node and its descendent nodes with the original donor may be retained and rerouted via the recovered path

- For the recovery of RLF occurring on one link for an IAB-MT with simultaneous inter-donor connectivity, all traffic can be rerouted to the other path without need for IAB-DU migration.

- LS to RAN2 on DAPS like solution agreed in R3-211326

Agreements on CHO and DAPS:

- Rel-16 CHO is supported for INTRA-donor migration of IAB-MT

- FFS whether the descendant nodes and UEs receive RRC reconfiguration messages before migrating IAB node connects to target path

- RAN3 further studies “DAPS-like” solution after RAN2 has conclusions

Agreement on reduction of service interruption:

- For intra-donor migration:

- Use concurrent TNL migration of all descendant nodes during intra-donor topology adaptation to reduce interruption time.

- Consider the following options to support transferring RRCReconfiguration for descendant IAB over source path

  - Sol1: the RRCReconfiguration for the child IAB is buffered in the parent DU, and it is only sent to the child IAB when a prerequisite step is satisfied/performed.

  - Sol2: the RRCReconfiguration for the child IAB is buffered in the child IAB-MT, and it is only executed when a prerequisite step is satisfied/performed.

  - Sol3: the RRCReconfiguration for the child IAB is not buffered in the parent DU or child IAB-MT, and is executed by the child IAB-MT upon reception.

  - Sol4: by CU proper implementation. CU control the time to send RRCreconfiguration for each descendent IAB-node, the parent node of each IAB-node does not need to buffer their RRCReconfiguration, and each IAB-node can apply the RRCReconfiguration just when receiving it.

- Agree inter-donor-DU re-routing can be used to address UL packet loss. FFS on other enhancement when re-routing cannot address UL packet loss or re-routing is unavailable; FFS on enhancement to address unnecessary DL transmission

- WA: MOBIKE can be used to reduce service interruption during Intra-Donor-CU Inter-Donor-DU Topology Adaptation. FFS whether it affects RAN3 specification.

- LS to SA3 on using MobIKE in IAB system agreed in R3-211297

Agreements on topology redundancy:

CP-UP separation:

- R3-211327 CR to TS 38.423 on CP-UP separation over Xn for Rel-17 IAB endorsed as BL

- R3-211329 CR to TS 38.420 on CP-UP separation over Xn for Rel-17 IAB endorsed as BL

- To support CP-UP separation, the node terminating F1 interface for the IAB-node determines the transfer path of F1-C traffic

Inter-donor topology redundancy:

- LS to RAN2 on inter-donor topology redundancy agreed in R3-211331

- In Rel-17, RAN3 agrees to support the following scenarios for inter-donor topology redundancy with the principle that an IAB-DU only has F1 interface with one Donor-CU:

  - Scenario 1: the IAB node is multi-connected with 2 Donors.

  - Scenario 2: the IAB node’s parent/ancestor node is multi-connected with 2 Donors.

- The F1-terminating donor initiates the traffic offload to the other donor’s topology

- To support the bearer mapping across two topologies at the boundary IAB node, the non-F1-termination donor CU needs to provide the ingress BH RLC CH ID(s) for DL traffic and egress BH RLC CH ID(s) for UL traffic to the F1-termination donor CU.

- The boundary IAB node belongs to two topologies of two donor CUs

- Inform RAN2 to consider the following options for BAP routing across two topologies, i.e.,

  - opt1 OAM based solution

  - opt3 routing via a new unique identity (e.g., extended BAP address with CU component, separate set of (e)LCIDs)

  - opt4 BAP header rewriting based on BAP routing ID at e.g. the boundary node

  - opt5 BAP header rewriting based on IP header at, e.g., the boundary node (seems to also impact RAN2)

- Both F1-termination node and non-F1-termination node can assign IP address(es) to the boundary IAB node

- In inter-donor topology redundancy, the granularities of the load balancing is per TNL association for F1-C traffic.

- The BH RLC channel management for each BH link is controlled by the CU who controls the topology containing the BH link.

Agreements on congestion mitigation:

- The CP-based congestion indication may contain reporting:

  - per BAP routing ID and/or

  - per child link and/or

  - BH RLC CH ID

(downselection is FFS).

- The CP-based congestion indication reuses the F1AP GNB-DU Status Indication procedure.

- The CP-based congestion indication pertains to DL congestion.

- Consider the following two options for the UP-based approach to IAB congestion mitigation:

  - No enhancements;

  - Packet marking-based approach.

- CR to BL TS 38.473 CP-based congestion indication for IAB agreed in R3-211330

Agreements on multi-hop performance:

- To address the potential UL packet discarding problem in inter-donor-DU re-routing case, discuss the following solutions (the case where donor DUs belong to different CUs is not precluded):

  - The target IAB-donor-DU is provided with the source IP address of re-routed packets

  - Suspend/disable the source IP filter in target IAB-donor-DU and transport network node(s)

  - Only allow re-routing among a configured subset of IAB-donor-DUs, where source IP filtering is not activated.

- In the inter-donor-DU re-routing case, the issue 2, i.e. how to achieve BAP routing towards the target donor DU for re-routed packets: wait for RAN2 progress

- LS to RAN2 on BAP routing for inter-donor-DU rerouting agreed in  R3-211298

#### 2.3.2	Remaining Open issues

15% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.4	RAN4

#### 2.4.1	Agreements

None. Work has not yet started.

#### 2.4.2	Remaining Open issues

All objectives from WID on enhancements to IAB for NR.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #104-e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2100219 | Resource multiplexing between backhaul and access for IAB duplexing enhancements | Huawei, HiSilicon |
| R1-2100220 | Enhancements for simultaneous operation of MT and DU | Huawei, HiSilicon |
| R1-2100221 | Other enhancements for IAB | Huawei, HiSilicon |
| R1-2100463 | Enhancement to resource multiplexing between child and parent links | vivo |
| R1-2100464 | Other enhancements for simultaneous operation of child and parent links | vivo |
| R1-2100465 | Other enhancements to Rel-17 IAB nodes | vivo |
| R1-2100670 | Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node | Intel Corporation |
| R1-2100671 | Other Enhancements for Simultaneous Operations | Intel Corporation |
| R1-2100690 | Discussions on enhancements to resource multiplexing between child and parent links of an IAB node | CEWiT |
| R1-2100717 | Discussions on IAB resource multiplexing enhancements | LG Electronics |
| R1-2100718 | Discussions on other enhancement for simultaneous operation | LG Electronics |
| R1-2100744 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | Fujitsu |
| R1-2100777 | Enhancements for IAB resource multiplexing | AT&T |
| R1-2100778 | Enhancements for IAB interference management | AT&T |
| R1-2100833 | Enhancements for resource multiplexing among IAB backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-2100834 | Other enhancements for simultaneous operation of IAB Parent and Child | Nokia, Nokia Shanghai Bell |
| R1-2100955 | Discussion on simultaneous operation of IAB-node’s child and parent links | CEWiT |
| R1-2100958 | Enhancements to the IAB resource multiplexing | ZTE, Sanechips |
| R1-2100959 | Enhancements for simultaneous operation of child and parent links | ZTE, Sanechips |
| R1-2100960 | Discussion on multiplexing capability for IAB | ZTE, Sanechips |
| R1-2100990 | Resource multiplexing in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2100991 | Timing, interference, and power control in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2101083 | Discussion on IAB resource multiplexing enhancements | ETRI |
| R1-2101084 | Discussion on simultaneous operation enhancements for eIAB | ETRI |
| R1-2101085 | Other enhancements to eIAB | ETRI |
| R1-2101227 | Enhancements to Resource Multiplexing for NR IAB | Samsung |
| R1-2101228 | Enhancements to Timing, Power Control and CLI for NR IAB | Samsung |
| R1-2101482 | Workplan for Rel-17 IAB | Qualcomm Incorporated |
| R1-2101483 | Resource management for enhanced duplexing | Qualcomm Incorporated |
| R1-2101484 | On enhancements for simultaneous operation of IAB-node's child and parent links | Qualcomm Incorporated |
| R1-2101628 | Resource multiplexing between child and parent links of an IAB node | NTT DOCOMO, INC. |
| R1-2101629 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | NTT DOCOMO, INC. |
| R1-2101695 | Resource multiplexing and DC in enhanced IAB | Ericsson |
| R1-2101696 | Timing, CLI and power control in enhanced IAB | Ericsson |
| R1-2101697 | Response to RAN3 LS on inter-donor topology redundancy | Ericsson |
| R1-2101810 | Enhancements for IAB interference management | AT&T |
| R1-2101855 | Summary #1 of [104-e-NR-eIAB-01] | Moderator (AT&T) |
| R1-2101878 | Summary #1 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2101880 | Reply LS on inter-donor topology redundancy | RAN1, Samsung |
| R1-2101889 | Summary of [104-e-NR-eIAB-01] – 1st Checkpoint | Moderator (AT&T) |
| R1-2101890 | Summary of [104-e-NR-eIAB-01] – 2nd Checkpoint | Moderator (AT&T) |
| R1-2101891 | Summary of [104-e-NR-eIAB-01] – 3rd Checkpoint | Moderator (AT&T) |
| R1-2101909 | Summary #2 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2102031 | Summary #3 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2102172 | Summary #4 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links | Moderator (Qualcomm) |
| R1-2102196 | Session notes for 8.10 (Enhancements to Integrated Access and Backhaul) | Ad-hoc Chair (Samsung) |

RAN2 #113-e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2100038 | LS on DAPS-like solution for service interruption reduction in Rel-17 IAB (R3-207184; contact: Samsung) | RAN3 |
| R2-2100040 | LS on CP-UP separation of Rel-17 IAB (R3-207198; contact: Samsung) | RAN3 |
| R2-2100041 | LS on inter-donor topology redundancy (R3-207199; contact: Samsung) | RAN3 |
| R2-2100225 | Consideration on topology-wide fairness, multi-hop latency and congestion mitigation | CATT |
| R2-2100226 | CHO and DAPS | CATT |
| R2-2100227 | RLF Indication and Local Rerouting | CATT |
| R2-2100358 | Discussion on Topology-wide fairness, latency and flow control enhancement | Intel Corporation |
| R2-2100359 | Discussion on Topology adaptation enhancements | Intel Corporation |
| R2-2100360 | Discussion on RAN3 LS of DAPS-like solution | Intel Corporation |
| R2-2100477 | Discussion on congestion, RLF and fairness handling | vivo |
| R2-2100478 | On inter-CU Topology Adaptation Enhancements | vivo |
| R2-2100479 | Duplexing enhancements of inter-carrier DC | vivo |
| R2-2100591 | Updated workplan for Rel-17 IAB | Qualcomm Incorporated (WI Rapporteur) |
| R2-2100592 | Report from email discussion [Post112-e][066][eIAB] Topology Adaptation | Qualcomm Incorporated |
| R2-2100593 | Simulations on fairness support in IAB topology | Qualcomm Incorporated |
| R2-2100594 | Enhancements to improve IAB multi-hop latency | Qualcomm Incorporated |
| R2-2100595 | Inter-donor-DU local rerouting for IAB | Qualcomm Incorporated |
| R2-2100611 | Re-routing enhancements in IAB | Nokia, Nokia Shanghai Bell |
| R2-2100612 | On CP_UP split for topology adapation enhancements | Nokia, Nokia Shanghai Bell |
| R2-2100708 | IAB fairness scheduling | NEC |
| R2-2100752 | Discussion on the fairness enforcement and congestion mitigation for IAB | Fujitsu |
| R2-2100753 | Consideration on multi-hop latency in IAB | Fujitsu |
| R2-2100754 | Handling of descendant nodes and UEs in inter-CU CHO and RLF recovery | Fujitsu |
| R2-2100801 | Consideration of topology-wide fairness and multi-hop latency enhancements for eIAB | Kyocera |
| R2-2100802 | Further consideration of topology adaptation enhancements for eIAB | Kyocera |
| R2-2100824 | An elaboration of required PDB for multi-hop latency | ITRI |
| R2-2100885 | Solutions to ensure fairness, latency bounds and mitigation of congestion impacts in eIAB Networks | Apple |
| R2-2100886 | Discussion on topology adaptation enhancements in eIAB Networks | Apple |
| R2-2100902 | Topology-wide fairness and Latency enhancements and congestion mitigation | Sony |
| R2-2100903 | Topology adaptation enhancements in IAB | Sony |
| R2-2101070 | Enhancements for topology-wide fairness, multi-hop latency and congestion mitigation | Huawei, HiSilicon |
| R2-2101071 | Consideration of topology adaptation enhancement for R17-IAB | Huawei, HiSilicon |
| R2-2101072 | Duplexing enhancements for R17 IAB | Huawei, HiSilicon |
| R2-2101086 | Fairness, latency and congestion – solutions | Samsung Electronics GmbH |
| R2-2101100 | Views on duplexing enhancements | Samsung Electronics GmbH |
| R2-2101109 | CHO in IAB system | Lenovo, Motorola Mobility |
| R2-2101142 | Discussion on IAB packet rerouting | Lenovo, Motorola Mobility |
| R2-2101168 | Report from email discussion [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung) | Samsung Electronics GmbH |
| R2-2101202 | Hop-by-hop flow control in uplink | Nokia, Nokia Shanghai Bell |
| R2-2101208 | Discussion on RLF indication enhancement and local routing for R17-IAB | CANON Research Centre France |
| R2-2101260 | Multi-hop scheduling and local routing enhancements for IAB | AT&T |
| R2-2101261 | Topology adaptation enhancements for IAB | AT&T |
| R2-2101262 | Duplexing enhancements for IAB | AT&T |
| R2-2101282 | Discussion on CP/UP separation | ZTE, Sanechips |
| R2-2101283 | Considerations on topology adaptation enhancements in IAB | ZTE, Sanechips |
| R2-2101284 | Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation | ZTE, Sanechips |
| R2-2101314 | On multi-hop latency, fairness and congestion mitigation | InterDigital |
| R2-2101315 | On IAB Topology Adaptation | InterDigital |
| R2-2101448 | On Topology-wide Fairness, Multi-hop Latency and Congestion Mitigation | Ericsson |
| R2-2101449 | On IAB Inter-donor Topology Adaptation | Ericsson |
| R2-2101450 | LS on DAPS-like solution for service interruption reduction | Ericsson |
| R2-2101502 | Consideration on identified issues for fairness, latency and congestion | LG Electronics |
| R2-2101503 | Consideration on local re-routing | LG Electronics |
| R2-2101514 | BH RLF indications with conditional mobility and local re-routing | LG Electronics |
| R2-2101766 | Discussion on Resource Reservation for CHO | ETRI |
| R2-2101798 | RAN2 impacts of Rel.17 IAB topology adaptation enhancements | Futurewei Technologies |
| R2-2101820 | Rel. 17 IAB enhancements for fairness, multi-hop latency reduction, and congestion mitigation | Futurewei Technologies |
| R2-2101905 | Issues on UL RLF notification and CP-UP separation | Samsung R&D Institute UK |
| R2-2102238 | Report from email discussion [Post112-e][066][eIAB] Topology Adaptation | Qualcomm Incorporated |
| R2-2102288 | Summary of [AT113-e][030][eIAB] Reply LS DAPS-like solution (Ericsson) | Ericsson |
| R2-2102364 | Reply LS on DAPS-like solution for service interruption reduction | RAN2 |

RAN3 #111-e

| TDoc | Title | Source |
| --- | --- | --- |
| R3-210100 | Inter IAB donor-CU topology adaptation | CATT |
| R3-210101 | CHO and DAPS for IAB | CATT |
| R3-210102 | Reducing the Service Interruption for IAB | CATT |
| R3-210103 | Inter-donor-DU local re-routing in IAB | CATT |
| R3-210207 | Further considerations on inter-donor IAB Node Migration procedure | ZTE |
| R3-210216 | Discussion on inter-donor migration procedure for Rel-17 IAB | Samsung |
| R3-210217 | Discussion on service interruption reduction for Rel-17 IAB | Samsung |
| R3-210218 | Discussion on CP-UP separation and inter-donor topology redundancy | Samsung |
| R3-210219 | CR on CP-UP separation over Xn for Rel-17 IAB | Samsung, Nokia, Nokia Shanghai Bell, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics, Ericsson |
| R3-210220 | Discussion on CP-based and UP-based congestion mitigation in Rel-17 IAB | Samsung |
| R3-210221 | Discussion on inter-donor-DU local re-routing in Rel-17 IAB | Samsung |
| R3-210346 | Updated Workplan for Rel-17 IAB | Qualcomm Incorporated (WI Rapporteur) |
| R3-210347 | TP for BL CR to 38.401 on Inter-donor Topology Adaptation Procedures | Qualcomm Incorporated |
| R3-210348 | Interruption time reduction for Intra-donor IAB-node Migration | Qualcomm Incorporated |
| R3-210349 | Backhaul transport for inter-donor redundancy | Qualcomm Incorporated |
| R3-210350 | Enhancements to congestion control for IAB | Qualcomm Incorporated |
| R3-210351 | Inter-donor-DU local rerouting for IAB | Qualcomm Incorporated |
| R3-210388 | Congestion Indication to CU-CP | Intel Deutschland GmbH |
| R3-210389 | Inter-Donor IAB Node Migration Discussion | Intel Deutschland GmbH |
| R3-210390 | Mitigation of Unnecessary Transmission | Intel Deutschland GmbH |
| R3-210429 | Considerations on top-down sequence during Inter-donor IAB node migration | KDDI Corporation |
| R3-210458 | Discussion on inter-donor IAB migration | Fujitsu |
| R3-210459 | Discussion on reduction of service interruption | Fujitsu |
| R3-210460 | Discussion on CP-based approach for DL and UL congestion mitigation in IAB networks | Fujitsu |
| R3-210487 | discussion on Inter-Donor IAB Node Migration | Nokia, Nokia Shanghai Bell |
| R3-210488 | Discussion on Reduction of Service Interruption during Intra-Donor Topology Adaptation | Nokia, Nokia Shanghai Bell |
| R3-210489 | discussion on Inter-CU topology redundancy | Nokia, Nokia Shanghai Bell |
| R3-210490 | Analysis on Congestion mitigation | Nokia, Nokia Shanghai Bell |
| R3-210491 | discussion on Inter-Donor-DU re-routing | Nokia, Nokia Shanghai Bell |
| R3-210536 | Considerations on topological redundancy for IAB | LG Electronics |
| R3-210541 | Discussion on inter-donor migration considering migration sequences | Google Inc. |
| R3-210547 | Inter-CU migration procedure | Huawei |
| R3-210548 | Inter-CU RLF recovery procedure | Huawei |
| R3-210549 | Discussion on inter-donor topology management | Huawei |
| R3-210550 | Discussion on IAB E2E flow control | Huawei |
| R3-210551 | Inter-donor-DU re-routing for IAB | Huawei |
| R3-210614 | Discussion on congestion mitigation for IAB | Lenovo, Motorola Mobility |
| R3-210615 | Discussion on IAB inter-donor topology redundancy | Lenovo, Motorola Mobility |
| R3-210616 | Discussion on IAB packet rerouting | Lenovo, Motorola Mobility |
| R3-210657 | Service interruption reduction for intra-donor migration of IAB-node with descendant nodes | AT&T |
| R3-210716 | Discussion on reduction of service interruption in intra-donor migration and RLF recovery scenario | ZTE |
| R3-210717 | Discussion on topology redundancy | ZTE |
| R3-210718 | Discussion on DL E2E flow and congestion control in R17-IAB | ZTE |
| R3-210719 | Considerations on inter-donor-DU re-routing | ZTE |
| R3-210721 | IAB Inter-donor Topology Adaptation | Ericsson |
| R3-210722 | Simultaneous Connectivity to Two IAB-donors and the Use of CHO | Ericsson |
| R3-210723 | Reduction of Service Interruption in IAB Networks | Ericsson |
| R3-210724 | Congestion Mitigation in IAB Networks | Ericsson |
| R3-210725 | CR TS 38.425 IAB End-to-End Flow Control Feedback | Ericsson |
| R3-210726 | Discussion on IAB duplexing enhancements | ZTE |
| R3-210781 | Issues on CP-based approach for DL congestion mitigation | LG Electronics |
| R3-211001 | CB: # 34_IAB_MigrationProcedureDetails - Summary of email discussion | Qualcomm - moderator |
| R3-211002 | CB: # 35_IAB_CHO-DAPS - Summary of email discussion | CATT - moderator |
| R3-211003 | CB: # 36_IAB_Reduction_of_SrvInt - Summary of email discussion | Nokia - moderator |
| R3-211004 | CB: # 37_IAB_TopoRed - Summary of email discussion | Samsung - moderator |
| R3-211005 | CB: # 38_IAB_CongestionMitigation - Summary of email discussion | Ericsson - moderator |
| R3-211006 | CB: # 39_IAB_MultiHopPerf - Summary of email discussion | Huawei - moderator |

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template