3GPP TSG-RAN Meeting #92-e	RP-211046

e-Meeting, June 14th – 18th, 2021

Agenda item: 		9.7.1.4

Source:				Nokia, Nokia Shanghai Bell

Title:	Discussion on way forward for Rel-17 DSS WI

WI:	NR_DSS-Core

Document for:		Discussion

# 1	Introduction

RAN#86 in December 2019 approved the WID on DSS enhancements with the following objectives [1]:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS):

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

For the second objective “Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI”, RAN1#104 concluded the study and submitted the observations to RAN#91[2]. RAN #91 discussed the RAN1 observations but did not at the time agree on a way forward. The following is minuted in the RAN#91 report [4]:

conclusion: The thread [91E][26][DSS_scope] was not able to agree to proceed to the specification phase with the feature, but the thread was not able to agree to stop the work on the feature either. RAN chair: no agreement to start the normative work on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI. There is no agreement to stop studying this either; trusts the RAN1 chair will schedule time for this in a way that ensures things don’t continue to go around in circles; companies need an offline breakthrough otherwise spending RAN1 time for this is waste

RAN1#105 in May discussed the issue related to the 2nd objective of the WI further and reached the following conclusion [5]:

Conclusion: Stop the RAN1 work on two-cell PDSCH scheduling via a single DCI for specification support in Rel-17 DSS - Due to Rel-17 WID scope only focusing on two DL carriers within FR1, RAN1 did not study the potential benefits for more than 2 carriers for both UL and DL within FR1 and FR2. - It is up to RAN to decide whether or not the feature is studied/specified in Rel-18 with extension to more than 2 carriers for both UL and DL within FR1 and FR2.

# 2	Discussion

RAN#92 needs to decide on the way forward with the DSS work item.

At least two possibilities exist:

Alternative 1: Agree on a revised WID that removes the 2nd objective as follows:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS):

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

Alternative 2: Stop the work on the DSS WI completely

The alternative 1 can be seen as the default assumption, as there is clearly no common ground in moving forward with that objective given the RAN#91 discussion and the RAN1#105 conclusion.

The alternative 2 can be seen as a more radical alternative that could be taken if RAN determines that the RAN1/RAN2 time budget can be used more productively on some other work than cross carrier scheduling from SCell to PCell.

Proposal: RAN to agree

- either to revised DSS WID deleting the 2nd objective,

- or to stop the work on the DSS WI completely.

# 3	References

- RP-193260	New WID on NR Dynamic spectrum sharing (DSS)

- RP-210433	Status report for WI: Core part: NR Dynamic spectrum sharing (DSS), Rapporteur (Ericsson)

- RP-210883	Moderator's summary for email discussion [91E][26][DSS_scope], Nokia

- Draft RAN#91 meeting report, March 30th 2021

- RP-211343 “Status report for WI: Core part: NR Dynamic spectrum sharing (DSS)”, Rapporteur (Ericsson)

- RP-211345 “Revised WID on NR Dynamic spectrum sharing (DSS)”, Ericsson