3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2109182

# Elbonia, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1820 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Distinguishing support of extended band n77 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN2 has been tasked to provide means to distinguish UEs that support only the current band n77 definition in the US (i.e. only 3700 - 3980 MHz range) and those that would support the extended definition of band n77 in the US as per latest FCC rulings (i.e. support of also 3450-3550 MHz). To ensure networks can distinguish the UEs, capability signalling is needed for indicating the support for the extended part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added a per-UE (inter-RAT) capability for support of NR band n77 extension in the US. Note that this optional capability has no impact to n77-capable UEs certified outside of US and such UEs are not be required to indicate this bit. But n77-capable UEs that wish to use the 3450-3550 MHz part of n77 frequency range in the US need to indicate this bit. Implementation of this CR by a Release 15 UE will not cause compatibility issues. Impact analysis Impacted functionality: Band n77 capability signalling. Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no inter-operability issues since the network considers UE doesn't support the n77 extension in the US and will not configure UE with 3450-3550 MHz frequency range. - If the UE is implemented according to the CR and the network is not, there are no inter-operability issues since the network will not comprehend the support for the n77 extension in the US and will not configure UE with 3450-3550 MHz frequency range. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for network to distinguish which version of band n77 UE supports in the US. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS36.331 CR4702R1, TS38.331 CR2747R1, TS38.306 CR0615R1 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 4.3.7	Inter-RAT parameters

<UNNECESSARY PARTS OMITTED>

#### 4.3.7.X	extendedBand-n77

This field defines whether the UE supports the band n77 in the US as specified in Note 12 of Table 5.2-1 in TS38.101-1 [33] version 17.3.0. UE only indicates this capability if it indicates support for the NR band n77.

End of Changes