3GPP TSG-RAN WG1 Meeting # 110	R1-2208225

# Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0322 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UE procedure for overlapping CG PUSCH and DG PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In Rel-15 and Rel-16, if a DG PUSCH overlaps with CG PUSCH in the time domain, the gap between the CG PUSCH and the PDCCH scheduling the DG PUSCH should not be smaller than N2 symbols. This can ensure that the UE have enough time to process the overlapping case. The UE just transmits one of the PUSCHs since the UE does not have the ability to cancel a transmission and transmit another one. In Rel-17, it was agreed that the UE can cancel LP PUSCH and transmit HP PUSCH. For the overlapping between HP DG PUSCH and LP CG PUSCH, a new timeline is defined so that the UE can cancel the CG PUSCH and transmit the DG PUSCH, which is specified in Clause 9 of TS 38.213. For the overlapping between HP CG PUSCH and LP DG PUSCH, it is up to UE implementation and no timeline is needed. Therefore, the defined N2 gap in Clause 6.1 of TS 38.214 is not necessary for those cases with different priorities between DG and CG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restriction of the gap between the CG PUSCH and PDCCH scheduling DG PUSCH in the case of overlapping between DG PUSCH and CG PUSCH with different L1 priorities in Rel-17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary time restriction imposed on the overlapping case between CG PUSCH and DG PUSCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding, which has no impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.1	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j, if the UE is not provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH, or the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH and the two PUSCHs have the same priority index as described in Clause 9 of [6, TS 38.213]. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

<Unchanged parts are omitted>